3GPP TSG-RAN WG4 #92		R4-1910399
Ljubljana, SI, 26 - 30 Aug 2019

Agenda item:	11.2.4

Source:	Keysight Technologies, Spirent Communications

Title:	TP to 38.827 on Channel Models

Document for:	Approval

# Introduction

This text proposal adds channel model implementation and scaling procedures to TR 38.827.

# Discussion

This text proposal adds channel model implementation aspects previously conditionally endorsed in [1] as well as channel model scaling aspects from [1]-[4].

Agreements on Direction of Travel (DOT) in [5] were incorporated in this text proposal.

Proposal: Include the draft text in TR 38.827.

# References

- R4-1904197, On Channel Model Implementations, Keysight Technologies, 3GPP TSG-RAN WG4 Meeting #90bis

- R4-1907109, CDL modification proposal, Spirent Communications, 3GPP TSG RAN WG4 Meeting #91

- R4-1907829, Channel model parameters for UMi and UMa CDL-A and C for FR1, Keysight Technologies, Spirent Communications, 3GPP TSG-RAN WG4 Meeting #91

- R4-1907830, Channel model parameters for UMi and InO CDL-A and C for FR2, Keysight Technologies, Spirent Communications, 3GPP TSG-RAN WG4 Meeting #91

- R4-1907926, Direction of Travel (DoT) for FR1, Spirent Communications, 3GPP TSG-RAN WG4 #92, August 2019

# 7	Channel Models

## 7.1	General

In order to describe unambiguously the procedure of generating realizations CDL channel models, various aspects need to be clarified, e.g., details of scaling procedure, inclusion of BS antenna arrays and beams to the model output, and removing unwanted randomness of model realizations.

The concept of angular scaling is based on rotating AoDs/ZoDs and scaling CDL model using the methods in TR 38.901 (section 7.7.5.1) to make them fit the median values in TR 38.901 Table 7.5-6 for the accepted scenarios.

## 7.2	Channel Models

This section describes amendments to the step-wise procedure of the CDL subclause 7.7.1 in TR 38.901 for generating fast fading radio channel realizations. This channel model methodology considers non-Jakes spectrum with the multi-path fading propagation conditions between the gNB emulator and test chamber probe modelled based on Clustered Delay Line (CDL) methodology.

First, the RMS delay spread values of CDL models are normalized first and they must be scaled in delay so that a desired RMS delay spread can be achieved. The scaled delays can be obtained according to the following equation:

in which

- is the normalized delay value of the nth cluster in a CDL in Tables 7.7.1.1 – 7.7.1.5 of [2]

- is the new delay value (in [ns]) of the nth cluster

- is the target delay spread (in [ns]).

Values of DSdesired for FR1/FR2 and for different model scenarios are specified in Table 7.2-1.

Table 7.2-1. Target delay spread values.

| Frequency | Scenario | DSdesired |
| --- | --- | --- |
| FR1 | UMi | 100 ns |
| FR1 | UMa | 365 ns |
| FR2 | UMi | 60 ns |
| FR2 | InO | 30 ns |

Subsequently, the departure and arrival angles (based on subclause 7.7.1 step 1 in TR38.901 are generated by combining 7.7-5 and part of step 7 in subclause 7.5. The arrival angles of azimuth using are generated using the following equation

,	(2)

where

-	n,AOA and cASA are the cluster AOA and the cluster-wise rms azimuth spread of arrival angles (cluster ASA), respectively, in Tables 7.7.1.1 – 7.7.1.5 of TR38.901

-	m denotes the ray offset angles within a cluster given by Table 7.5-3,

-	 is the mean angle of the original channel model table in NLOS case (equation is specified in Annex A.2 of TR38.901) and the LOS angle  in LOS case,

-	Tables 7.2-2 and 7.2.-3 contain the non-circular angle spread values of the original CDL models of TR38.901 before any angular scaling,  are the angular spreads derived from the original CDL Tables 7.7.1.1 – 7.7.1.5 of TR38.901. TR25.996 describes μθ:

The values are calculated for the AOD, AOA, ZOD, and ZOA angles after removing the mean angle following the definition of rms angular spread in TR25.996, without finding the minimum over circular shifts. Here, the calculation is performed after removing the mean angle first and subsequently equation A-2 from Annex A of TR38.901

is used to rotate  to zero (and also wrap AOAs within +/-180). Equations A-3

and A-1 of TR 25.996

are used to calculate the ASmodel. Note that equation A-2 of TR 25.996 is not applied to ASmodel calculations, the following equation is used instead

-	 is the target angular spread. Table 7.2-4 specifies  values for CDL-A,B,C,D,E UMi and UMa at FR1 and Table 7.2-5 specifies the corresponding  values at FR2. These target values are obtained by determining median angular spreads of Table 7.5-6 of TR38.901.

The angular scaling is applied to the ray angles and no further scaling is performed. The generation of AOD (), ZOA (), and ZOD () follows a procedure similar to AOA as described above. Here, the azimuth angles may need to be wrapped around to be within [0, 360] degrees, while the zenith angles may need to be clipped to be within [0, 180] degrees.

Each CDL parameter table of contains two sets of three rows, i.e., three clusters, with exactly same angular parameters. This is harmful for the statistical properties of the models as they become non-WSS across the ensemble of model realizations. Instead of making the angular parameters non-equal by introducing small offsets to angles of the three rows, the problematic clusters are treated as midpaths as intended when the CDLs where drawn from statistical distributions which works across all frequency ranges. For the clusters that look like midpaths, e.g., Cluster 2-4 and 5-7 for CDL-A and Cluster 2-4 and 6-8 for CDL-C, the powers for each of the three clusters are added and using the regular midpath power distribution of 0.5, 0.3, and 0.2 specified in Table 7.5-5 of TR38.901, the powers for the each of the midpaths are calculated. Notice that the intra cluster delay spread in Table 7.5-5 of TR38.901 is not followed, and the same delays as the original CDL are followed for the midpaths (aka Sub-Cluster). This helps keeping the rms DS of the modified CDL to 1s.

Table 7.2-2: Original (non-circular) angle spreads of CDL models UMi and UMa (K-factor 9 dB)

| Model | [deg] |  |  |  |
| --- | --- | --- | --- | --- |
|  | ASD | ASA | ZSD | ZSA |
| CDL-A | 73.6985 | 85.2676 | 28.5575 | 21.0831 |
| CDL-B | 41.5917 | 59.3326 | 5.9633 | 10.3818 |
| CDL-C | 39.0949 | 71.1175 | 4.0666 | 10.4245 |
| CDL-D | 15.6771 | 17.3604 | 2.4462 | 1.5362 |
| CDL-E | 13.1544 | 37.5640 | 1.4577 | 2.4601 |

Table 7.2-3: Original (non-circular) angle spreads of CDL-D and CDL-E models InO (K-factor 7 dB)

| Model | [deg] |  |  |  |
| --- | --- | --- | --- | --- |
|  | ASD | ASA | ZSD | ZSA |
| CDL-D | 18.9859 | 21.0747 | 2.9629 | 1.8735 |
| CDL-E | 15.7784 | 45.3434 | 1.7692 | 2.9982 |

Table 7.2-4: Desired AS for UMi and UMa at 3.5 GHz (FR1).

| Model | ASdesired [deg] |  |  |  |
| --- | --- | --- | --- | --- |
|  | ASD | ASA | ZSD | ZSA |
| UMi NLOS (CDL-A, B, C) | 23.9751 | 57.2457 | 0.7762 | 7.8320 |
| UMi LOS (CDL-D, E) | 15.0432 | 47.6149 | 0.6166 | 4.6204 |
| UMa NLOS (CDL-A, B, C) | 25.7620 | 74.1138 | 4.8978 | 18.2050 |
| UMa LOS (CDL-D, E) | 14.0180 | 64.5654 | 3.4674 | 8.9125 |
| Note: For UMa frequency fc = 6 as stated in [2], and other parameters hUMa = 25, hUMi = 10, hUT = 1.5, and D2D = 100. |  |  |  |  |

Table 7.2-5: Desired AS for UMi and InO at 28 GHz (FR2).

| Model | ASdesired [deg] |  |  |  |
| --- | --- | --- | --- | --- |
|  | ASD | ASA | ZSD | ZSA |
| UMi NLOS (CDL-A, B, C) | 15.6188 | 49.3183 | 0.7762 | 7.2695 |
| UMi LOS (CDL-D, E) | 13.7050 | 41.0212 | 0.6166 | 3.8350 |
| InO NLOS (CDL-A, B, C) | 41.6869 | 50.3659 | 12.0226 | 14.7109 |
| InO LOS (CDL-D, E) | 39.8107 | 31.8526 | 1.3702 | 11.4756 |

Subsequently, the AOD angles are coupled to AOA angles within a cluster n. Instead of random procedure, the coupling is performed using the fixed coupling pattern specified in Table 7.2-6.  The same fixed coupling pattern is applied for all clusters n.

Table 7.2-6: Fixed coupling pattern of ray angles to be applied for each cluster.

|  | m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 6 | 12 | 5 | 10 | 8 | 11 | 16 | 14 | 18 | 9 | 20 | 4 | 2 | 15 | 7 | 13 | 19 | 17 | 3 | 1 |
|  | 20 | 9 | 12 | 1 | 13 | 18 | 10 | 4 | 8 | 2 | 6 | 14 | 11 | 19 | 7 | 3 | 17 | 5 | 15 | 16 |
|  | 2 | 16 | 3 | 11 | 18 | 9 | 5 | 17 | 4 | 19 | 15 | 20 | 13 | 7 | 10 | 1 | 8 | 12 | 6 | 14 |
|  | 15 | 18 | 13 | 1 | 12 | 9 | 6 | 7 | 5 | 3 | 2 | 8 | 14 | 17 | 19 | 16 | 11 | 20 | 10 | 4 |

In the next steps, the linear cross polarization power ratios (XPR) are calculated for each ray m of each cluster n as

where X is the per-cluster XPR in dB from Tables 7.7.1.1 – 7.7.1.5 of TR38.901.

The gNB beam pattern including the assumptions for gNB antenna for definitions and symbols of subclause 7.3 of TR38.901 for FR1 and FR2 are summarized in Table 7.2-7.

Table 7.2-7: BS Antenna Parameters

| Parameter description | Symbol | Parameter value |  |  |
| --- | --- | --- | --- | --- |
|  |  | FR1 ≤2.5GHz | FR1 >2.5GHz | FR2 |
| Antenna panels in vertical dimension | Mg | 1 | 1 | 1 |
| Antenna panels in horizontal dimension | Ng | 1 | 1 | 1 |
| Elements per panel in vertical dimension | Me | 4 | 8 | 8 |
| Elements per panel in horizontal dimension | Ne | 8 | 8 | 16 |
| Number of polarizations per panel | P | 2 | 2 | 2 |
| Element spacing in horizontal dimension () | dH | 0.5 | 0.5 | 0.5 |
| Element spacing in vertical dimension () | dV | 0.5 | 0.5 | 0.5 |

Antenna element radiation patterns, including orientation of the element main polarization components as well as orientation of the antenna array for both FR1 and FR2 are as in the example pattern in Table 7.3-1 of TR38.901. The antenna element parameters are 3dB = 65, 3dB = 65, Amax = 30dB, SLAv = 30dB, GE,max =8 dBi.

It is assumed the co-polarized elements of the array are combined to a single RF port, i.e. they compose an antenna array that can form beams by setting certain weights per element. Weight vector for the first polarization and for the second polarization is

α1,1…αMe,Ne=1MeNeexp-j2πrtx,max T ∙ dtx,me,neλ0∈C1×MeNe

where dtx,me,ne is the location vector of transmit antenna element   and , and rtx,max is a spherical unit vector denoting the target beam direction. Determination of beam directions rtx,max is TBD.

Random initial phase   are not used for the different polarization combinations (θθ, θϕ, ϕθ, ϕϕ). Instead, a fixed and pre-defined set of initial phases Φmθθ,Φmθϕ,Φmϕθ,ΦmϕϕΦmθθ,Φmθϕ,Φmϕθ,Φmϕϕ of Table 7.2-8 and a scalar random initial phase  term Φn,m~Uni(-π,π) is used for each ray m of each cluster n.

The set of fixed initial phases can be same for all clusters, i.e. Φn,mθθ=Φmθθ ∀ n=1,…,N etc. for all four polarization combinations. These 20×4 initial phase values can be specified either by a table of values or by setting a random number generator and a fixed seed number. The distribution of scalar initial phases Φn,m is uniform within -π,π. Its purpose is to enable generation of different fading sequences on different uses of the model, but still maintaining the power angular distribution of the model. The scalar initial phases can be fixed (or removed) if completely deterministic process, i.e. exactly same fading sequences at each model use, is aimed at.

Table 7.2-8: Fixed initial phases for 2x2 polarization matrices. These values are drawn from uniform distribution.

| m | Φmθθ [rad] | Φmϕθ [rad] | Φmθϕ [rad] | Φmϕϕ [rad] |
| --- | --- | --- | --- | --- |
| 1 | 1.7609 | -0.6928 | -1.6230 | -0.6037 |
| 2 | -2.5356 | -2.3124 | 2.7775 | 2.8660 |
| 3 | 0.4725 | -2.7660 | -1.6664 | -0.9226 |
| 4 | 2.0181 | -3.0448 | -2.8713 | -2.0798 |
| 5 | 0.9369 | 1.4560 | 0.9283 | -0.3084 |
| 6 | 0.2954 | -1.2798 | 1.5375 | -1.9544 |
| 7 | 1.1735 | -1.9886 | -0.8263 | 0.7893 |
| 8 | 1.7607 | -2.6319 | 2.6979 | 1.7324 |
| 9 | -0.0830 | -0.4030 | -0.3344 | -1.2167 |
| 10 | 0.0535 | 0.0677 | 1.9957 | 1.8525 |
| 11 | 0.9068 | -0.7627 | 1.9577 | 0.2062 |
| 12 | -0.9379 | 2.7583 | 2.3621 | 0.3151 |
| 13 | 0.7695 | 0.5469 | -1.8363 | -1.2488 |
| 14 | -0.1827 | -1.6934 | 2.1634 | -1.9179 |
| 15 | -1.7221 | -2.0690 | -1.7111 | -0.4040 |
| 16 | -1.1869 | 2.6602 | -0.4385 | -1.9804 |
| 17 | 2.5439 | 3.0143 | -0.3841 | -2.4434 |
| 18 | -1.5201 | -0.5735 | 0.5962 | -1.4941 |
| 19 | 0.6462 | 1.3271 | -1.7483 | -2.4038 |
| 20 | -1.2775 | -1.1386 | -0.4765 | 0.0494 |

To determine the channel all clusters are treated as "weaker cluster", i.e., no further sub-clusters in delay should be generated. The BS beamforming weights αme,ne for antenna elements are used and the BS antenna signals are summed for BS hybrid beamforming. Here, the random initial phases Φn,m are used for sub-paths, but not for the different polarization combinations (θθ, θϕ, ϕθ, ϕϕ). The following equations apply for the NLOS clusters and the LOS path, respectively:

Hu,s,nNLOSt=PnMm=1Mne=1Neme=1MeFrx,u,θθn,m,ZOA,φn,m,AOAFrx,u,φθn,m,ZOA,φn,m,AOATexpΦmθθκn,m-1expΦmθϕκn,m-1expΦmϕθexpΦmϕϕFtx,s,θθn,m,ZOD,φn,m,AODFtx,s,φθn,m,ZOD,φn,m,AOD

αme,neexpjΦn,mexpj2πrrx,n,mT.drx,uλ0expj2πrtx,n,mT.dtx,sλ0expj2πrrx,n,mT.vλ0t				(5)

Hu,s,nLOSt=ne=1Neme=1MeFrx,u,θθLOS,ZOA,φLOS,AOAFrx,u,φθLOS,ZOA,φLOS,AOAT100-1Ftx,s,θθLOS,ZOD,φLOS,AODFtx,s,φθLOS,ZOD,φLOS,AODαme,ne

expj2πrrx,LOST∙drx,uλ0expj2πrtx,LOST∙dtx,sλ0expj2πrrx,LOST∙vλ0t

where Frx,u,θ , Frx,u,ϕ, Ftx,s,θ, Ftx,s,ϕ, , , , and are determined as in TR 38.901. UE velocity vector is determined as

UE velocity v is defined as follows: 30km/h for FR1 vs 3 km/h (Indoor Office) and 12 km/h (UMi) for FR2. The UE travelling direction (v, v) are as follows for FR1:

- ([251.2923°],90o) for UMi CDL A channel model

- ([127.0455°],90o) for UMi CDL C channel model

- ([182.1659°],90o) for UMa CDL A channel model

- ([208.7405°],90o) for UMa CDL C channel model

For FR2, the UE direction of travel is FFS.

### 7.2.1	Channel Models for FR1

The Channel model parameter tables for CDL-A, B, C, D, and E for UMa and UMi at 3.5 GHz are presented in this subclause without the effect of base station antenna filtering.

For FR1, the baseline emulated propagation environment for FR1 MIMO OTA is 2D without elevation modelling, i.e.,  all ZOA are set 90° and ZSA is 0° in the following tables.

Tables 7.2.1-1—7.2.1-5 show the model parameters, UMi CDL-A—CDL-E models, respectively. For the determination of desired zenith spread of departure (ZSDdesired) from table 7.5-8 of TR38.901, the following parameters are used hBS  = 10 m, hUT = 1.5 m, and d2d = 100 m.

Table 7.2.1-1: Channel model parameters for UMi CDL-A at 3.5 GHz.

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -13.4014 | -59.324 | 98.721 | 95.9936 | 90 |
| 2 | 38.19 | 0 | -2.752 | -156.546 | 97.1624 | 90 |
| 3 | 40.25 | -2.2185 | -2.752 | -156.546 | 97.1624 | 90 |
| 4 | 58.68 | -3.9794 | -2.752 | -156.546 | 97.1624 | 90 |
| 5 | 46.1 | -5.9799 | 27.9576 | 115.7066 | 97.9452 | 90 |
| 6 | 53.75 | -8.1984 | 27.9576 | 115.7066 | 97.9452 | 90 |
| 7 | 67.08 | -9.9593 | 27.9576 | 115.7066 | 97.9452 | 90 |
| 8 | 57.5 | -10.5014 | 38.1399 | -55.2369 | 98.7118 | 90 |
| 9 | 76.18 | -7.5014 | -27.9638 | -82.1587 | 96.1295 | 90 |
| 10 | 153.75 | -15.9014 | 50.144 | 127.5226 | 95.3467 | 90 |
| 11 | 189.78 | -6.6014 | -28.3867 | 99.1238 | 98.0648 | 90 |
| 12 | 222.42 | -16.7014 | 42.4666 | -131.84 | 99.2935 | 90 |
| 13 | 217.18 | -12.4014 | -51.1586 | 82.1383 | 98.7444 | 90 |
| 14 | 249.42 | -15.2014 | -57.3396 | 115.7066 | 98.902 | 90 |
| 15 | 251.19 | -10.8014 | -43.6439 | -58.728 | 95.912 | 90 |
| 16 | 305.82 | -11.3014 | -45.6283 | -69.4699 | 95.7272 | 90 |
| 17 | 408.1 | -12.7014 | 52.4212 | -85.7169 | 95.806 | 90 |
| 18 | 445.79 | -16.2014 | 46.8909 | 138.3987 | 99.0271 | 90 |
| 19 | 456.95 | -18.3014 | 41.7835 | 161.2923 | 94.9226 | 90 |
| 20 | 479.66 | -18.9014 | -39.9679 | 168.543 | 95.083 | 90 |
| 21 | 500.66 | -16.6014 | -51.5165 | 132.7593 | 99.2962 | 90 |
| 22 | 530.43 | -19.9014 | 39.7665 | -155.942 | 95.2461 | 90 |
| 23 | 965.86 | -29.7014 | -19.6683 | 101.3393 | 98.5677 | 90 |
| Per-Cluster Parameters |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 1.6266 | 7.385 | 0.0815 | 0 | 10 |  |

Table 7.2.1-2: Channel model parameters for UMi CDL-B at 3.5 GHz.

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 3.2594 | -173.655 | 105.6621 | 90 |
| 2 | 10.72 | -2.2185 | 3.2594 | -173.655 | 105.6621 | 90 |
| 3 | 21.55 | -3.9794 | 3.2594 | -173.655 | 105.6621 | 90 |
| 4 | 20.95 | -3.2014 | -21.7581 | 127.2975 | 106.8987 | 90 |
| 5 | 28.7 | -9.8014 | -39.8007 | -91.3552 | 107.4194 | 90 |
| 6 | 29.86 | -1.2 | -8.6729 | 155.8564 | 105.3237 | 90 |
| 7 | 37.52 | -3.4185 | -8.6729 | 155.8564 | 105.3237 | 90 |
| 8 | 50.55 | -5.1794 | -8.6729 | 155.8564 | 105.3237 | 90 |
| 9 | 36.81 | -7.6014 | -40.8383 | -93.0919 | 107.2762 | 90 |
| 10 | 36.97 | -3.0014 | 28.1617 | 133.6653 | 105.1674 | 90 |
| 11 | 57 | -8.9014 | -43.6052 | -87.11 | 104.9592 | 90 |
| 12 | 52.83 | -9.0014 | 40.7281 | 98.1597 | 104.6858 | 90 |
| 13 | 110.21 | -4.8014 | -32.1917 | 106.2642 | 105.3497 | 90 |
| 14 | 127.56 | -5.7014 | -31.2117 | -91.1623 | 105.2325 | 90 |
| 15 | 154.74 | -7.5014 | 33.292 | -95.697 | 105.0893 | 90 |
| 16 | 178.42 | -1.9014 | 15.5376 | -140.658 | 105.2976 | 90 |
| 17 | 201.69 | -7.6014 | -43.8934 | -93.8638 | 104.9071 | 90 |
| 18 | 282.94 | -12.2014 | -54.327 | 62.7505 | 106.8857 | 90 |
| 19 | 302.19 | -9.8014 | -46.8333 | -82.6718 | 104.9722 | 90 |
| 20 | 361.87 | -11.4014 | -49.7155 | 69.6972 | 107.4584 | 90 |
| 21 | 410.67 | -14.9014 | -61.8207 | 57.058 | 107.3413 | 90 |
| 22 | 427.9 | -9.2014 | 41.4774 | 91.7918 | 107.2241 | 90 |
| 23 | 478.34 | -11.3014 | -46.8333 | -64.726 | 106.9508 | 90 |
| Per-Cluster Parameters |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 5.7644 | 21.2262 | 0.3905 | 0 | 8 |  |

Table 7.2.1-3: Channel model parameters for UMi CDL-C at 3.5 GHz.

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4215 | -36.1891 | -122.2815 | 98.9242 | 90 |
| 2 | 20.99 | -1.25 | -21.5937 | 125.831 | 99.1915 | 90 |
| 3 | 22.19 | -3.4684 | -21.5937 | 125.831 | 99.1915 | 90 |
| 4 | 23.29 | -5.2294 | -21.5937 | 125.831 | 99.1915 | 90 |
| 5 | 21.76 | -2.5215 | -32.5709 | -143.6126 | 99.5732 | 90 |
| 6 | 63.66 | 0 | -7.4275 | 166.4003 | 99.306 | 90 |
| 7 | 64.48 | -2.2185 | -7.4275 | 166.4003 | 99.306 | 90 |
| 8 | 65.6 | -3.9794 | -7.4275 | 166.4003 | 99.306 | 90 |
| 9 | 65.84 | -7.4215 | 37.2175 | 73.8315 | 100.4513 | 90 |
| 10 | 79.35 | -7.1215 | -47.1664 | 82.7664 | 98.5616 | 90 |
| 11 | 82.13 | -10.7215 | 41.5716 | -79.6999 | 100.6231 | 90 |
| 12 | 93.36 | -11.1215 | -67.1585 | 66.9895 | 98.218 | 90 |
| 13 | 122.85 | -5.1215 | -41.5244 | 84.0543 | 100.165 | 90 |
| 14 | 130.83 | -6.8215 | -47.0437 | -96.2818 | 100.2604 | 90 |
| 15 | 217.04 | -8.7215 | -55.7519 | 94.8406 | 98.1225 | 90 |
| 16 | 271.05 | -13.2215 | 55.3698 | 53.9494 | 100.2604 | 90 |
| 17 | 425.89 | -13.9215 | 53.2234 | 16.0364 | 98.4852 | 90 |
| 18 | 460.03 | -13.9215 | 46.8456 | 32.2963 | 98.1416 | 90 |
| 19 | 549.02 | -15.8215 | -70.1021 | 18.2098 | 97.9698 | 90 |
| 20 | 560.77 | -17.1215 | 48.9306 | 37.0455 | 100.7376 | 90 |
| 21 | 630.65 | -16.0215 | 49.6052 | 33.7452 | 98.1225 | 90 |
| 22 | 663.74 | -15.7215 | 57.7615 | 29.801 | 98.1034 | 90 |
| 23 | 704.27 | -21.6215 | 65.6725 | 11.6092 | 100.4513 | 90 |
| 24 | 865.23 | -22.8215 | -83.5324 | 56.2837 | 100.9476 | 90 |
| Per-Cluster Parameters |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 1.2265 | 12.0742 | 0.5726 | 0 | 7 |  |

Table 7.2.1-4: Channel model parameters for UMi CDL-D at 3.5 GHz.

| Cluster # | Cluster PAS | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Specular (LOS path) | 0 | -0.2 | 0 | 180 | 98.5 | 90 |
|  | Laplacian | 0 | -13.8303 | 0 | -180 | 98.5 | 90 |
| 2 | Laplacian | 3.5275 | -19.1289 | 85.5931 | -69.0402 | 95.2232 | 90 |
| 3 | Laplacian | 61.6807 | -21.3474 | 85.5931 | -69.0402 | 95.2232 | 90 |
| 4 | Laplacian | 137.3705 | -23.1083 | 85.5931 | -69.0402 | 95.2232 | 90 |
| 5 | Laplacian | 141.6035 | -18.2289 | 12.4743 | 133.3735 | 98.2479 | 90 |
| 6 | Laplacian | 181.8169 | -20.4474 | 12.4743 | 133.3735 | 98.2479 | 90 |
| 7 | Laplacian | 261.6389 | -22.2083 | 12.4743 | 133.3735 | 98.2479 | 90 |
| 8 | Laplacian | 178.8941 | -23.2303 | 33.2009 | -62.0624 | 98.5 | 90 |
| 9 | Laplacian | 407.3746 | -28.1303 | -61.8919 | -109.358 | 95.9542 | 90 |
| 10 | Laplacian | 799.9337 | -23.9303 | -31.5697 | 36.555 | 96.6852 | 90 |
| 11 | Laplacian | 949.8016 | -25.1303 | 50.4731 | -14.3389 | 99.8359 | 90 |
| 12 | Laplacian | 978.4246 | -30.3303 | -126.759 | -71.2668 | 93.9125 | 90 |
| 13 | Laplacian | 1262.337 | -28.0303 | 74.0784 | 83.851 | 95.4753 | 90 |
| Per-Cluster Parameters |  |
| Parameter |  | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value |  | 4.7978 | 21.9419 | 0.7562 | 0 | 11 |  |

Table 7.2.1-5: Channel model parameters for UMi CDL-E at 3.5 GHz.

| Cluster # | Cluster PAS | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Specular (LOS path) | 0 | -0.03 | 0 | 180 | 99.6 | 90 |
|  | Laplacian | 0 | -22.2442 | 0 | -180 | 99.6 | 90 |
| 2 | Laplacian | 49.9706 | -16.0427 | 65.756 | -25.0925 | 101.5457 | 90 |
| 3 | Laplacian | 52.9593 | -18.2612 | 65.756 | -25.0925 | 101.5457 | 90 |
| 4 | Laplacian | 54.8089 | -20.0221 | 65.756 | -25.0925 | 101.5457 | 90 |
| 5 | Laplacian | 52.9593 | -23.1142 | -22.986 | 80.8762 | 99.5154 | 90 |
| 6 | Laplacian | 69.2365 | -22.6142 | 18.526 | 94.9462 | 100.1076 | 90 |
| 7 | Laplacian | 185.8637 | -18.8128 | 10.6353 | -148.945 | 99.2616 | 90 |
| 8 | Laplacian | 187.8204 | -21.0313 | 10.6353 | -148.945 | 99.2616 | 90 |
| 9 | Laplacian | 190.702 | -22.7922 | 10.6353 | -148.945 | 99.2616 | 90 |
| 10 | Laplacian | 257.2613 | -22.5142 | 21.7281 | -133.48 | 100.1076 | 90 |
| 11 | Laplacian | 361.5249 | -25.8142 | 37.3951 | -71.8765 | 98.2465 | 90 |
| 12 | Laplacian | 530.7998 | -20.4142 | 0.5718 | 138.1703 | 99.3039 | 90 |
| 13 | Laplacian | 1168.55 | -30.0142 | 63.9262 | 2.2763 | 97.9081 | 90 |
| 14 | Laplacian | 2009.522 | -29.4142 | 65.8703 | 15.2055 | 101.7149 | 90 |
| Per-Cluster Parameters |  |
| Parameter |  | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value |  | 5.7179 | 13.9432 | 1.2689 | 0 | 8 |  |

Tables 7.2.1-6—7.2.1-10 tabulate channel model parameters for UMa CDL-A—CDL-E models at 3.5 GHz, respectively.

In the determination of desired angle spreads (ASdesired), frequency is set 6 GHz as stated in Table 7.5-6 Part-1 of TR38.901.

For the determination of desired zenith spread of departure (ZSDdesired) from table 7.5-7 of TR38.901, the following parameters are used hUT = 1.5 m, and d2d = 100 m.

Table 7.2.1-6: Channel model parameters for UMa CDL-A at 3.5 GHz.

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -13.4014 | -63.5923 | 70.1754 | 89.1998 | 90 |
| 2 | 139.3935 | 0 | -2.804 | -154.231 | 96.5746 | 90 |
| 3 | 146.9125 | -2.2185 | -2.804 | -154.231 | 96.5746 | 90 |
| 4 | 214.182 | -3.9794 | -2.804 | -154.231 | 96.5746 | 90 |
| 5 | 168.265 | -5.9799 | 30.1944 | 92.1659 | 101.5139 | 90 |
| 6 | 196.1875 | -8.1984 | 30.1944 | 92.1659 | 101.5139 | 90 |
| 7 | 244.842 | -9.9593 | 30.1944 | 92.1659 | 101.5139 | 90 |
| 8 | 209.875 | -10.5014 | 41.1356 | -23.0699 | 106.3504 | 90 |
| 9 | 278.057 | -7.5014 | -29.8948 | -57.9245 | 90.0573 | 90 |
| 10 | 561.1875 | -15.9014 | 54.0343 | 107.4637 | 85.1179 | 90 |
| 11 | 692.697 | -6.6014 | -30.3493 | 70.6969 | 102.2686 | 90 |
| 12 | 811.833 | -16.7014 | 45.7847 | -122.245 | 110.0206 | 90 |
| 13 | 792.707 | -12.4014 | -54.8184 | 48.7064 | 106.5562 | 90 |
| 14 | 910.383 | -15.2014 | -61.46 | 92.1659 | 107.551 | 90 |
| 15 | 916.8435 | -10.8014 | -46.7436 | -27.5897 | 88.6853 | 90 |
| 16 | 1116.243 | -11.3014 | -48.8759 | -41.4968 | 87.519 | 90 |
| 17 | 1489.565 | -12.7014 | 56.4812 | -62.5312 | 88.0164 | 90 |
| 18 | 1627.1335 | -16.2014 | 50.5387 | 121.5446 | 108.3399 | 90 |
| 19 | 1667.8675 | -18.3014 | 45.0506 | 151.184 | 82.4424 | 90 |
| 20 | 1750.759 | -18.9014 | -42.7936 | 160.5712 | 83.4543 | 90 |
| 21 | 1827.409 | -16.6014 | -55.2029 | 114.2434 | 110.0378 | 90 |
| 22 | 1936.0695 | -19.9014 | 42.8834 | -153.449 | 84.4834 | 90 |
| 23 | 3525.389 | -29.7014 | -20.9811 | 73.5652 | 105.4414 | 90 |
| Per-Cluster Parameters |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 1.7478 | 9.5611 | 0.5145 | 0 | 10 |  |

Table 7.2.1-7: Channel model parameters for UMa CDL-B at 3.5 GHz.

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 3.8721 | -170.785 | 105.7717 | 90 |
| 2 | 39.128 | -2.2185 | 3.8721 | -170.785 | 105.7717 | 90 |
| 3 | 78.6575 | -3.9794 | 3.8721 | -170.785 | 105.7717 | 90 |
| 4 | 76.4675 | -3.2014 | -23.0099 | 112.7684 | 113.5742 | 90 |
| 5 | 104.755 | -9.8014 | -42.3972 | -64.2347 | 116.8595 | 90 |
| 6 | 108.989 | -1.2 | -8.9495 | 149.7425 | 103.6362 | 90 |
| 7 | 136.948 | -3.4185 | -8.9495 | 149.7425 | 103.6362 | 90 |
| 8 | 184.5075 | -5.1794 | -8.9495 | 149.7425 | 103.6362 | 90 |
| 9 | 134.3565 | -7.6014 | -43.5122 | -66.4831 | 115.956 | 90 |
| 10 | 134.9405 | -3.0014 | 30.6303 | 121.0127 | 102.6507 | 90 |
| 11 | 208.05 | -8.9014 | -46.4853 | -58.7386 | 101.3365 | 90 |
| 12 | 192.8295 | -9.0014 | 44.1333 | 75.0448 | 99.6118 | 90 |
| 13 | 402.2665 | -4.8014 | -34.2211 | 85.5375 | 103.8005 | 90 |
| 14 | 465.594 | -5.7014 | -33.1681 | -63.9849 | 103.0613 | 90 |
| 15 | 564.801 | -7.5014 | 36.143 | -69.8558 | 102.1579 | 90 |
| 16 | 651.233 | -1.9014 | 17.0654 | -128.065 | 103.472 | 90 |
| 17 | 736.1685 | -7.6014 | -46.795 | -67.4824 | 101.008 | 90 |
| 18 | 1032.731 | -12.2014 | -58.0062 | 29.2019 | 113.4921 | 90 |
| 19 | 1102.994 | -9.8014 | -49.9539 | -52.9926 | 101.4187 | 90 |
| 20 | 1320.826 | -11.4014 | -53.051 | 38.1956 | 117.1059 | 90 |
| 21 | 1498.946 | -14.9014 | -66.0584 | 21.8321 | 116.3667 | 90 |
| 22 | 1561.835 | -9.2014 | 44.9385 | 66.8006 | 115.6275 | 90 |
| 23 | 1745.941 | -11.3014 | -49.9539 | -29.7588 | 113.9027 | 90 |
| Per-Cluster Parameters |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 6.194 | 27.4808 | 2.464 | 0 | 8 |  |

Table 7.2.1-8: Channel model parameters for UMa CDL-C at 3.5 GHz.

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4215 | -37.4195 | -96.4031 | 96.7645 | 90 |
| 2 | 76.6135 | -1.25 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 3 | 80.9935 | -3.4684 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 4 | 85.0085 | -5.2294 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 5 | 79.424 | -2.5215 | -33.5316 | -124.0196 | 100.8594 | 90 |
| 6 | 232.359 | 0 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 7 | 235.352 | -2.2185 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 8 | 239.44 | -3.9794 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 9 | 240.316 | -7.4215 | 41.4581 | 51.4188 | 106.3995 | 90 |
| 10 | 289.6275 | -7.1215 | -49.2149 | 62.9864 | 94.4761 | 90 |
| 11 | 299.7745 | -10.7215 | 46.1367 | -41.2744 | 107.4834 | 90 |
| 12 | 340.764 | -11.1215 | -70.697 | 42.5606 | 92.3083 | 90 |
| 13 | 448.4025 | -5.1215 | -43.1524 | 64.6538 | 104.5929 | 90 |
| 14 | 477.5295 | -6.8215 | -49.0831 | -62.7423 | 105.1951 | 90 |
| 15 | 792.196 | -8.7215 | -58.4403 | 78.6184 | 91.7061 | 90 |
| 16 | 989.3325 | -13.2215 | 60.9633 | 25.6781 | 105.1951 | 90 |
| 17 | 1554.4985 | -13.9215 | 58.6569 | -23.4063 | 93.9944 | 90 |
| 18 | 1679.1095 | -13.9215 | 51.8037 | -2.3553 | 91.8265 | 90 |
| 19 | 2003.923 | -15.8215 | -73.86 | -20.5926 | 90.7426 | 90 |
| 20 | 2046.8105 | -17.1215 | 54.0442 | 3.7933 | 108.2061 | 90 |
| 21 | 2301.8725 | -16.0215 | 54.7691 | -0.4794 | 91.7061 | 90 |
| 22 | 2422.651 | -15.7215 | 63.5332 | -5.5859 | 91.5856 | 90 |
| 23 | 2570.5855 | -21.6215 | 72.0338 | -29.1381 | 106.3995 | 90 |
| 24 | 3158.0895 | -22.8215 | -88.2912 | 28.7003 | 109.5309 | 90 |
| Per-Cluster Parameters |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 1.3179 | 15.632 | 3.6131 | 0 | 7 |  |

Table 7.2.1-9: Channel model parameters for UMa CDL-D at 3.5 GHz.

| Cluster # | Cluster PAS | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Specular (LOS path) | 0 | -0.2 | 0 | 180 | 98.5 | 90 |
|  | Laplacian | 0 | -13.8303 | 0 | -180 | 98.5 | 90 |
| 2 | Laplacian | 12.8753 | -19.1289 | 79.7601 | -157.697 | 80.0732 | 90 |
| 3 | Laplacian | 225.1344 | -21.3474 | 79.7601 | -157.697 | 80.0732 | 90 |
| 4 | Laplacian | 501.4023 | -23.1083 | 79.7601 | -157.697 | 80.0732 | 90 |
| 5 | Laplacian | 516.8527 | -18.2289 | 11.6242 | 116.7749 | 97.0826 | 90 |
| 6 | Laplacian | 663.6315 | -20.4474 | 11.6242 | 116.7749 | 97.0826 | 90 |
| 7 | Laplacian | 954.9819 | -22.2083 | 11.6242 | 116.7749 | 97.0826 | 90 |
| 8 | Laplacian | 652.9634 | -23.2303 | 30.9383 | -20.0777 | 98.5 | 90 |
| 9 | Laplacian | 1486.917 | -28.1303 | -57.6741 | 147.6324 | 84.1838 | 90 |
| 10 | Laplacian | 2919.758 | -23.9303 | -29.4182 | -14.5102 | 88.2944 | 90 |
| 11 | Laplacian | 3466.776 | -25.1303 | 47.0334 | 44.6351 | 106.0125 | 90 |
| 12 | Laplacian | 3571.25 | -30.3303 | -118.12 | 95.5983 | 72.7025 | 90 |
| 13 | Laplacian | 4607.53 | -28.0303 | 69.03 | 177.7798 | 81.4907 | 90 |
| Per-Cluster Parameters |  |
| Parameter |  | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value |  | 4.4709 | 29.753 | 4.2523 | 0 | 11 |  |

Table 7.2.1-10: Channel model parameters for UMa CDL-E at 3.5 GHz.

| Cluster # | Cluster PAS | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Specular (LOS path) | 0 | -0.03 | 0 | 180 | 99.6 | 90 |
|  | Laplacian | 0 | -22.2442 | 0 | -180 | 99.6 | 90 |
| 2 | Laplacian | 182.3926 | -16.0427 | 61.2748 | -98.1037 | 110.5415 | 90 |
| 3 | Laplacian | 193.3013 | -18.2612 | 61.2748 | -98.1037 | 110.5415 | 90 |
| 4 | Laplacian | 200.0527 | -20.0221 | 61.2748 | -98.1037 | 110.5415 | 90 |
| 5 | Laplacian | 193.3013 | -23.1142 | -21.4195 | 45.5889 | 99.1243 | 90 |
| 6 | Laplacian | 252.7131 | -22.6142 | 17.2635 | 64.6678 | 102.4543 | 90 |
| 7 | Laplacian | 678.4024 | -18.8128 | 9.9105 | -137.889 | 97.6971 | 90 |
| 8 | Laplacian | 685.5446 | -21.0313 | 9.9105 | -137.889 | 97.6971 | 90 |
| 9 | Laplacian | 696.0624 | -22.7922 | 9.9105 | -137.889 | 97.6971 | 90 |
| 10 | Laplacian | 939.0038 | -22.5142 | 20.2473 | -116.92 | 102.4543 | 90 |
| 11 | Laplacian | 1319.566 | -25.8142 | 34.8467 | -33.3854 | 91.9885 | 90 |
| 12 | Laplacian | 1937.419 | -20.4142 | 0.5328 | 123.2792 | 97.935 | 90 |
| 13 | Laplacian | 4265.208 | -30.0142 | 59.5698 | 67.1651 | 90.0856 | 90 |
| 14 | Laplacian | 7334.755 | -29.4142 | 61.3814 | 84.697 | 111.493 | 90 |
| Per-Cluster Parameters |  |
| Parameter |  | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value |  | 5.3282 | 18.9069 | 7.1358 | 0 | 8 |  |

### 7.2.2	Channel Models for FR2

Channel model parameter tables for CDL-A and C for UMi and InO at 28 GHz are presented in this subclause. The channel model tables presented here are only the propagation parameters, i.e., without the effect of base station antenna filtering.

Tables 7.2.2-1—7.2.2.5 show the model parameters, UMi CDL-A—CDL-E models, respectively. For the determination of desired zenith spread of departure (ZSDdesired) from table 7.5-8 of TR38.901, the following parameters are used hBS  = 10 m, hUT = 1.5 m, and d2d = 100 m.

Table 7.2.2-1: Channel model parameters for UMi CDL-A at 28 GHz.

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -13.4014 | -39.363 | 112.1365 | 95.9936 | 100.6889 |
| 2 | 22.914 | 0 | -2.5086 | -157.634 | 97.1624 | 88.9311 |
| 3 | 24.15 | -2.2185 | -2.5086 | -157.634 | 97.1624 | 88.9311 |
| 4 | 35.208 | -3.9794 | -2.5086 | -157.634 | 97.1624 | 88.9311 |
| 5 | 27.66 | -5.9799 | 17.4974 | 126.7698 | 97.9452 | 89.8621 |
| 6 | 32.25 | -8.1984 | 17.4974 | 126.7698 | 97.9452 | 89.8621 |
| 7 | 40.248 | -9.9593 | 17.4974 | 126.7698 | 97.9452 | 89.8621 |
| 8 | 34.5 | -10.5014 | 24.1308 | -70.3543 | 98.7118 | 73.6909 |
| 9 | 45.708 | -7.5014 | -18.9331 | -93.5479 | 96.1295 | 76.7596 |
| 10 | 92.25 | -15.9014 | 31.951 | 136.9496 | 95.3467 | 67.8293 |
| 11 | 113.868 | -6.6014 | -19.2086 | 112.4835 | 98.0648 | 77.7251 |
| 12 | 133.452 | -16.7014 | 26.9494 | -136.349 | 99.2935 | 66.4156 |
| 13 | 130.308 | -12.4014 | -34.0436 | 97.8501 | 98.7444 | 74.415 |
| 14 | 149.652 | -15.2014 | -38.0703 | 126.7698 | 98.902 | 106.7919 |
| 15 | 150.714 | -10.8014 | -29.1481 | -73.3619 | 95.912 | 97.9305 |
| 16 | 183.492 | -11.3014 | -30.4408 | -82.6162 | 95.7272 | 99.7579 |
| 17 | 244.86 | -12.7014 | 33.4345 | -96.6134 | 95.806 | 99.9303 |
| 18 | 267.474 | -16.2014 | 29.8317 | 146.3196 | 99.0271 | 68.6913 |
| 19 | 274.17 | -18.3014 | 26.5044 | 166.0428 | 94.9226 | 66.8293 |
| 20 | 287.796 | -18.9014 | -26.7533 | 172.2895 | 95.083 | 62.6917 |
| 21 | 300.396 | -16.6014 | -34.2767 | 141.4611 | 99.2962 | 107.7918 |
| 22 | 318.258 | -19.9014 | 25.1904 | -157.113 | 95.2461 | 109.4124 |
| 23 | 579.516 | -29.7014 | -13.5289 | 114.3922 | 98.5677 | 111.2743 |
| Per-Cluster Parameters |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 1.0596 | 6.3623 | 0.0815 | 1.0344 | 10 |  |

Table 7.2.2-2: Channel model parameters for UMi CDL-B at 28 GHz.

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0.3941 | -175.004 | 105.6621 | 76.7603 |
| 2 | 6.432 | -2.2185 | 0.3941 | -175.004 | 105.6621 | 76.7603 |
| 3 | 12.93 | -3.9794 | 0.3941 | -175.004 | 105.6621 | 76.7603 |
| 4 | 12.57 | -3.2014 | -15.9038 | 134.1256 | 106.8987 | 65.837 |
| 5 | 17.22 | -9.8014 | -27.6578 | -104.101 | 107.4194 | 63.4563 |
| 6 | 17.916 | -1.2 | -7.3793 | 158.7297 | 105.3237 | 68.7779 |
| 7 | 22.512 | -3.4185 | -7.3793 | 158.7297 | 105.3237 | 68.7779 |
| 8 | 30.33 | -5.1794 | -7.3793 | 158.7297 | 105.3237 | 68.7779 |
| 9 | 22.086 | -7.6014 | -28.3337 | -105.597 | 107.2762 | 79.3511 |
| 10 | 22.182 | -3.0014 | 16.6169 | 139.6116 | 105.1674 | 67.9377 |
| 11 | 34.2 | -8.9014 | -30.1363 | -100.444 | 104.9592 | 64.6467 |
| 12 | 31.698 | -9.0014 | 24.8034 | 109.0228 | 104.6858 | 62.1259 |
| 13 | 66.126 | -4.8014 | -22.7008 | 116.0051 | 105.3497 | 76.2702 |
| 14 | 76.536 | -5.7014 | -22.0624 | -103.935 | 105.2325 | 78.931 |
| 15 | 92.844 | -7.5014 | 19.9591 | -107.841 | 105.0893 | 65.2068 |
| 16 | 107.052 | -1.9014 | 8.3929 | -146.576 | 105.2976 | 76.1301 |
| 17 | 121.014 | -7.6014 | -30.324 | -106.262 | 104.9071 | 64.1565 |
| 18 | 169.764 | -12.2014 | -37.1211 | 78.5171 | 106.8857 | 79.5612 |
| 19 | 181.314 | -9.8014 | -32.2392 | -96.62 | 104.9722 | 64.0865 |
| 20 | 217.122 | -11.4014 | -34.1169 | 84.5019 | 107.4584 | 61.6358 |
| 21 | 246.402 | -14.9014 | -42.0029 | 73.6129 | 107.3413 | 63.4563 |
| 22 | 256.74 | -9.2014 | 25.2916 | 103.5368 | 107.2241 | 63.5964 |
| 23 | 287.004 | -11.3014 | -32.2392 | -81.1593 | 106.9508 | 65.1368 |
| Per-Cluster Parameters |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 3.7533 | 18.2868 | 0.3905 | 4.9015 | 8 |  |

Table 7.2.2-3: Channel model parameters for UMi CDL-C at 28 GHz.

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4215 | -30.4353 | -134.4434 | 98.9242 | 83.3318 |
| 2 | 12.594 | -1.25 | -20.9269 | 129.1633 | 99.1915 | 72.5229 |
| 3 | 13.314 | -3.4684 | -20.9269 | 129.1633 | 99.1915 | 72.5229 |
| 4 | 13.974 | -5.2294 | -20.9269 | 129.1633 | 99.1915 | 72.5229 |
| 5 | 13.056 | -2.5215 | -28.0782 | -152.8206 | 99.5732 | 71.1282 |
| 6 | 38.196 | 0 | -11.6982 | 164.1145 | 99.306 | 74.7544 |
| 7 | 38.688 | -2.2185 | -11.6982 | 164.1145 | 99.306 | 74.7544 |
| 8 | 39.36 | -3.9794 | -11.6982 | 164.1145 | 99.306 | 74.7544 |
| 9 | 39.504 | -7.4215 | 17.3861 | 84.3647 | 100.4513 | 69.2454 |
| 10 | 47.61 | -7.1215 | -37.5865 | 92.0623 | 98.5616 | 66.7349 |
| 11 | 49.278 | -10.7215 | 20.2226 | -97.7585 | 100.6231 | 72.0348 |
| 12 | 56.016 | -11.1215 | -50.6106 | 78.4702 | 98.218 | 64.4337 |
| 13 | 73.71 | -5.1215 | -33.911 | 93.1719 | 100.165 | 85.4238 |
| 14 | 78.498 | -6.8215 | -37.5066 | -112.0441 | 100.2604 | 64.1548 |
| 15 | 130.224 | -8.7215 | -43.1797 | 102.4645 | 98.1225 | 64.7824 |
| 16 | 162.63 | -13.2215 | 29.2116 | 67.2359 | 100.2604 | 92.467 |
| 17 | 255.534 | -13.9215 | 27.8133 | 34.5731 | 98.4852 | 65.6889 |
| 18 | 276.018 | -13.9215 | 23.6584 | 48.5813 | 98.1416 | 68.7572 |
| 19 | 329.412 | -15.8215 | -52.5282 | 36.4455 | 97.9698 | 59.1339 |
| 20 | 336.462 | -17.1215 | 25.0168 | 52.6729 | 100.7376 | 65.3402 |
| 21 | 378.39 | -16.0215 | 25.4562 | 49.8296 | 98.1225 | 58.4365 |
| 22 | 398.244 | -15.7215 | 30.7697 | 46.4316 | 98.1034 | 65.2705 |
| 23 | 422.562 | -21.6215 | 35.9234 | 30.759 | 100.4513 | 62.6903 |
| 24 | 519.138 | -22.8215 | -61.2775 | 69.2469 | 100.9476 | 61.993 |
| Per-Cluster Parameters |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 10.4021 | 0.5726 | 4.8814 | 7 |  |

Table 7.2.2-4: Channel model parameters for UMi CDL-D at 28 GHz.

| Cluster # | Cluster PAS | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Specular (LOS path) | 0 | -0.2 | 0 | 180 | 98.5 | 81.5 |
|  | Laplacian | 0 | -13.8303 | 0 | -180 | 98.5 | 81.5 |
| 2 | Laplacian | 2.1165 | -19.1289 | 77.9794 | -34.5531 | 95.2232 | 94.9807 |
| 3 | Laplacian | 37.0084 | -21.3474 | 77.9794 | -34.5531 | 95.2232 | 94.9807 |
| 4 | Laplacian | 82.4223 | -23.1083 | 77.9794 | -34.5531 | 95.2232 | 94.9807 |
| 5 | Laplacian | 84.9621 | -18.2289 | 11.3647 | 139.8304 | 98.2479 | 76.2575 |
| 6 | Laplacian | 109.0901 | -20.4474 | 11.3647 | 139.8304 | 98.2479 | 76.2575 |
| 7 | Laplacian | 156.9833 | -22.2083 | 11.3647 | 139.8304 | 98.2479 | 76.2575 |
| 8 | Laplacian | 107.3364 | -23.2303 | 30.2476 | -78.3945 | 98.5 | 73.2618 |
| 9 | Laplacian | 244.4247 | -28.1303 | -56.3864 | -69.288 | 95.9542 | 61.7782 |
| 10 | Laplacian | 479.9602 | -23.9303 | -28.7614 | 56.4193 | 96.6852 | 73.5114 |
| 11 | Laplacian | 569.8809 | -25.1303 | 45.9833 | -37.2797 | 99.8359 | 95.2304 |
| 12 | Laplacian | 587.0548 | -30.3303 | -115.483 | -136.177 | 93.9125 | 54.2889 |
| 13 | Laplacian | 757.4022 | -28.0303 | 67.4889 | 47.3128 | 95.4753 | 60.0307 |
| Per-Cluster Parameters |  |
| Parameter |  | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value |  | 4.371 | 18.9034 | 0.7562 | 7.4893 | 11 |  |

Table 7.2.2-5: Channel model parameters for UMi CDL-E at 28 GHz.

| Cluster # | Cluster PAS | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Specular (LOS path) | 0 | -0.03 | 0 | 180 | 99.6 | 80.4 |
|  | Laplacian | 0 | -22.2442 | 0 | -180 | 99.6 | 80.4 |
| 2 | Laplacian | 29.9823 | -16.0427 | 59.9068 | 3.3088 | 101.5457 | 80.4 |
| 3 | Laplacian | 31.7756 | -18.2612 | 59.9068 | 3.3088 | 101.5457 | 80.4 |
| 4 | Laplacian | 32.8854 | -20.0221 | 59.9068 | 3.3088 | 101.5457 | 80.4 |
| 5 | Laplacian | 31.7756 | -23.1142 | -20.9413 | 94.6029 | 99.5154 | 81.0235 |
| 6 | Laplacian | 41.5419 | -22.6142 | 16.8781 | 106.7245 | 100.1076 | 89.5973 |
| 7 | Laplacian | 111.5182 | -18.8128 | 9.6893 | -153.245 | 99.2616 | 83.9854 |
| 8 | Laplacian | 112.6923 | -21.0313 | 9.6893 | -153.245 | 99.2616 | 83.9854 |
| 9 | Laplacian | 114.4212 | -22.7922 | 9.6893 | -153.245 | 99.2616 | 83.9854 |
| 10 | Laplacian | 154.3568 | -22.5142 | 19.7953 | -139.922 | 100.1076 | 84.2971 |
| 11 | Laplacian | 216.9149 | -25.8142 | 34.0687 | -86.8495 | 98.2465 | 92.2473 |
| 12 | Laplacian | 318.4799 | -20.4142 | 0.5209 | 143.9629 | 99.3039 | 81.3353 |
| 13 | Laplacian | 701.1301 | -30.0142 | 58.2398 | -22.9654 | 97.9081 | 93.1826 |
| 14 | Laplacian | 1205.713 | -29.4142 | 60.011 | -11.8267 | 101.7149 | 77.1264 |
| Per-Cluster Parameters |  |
| Parameter |  | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value |  | 5.2093 | 12.0124 | 1.2689 | 10.912 | 8 |  |

Tables 7.2.2-6—7.2.2.10 tabulate channel model parameters for InO CDL-A—CDL-E models at 28 GHz, respectively.

Table 7.2.2-6: Channel model parameters for InO CDL-A at 28 GHz.

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -13.4014 | -101.633 | 110.3637 | 77.4554 | 114.0008 |
| 2 | 11.457 | 0 | -3.2678 | -157.49 | 95.5583 | 90.2073 |
| 3 | 12.075 | -2.2185 | -3.2678 | -157.49 | 95.5583 | 90.2073 |
| 4 | 17.604 | -3.9794 | -3.2678 | -157.49 | 95.5583 | 90.2073 |
| 5 | 13.83 | -5.9799 | 50.1288 | 125.3079 | 107.6831 | 92.0912 |
| 6 | 16.125 | -8.1984 | 50.1288 | 125.3079 | 107.6831 | 92.0912 |
| 7 | 20.124 | -9.9593 | 50.1288 | 125.3079 | 107.6831 | 92.0912 |
| 8 | 17.25 | -10.5014 | 67.8333 | -68.3566 | 119.5552 | 59.3663 |
| 9 | 22.854 | -7.5014 | -47.105 | -92.0429 | 79.5604 | 65.5763 |
| 10 | 46.125 | -15.9014 | 88.7055 | 135.7039 | 67.4357 | 47.5044 |
| 11 | 56.934 | -6.6014 | -47.8403 | 110.7181 | 109.5355 | 67.5301 |
| 12 | 66.726 | -16.7014 | 75.3564 | -135.753 | 128.5646 | 44.6436 |
| 13 | 65.154 | -12.4014 | -87.4352 | 95.7739 | 120.0604 | 60.8316 |
| 14 | 74.826 | -15.2014 | -98.1824 | 125.3079 | 122.5022 | 126.3512 |
| 15 | 75.357 | -10.8014 | -74.3689 | -71.4282 | 76.1924 | 108.4188 |
| 16 | 91.746 | -11.3014 | -77.8193 | -80.879 | 73.3297 | 112.1169 |
| 17 | 122.43 | -12.7014 | 92.665 | -95.1735 | 74.5505 | 112.4658 |
| 18 | 133.737 | -16.2014 | 83.0491 | 145.2729 | 124.4388 | 49.2488 |
| 19 | 137.085 | -18.3014 | 74.1685 | 165.4151 | 60.8681 | 45.4809 |
| 20 | 143.898 | -18.9014 | -67.9771 | 171.7944 | 63.352 | 37.1078 |
| 21 | 150.198 | -16.6014 | -88.0574 | 140.3112 | 128.6067 | 128.3747 |
| 22 | 159.129 | -19.9014 | 70.6615 | -156.959 | 65.878 | 131.6542 |
| 23 | 289.758 | -29.7014 | -32.6811 | 112.6674 | 117.3239 | 135.4221 |
| Per-Cluster Parameters |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 2.8282 | 6.4975 | 1.263 | 2.0933 | 10 |  |

Table 7.2.2-7: Channel model parameters for InO CDL-B at 28 GHz.

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 9.3327 | -174.826 | 105.9611 | 81.8761 |
| 2 | 3.216 | -2.2185 | 9.3327 | -174.826 | 105.9611 | 81.8761 |
| 3 | 6.465 | -3.9794 | 9.3327 | -174.826 | 105.9611 | 81.8761 |
| 4 | 6.285 | -3.2014 | -34.1667 | 133.2233 | 125.1141 | 59.7711 |
| 5 | 8.61 | -9.8014 | -65.5384 | -102.417 | 133.1785 | 54.9534 |
| 6 | 8.958 | -1.2 | -11.4147 | 158.35 | 100.7192 | 65.7225 |
| 7 | 11.256 | -3.4185 | -11.4147 | 158.35 | 100.7192 | 65.7225 |
| 8 | 15.165 | -5.1794 | -11.4147 | 158.35 | 100.7192 | 65.7225 |
| 9 | 11.043 | -7.6014 | -67.3425 | -103.945 | 130.9608 | 87.119 |
| 10 | 11.091 | -3.0014 | 52.6316 | 138.8259 | 98.2999 | 64.0221 |
| 11 | 17.1 | -8.9014 | -72.1535 | -98.6816 | 95.0741 | 57.3623 |
| 12 | 15.849 | -9.0014 | 74.4815 | 107.5873 | 90.8403 | 52.2611 |
| 13 | 33.063 | -4.8014 | -52.3082 | 114.7179 | 101.1224 | 80.8842 |
| 14 | 38.268 | -5.7014 | -50.6043 | -102.247 | 99.308 | 86.2688 |
| 15 | 46.422 | -7.5014 | 61.552 | -106.237 | 97.0902 | 58.4959 |
| 16 | 53.526 | -1.9014 | 30.6814 | -145.794 | 100.316 | 80.6008 |
| 17 | 60.507 | -7.6014 | -72.6547 | -104.624 | 94.2677 | 56.3704 |
| 18 | 84.882 | -12.2014 | -90.7961 | 76.4337 | 124.9125 | 87.5441 |
| 19 | 90.657 | -9.8014 | -77.7663 | -94.7768 | 95.2758 | 56.2287 |
| 20 | 108.561 | -11.4014 | -82.7778 | 82.5456 | 133.7833 | 51.2692 |
| 21 | 123.201 | -14.9014 | -103.826 | 71.4253 | 131.9688 | 54.9534 |
| 22 | 128.37 | -9.2014 | 75.7845 | 101.9848 | 130.1543 | 55.2368 |
| 23 | 143.502 | -11.3014 | -77.7663 | -78.9878 | 125.9205 | 58.3542 |
| Per-Cluster Parameters |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 10.0229 | 18.6752 | 6.0483 | 9.9189 | 8 |  |

Table 7.2.2-8: Channel model parameters for InO CDL-C at 28 GHz.

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4215 | -48.3848 | -132.8363 | 93.0309 | 93.3988 |
| 2 | 6.297 | -1.25 | -23.0068 | 128.723 | 97.1699 | 71.5254 |
| 3 | 6.657 | -3.4684 | -23.0068 | 128.723 | 97.1699 | 71.5254 |
| 4 | 6.987 | -5.2294 | -23.0068 | 128.723 | 97.1699 | 71.5254 |
| 5 | 6.528 | -2.5215 | -42.0936 | -151.6038 | 103.0827 | 68.703 |
| 6 | 19.098 | 0 | 1.6247 | 164.4166 | 98.9437 | 76.0412 |
| 7 | 19.344 | -2.2185 | 1.6247 | 164.4166 | 98.9437 | 76.0412 |
| 8 | 19.68 | -3.9794 | 1.6247 | 164.4166 | 98.9437 | 76.0412 |
| 9 | 19.752 | -7.4215 | 79.2514 | 82.9728 | 116.6821 | 64.8928 |
| 10 | 23.805 | -7.1215 | -67.4716 | 90.8339 | 87.4138 | 59.8126 |
| 11 | 24.639 | -10.7215 | 86.8222 | -95.3722 | 119.3429 | 70.5376 |
| 12 | 28.008 | -11.1215 | -102.233 | 76.9531 | 82.0922 | 55.1556 |
| 13 | 36.855 | -5.1215 | -57.6616 | 91.9671 | 112.2475 | 97.6324 |
| 14 | 39.249 | -6.8215 | -67.2583 | -109.9613 | 113.7257 | 54.5912 |
| 15 | 65.112 | -8.7215 | -82.3998 | 101.457 | 80.614 | 55.8612 |
| 16 | 81.315 | -13.2215 | 110.8139 | 65.4801 | 113.7257 | 111.8854 |
| 17 | 127.767 | -13.9215 | 107.0819 | 32.1236 | 86.2312 | 57.6958 |
| 18 | 138.009 | -13.9215 | 95.9924 | 46.4294 | 80.9097 | 63.905 |
| 19 | 164.706 | -15.8215 | -107.3512 | 34.0358 | 78.2489 | 44.4306 |
| 20 | 168.231 | -17.1215 | 99.6178 | 50.6078 | 121.1167 | 56.9902 |
| 21 | 189.195 | -16.0215 | 100.7907 | 47.7042 | 80.614 | 43.0194 |
| 22 | 199.122 | -15.7215 | 114.9725 | 44.234 | 80.3184 | 56.8491 |
| 23 | 211.281 | -21.6215 | 128.7278 | 28.2285 | 116.6821 | 51.6277 |
| 24 | 259.569 | -22.8215 | -130.7032 | 67.5339 | 124.3688 | 50.2165 |
| Per-Cluster Parameters |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 2.1326 | 10.6231 | 8.8692 | 9.8783 | 7 |  |

Table 7.2.2-9: Channel model parameters for InO CDL-D at 28 GHz.

| Cluster # | Cluster PAS | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Specular (LOS path) | 0 | -0.2 | 0 | 180 | 98.5 | 81.5 |
|  | Laplacian | 0 | -12.1055 | 0 | -180 | 98.5 | 81.5 |
| 2 | Laplacian | 0.8738 | -17.404 | -172.96 | 42.7638 | 92.4883 | 114.5755 |
| 3 | Laplacian | 15.2798 | -19.6225 | -172.96 | 42.7638 | 92.4883 | 114.5755 |
| 4 | Laplacian | 34.0301 | -21.3834 | -172.96 | 42.7638 | 92.4883 | 114.5755 |
| 5 | Laplacian | 35.0787 | -16.504 | 27.2592 | 154.306 | 98.0376 | 68.6373 |
| 6 | Laplacian | 45.0405 | -18.7225 | 27.2592 | 154.306 | 98.0376 | 68.6373 |
| 7 | Laplacian | 64.8144 | -20.4834 | 27.2592 | 154.306 | 98.0376 | 68.6373 |
| 8 | Laplacian | 44.3165 | -21.5055 | 72.5514 | -115.009 | 98.5 | 61.2872 |
| 9 | Laplacian | 100.9167 | -26.4055 | -135.248 | 20.546 | 93.8293 | 33.1118 |
| 10 | Laplacian | 198.1633 | -22.2055 | -68.9868 | 100.9531 | 95.1704 | 61.8997 |
| 11 | Laplacian | 235.2893 | -23.4055 | 110.295 | -88.7107 | 100.9509 | 115.188 |
| 12 | Laplacian | 242.3799 | -28.6055 | 83.0045 | 78.3003 | 90.0836 | 14.7365 |
| 13 | Laplacian | 312.7121 | -26.3055 | 161.8778 | -34.6022 | 92.9507 | 28.8242 |
| Per-Cluster Parameters |  |
| Parameter |  | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value |  | 10.4843 | 12.0913 | 1.3873 | 18.3753 | 11 |  |

Table 7.2.2-10: Channel model parameters for InO CDL-E at 28 GHz.

| Cluster # | Cluster PAS | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Specular (LOS path) | 0 | -0.03 | 0 | 180 | 99.6 | 80.4 |
|  | Laplacian | 0 | -20.5194 | 0 | -180 | 99.6 | 80.4 |
| 2 | Laplacian | 12.3987 | -14.3178 | 145.0787 | 66.3397 | 103.1625 | 80.4 |
| 3 | Laplacian | 13.1402 | -16.5363 | 145.0787 | 66.3397 | 103.1625 | 80.4 |
| 4 | Laplacian | 13.5992 | -18.2972 | 145.0787 | 66.3397 | 103.1625 | 80.4 |
| 5 | Laplacian | 13.1402 | -21.3894 | -50.7145 | 125.0665 | 99.4451 | 81.931 |
| 6 | Laplacian | 17.1789 | -20.8894 | 40.8743 | 132.864 | 100.5293 | 102.9826 |
| 7 | Laplacian | 46.1165 | -17.0879 | 23.4649 | -162.789 | 98.9804 | 89.2034 |
| 8 | Laplacian | 46.602 | -19.3064 | 23.4649 | -162.789 | 98.9804 | 89.2034 |
| 9 | Laplacian | 47.317 | -21.0673 | 23.4649 | -162.789 | 98.9804 | 89.2034 |
| 10 | Laplacian | 63.8316 | -20.7894 | 47.939 | -154.219 | 100.5293 | 89.9689 |
| 11 | Laplacian | 89.7015 | -24.0894 | 82.5056 | -120.079 | 97.1217 | 109.4894 |
| 12 | Laplacian | 131.7019 | -18.6894 | 1.2616 | 156.8183 | 99.0579 | 82.6965 |
| 13 | Laplacian | 289.9404 | -28.2894 | 141.0417 | -78.9842 | 96.5022 | 111.7859 |
| 14 | Laplacian | 498.6022 | -27.6894 | 145.331 | -71.819 | 103.4723 | 72.3621 |
| Per-Cluster Parameters |  |
| Parameter |  | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value |  | 12.6155 | 7.7272 | 2.3234 | 26.7929 | 8 |  |

## 7.3	Verification of Channel Model implementation

<Editor’s note: whether different verification procedures are introduced for FR1 and FR2 is FFS; these can be captured as additional sub-clauses >