3GPP TSG RAN WG4 meeting #92-bis						R4-1910924

Chongqing, China, October 14 - 18, 2019

Agenda Item: 8.8.2.5

Source: MediaTek

Title: Discussion on PRS measurement without gaps and differential RSRP consideration for test cases

Document for:	Discussion

# Introduction

In LTE and NR, the measurement gaps are need for the UE to measure inter-frequency RSTDs. In NR, the concept of BWP has been introduced and defined. UEs are expected to receive only within the frequency range configured for the active BWPs with the associated numerologies. The exception is that UEs may perform RRM measurement outside of its active BWP within measurement gaps.

In this contribution, we propose that under some conditions, UE may perform PRS measurement without measurement gaps even if PRSs from different cells are transmitted on different BWPs.

On the other aspect, it has been agreed that NR supports DL-AoD based positioning. For such positioning technique, a UE needs to report PRS-RSRPs measured from cells, the network then estimates the UE’s location based on UE’s report and the related PRS beam direction informations of cells.

Differential PRS-RSRP reporting can save UE’s reporting overhead. In this contribution, we show the positioning performance of DL-AoD based positioning with differential PRS-RSRP reporting. We then propose that PRS-RSRP differential reporting should be taken into consideration for future discussion on test cases.

# PRS Measurement without Gaps

In current specification, SSB-based intra-frequency measurement is defined, but there is no definition on the PRS-based intra-frequency measurement.

In order to boost the accuracy of positioning, it is very likely that PRS transmission bandwidth would be larger than the active BWP for a UE. In general, BWP switching includes the adjustment of centre frequency and the bandwidth adaptation.

In the case that the centre frequency of PRS bandwidth and active BWP are the same, the UE doesn’t need to adjust the centre frequency, and instead only the RF filter bandwidth adjustment is needed for PRS measurement. In such cases, the UE may require a relatively small time period for bandwidth adaptation.

However, the current agreements indicate that, the PRS measurement needs to be performed within the measurement gap, even if only a small bandwidth adaptation time period is required.

Observation 1: The restriction that PRS measurement must be performed within measurement gaps may lead to the increase measurement gap occasions and the increase of UE’s reporting delay

Note that assigning more measurement objects to be measured within the measurement gaps may lead to the increase of measurement gap occasions. Note also that the network may only provide a single per-UE measurement gap pattern for concurrent monitoring all frequency layers. Thus, we have the following observation:

Proposal 1: In the case that the centre frequency of PRS bandwidth and active BWP are the same, the measurement gaps may not be required for the UE to perform PRS measurement. Instead, the network may configure a small time period before PRS slots and configure another small time period after PRS slots such that UE may perform bandwidth adaptation within those configured time periods.

The detailed specification is FFS.

To improve the system efficiency, we have the following proposal:

The concept of proposal 1 is illustrated in the following figure:

Figure 1: An illustration that UE performs PRS measurement with measurement gaps

In the above figure, there are several symbols before and after PRS transmission where there is no data transmission, such that the UE may perform bandwidth adaptation within the time periods of these symbol. The UE is then able to measure PRSs transmitted from 3 cells without measurement gaps.

# Differential RSRP reporting for DL-AoD Based Positioning

The DL-AoD positioning may work as follows (see Figure 2 for an illustration):

Step 1: The network configures an UE to measure RSRP for several PRS resource sets

- Note: A PRS resource set is associated with a TRP

Step 2: Each TRP transmits PRS on time/frequency resources specified by PRS resources of a PRS resource set

- Note: A PRS resource is associated with a Tx beam and a PRS resource ID

Step 3: The UE measures PRS transmitted from TRPs and reports measurement results to the network

Step 4: The network estimates the AoDs based on the UE’s RSRP report

Step 5: The location server estimates the UE’s position by using the estimated AoDs

Figure 2: an illustration of DL-AoD positioning framework

To reduce the UE reporting overhead, instead of reporting absolute PRS-RSRP for each measured PRS beam, UE may report differential PRS-RSRP as follows:

Consider PRS resources in a PRS resource set,

- Suppose pk is the RSRP measured from PRS resource with ID k, k = 1,2,…,N

- Suppose pm is the maximum RSRP, i.e.,  pm≥pk  for k = 1,2,…,N

- Then UE reports the PRS resource ID m, but UE does not report pm

- For PRS resource with ID j, where j≠m, UE reports j and xj to the network, where xj=10logpm-10logpj

In the following, we show the performance of DL-AoD based positioning with differential PRS-RSRP reporting for UMi and indoor office scenario (the detailed simulation assumptions are given in Appendix).

Figure 3: Performance of DL-AoD based positioning with differential PRS-RSRP reporting for UMi

Figure 4: Performance of DL-AoD based positioning with differential PRS-RSRP reporting for indoor office FR2

Proposal 2: PRS-RSRP differential reporting should be taken into consideration for future discussion on test cases

Observation 2: Differential PRS-RSRP reporting can save UE’s reporting overhead. In addition, with differential PRS-RSRP reporting, the DL-AoD positioning can meet the regulatory accuracy requirement under UMi scenario, and can meet commercial accuracy requirement under the indoor office scenario

We have the following observation and proposal:

# Conclusions

The observations and proposals are summarized below:

Observation 1: The restriction that PRS measurement must be performed within measurement gaps may lead to the increase measurement gap occasions and the increase of UE’s reporting delay

Observation 2: Differential PRS-RSRP reporting can save UE’s reporting overhead. In addition, with differential PRS-RSRP reporting, the DL-AoD positioning can meet the regulatory accuracy requirement under UMi scenario, and can meet commercial accuracy requirement under the indoor office scenario

Proposal 1: In the case that the centre frequency of PRS bandwidth and active BWP are the same, the measurement gaps may not be required for the UE to perform PRS measurement. Instead, the network may configure a small time period before PRS slots and configure another small time period after PRS slots such that UE may perform bandwidth adaptation within those configured time periods.

The detailed specification is FFS.

Proposal 2: PRS-RSRP differential reporting should be taken into consideration for future discussion on test cases

# Reference

[1] R4-1910002

# Appendix

Basically we follow the simulation settings for UMi and office scenario as given in [1]. Modified parameters, PRS settings, and positioning algorithm are described below:

| Parameter | UMi, FR1 | Office, FR2 |
| --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline | Baseline |
| Carrier frequency | 4Gz | 30GHz |
| Subcarrier spacing | 15kHz | 60kHz |
| Reference Signal Transmission Bandwidth | 20MHz | 36MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | In a two-symbol PRS occasion: 1st symbol is comb-4 starting at RE 0 2nd symbol is comb-4 starting at RE 2 | In a two-symbol PRS occasion: 1st symbol is comb-4 starting at RE 0 2nd symbol is comb-4 starting at RE 2 |
| Reference signal (type of sequence, number of ports, …) | PN sequence, 1 port | PN sequence, 1 port |
| (Number of sites, number sectors) | (7, 3) | (12, 3) |
| Number of symbols used per occasion | 2 | 2 |
| number of occasions used per positioning estimate | 10 | 10 |
| Power-boosting level | 6.02 dB | 6.02 dB |
| Uplink power control (applied/not applied) | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | RSRP = average power per RE | RSRP = average power per RE |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | AoD estimation: Cell beam pattern is known in advance, so a look-up table (LUT) saving the AoD and power of each beam can be constructed. UE reports the measured differential PRS-RSRP of beams of a TP to the network AoDs are estimated by using the LUT and RSRP reported from UE Transforming AoD estimation results to UE location: Least-square estimator | AoD estimation: Cell beam pattern is known in advance, so a look-up table (LUT) saving the AoD and power of each beam can be constructed. UE reports the measured differential PRS-RSRP of beams of a TP to the network AoDs are estimated by using the LUT and RSRP reported from UE Transforming AoD estimation results to UE location: Least-square estimator |
| Network synchronization assumptions | Ideal | Ideal |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Each cell performs beamsweeping with LCS α angle = [-52.5 -37.5 -22.5 -7.5 7.5 22.5 37.5 52.5] degrees. Note these angles are equally 15-degree spaced and are centered at zero | Each cell performs beamsweeping with LCS α angle = [-52.5 -37.5 -22.5 -7.5 7.5 22.5 37.5 52.5] degrees. Note these angles are equally 15-degree spaced and are centered at zero |
| Precoding assumptions | N/A | N/A |
| Additional notes, if any | UE is equipped with one omni-directional antenna element | Unless otherwise specified, UE is equipped with one omni-directional antenna element |