3GPP TSG-RAN4 #92Bis								R4-1911680

Chongqing, China, 14 - 18 October 2019

Source:	Qualcomm Incorporated

Title:	FR2 UE demodulation and CSI testing methodology

Agenda item: 	10.1.4

Document for:	Approval

# 1	Introduction

For frequency bands in FR2, conducted antenna connectors are not available and the OTA testing is considered as the baseline approach for NR UE demodulation and CSI test methodology. The detail of UE demodulation and CSI testing OTA methodology were specified in TR38.810 [1].

In the current methdology, UE demoluation and CSI testing should be under the princple of aligning downlink signal and noise to Rx beam peak direction. To find out the Rx beam peak direction, the EIS based seraching is needed before the UE demodulation which will lead to a large amount of testing time e.g. about 12-14 hours fo find the beam peak direction.

In RAN4#92 meeting, we submitted a discssion paper on introduing alternative testing direcion for FR2 UE demodualtion and CSI testing to reduce the testing time but it was not apporved [2]. In this meeting, we re-submit the paper and corresponding CRs with some clarification.

# 2	Dicussion

The baseline measurement setup of NR UE demodulation and CSI characteristics for FR2 is capable of establishing an OTA link between the DUT and gNB sources with one angle of arrival (AoA) to the UE. The setup is expected to be fixed and to be used with UE Tx/Rx beamlock to allow testing of DUT baseband perforance under a “virtually cabled” scenario. Therefore, in UE demodulation testing, there is no need to verify the RF performance i.e. EIS requirements.

As shown in Figure 1, in UE demodulation tesing, the performance requirements will be specified at referene point. And during the testing, based on the setup in TR38.810 [1], downlink signal and noise should be aligned to the Rx beam peak direction.

Figure 1: DL SNR reference point for UE Demodulation and CSI testing methodology [1]

To find out the Rx basem peak direction, the peak EIS measurement procedure will be performed before the UE demodulation started. If we check the beam peak serach measurement grid defined in [1]. The following measurement grids are recommended:

- Constant density grid (using the charged particle implementation) with at least 800 grid points.

- Constant step size grid with at least 1106 grid points, corresponding to an angular step size of 7.5º.

Based on the testing experience, with the constant step grid, over 12 hours are needed just to find the beam peak direction which is very time consuming. And we have to re-search the beam peak direction with any UE update during the test.

Observation 1: In UE demodulation and CSI testing, with the constant step grid, over 12 hours are needed just to find the beam peak direction which is very time consuming. And we have to re-search the beam peak direction with any UE update during the test.

If checking the UE demodulation setup and test parameters, we can find out the intention of testing DUT at the Rx beam peak direction is to make sure DUT can have larger SNR range with certain Noc level at the testing direction. The following description is copied from [1]:

The Noc values are based on REFSENS for the operating band and on the UE Power class, and taking a baseline of UE Power Class 3 in band n260.

Noc = RESFENSPC3, n260, 50MHz -10log10(SCSREFSENS x PRBREFSENS x 12) - SNRREFSENS + ∆thermal

where:

-	REFSENSPC3, n260, 50MHz is the REFSENS value in dBm specified for Power Class 3 UE in band n260 for 50MHz channel bandwidth in TS 38.101-2 [16] Table 7.3.2.3-1, [dBm/Hz]

-	SCSREFSENS is a subcarrier spacing associated with NRB for 50MHz in TS 38.101-2 [16] Table 5.3.2-1, chosen as 120kHz.

-	PRBREFSENS is NRB associated with subcarrier spacing 120kHz for 50MHz in TS 38.101-2 [16] Table 5.3.2-1 and is 32.

-	12 is the number of subcarriers in a PRB

-	SNRREFSENS = -1 dB is the SNR used for simulation of REFSENS

-	∆thermal is the amount of dB that the wanted noise is set above UE thermal noise, giving a rise in total noise of ∆BB. ∆thermal = 6dB, giving a rise in total noise of 1dB.

Therefore, we can find out that the Noc level in Mode 1 is derived based on the assumption that DUT can meet REFSENS defined in TS38.101-2 at the test point. Moreover, the SNR range is also obtained based on assumption that the testing point meets REFSENS requirements.

Observation 2: FR2 Demodulation testing Noc level is derived based the assumption that DUT can meet the REFSENS in the testing direction.

While in UE implementation, except Rx beam peak direction, it is possible to have many directions passing the REFSENS requirements. Therefore, finding Rx beam direction is not necessary, and the directions which satisfy the REFSENS can be selected for demodulation testing.

Observation 3: Aligning signal and noise level to Rx beam direction is not necessary in demodulation testing. The directions which satisfy the REFSENS can be selected for demodulation testing.

With above analysis, we can try to find more efficient approaches to find the test direction performing the FR2 UE demodulation testing. For instance, SS-RSRPB based Rx beam peak searching was proposed when EIS peak measurement procedure was discussed in the testability SI. However, since the accuracy of RSRP is +/-6dB, this approach did not reach consensus. But in UE demodulation testing, as mentioned in observation 3, we don’t need to find the Rx beam peak direction. Therefore, we can use RSRPB reporting to find the testing direction which satisfy the REFSENS and verify the demodulation performance more efficiently.

There was some related discussion in RAN5 #84 meeting. An example finding UE direction for demodulation is given as follows [3]:

Example procedure for finding UE direction for DEMOD testing:

- Enable periodic RSRPB reporting from the UE.

- Set of grid points for the UE scan.

- For each grid point, record RSRPB on both PolLink=  polarization and PolLink=  polarization.

- Sort the grid points based on reporting RSRPB e.g. the linear sum of RSRPB values (2 each for  and   polarization).

- For the top X % grid points, run the REFSENS throughput test

- Run the isolation and RI check for the grid points which pass the REFSENS throughput test.

  - The grid points pass the isolation and /or RI check based on the specific test cases can be sued for running DEMOD tests

- If no grid points pass the isolation and /or RI check based on the specific test cases, then fall back to use Rx beam peak approach.

We should note that here is just an example procedure and the details of the procedure for finding UE demodulation testing direction will be discussed and specified in RAN5.

Observation 4: The details of the procedure for finding UE demodulation testing direction will be discussed and specified in RAN5.

From the example, we can observe that if the approach can’t find the direction which satisfies the REFSENS requirements, then the EIS based search can be used as the fallback approach which can guarantee the robustness of the proposed approach.

Therefore, to introduce more efficient approaches for FR2 UE demodulation testing meanwhile not change the current test setup and parameters, we have the following proposal:

Proposal 1: FR2 UE demodulation and CSI testing can be performed in the directions which satisfy the REFSENS defined in TS38.101-2. Approve the corresponding draft CR on TR38.810 [4] and TS38.101-4 [5].

# 3	Conclusion

In this paper, we provide the views on the FR2 UE demodulation and CSI testing methodology. To introduce more efficient apporached, we have the following observations and proposal:

Observation 1: In UE demodulation and CSI testing, with the constant step grid, over 12 hours are needed just to find the beam peak direction which is very time consuming. And we have to re-search the beam peak direction with any UE update during the test.

Observation 2: FR2 Demodulation testing Noc level is derived based the assumption that DUT can meet the REFSENS in the testing direction.

Observation 3: Aligning signal and noise level to Rx beam direction is not necessary in demodulation testing. The directions which satisfy the REFSENS can be selected for demodulation testing.

Observation 4: The details of the procedure for finding UE demodulation testing direction will be discussed and specified in RAN5.

Proposal 1: FR2 UE demodulation and CSI testing can be performed in the directions which satisfy the REFSENS defined in TS38.101-2. Approve the corresponding draft CR on TR38.810 [4] and TS38.101-4 [5].

# 4	Reference

- 3GPP TR38.810, Study on test methods, v16.3.0

- R4-1909577	, FR2 UE demodulation and CSI testing methodology, Qualcomm, August, 2019.

- R5-197552, Discussion on UE positioning procedure for FR2 Demod tests, Discussion on UE positioning procedure for FR2 Demod tests, Qualcomm, Aug. 2019.

- R4-1911681, Draft CR for TR38.810: UE demudulation and CSI testing methodlogy, Qualcomm Incorporated, Oct 2019.

- R4-1911683, Draft CR for TS38.101-4: Angle of arrival for radiated UE demodulation testing, Qualcomm Incorporated, Oct 2019.