3GPP TSG-RAN WG4 Meeting #92                	R4-1912285

Chongqing, China, October 14th – 18th  2019

Source:	Nokia, Nokia Shanghai Bell

Title:	Inter-frequency measurement requirements in NR-U

Agenda item:	8.1.4.9

Document for:	Discussion

# 1	Introduction

The following was agreed in RAN4 regarding inter-frequency measurements in NR-U [1]

Inter-frequency Measurements (RSRP, RSRQ, SINR)

- For inter-frequency, use the same approach as for intra-frequency:

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max(600ms, (8+LPSS/SSS) x max(MGRP, DMTC period)) x CSSFinter |
| DRX cycle ≤ 320ms | max(600ms, ceil((8+LPSS/SSS )x1.5) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter |
| DRX cycle > 320ms | (8+LPSS/SSS) x DRX cycle x CSSFinter |

| Condition NOTE1,2 | TSSB_measurement_period_inter |
| --- | --- |
| No DRX | max(200ms, (8+Lmeas) x max(MGRP, DMTC period)) x CSSFinter |
| DRX cycle ≤ 320ms | max(200ms, ceil((8+ Lmeas)x1.5) x max(MGRP, DMTC period, DRX cycle)) x CSSFinter |
| DRX cycle > 320ms | (8 +Lmeas) x DRX cycle x CSSFinter |

- Define maximum values for L-values in the above

  - Note: CSSF for NR-U is being discussed separately

  - Note: the same approach is applicable for SSB index identification, unless RAN2 defines a different procedure

  - Note: the above can be revisited when RAN1 design is concluded

In this contribution we discuss the maximum extension of the inter-frequency cell identification and measurement periods for NR-U.

2	Discussion

The approach defined in last RAN4 meeting to deal with missed LBT samples in NR-U follows the same approach defined for LTE-LAA: extending the measurement or the cell identification periods. As agreed in last meeting, a maximum extension of the cell identification or measurement periods needs to be defined, since the network needs to have an estimate of when to expect the UE to report measurements, while not decreasing the probability of having valid measurements under LBT failure.

In our paper [2], it was shown the maximum extension of the intra-frequency measurement period for different probabilities of DRS not being available at the UE due to LBT failure. Considering the same approach for inter-frequency measurements, considering different probabilities of the DRS not being available during the measurement period, the following maximum inter-frequency cell identification and measurement periods would be defined as in Table 1 and Table 2. Though the exact DRS periodicities are not yet agreed in RAN1, these tables consider DMTC periodicities larger than 20 ms, as proposed in RAN1 [4].

Table 1 – Inter-frequency cell identification times, with gaps, considering different maximum values of LPSS/SSS

| Top of Form Maximum TPSS/SSS_sync_inter_NR_U [s] Bottom of Form |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No DRX |  |  |  |  |  |  |
| max(DMTC periodicity,MGRP) | LPSS/SSS = 0 | LPSS/SSS = 1 | LPSS/SSS = 3 | LPSS/SSS = 8 | LPSS/SSS = 12 | LPSS/SSS = 24 |
| 20 ms | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.64 |
| 40 ms | 0.6 | 0.6 | 0.6 | 0.64 | 0.8 | 1.28 |
| 80 ms | 0.64 | 0.72 | 0.88 | 1.28 | 1.6 | 2.56 |
| 160 ms | 1.28 | 1.44 | 1.76 | 2.56 | 3.2 | 5.12 |
| DRX cycle= 320ms |  |  |  |  |  |  |
| all | 3.84 | 4.32 | 5.28 | 7.68 | 9.6 | 15.36 |
| DRX cycle= 2560 ms |  |  |  |  |  |  |
| all | 20.48 | 23.04 | 28.16 | 40.96 | 51.2 | 81.92 |

Table 2 – Inter-frequency measurement times, with gaps, considering different maximum values of LPSS/SSS

| Maximum TSSB_measurement_period_inter [s] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No DRX |  |  |  |  |  |  |
| max(DMTC periodicity,MGRP) | Lmeas= 0 | Lmeas = 1 | Lmeas= 3 | Lmeas= 8 | Lmeas = 12 | Lmeas = 24Bottom of Form |
| 20 ms | 0.2 | 0.2 | 0.22 | 0.32 | 0.4 | 0.64 |
| 40 ms | 0.32 | 0.36 | 0.44 | 0.64 | 0.8 | 1.28Bottom of Form |
| 80 ms | 0.64 | 0.72 | 0.88 | 1.28 | 1.6 | 2.56 |
| 160 ms | 1.28 | 1.44 | 1.76 | 2.56 | 3.2 | 5.12Top of Form |
| DRX cycle= 320ms |  |  |  |  |  |  |
| all | 3.84 | 4.32 | 5.28 | 7.68 | 9.6 | 15.36 |
| DRX cycle= 2560 ms |  |  |  |  |  |  |
| all | 20.48 | 23.04 | 28.16 | 40.96 | 51.2 | 81.92Bottom of Form |

Similarly to the maximum cell identification period in intra-frequency measurements, the inter-frequency cell identification significantly vary depending on the DRX cases. With no DRX, LPSS/SSS can assume large values without impacting significantly on the cell identification period, especially with the shortest DMTC periodicity, which is being proposed in

However, with longer DRX cycles, if the same approach is defined, the maximum distance between two valid DRS samples can be in the order of tenths of seconds, which is already more than the 5s in which the cell is known. In LAA, this time was extended to 8s.

Selecting the same value of LPSS/SSS or Lmeas for all cases with or without DRX can lead either to very long time between valid DRS samples, or to too restrictive requirements in some cases.

To avoid decreasing the probability of a valid measurement, while balancing the extension of the cell identification and measurement times, we propose that the same approach proposed in [2] is also defined for inter-frequency measurements.

Define distinct maximum values of LPSS/SSS and Lmeas, considering the different combinations of DRX cycle and DRS periodicity.

To avoid a long interval between valid samples, define also a maximum interval between valid DRS samples for long DRX cycles.

Define the inter-frequency cell identification time as:

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max(600ms, (8+LPSS/SSS(1)) x max(MGRP, DMTC period)) x CSSFinter |
| DRX cycle ≤ 320ms | max(600ms, ceil((8+LPSS/SSS (2))x1.5) x max(MGRP, DMTC period, DRX cycle)) x CSSFinter |
| DRX cycle > 320ms | (8+LPSS/SSS (3)) x DRX cycle x CSSFinter |
| Note 1: The requirements apply provided that the cell identification time does not exceed max(600ms, ([20] x max(MGRP, SMTC period)) x CSSFinter for DRS periodicities ≤ 80 ms, max(600ms, ([16] x max(MGRP, SMTC period)) x CSSFinter otherwise. Note 2: The requirements apply provided that the cell identification time does not exceed max(600ms, ([16]x1.5 x max(MGRP, DMTC period, DRX cycle)) x CSSFinter Note 3: The requirements apply provided that the cell identification time does not exceed [11] x DRX cycle x CSSFinter |  |

Define the inter-frequency measurement time as:

| Condition NOTE1,2 | TSSB_measurement_period_inter |
| --- | --- |
| No DRX | max(600ms, (8+LPSS/SSS(1)) x max(MGRP, SMTC period)) x CSSFinter |
| DRX cycle ≤ 320ms | max(600ms, ceil((8+LPSS/SSS (2))x1.5) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter |
| DRX cycle > 320ms | (8+LPSS/SSS (3)) x DRX cycle x CSSFinter |
| Note 1: The requirements apply provided that the cell identification time does not exceed max(600ms, ([16] x max(MGRP, SMTC period)) x CSSFinter Note 2: The requirements apply provided that the cell identification time does not exceed max(600ms, ([16]x1.5 x max(MGRP, DMTC period, DRX cycle)) x CSSFinter Note 3: The requirements apply provided that the cell identification time does not exceed [11] x DRX cycle x CSSFinter |  |

3. Summary

- In this document, the extension of the intra-frequency cell identification and measurement requirements in NR-U was discussed. The following observations were made:

- Selecting the same value of LPSS/SSS or Lmeas for all cases with or without DRX can lead either to very long time between valid DRS samples, or to too restrictive requirements in some cases.

Define distinct maximum values of LPSS/SSS and Lmeas, considering the different combinations of DRX cycle and DRS periodicity.

To avoid a long interval between valid samples, define also a maximum interval between valid DRS samples for long DRX cycles.

Define the inter-frequency cell identification time as:

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max(600ms, (8+LPSS/SSS(1)) x max(MGRP, DMTC period)) x CSSFinter |
| DRX cycle ≤ 320ms | max(600ms, ceil((8+LPSS/SSS (2))x1.5) x max(MGRP, DMTC period, DRX cycle)) x CSSFinter |
| DRX cycle > 320ms | (8+LPSS/SSS (3)) x DRX cycle x CSSFinter |
| Note 1: The requirements apply provided that the cell identification time does not exceed max(600ms, ([20] x max(MGRP, SMTC period)) x CSSFinter for DRS periodicities ≤ 80 ms, max(600ms, ([16] x max(MGRP, SMTC period)) x CSSFinter otherwise. Note 2: The requirements apply provided that the cell identification time does not exceed max(600ms, ([16]x1.5 x max(MGRP, DMTC period, DRX cycle)) x CSSFinter Note 3: The requirements apply provided that the cell identification time does not exceed [11] x DRX cycle x CSSFinter |  |

Define the inter-frequency measurement time as:

| Condition NOTE1,2 | TSSB_measurement_period_inter |
| --- | --- |
| No DRX | max(600ms, (8+LPSS/SSS(1)) x max(MGRP, SMTC period)) x CSSFinter |
| DRX cycle ≤ 320ms | max(600ms, ceil((8+LPSS/SSS (2))x1.5) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter |
| DRX cycle > 320ms | (8+LPSS/SSS (3)) x DRX cycle x CSSFinter |
| Note 1: The requirements apply provided that the cell identification time does not exceed max(600ms, ([16] x max(MGRP, SMTC period)) x CSSFinter Note 2: The requirements apply provided that the cell identification time does not exceed max(600ms, ([16]x1.5 x max(MGRP, DMTC period, DRX cycle)) x CSSFinter Note 3: The requirements apply provided that the cell identification time does not exceed [11] x DRX cycle x CSSFinter |  |

References

- R4-1910551, “WF on RRM requirements for NR-U”, Ericsson, Qualcomm Inc, Nokia, Nokia Shanghai Bell.

- R4-1912285, “Intra-frequency measurements in NR-U”, Nokia, Nokia Shanghai Bell

- R1-1910614, “On Enhancements to Initial Access Procedures for NR-U”, Nokia, Nokia Shanghai Bel

- R4-79AH-0007. “On preventing infinite measurement and cell identification time in LAA”, Nokia

- R4-165299, “Known cell requirements for LAA Rel-14”, Ericsson