3GPP TSG-RAN WG4 Meeting #93                       	 R4- 1913309

# Reno, Nevada, USA, 18th - 22th Nov, 2019

Agenda Item:	9.4.5.4

Source:	MediaTek Inc.

Title:	Discussion on NR V2X interruption requirement

Document for:		Discussion

- Introduction

In last RAN4 #92bis meeting, a WF about NR V2X RRM requirement was approved. The open issue about interruption is listed below.

- Interruption when NR Uu BWP switching, and NR V2X SL BWP configuration/reconfiguration (if supported) are performed simultaneously Agreement Keep 2 options, one of options will be selected in next meeting - Opt1 : Not define the interruption requirement and NR Uu BWP switching delay is not impacted - Opt2 : NR Uu BWP switching delay could be extended

In this paper, we will discuss the interruption requirement in NR V2X RRM.

- Interruption

This issue is basically similar as when UE switches its active BWP on one active serving cell and receives RRC reconfiguration to add one new SCell. In NR, there is no requirement on this combination procedures. Thus, it is suggested that not to define combination procedures requirement in NR V2X. Actually, network can avoid these two behavior happens at the same time because both RRC configurations are controlled by network.

Proposal 1: Do not define the requirement when NR Uu BWP switching, and NR V2X SL BWP configuration/reconfiguration (if supported) are performed simultaneously.

Even if these two procedures will happen at the same time, the NR Uu BWP switching should have higher priority. If NR Uu BWP switching delay will be extended, this extension will have much impact to all the UE’s behavior about the interruption and delay requirements in NR Uu link. RRC reconfiguration for sidelink communication initiation and termination will happen occasionally, and the procedure if NR Uu BWP switching is much faster, it is suggested that NR Uu BWP switching won’t be interrupted by NR SL RRC reconfiguration.

Proposal 2: NR Uu BWP switching won’t be interrupted by NR SL RRC reconfiguration.

- Summary

In this paper, we give our views on interruption case when NR Uu BWP switching, and NR V2X RRC reconfiguration are performed simultaneously.

Proposal 1: Do not define the requirement when NR Uu BWP switching, and NR V2X SL BWP configuration/reconfiguration (if supported) are performed simultaneously.

Proposal 2: NR Uu BWP switching won’t be interrupted by NR SL RRC reconfiguration.