3GPP TSG-RAN WG4 Meeting #93              	R4-1913368

Reno, USA, 18th – 22th Nov, 2019

Agenda item:	8.13.4.2

Source:	Samsung

Title:	Simulation results for HST PRACH in LTE Rel-16

Document for:	Discussion

# 1	Introduction

During RAN4#92 meeting, the general issues about BS demodulation scenarios and requirements for high speed. The simulation assumption for requirement of HST PRACH was agreed and captured in the WF [1].

In this contribution, based on the simulation assumption, the ideal and impairment results are presented for finalize the requirements of HST PRACH.

# 2	Simulation results

In this section, the ideal and impairment simulation results are provided with frequency offset for 1944Hz as follows table 1 and table 2:

Table 1: ideal simulation results for high speed mode restricted set type B

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix | Frequency offset | SNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| 1 | 2 | AWGN | 1944Hz | -13.78 | -13.43 | -16.04 | -16.04 |
|  | 4 | AWGN | 1944Hz | -16.22 | -15.89 | -18.34 | -18.32 |
|  | 8 | AWGN | 1944Hz | -18.48 | -18.19 | -20.49 | -20.49 |

Table 2: impairment simulation results for high speed mode restricted set type B

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix | Frequency offset | SNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| 1 | 2 | AWGN | 1944Hz | -11.78 | -11.43 | -14.04 | -14.04 |
|  | 4 | AWGN | 1944Hz | -14.22 | -13.89 | -16.34 | -16.32 |
|  | 8 | AWGN | 1944Hz | -16.48 | -16.19 | -18.49 | -18.49 |

# 3	Conclusion

In this contribution, the ideal and impairment simulation results are presented for HST PRACH requirement.

# References

R4-1907747, Way forward on LTE HST BS demodulation performance requirement, NTT DOCOMO