3GPP TSG-RAN WG4 Meeting #93                 	R4-1913400

Reno, Nevada, US, 18th – 12th November 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	NR Rel-16 HST BS demodulation PUSCH simulation results

Agenda item:	9.17.2.3.1

Document for:	Discussion

Introduction

In the RAN4#92-bis meeting, the foundations towards Rel-16 PUSCH HST BS demodulation performance requirements were discussed and the next steps were captured in a WF [1].

For 350kph, a baseline test case configuration was agreed in RAN4#92-bis, with some open questions on l0 and 1Rx.

For 500kph, a baseline test case configuration was agreed in RAN4#92-bis. However, all, except one, company that supplied simulation results, had doubts on the feasibility of the 15kHz scenario with 2.1GHz centre frequency. Further open questions persist with respect to l0 and 8Rx.

In this contribution we provide the results of our simulation campaign on possible performance requirement parametrization in PUSCH. These results are discussed in our companion discussion paper on HST BS demodulation PUSCH performance requirements.
Additional simulation results, including full TPUT vs. SNR curves, can be found in our earlier contribution [2].

PUSCH Simulation results

Unless otherwise stated in the following tables, the simulation setup follows the HST PUSCH baseline as captured in [1]:

Table 1: PUSCH HST baseline configuration

| Parameter | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | v = 350km/h |  | v = 500km/h |  |  |
| Transform precoding | Disabled |  | Disabled |  |  |
| Number of Tx | 1 |  | 1 |  |  |
| Number of Rx | Option 1: 1 Rx Option 2: 2 Rx (baseline) |  | Option 1: 2 Rx (baseline) Option 2: 8 Rx |  |  |
| Number of layers | 1 |  | 1 |  |  |
| SCS | 15kHz, 30kHz |  | 15kHz, 30kHz |  |  |
| Reference signal | DMRS type 1 with 1+1+1 |  | DMRS type 1 with 1+1+1 |  |  |
| l0 | Option 1: l0 = 3 Option 2: l0 = 2 |  | Option 1: l0 = 3 Option 2: l0 = 2 |  |  |
| symbols length | 14 |  | 14 |  |  |
| start symbol index | 0 |  | 0 |  |  |
| Time domain resource allocation type | type A |  | type A |  |  |
| Frequency domain resource | Full applicable test bandwidth |  | Full applicable test bandwidth |  |  |
| MCS index | 2 and 16 (both for alignment) |  | 2 and 16 (both for alignment) |  |  |
| Maximum Doppler shift | 15kHz SCS: 1340Hz | 30kHz SCS: 2334Hz | 15kHz SCS: Option 1: 1944Hz Option 2: 1750Hz |  | 30kHz SCS: Option 1: 3334Hz Option 2: 3000Hz |
| Carrier frequency (GHz) | 15kHz SCS: 2.1GHz 30kHz SCS: 3.6GHz |  | 15kHz SCS: [2.1GHz or 1.9GHz] 30kHz SCS: 3.6GHz |  |  |
| Propagation condition | HST single-tap channel model: Tunnel: Ds=300m, Dmin=2m Open space: Ds=700m, Dmin=150m |  |  |  |  |
| SCS and BW | 15kHz: 10MHz; 30kHz: 40MHz |  |  |  |  |
| Timing offset | 0 |  |  | 0 |  |
| Frequency offset | 0 |  |  | 0 |  |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled |  |  | Disabled |  |
| Number of HARQ transmissions | 4 |  |  | 4 |  |
| Testing metric | SNR @70% of maximum throughput |  |  | SNR @70% of maximum throughput |  |

350kph baseline

Table 2: PUSCH, 350kph, CP-OFDM, 1T2R, Type A TDRA: simulation setup and results summary

| ID | Freq/Speed | Propagation condition | BW/SCS | DM-RS | MCS | SNR@ 70% TPUT |
| --- | --- | --- | --- | --- | --- | --- |
| 3001 | 2.1GHz/350kph | Single Tap-Scen3-1340Hz | 10MHz/15kHz | 1+1+1 l0=2 | 2 | -5,93 |
| 3002 |  |  |  |  | 16 | 5,75 |
| 3003 |  |  |  | 1+1+1 l0=3 | 2 | -5,94 |
| 3004 |  |  |  |  | 16 | 5,75 |
| 3005 |  | Single Tap-Scen1-1340Hz-option2 | 10MHz/15kHz | 1+1+1, l0=2 | 2 | -6,00 |
| 3006 |  |  |  |  | 16 | 5,73 |
| 3007 |  |  |  | 1+1+1 l0=3 | 2 | -6,00 |
| 3008 |  |  |  |  | 16 | 5,73 |
| 3009 | 3.6GHz/350kph | Single Tap-Scen3-2334Hz | 40MHz/30kHz | 1+1+1 l0=2 | 2 | -5,97 |
| 3010 |  |  |  |  | 16 | 5,74 |
| 3011 |  |  |  | 1+1+1 l0=3 | 2 | -5,97 |
| 3012 |  |  |  |  | 16 | 5,75 |
| 3013 |  | Single Tap-Scen1-2334Hz-option2 | 40MHz/30kHz | 1+1+1, l0=2 | 2 | -6,01 |
| 3014 |  |  |  |  | 16 | 5,72 |
| 3015 |  |  |  | 1+1+1 l0=3 | 2 | -6,02 |
| 3016 |  |  |  |  | 16 | 5,72 |

500kph l0 comparison

Table 3: PUSCH, 500kph, CP-OFDM, 1T2R, Type A TDRA: simulation setup and results summary

| ID | Freq/Speed | Propagation condition | BW/SCS | DM-RS | MCS | SNR@ 30% TPUT | SNR@ 70% TPUT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3017 | 2.1GHz/500kph | Single Tap-Scen3-1944Hz | 10MHz/15kHz | 1+1+1 l0=2 | 2 | NaN | NaN |
| 3018 |  |  |  |  | 16 | NaN | NaN |
| 3019 |  |  |  | 1+1+1 l0=3 | 2 | NaN | NaN |
| 3020 |  |  |  |  | 16 | NaN | NaN |

Conclusion

In this contribution we provided the results of our simulation campaign on possible performance requirement parametrization in PUSCH for NR_HST Rel-16 BS demodulation performance requirements.
These results are discussed in detail in our companion discussion paper.

- References

- R4-1912809, Way forward on NR HST PUSCH demodulation, Huawei, RAN4#92-Bis.

- R4-1911192, NR Rel-16 HST BS demodulation PUSCH simulation results, Nokia, Nokia Shanghai Bell, RAN4#92-Bis.