3GPP TSG-RAN WG4 Meeting #93                 	R4-1913401

Reno, Nevada, US, 18th – 12th November 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	NR Rel-16 HST BS demodulation PRACH simulation results

Agenda item:	9.17.2.3.2

Document for:	Discussion

Introduction

In the RAN4#92-bis meeting, the foundations of Rel-16 PUSCH HST BS demodulation performance requirements were discussed and the next steps were captured in a WF [1].

For 350kph, a baseline test case configuration was agreed, with some open questions concerning the exact preamble Ids, low mobility scenarios, and restricted set Type B max FO.

Also, for 500kph a baseline test case configuration was agreed. Open questions persist with respect to usage of format 0 and exact preamble Ids.

A further open question is the re-use of timing error tolerances from non-HST testing.

In this contribution we provide the results of our simulation campaign on possible performance requirement parametrization in PRACH. These results are discussed in our companion discussion paper on HST BS demodulation PRACH performance requirements.
Additional simulation results can be found in our earlier contribution [2].

PRACH Simulation results

Unless otherwise stated in the following tables, the simulation setup follows the HST PRACH baseline as captured in [1]:

Table 1: PRACH HST baseline configuration

| Parameter | Value |  |
| --- | --- | --- |
|  | v = 350km/h | v = 500km/h |
| Frequency range | FR1 |  |
| Number of Tx | 1 |  |
| Number of Rx | 2Rx, [4Rx], [8 Rx] |  |
| Formats | 0 | A2, B4, C2, [0] (B3 option not in WF) |
| SCS | 1.25kHz | 15kHz, 30kHz, [1.25kHz] |
| Restricted set format 0 | {Type A, Type B} | {Type A, Type B} |
| (Ncs, logical sequence index, v) | Set A: Option 1: (15, 384, 0) Option 2: (15, 384, 36) Set B: Option 1: (15, 30, 30) Option 2: (15, 384, 36) | Short - 15kHz: (23, 0, 0) Short - 30kHz: (46, 0, 0) [Long: See 350km/h.] |
| Max timing error | Format 0, AWGN: 1.04us Format 0, TDLC300: 2.55us | 15kHz, AWGN: 0.52us 15kHz, TDLC300: 2.03us 30kHz, AWGN: 0.26us 30kHz, TDLC300: 1.66us [Format 0: See 350km/h] |
| Maximum Doppler shift | Set A: 1340 Hz, [0Hz] Set B: Option 1: 2334 Hz, [0Hz] Option 2: 1875 Hz, [0Hz] | Short: Align with HST PUSCH. [Long: Not discussed.] |
| Carrier frequency (GHz) | Set A: 2.1GHz Set B: 3.6GHz | Short: Align with HST PUSCH. Long: See 350km/h. |
| Propagation condition | {AWGN with max Doppler shift from above, TDLC[300-100] with FO=400Hz} |  |
| Testing metric | SNR @ 0.1% false alarm probability and 99% detection probability. |  |

FR1 1T2R

Table 1: PRACH FR1 1T2R.

| PRACH FR1 1T2R |  | Propagation condition | Frequency offset (Hz) | Nokia | Ncs | LogRootId | SignatureId |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Ideal |  |  |  |
| Format 0 (SetA) | 1.25kHz | AWGN | 0 | -15.31 | 15 | 384 | 0 |
|  |  | AWGN | 1340 | -15.2 |  |  |  |
|  |  | TDLC300-100 | 400 | -7.52 |  |  |  |
|  |  | AWGN | 0 | -15.24 | 15 | 384 | 30 |
|  |  | AWGN | 1340 | -15.13 |  |  |  |
|  |  | TDLC300-100 | 400 | -7.75 |  |  |  |
|  |  | AWGN | 0 | -15.33 | 15 | 384 | 36 |
|  |  | AWGN | 1340 | -14.98 |  |  |  |
|  |  | TDLC300-100 | 400 | -7.38 |  |  |  |
| Format 0 (SetB) | 1.25kHz | AWGN | 1875 | -13.78 | 15 | 384 | 0 |
|  |  | AWGN | 1944 | -13.84 |  |  |  |
|  |  | AWGN | 2334 | -14.61 |  |  |  |
|  |  | TDLC300-100 | 400 | -6.93 |  |  |  |
|  |  | AWGN | 0 | -14.74 | 15 | 30 | 30 |
|  |  | AWGN | 1875 | -13.8 |  |  |  |
|  |  | AWGN | 2334 | -14.61 |  |  |  |
|  |  | TDLC300-100 | 400 | -6.89 |  |  |  |
|  |  | AWGN | 0 | -14.68 | 15 | 384 | 36 |
|  |  | AWGN | 1875 | -13.74 |  |  |  |
|  |  | AWGN | 2334 | -14.37 |  |  |  |
|  |  | TDLC300-100 | 400 | -6.92 |  |  |  |
| A2 | 15KHz | AWGN | 1750 | -13.61 | 23 | 0 | 0 |
|  |  | AWGN | 1944 | -13.51 |  |  |  |
|  | 30kHz | AWGN | 3000 | -13.68 | 46 | 0 | 0 |
|  |  | AWGN | 3334 | -13.54 |  |  |  |
| B4 | 15KHz | AWGN | 1750 | -17.36 | 23 | 0 | 0 |
|  |  | AWGN | 1944 | -17.18 |  |  |  |
|  | 30kHz | AWGN | 3000 | -17.46 | 46 | 0 | 0 |
|  |  | AWGN | 3334 | -17.31 |  |  |  |
| C2 | 15KHz | AWGN | 1750 | -13.65 | 23 | 0 | 0 |
|  |  | AWGN | 1944 | -13.5 |  |  |  |
|  | 30kHz | AWGN | 3000 | -13.67 | 46 | 0 | 0 |
|  |  | AWGN | 3334 | -13.55 |  |  |  |

FR1 1T4R

Table 1: PRACH FR1 1T4R.

| PRACH FR1 1T4R |  | Propagation condition | Frequency offset (Hz) | Nokia | Ncs | LogRootId | SignatureId |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Ideal |  |  |  |
| Format 0 (SetA) | 1.25kHz | AWGN | 0 | -17.56 | 15 | 384 | 0 |
|  |  | AWGN | 1340 | -17.51 |  |  |  |
|  |  | TDLC300-100 | 400 | -12.67 |  |  |  |
|  |  | AWGN | 0 | -17.46 | 15 | 384 | 30 |
|  |  | AWGN | 1340 | -17.29 |  |  |  |
|  |  | TDLC300-100 | 400 | -12.61 |  |  |  |
|  |  | AWGN | 0 | -17.59 | 15 | 384 | 36 |
|  |  | AWGN | 1340 | -17.29 |  |  |  |
|  |  | TDLC300-100 | 400 | -12.54 |  |  |  |
| Format 0 (SetB) | 1.25kHz | AWGN | 1875 | -15.99 | 15 | 384 | 0 |
|  |  | AWGN | 1944 | -15.96 |  |  |  |
|  |  | AWGN | 2334 | -16.77 |  |  |  |
|  |  | TDLC300-100 | 400 | -12.06 |  |  |  |
|  |  | AWGN | 0 | -16.91 | 15 | 30 | 30 |
|  |  | AWGN | 1875 | -15.98 |  |  |  |
|  |  | AWGN | 2334 | -16.72 |  |  |  |
|  |  | TDLC300-100 | 400 | -12.12 |  |  |  |
|  |  | AWGN | 0 | -16.8 | 15 | 384 | 36 |
|  |  | AWGN | 1875 | -15.88 |  |  |  |
|  |  | AWGN | 2334 | -16.62 |  |  |  |
|  |  | TDLC300-100 | 400 | -12.04 |  |  |  |
| A2 | 15KHz | AWGN | 1750 | -15.8 | 23 | 0 | 0 |
|  |  | AWGN | 1944 | -15.6 |  |  |  |
|  | 30kHz | AWGN | 3000 | -15.79 | 46 | 0 | 0 |
|  |  | AWGN | 3334 | -15.68 |  |  |  |
| B4 | 15KHz | AWGN | 1750 | -19.32 | 23 | 0 | 0 |
|  |  | AWGN | 1944 | -19.13 |  |  |  |
|  | 30kHz | AWGN | 3000 | -19.38 | 46 | 0 | 0 |
|  |  | AWGN | 3334 | -19.26 |  |  |  |
| C2 | 15KHz | AWGN | 1750 | -15.76 | 23 | 0 | 0 |
|  |  | AWGN | 1944 | -15.59 |  |  |  |
|  | 30kHz | AWGN | 3000 | -15.81 | 46 | 0 | 0 |
|  |  | AWGN | 3334 | -15.7 |  |  |  |

FR1 1T8R

Table 1: PRACH FR1 1T8R.

| PRACH FR1 1T8R |  | Propagation condition | Frequency offset (Hz) | Nokia | Ncs | LogRootId | SignatureId |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Ideal |  |  |  |
| Format 0 (SetA) | 1.25kHz | AWGN | 0 | -19.67 | 15 | 384 | 0 |
|  |  | AWGN | 1340 | -19.62 |  |  |  |
|  |  | TDLC300-100 | 400 | -16.63 |  |  |  |
|  |  | AWGN | 0 | -19.55 | 15 | 384 | 30 |
|  |  | AWGN | 1340 | -19.35 |  |  |  |
|  |  | TDLC300-100 | 400 | -16.57 |  |  |  |
|  |  | AWGN | 0 | -19.65 | 15 | 384 | 36 |
|  |  | AWGN | 1340 | -19.3 |  |  |  |
|  |  | TDLC300-100 | 400 | -16.56 |  |  |  |
| Format 0 (SetB) | 1.25kHz | AWGN | 1875 | -18.08 | 15 | 384 | 0 |
|  |  | AWGN | 1944 | -18.05 |  |  |  |
|  |  | AWGN | 2334 | -18.79 |  |  |  |
|  |  | TDLC300-100 | 400 | -15.92 |  |  |  |
|  |  | AWGN | 0 | -18.91 | 15 | 30 | 30 |
|  |  | AWGN | 1875 | -18.08 |  |  |  |
|  |  | AWGN | 2334 | -18.73 |  |  |  |
|  |  | TDLC300-100 | 400 | -15.93 |  |  |  |
|  |  | AWGN | 0 | -18.86 | 15 | 384 | 36 |
|  |  | AWGN | 1875 | -17.96 |  |  |  |
|  |  | AWGN | 2334 | -18.56 |  |  |  |
|  |  | TDLC300-100 | 400 | -16.08 |  |  |  |
| A2 | 15KHz | AWGN | 1750 | -17.45 | 23 | 0 | 0 |
|  |  | AWGN | 1944 | -17.3 |  |  |  |
|  | 30kHz | AWGN | 3000 | -17.53 | 46 | 0 | 0 |
|  |  | AWGN | 3334 | -17.43 |  |  |  |
| B4 | 15KHz | AWGN | 1750 | -20.87 | 23 | 0 | 0 |
|  |  | AWGN | 1944 | -20.73 |  |  |  |
|  | 30kHz | AWGN | 3000 | -20.91 | 46 | 0 | 0 |
|  |  | AWGN | 3334 | -20.8 |  |  |  |
| C2 | 15KHz | AWGN | 1750 | -17.45 | 23 | 0 | 0 |
|  |  | AWGN | 1944 | -17.27 |  |  |  |
|  | 30kHz | AWGN | 3000 | -17.52 | 46 | 0 | 0 |
|  |  | AWGN | 3334 | -17.39 |  |  |  |

Conclusion

In this contribution we provided the results of our simulation campaign on possible performance requirement parametrization in PRACH for NR_HST Rel-16 BS demodulation performance requirements.
These results are discussed in detail in our companion discussion paper.

- References

- R4-1912729, Way forward on NR HST PRACH demodulation, Samsung, RAN4#92-Bis.

- R4-1911193, NR Rel-16 HST BS demodulation PRACH simulation results, Nokia, Nokia Shanghai Bell, RAN4#92-Bis