3GPP TSG-RAN WG4 #93	R4-1913412

Reno, US, 18th – 22nd November 2019

Agenda item:	9.5.4

Source: 	Qualcomm Incorporated

Title: 	Overview of IAB RF requirements in FR2

Document for:	Approval

# Introduction

In RAN#82, a WID on Integrated Access and Backhaul for NR (IAB) was approved in [1]. Among the objectives of the work item, RAN4 is tasked to define RF and RRM requirements for both backhaul (BH) and access links of an IAB-node including requirements for co-existence (e.g. ACLR, ACS).

In this contribution we present our views on the definition of IAB RF requirements for FR2. We believe that many of the requirements currently defined for BS and UE can be directly re-used for IAB nodes and RAN4 should focus their effort on the definition of those requirements that may be subject to change.

# Discussion

Table 1 shows our views on the definition of IAB RF requirements in FR2. We believe that all IAB-DU requirements should be imported from BS specifications whereas more discussion will be needed for IAB-MT requirements. Based on Table 1, we propose to focus on the requirements that need more discussion and agree on those that can be directly imported from UE and BS specifications.

Proposal 1: use Table 1 as baseline for definition of IAB-MT and IAB-DU RF requirements

Table 1. IAB-MT and IAB-DU RF requirements

| RF Requirement | IAB-MT | IAB-DU |
| --- | --- | --- |
| Radiated transmit power | Import from BS specs - Modifications may be needed | Import from BS specs |
| IAB output power | Import from BS specs - Modifications may be needed | Import from BS specs |
| OTA output power dynamics | Currently under discussion in RAN4 | Import from BS specs |
| OTA transmit ON/OFF power | Import from UE specs | Import from BS specs |
| OTA transmitted signal quality | Import from UE specs | Import from BS specs |
| Beam correspondence | Under discussion | Not needed |
| OTA unwanted emissions | ACLR: under discussion | Import from BS specs |
|  | Emission mask: Import from BS OBUE specification |  |
|  | Spurious emission: Import from BS specs |  |
| OTA reference sensitivity level | REFSENS shall be declared following BS approach | Import from BS specs |
| OTA in-band selectivity and blocking | ACS: under discussion | Import from BS specs |
|  | In-band blocking: under discussion |  |
| OTA maximum input level | Under discussion | Not needed |
| OTA out-of-band blocking | May not be needed. Currently not defined for UE | Import from BS specs |
| OTA receiver spurious emissions | Import from BS specs | Import from BS specs |
| OTA receiver intermodulation | May not be needed. Currently not defined for UE | Import from BS specs |
| OTA in-channel selectivity | Not needed | Import from BS specs |

# Conclusions

In this contribution we presented our views on the definition of IAB RF requirements. Considering the time left for the completion of the work item, we believe that RAN4 should focus their effort on the definition of the more fundamental requirements, trying to re-use as much as possible Rel-15 UE and BS specifications.

Based on the analysis shown in Table 1 we made the following proposal:

Proposal 1: use Table 1 as baseline for definition of IAB-MT and IAB-DU RF requirements

# Reference

- RP-182882, “New WID: Integrated Access and Backhaul for NR”, Qualcomm Incorporated.

- 3GPP TS 38.174, “Integrated access and backhaul radio transmission and reception”.