3GPP TSG-RAN WG4 Meeting #92bis						R4-1913522

Reno, USA, 18th – 22th November 2019

Source:	Samsung

Title:	Discussion on FR1 MIMO OTA environmental condition

Agenda item:	11.2.3.1

Document for:	Discussion

- Background

At the beginning of this work item, FR1 environmental condition was intensively discussed. The pros and cons of UE noise-limited condition and SIR based condition were fully compared.

Debates keep ongoing until RAN4#92 meeting. Though continued discussion is allowed until Nov RAN4 meeting in the WF [1], tentative agreement was recorded in the ad-hoc meeting notes [2], i.e., UE noise-limited environmental condition as baseline:

Agreements:

Select UE noise-limited environmental condition as the baseline for FR1 NR MIMO OTA testing.

Apple share concerns on this agreements.

In RAN4#92bis meeting, it is agreed to make final decision on FR1 environmental decision on this meeting, as captured in the WF [3]:

The Environmental condition for FR1:

- Companies provide analysis results or measurement results next meeting to make the final decision.

In this paper, the pros and cons of the two environmental conditions based on companies’ previous studies are summarized, and SNR range issue is further analyzed and emphasized.

# Discussion

## Summary of two environmental conditions’ comparison

Adequate discussions on the pros and cons of the two environmental conditions have been done in previous meetings. A good comparison table is summarized in [4] and reproduced as below table 2.1-1:

Table 2.1-1 summary of environmental conditions comparison (from [4])

| Parameters for consideration | UE noise-limited (varied DL signal) | SIR-based (fixed DL signal, varied AWGN) |
| --- | --- | --- |
| DL signal level | Low | High (typically 40dB higher than REFSENSE at test zone) |
| Artificial Noise in the chamber | No (ideally, only UE noise floor ) | Yes (high, ~10dB higher than UE noise-floor ) |
| Antenna gain impacts | High | Low |
| Antenna gain imbalance impacts | Not confirmed | Not confirmed |
| Antenna correlation impacts | High | High |
| UE Self-interference impacts | High | Low |
| Close to UE conducted demodulation test? | No | Yes |
| MIMO Network scenario | Edge of a cell’s coverage (typically TM2 scenario) | High SNR with interference, closer to actual networks |
| Performance difference of different devices | High (typically in ~10dB, similar trend to TIS ) | Low (typically in ~3dB) |
| Performance difference of low and high bands | High (typically ~4dB, similar trend to TIS) | No significant trends (typically in ~1dB) |
| Test System implementation complexity | Difficult to guarantee minimum noise (i.e. pure UE noise-limited) condition for different bands | Need to generate high DL power near maximum linear output of the system |

According to above comparison table, we can see that SIR based method has the advantage of more matching with actual network scenario, however, its disadvantage is also serious, i.e., it can not distinguish good UE and bad UE, typically the performance difference of different devices is within 3dB, and some practical test shows UE performance difference within 2dB in [5]. If a test case can not distinguish good device and bad device, then the test seems not so meaningful.

Though UE noise-limited condition has the disadvantage of cell edge scenario, it is not so serious because it still can reflect its performance potentiality in other network scenarios indirectly. UE noise-limited conditions can verify more UE performance factors including antenna gain, antenna correlation, UE noise, etc.

Observation 1:	SIR based condition has scenario advantage but its disadvantage of not distinguishing good UE and bad UE is also serious. UE noise-limited condition is more balanced and can verify more UE performance factors.

## Further analysis of high DL power issue of SIR based condition

As highlighted in Table 2.1-1, high DL signal power is another disadvantage of SIR based condition. It may not be an issue for LTE but may be an issue for NR. In the following we make further analysis on this point.

According to Table 2.4.2-1 and Table 2.4.2-2 of CTIA MIMO test plan, the signal level at test volume shall be no less than REFSENS plus 40dB. Taking LTE B41 and NR n79 as examples, LTE B41 REFSENS=-92dBm, NR n79 REFSENS=-85.6dBm, after plus 40dB, we get the signal level at test volume will be at least: -52dBm for LTE B41 and -45.6dBm for NR n79.

As agreed in last meeting, the minimum test distance is d=1.2m. So the air loss from test volume to TE probe antennas can be calculated by the equation

AirLossdB=10log4πdλ2

We can get AirLoss is 42.6dB for LTE B41and 48dB for NR n79.

Take the cable loss, TE antenna gain into account, assume it is 5dB loss in total. The necessary output power of TE’s PA shall be (-52+42.6+5)=-4.4dBm for LTE B41 and (-45.6+48+5)=7.4dBm for NR n79.

Considering NR frequency range is up to 7.125GHz, and the test distance can be longer than 1.2m, the necessary output power of TE’s PA under different frequency and different test distance is calculated as shown in Table 2.2-1.

Table 2.2-1 necessary output power of TE’s PA to guarantee SNR range for SIR based condition

|  | LTE B41 |  |  | NR n79 |  |  | NR 7.125GHz |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| REFSENS | -92 |  |  | -85.6 |  |  | ~-85.6 |  |  |
| Signal level at test zone | -52 |  |  | -45.6 |  |  | ~-45.6 |  |  |
| Measurement distance (d) | d=1.2m | d=2m | d=3m | d=1.2m | d=2m | d=3m | d=1.2m | d=2m | d=3m |
| AirLoss | 42.6 | 47 | 50.6 | 48 | 52.4 | 56 | 51.1 | 55.5 | 59 |
| Cable loss – TE antenna gain | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
| TE PA output power | -4.4 | 0 | 3.6 | 7.4 | 11.8 | 15.4 | ~10.5 | ~14.9 | ~18.4 |

For LTE B41, it can be seen that TE’s PA output power around 0dBm is enough to achieve necessary SNR range for SIR based condition. For NR n79 or higher frequency, TE’s PA need to transmit higher than 10dBm power even near to 20dBm power to achieve necessary SNR at test zone, which is near PA’s linear point or out of PA’s linear range.

Considering FR1 also needs to cover 4x4 DL MIMO test, the high DL signal level issue deteriorates further.

Observation 2:	For NR MIMO OTA, the high DL signal level issue for SIR based environmental condition deteriorates due to wider BW, higher frequency, higher MIMO rank compared with LTE. It may affect the SNR range and will be a limitation for test system.

Some company also proposes to apply different environmental condition for different bands [6]. However, in previous meetings there are already agreements that finally only one environmental condition will be selected. So we have to select a better one in this meeting.

Based on above analysis, we make the following proposals

Proposal 1:	apply only one environmental condition for NR FR1 MIMO OTA

Proposal 2:	UE noise-limited environmental condition is more favorable since it is more balanced, no serious issue and more extendable.

# Conclusion

Observation 1:	SIR based condition has scenario advantage but its disadvantage of not distinguishing good UE and bad UE is also serious. UE noise-limited condition is more balanced and can verify more UE performance factors.

Observation 2:	For NR MIMO OTA, the high DL signal level issue for SIR based environmental condition deteriorates due to wider BW, higher frequency, higher MIMO rank compared with LTE. It may affect the SNR range and will be a limitation for test system.

Proposal 1:	apply only one environmental condition for NR FR1 MIMO OTA

Proposal 2:	UE noise-limited environmental condition is more favorable since it is more balanced, no serious issue and more extendable.

# References

- R4-1910609 	“WF on NR MIMO OTA”, CAICT

- R4-1919932 	“NR MIMO OTA Ad-hoc meeting notes”, CAICT

- R4-1911637 	“WF on NR MIMO OTA”, CAICT, Spirent, CMCC, SGS Wireless, Samsung, Qualcomm, vivo

- R4-1815899 	“NR MIMO OTA Ad-hoc meeting notes”, CAICT

- R4-1911682 	“Environmental conditions for NR FR1 MIMO OTA”, Qualcomm

- R4-1911515 	“Further views on the environmental conditions for FR1 MIMO OTA”, Apple