3GPP TSG-RAN4 Meeting #93	R4-1913743

# Reno, USA, 18th Nov – 22th Nov, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6686 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 36.133 on interruption requirements for NR V2X |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  |  | Date: |  | 2019-10-25 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduce interruption requirements on LTE serving cell(s) due to NR V2X RRC reconfiguration |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following interruption requirements are introduced to 36.133 for NR V2X: - Interruption requirement on LTE serving cell(s) due to V2X RRC re-configuration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interruption requirements on LTE serving cell(s) due to V2X RRC re-configuration are missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 13.7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 36.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change>>

13.7.4	Interruptions to WAN due to NR V2X Sidelink Communication

This sub-clause contains the requirements related to the interruptions on the serving cell(s) due to NR V2X sidelink communication.

A UE capable of NR V2X sidelink communication may indicate its interest (initiation or termination) in NR V2X sidelink communication to the connected gNodeB/eNodeB using IE [SidelinkUEInformationNR].

The UE is allowed an interruption of up to 2 subframes on the serving cell(s) during the RRC reconfiguration procedure that includes the NR V2X sidelink communication configuration message [sl-V2X-ConfigDedicatedNR] (setup and release). This interruption is for both uplink and downlink of the serving cell(s).

<< End of Change>>