3GPP TSG-RAN WG4 Meeting #93	R4-1913835

Reno Nevada, USA, 18th – 22nd November, 2019

Source:	Verizon, Nokia, Qualcomm

Title:	TP for TR 38.716-02-00: CA_n2A_n5A

Agenda item:	10.2.2

Document for:	Approval

# Introduction

In RAN#85 meeting, the revised WID on Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1, 2) [1]. This contribution is a text proposal for TR 38.716-02-00 to include CA_n2A_n5A.

# Reference

[1] 	RP-191822, Revised WID on Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1, 2), ZTE

# Text Proposal

<Start of Text Proposal>

### 8.x		CA_n2_n5

### 8.x.1		Common for 1 band UL and 2 bands UL CA

#### 8.x.1.1		Operating bands for CA

Table 8.x.1.1-1: CA band combination of band n2+n5

| NR CA Band | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n2-n5 | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |

#### 8.x.1.2		Channel bandwidths per operating band for CA

Table 8.x.1.2-1: Supported bandwidths per CA band combination of band n2+n5

| CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | NR Uplink CA configuration | NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n2A-n5A | CA_n2A-n5A | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 8.x.1.3		Co-existence studies

Table 6.36.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n2-n5.

Table 8.x.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n2 | 1850 | 1910 | 1930 | 1990 | 3700 | 3820 | 5550 | 5730 | 7400 | 7640 | 9250 | 9550 |
| n5 | 824 | 849 | 869 | 894 | 1648 | 1698 | 2472 | 2547 | 3296 | 3396 | 4120 | 4245 |

Based on Table 6.x.1.3-1, it can be seen that there is no harmonic skirt interference from band n5 to impact the receive frequency of band n2.

#### 8.x.1.4		∆TIB and ∆RIB values

For CA_n2-n5, the TIB,c and RIB,c values for UEs are given in the tables below.

Table 8.x.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n2-n5 | n2 | 0.3 |
|  | n5 | 0.3 |

Table 8.x.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n2-n5 | n2 | 0 |
|  | n5 | 0 |

#### 8.x.1.5		REFSEN requirements

Same REFSENS requirements will be applied from 38.101-1. There is no need for additional REFSENS requirements.

### 8.x.2	Specific for 2 bands UL CA

#### 8.x.2.1		UE co-existence studies

Table 8.X.2.1-1 lists Band n2 + Band n5 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 8.x.2.1-1: Band n2 and Band n5 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 824 | 849 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 1648 | 1698 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 2472 | 2547 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7640 | 3296 | 3396 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9250 | 9550 | 4120 | 4245 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 1086 | 1001 | 2674 | 2759 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2851 | 2996 | 262 | 152 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 4524 | 4669 | 3498 | 3608 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 4524 | 4669 | 3498 | 3608 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 4701 | 4906 | 562 | 697 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 6374 | 6579 | 4322 | 4457 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 2002 | 2172 | 5348 | 5518 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1546 | 1386 | 6816 | 6551 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5146 | 5306 | 8224 | 8489 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1153 | 1348 | 4082 | 3852 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6172 | 6367 | 7198 | 7428 |

Based on Table 8.x.2.1-1, there is no harmonics product found to fall into Rx frequencies of band n2 and no IMD falls into its band n2 Rx frequencies.

Based on the Table 8.x.2.1-1;

- - the 2nd  harmonics may fall into n43, n77 and n78 Rx frequency

- - the 3rd harmonics may fall into B46, n41 and n90 Rx frequency

- - the 4th harmonics may fall into B53 and n77 and n78 Rx frequency

- - the 5th harmonics may fall into n77 Rx frequency

- - the 2nd IMD may fall into n7, n41 and n90 Rx frequency

- - the 3rd IMD may fall into B22, B47, n49, B41, n77, n78, n79 Rx frequency

- - the 4th IMD may fall into n1, B4, B10, n34, B46, n65, n66, n70, n79 Rx frequency

- - the 5th IMD may fall into B46, n77 Rx frequency

Table 8.x.2.1-2 lists the protected bands required for the 2UL bands CA configuration.

Table 8.x.2.1-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) |  | NOTE |
| CA_n2-n5 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 18, 19, 24, 25, 26, 27, 28, 29, 30, 31, 34, 38, 40, 41, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74, 85, n90 | FDL_low | - | FDL_high | -50 |  | 1 |  |
|  | E-UTRA Band 41, 52 | FDL_low | - | FDL_high | -50 |  | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 |  | 1 | 39 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 |  | 0.3 | 8,39 |

<Start of Text Proposal>