3GPP TSG-RAN WG4 Meeting #93	R4-1914081

Reno, US, 18th– 22nd November 2019

Agenda item:	9.12.1	General (Ad-hoc MoM/TR maintenance)	[NR_DL256QAM_FR2]

Source: 		NTT DOCOMO, INC.

Title: 	TP to TR 38.883 Updated FR2 DL 256QAM link level simulation results

Document for:	Approval

- Introduction

In RAN4 #92bis, TP to TR 38.883 to capture link level and system level simulation results was approved [1]. In this TP, some errors are corrected.

References

- R4-1913051, “TP to TR 38.822 NR FR2 256QAM link level and system level simulation results”, NTT DOCOMO, INC, RAN4 #92bis, October 2019

--------------Start of text proposal-------------

5.2.1.1	Simulation assumptions

The link level simulation assumptions are listed as in table 5.2.1.1-1, based on which, to evaluate the throughput difference between 64QAM and 256QAM. The study aims to identify conditions where DL 256QAM provides performance benefits.

Table 5.2.1.1-1 link level simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 29 GHz (n257) and 39 GHz (n260) |
| CBW | 50 MHz, 100MHz |
| SCS | 60kHz, 120 kHz; |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 30ns delay spread, 35Hz Doppler frequency TDL-D 30ns delay spread, 35Hz Doppler frequency Static (AWGN) |
| MCS | 64QAM: MCS 23, 24, 26, 28 in TS 38.214 Table 5.1.3.1-1, and other MCSs are not precluded 256QAM: MCS 21, 23, 25, 27 in TS 38.214 Table 5.1.3.1-2, and other MCSs are not precluded Baseline: fixed MCSs |
| Precoding | Precoding configuration defined in 38.101-4 Section 7.2 for fading channels and Section 7.5 for static channel; follow PMI |
| Symbol type | CP-OFDM |
| HARQ | 8, None |
| Antenna configuration | Fading channel: 2x2 for Rank1 and Rank2, Low correlation Static channel: 1x2 for Rank1, 2x2 for Rank2 |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PDSCH configuration | Type A mapping, Start symbol 1, Duration 13 (for D slots) |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| PTRS configuration | KPTRS : 2 (every 2 RBs), LPTRS : 1 (every 1 symbol) |
| Phase noise compensation | Practical based on PTRS |
| Phase noise model | TR 38.803 model (in section 6.1.10 and section 6.1.11) modelled Phase noise for TX and RX Option a): example1 (BS) + example1(UE) Option b): example2 (BS) + example2(UE) Option c): example2 (BS) + example2(BS) Option d):example2 (BS) + PN model config1: example1(UE) Option e): Other phase noise models, e.g. ones extracted from commercially available components or published results, are not excluded |
| txEVM + rxEVM excluding phase noise for 256QAM | txEVM: [1.0%-5.0%], rxEVM: [1.0%-5.0%] Option 1: txEVM <= rxEVM; Option2: no restriction |
| Other parameters | follow assumptions in 38.101-4 Section 7.2 for fading channels (e.g., case 2-6) and Section 7.5 for static channels |

The assumptions adopted by each company are shown as following table 5.2.1.1-2 which are down-selected based on the table 5.2.1.1-1.

Table 5.2.1.1-2 link level simulation assumptions down-selected by companies

| Parameter |  |  |  | CTC[5] | Nokia[6][15] | Docomo[7] | Huawei[8] | Ericsson[9] | CATT[10] | Intel[11] | Qualcomm[12] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency |  |  |  | 29 GHz | 29 GHz | 29 GHz | 29 GHz |  | 29 GHz |  |  |
| CBW |  |  |  | 50MHz | 100MHz | 100MHz | 50MHz | 50 MHz | 50MHz | 50MHz | 100MHz |
| SCS |  |  |  | 120kHz | 60kHz | 120kHz | 120kHz | 60kHz | 60kHz | 60kHz | 120kHz |
| Allocated RBs |  |  |  |  |  |  |  |  |  |  |  |
| Propagation |  |  | TDL-A |  |  |  |  |  |  |  |  |
|  |  |  | TDL-D |  |  |  |  |  |  |  |  |
|  |  |  | Static |  |  |  |  |  |  |  |  |
| MCS |  |  | 64QAM | 28 | 26,28 | 24,25,26,28 | 23,24,26,28 | 23,24,26,28 | 23 | 23,24,26,28 | 26,27,28 |
|  |  |  | 256QAM | 23,27 | 21,23,25,27 | 21,23,25,27 | 21,23,25,27 | 21,23,25,27 | 21 | 21,23,25,27 | 20,21,22 |
| Precoding |  |  |  |  |  |  |  |  |  |  |  |
| Symbol type |  |  |  |  |  |  |  |  |  |  |  |
| HARQ |  |  |  | None | None | None |  |  | 8 | 8 | 8 |
| Antenna configuration |  | Fading |  | 2x2 for Rank1 | 2x2 for Rank1 | 2x2 for Rank2 |  | 1x2, 2x2 for Rank1 | 2x2 for Rank1 |  | 2x2 for Rank2 |
|  |  | Static |  | 1x2 for Rank1 |  | 2x2 for Rank2 |  |  | 1x2 for Rank1 |  | 2x2 for Rank2 |
| Channel estimation |  |  |  |  |  | Practical for AWGN Ideal for TDL-D |  |  |  |  |  |
| Receiver type |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH configuration |  |  |  |  |  |  |  |  |  |  |  |
| DMRS configuration |  |  |  |  |  |  |  | No additional |  |  |  |
| PTRS configuration |  |  |  | None |  |  |  |  |  |  |  |
| Phase noise compensation |  |  |  | None |  | Ideal |  |  |  |  |  |
| Phase noise model | Option a) |  |  |  |  |  |  |  |  |  |  |
|  | Option b) |  |  |  |  |  |  |  |  |  |  |
|  | Option c) |  |  |  |  |  |  |  |  |  |  |
|  | Option d) |  |  |  |  |  |  |  |  |  |  |
|  | Option e) |  |  |  |  |  |  |  |  |  | example1BS+example2UE internal PN model |
| txEVM + rxEVM excluding phase noise for 256QAM |  |  |  | Tx+Rx: 3%, 4% | txEVM: 2%-3%, rxEVM: 2%-5% | txEVM: 0%, 2%, 3%, rxEVM: 0%, 2%, 3% | txEVM: 1%-3%, rxEVM: 1%-3% |  |  | txEVM: 1%-3%, rxEVM: 1%-3% | Tx:3% Rx:internal |
| Other parameters |  |  |  |  |  |  |  |  |  |  |  |
| Note: The symbol of means selecting the parameters corresponding to table 5.2.1.1-1. |  |  |  |  |  |  |  |  |  |  |  |

--------------Omitted unchanged sections-------------

5.2.1.4	Results from NTT DOCOMO [7]

Figure 5.2.1.4-1 and 5.2.1.4-2 show link level simulation results compared between 64QAM and 256QAM. On static channel and TDL-D with antenna configuration 2x2 and Rank 2, with  3% Tx EVM and 3% Rx EVM, the performance gain compared to 64QAM modulation is obtained over 21dB and 25dB SNR, respectively.

a) Tx EVM 0% and Rx EVM 0%						 b) Tx EVM 2% and Rx EVM 2%

c) Tx EVM 3% and Rx EVM 3%

Figure 5.2.1.4-1: Simulation results on static channel

a) Tx EVM 0% and Rx EVM 0%						 b) Tx EVM 2% and Rx EVM 2%

c) Tx EVM 3% and Rx EVM 3%

Figure 5.2.1.4-2: Simulation results on TDL-D channel

--------------Omitted unchanged sections-------------

5.2.2.5	Results from NTT DOCOMO

Figure 5.2.2.5-1 shows the system level simulation result [7]. From the result, around 30% of UEs can achieve more than 25dB SINR.

Figure 5.2.2.5-1: System level simulation results

--------------End of text proposal-------------