3GPP TSG-RAN WG4 Meeting #93	R4-1914095

Reno, US, 18th– 22nd November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | 0091 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.104: Introduction of NR PUSCH performance requirements for HST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST-Perf |  |  |  |  |  | Date: |  |  | 2019-11-8 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR is to introduce PUSCH requirements under high speed train conditions assuming a UE velocity of up to 350km/h. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add new section for PUSCH performance under high speed train conditions assuming a UE velocity up to 350km/h - Add HST scenario 1-NR350 and scenario 3-NR350 to Annex G.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PUSCH performance under high speed train conditions assuming a UE velocity of up to 350km/h is not ensured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4, 8.2.5, 11.2.1.4, 11.2.1.5, G.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.141-1, TS 38.141-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------Start of text proposal-------------

8.2.4	Requirements for PUSCH with transform precoding disabled for high speed train

8.2.4.1	General

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ retransmissions. The performance requirements for high speed train are optional.

Table: 8.2.4.1-1 Test parameters for testing PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Uplink-downlink allocation for TDD |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | Pos2 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |

8.2.4.2	Minimum requirements

The throughput shall be equal to or larger than the fraction of maximum throughput for the FRCs stated in tables 8.2.4.2-1 to 8.2.4.2-4 at the given SNR for 1Tx. FRCs are defined in annex A.

Table 8.2.4.2-1: Minimum requirements for PUSCH, Type A, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | TBD | Normal | HST Scenario 1-NR350 | 70 % | TBD | pos2 | TBD |
|  | TBD | Normal | HST HST Scenario 3-NR350 | 70 % | TBD | pos2 | TBD |

Table 8.2.4.2-2: Minimum requirements for PUSCH, Type A, 10 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | TBD | Normal | HST Scenario 1-NR350 | 70 % | TBD | pos2 | TBD |
|  | TBD | Normal | HST HST Scenario 3-NR350 | 70 % | TBD | pos2 | TBD |

Table 8.2.4.2-3: Minimum requirements for PUSCH, Type A, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | TBD | Normal | HST Scenario 1-NR350 | 70 % | TBD | pos2 | TBD |
|  | TBD | Normal | HST Scenario 3-NR350 | 70 % | TBD | pos2 | TBD |

Table 8.2.4.2-4: Minimum requirements for PUSCH, Type A, 40 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | TBD | Normal | HST Scenario 1-NR350 | 70 % | TBD | pos2 | TBD |
|  | TBD | Normal | HST Scenario 3-NR350 | 70 % | TBD | pos2 | TBD |

8.2.5	Requirements for PUSCH with transform precoding enabled for high speed train

Editor's note: Requirement for PUSCH with transform precoding enabled under HST will be added lator.

--------------Omitted unchanged sections-------------

11.2.1.4	Requirements for PUSCH with transform precoding disabled for high speed train

Apply the requirements defined in subclause 8.2.4 for 2Rx.

11.2.1.5	Requirements for PUSCH with transform precoding enabled for high speed train

Apply the requirements defined in subclause 8.2.5 for 2Rx.

--------------Omitted unchanged sections-------------

G.3	High speed train condition

High speed train condition is as follow:

Scenario 1-NR350: Open space

Scenario 3-NR350: Tunnel for multi-antennas

The high speed train condition for the test of the baseband performance is two non-fading propagation channels. For BS with Rx diversity defined in scenario 3, the Doppler shift variation is the same between antennas.

Doppler shift for both scenarios is given by:

(G.3.1)

where  is the Doppler shift and  is the maximum Doppler frequency. The cosine of angle is given by:

, 	(G.3.2)

,                            (G.3.3)

,                            (G.3.4)

where  is the initial distance of the train from BS, and  is BS-Railway track distance, both in meters;  is the velocity of the train in m/s,  is time in seconds.

Doppler shift and cosine angle is given by equation G.3.1 and G.3.2-G.3.4 respectively, where the required input parameters listed in table G.3-1. and the resulting Doppler shift shown in Figure G.3-1, G.3-2, G.3-3 and G.3-4 are applied for all frequency bands.

Table G.3-1: Parameters for high speed train conditions

| Parameter | Value |  |
| --- | --- | --- |
|  | Scenario 1-NR350 | Scenario 3-NR350 |
|  | 700 m | 300 m |
|  | 150 m | 2 m |
|  | 350 km/h | 350 km/h |
|  | 1340 Hz for 15kHz SCS 2334 Hz for 30kHz SCS | 1340 Hz for 15kHz SCS 2334 Hz for 30kHz SCS |

NOTE1:	Parameters for HST conditions in table G.3-1 including  and Doppler shift trajectories presented on figures G.3-1 , G.3-2, G.3-3 and G.3-4 were derived from Band n1 for 15 kHz SCS and Band n77 for 30kHz SCS, and are applied for performance verification in all frequency bands.

Figure G.3-1: Doppler shift trajectory for scenario 1-NR350 (15 kHz SCS)

Figure G.3-2: Doppler shift trajectory for scenario 3-NR350 (15 kHz SCS)

Figure G.3-3: Doppler shift trajectory for scenario 1-NR350 (30 kHz SCS)

Figure G.3-4: Doppler shift trajectory for scenario 3-NR350 (30 kHz SCS)

--------------End of text proposal-------------