3GPP TSG-RAN WG4 Meeting #93                	R4-1914177

Reno, USA, November 18th – 22nd, 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	Intra-frequency measurement requirements in NR-U

Agenda item:	9.1.4.9

Document for:	Discussion

# 1	Introduction

The following was agreed in RAN4 #92bis:

- The following L-max values are to be decided in RAN4#93 for intra-frequency measurement requirements

- Other DRX ranges are not precluded

| Procedure | Rel-15 samples | Maximum number of DL LBT failures |  |  |
| --- | --- | --- | --- | --- |
|  |  | Parameter name | Parameter value | Condition |
| PSS/SSS detection, no gaps | 5 | LPSS/SSS,max | TBD | Max(TDRX,TSMTC)≤40 |
|  |  |  | TBD | 40<Max(TDRX,TSMTC)≤320 |
|  |  |  | TBD | TDRX>320 |
| PSS/SSS detection for deactivated SCell, no gaps | 5 | LPSS/SSS,deact,max | TBD | Max(TDRX, measCycleSCell)≤40 |
|  |  |  | TBD | 40< Max(TDRX, measCycleSCell)≤320 |
|  |  |  | TBD | TDRX>320 |
| Time index detection, no gaps | 3 | Lind,max | TBD | Max(TDRX, TSMTC)≤40 |
|  |  |  | TBD | 40< Max(TDRX, TSMTC)≤320 |
|  |  |  | TBD | TDRX>320 |
| Time index detection for deactivated SCell, no gaps | 3 | Lind,deact,max | TBD | Max(TDRX, measCycleSCell)≤40 |
|  |  |  | TBD | 40< Max(TDRX, measCycleSCell)≤320 |
|  |  |  | TBD | TDRX>320 |
| Measurement, no gaps | 5 | Lmeas,max | TBD | Max(TDRX, TSMTC)≤40 |
|  |  |  | TBD | 40< Max(TDRX, TSMTC)≤320 |
|  |  |  | TBD | TDRX>320 |
| Measurement on deactivated SCell, no gaps | 5 | Lmeas,deact,max | TBD | Max(TDRX, measCycleSCell)≤40 |
|  |  |  | TBD | 40< Max(TDRX, measCycleSCell)≤320 |
|  |  |  | TBD | TDRX>320 |
| PSS/SSS detection, with gaps | 5 | LPSS/SSS,gaps,max | TBD | Max(TDRX,TSMTC, MGRP)≤40 |
|  |  |  | TBD | 40<Max(TDRX,TSMTC, MGRP)≤320 |
|  |  |  | TBD | TDRX>320 |
| Time index detection, with gaps | 3 | Lind,gaps,max | TBD | Max(TDRX, TSMTC, MGRP)≤40 |
|  |  |  | TBD | 40< Max(TDRX, TSMTC, MGRP)≤320 |
|  |  |  | TBD | TDRX>320 |
| Measurement, with gaps | 5 | Lmeas,gaps,max | TBD | Max(TDRX, TSMTC, MGRP)≤40 |
|  |  |  | TBD | 40< Max(TDRX, TSMTC, MGRP)≤320 |
|  |  |  | TBD | TDRX>320 |

- In this contribution we discuss the maximum extension of the intra-frequency cell identification, time index detection and measurement periods for NR-U. Once agreement has been reached for intra-frequency a similar approach most likely can be adopted to inter-frequency NR-U measurements.

2	Discussion

- One important difference between NR-U and LAA is that for NR-U the cell detection and measurement period also apply for the serving PCell and PSCell, while for LAA the requirements only apply for SCells: for NR-U the latencies are more important and will have direct impact on the UE connectivity and mobility. On the other hand, in NR-U, in each DRS transmission window, multiple occasions for QCLed beams can be configured. In this case, an LBT failure would be result of failure in all these opportunities. Therefore, a complete LBT failure is less probable, if the multiple transmission occasions within the DRS TX window are considered.

In NR-U, in each DRS transmission window, multiple occasions for QCLed beams can be configured. In this case, an LBT failure would be result of failure in all these opportunities.

- In NR-U, the UE shall monitor the DRS transmission window until it detects at least one SSB. Only when the entire DRS transmission window is monitored, and no SSB is detected, the UE can determine that the SSB is not available.

The approach defined in last two RAN4 meetings to deal with missed LBT samples in NR-U follows the same approach defined for LTE-LAA:

- Extending the measurement or the cell identification periods.

As agreed in last meeting, a maximum extension of the cell identification or measurement periods needs to be defined, since the network needs to have an estimate of when to expect the UE to report measurements, while not decreasing the probability of having valid measurements even under LBT failure.

By defining a short extension of the measurement / cell identification periods, the UE might not be able to complete the measurement due to LBT failure.

By defining a large extension of the measurement / cell identification periods, in the worst case, the LBT failures can be consecutive, leading to a long time between two valid measurements.

As agreed in last meeting, the extension might be different for different SSB periodicities, or DRX cycles. In order to evaluate the extension of the cell identification time in each case, Table 1 shows a set of examples considering L PSS/SSS max = 0, 2, 5, 10, 15 and 20. For short SMTC periodicities, 20 ms or 40ms, for example,  the cell identification time does not vary significantly disregarding the chosen maximum value for LPSS/SSS, therefore, larger values of L PSS/SSS max are still acceptable. As the SMTC period increases, the measurement time increases, as well as the period between valid samples, in case the LBT failure is consecutive.

Table 1- Maximum cell identification time considering different values of L PSS/SSS max

|  | max(DRX,SMTC) | LPSS/SSS = 0 | LPSS/SSS = 2 | LPSS/SSS = 5 | LPSS/SSS = 10 | LPSS/SSS = 15 | LPSS/SSS = 20 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No DRX | 20 ms | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 |
|  | 40 ms | 0.6 | 0.6 | 0.6 | 0.6 | 0.8 | 1 |
|  | 80 ms | 0.6 | 0.6 | 0.8 | 1.2 | 1.6 | 2 |
|  | 160 ms | 0.8 | 1.12 | 1.6 | 2.4 | 3.2 | 4 |
| DRX | 40 ms | 0.6 | 0.6 | 0.6 | 0.92 | 1.2 | 1.52 |
|  | 80 ms | 0.64 | 0.88 | 1.2 | 1.84 | 2.4 | 3.04 |
|  | 160 ms | 1.28 | 1.76 | 2.4 | 3.68 | 4.8 | 6.08 |
|  | 320 ms | 2.56 | 3.52 | 4.8 | 7.36 | 9.6 | 12.16 |
|  | 640 ms | 3.2 | 4.48 | 6.4 | 9.6 | 12.8 | 16 |
|  | 1.28 s | 6.4 | 8.96 | 12.8 | 19.2 | 25.6 | 32 |
|  | 2.56 s | 12.8 | 17.92 | 25.6 | 38.4 | 51.2 | 64 |

In order to avoid a very long time between valid SSBs, while maximizing the probability that the UE gets valid samples within the measurement period even under high LBT failure probability, there are two options:

- Limiting the extension so that even in the worst case the time between valid SSBs is within reasonable limits.

- Introduce an additional condition, to limit the number of consecutive failures.

If the first option is chosen, and the maximum period between valid SSBs is chosen as 5s, for example, considering the 2.56 s DRX cycle, Lin_max needs to equal to 1. This will certainly decrease the probability of the UE getting enough samples within the measurement / evaluation period. Therefore, the second option seems to allow for more flexibility than the first option.

Introduce a new criterion to limit the maximum acceptable period between two available SSBs within the measurement or cell identification period.

For longer DRX cycles > 640 ms, an additional criterion applies, to limit the acceptable period of consecutive LBT failures to [X] seconds.

- Additionally, to maximize the probability of receiving SSBs within the measurement period even under high probability of LBT failure, while keeping the measurement period within reasonable limits, it is proposed that the maximum extension of the measurement / cell identification period is defined as the one leading to, at most, two times the baseline period.

Define the extensions for intra-frequency measurement as:

| Procedure | Rel-15 samples | Maximum number of DL LBT failures |  |  |
| --- | --- | --- | --- | --- |
|  |  | Parameter name | Parameter value | Condition |
| PSS/SSS detection, no gaps | 5 | LPSS/SSS,max | 20 | Max(TDRX,TSMTC)≤40 |
|  |  |  | 10 | 40<Max(TDRX,TSMTC)≤80 |
|  |  |  | 5 | 80<Max(TDRX,TSMTC)≤640 |
|  |  |  | 5NOTE 1 | TDRX>640 |
| PSS/SSS detection for deactivated SCell, no gaps | 5 | LPSS/SSS,deact,max | 5 | Max(TDRX, measCycleSCell)≤640 |
|  |  |  | 5 NOTE 1 | 640 < Max(TDRX, measCycleSCell) |
| Time index detection, no gaps | 3 | Lind,max | 9 | Max(TDRX, TSMTC)≤40 |
|  |  |  | 3 | 40< Max(TDRX, TSMTC)≤1280 |
|  |  |  | 3 NOTE 1 | TDRX>1280 |
| Time index detection for deactivated SCell, no gaps | 3 | Lind,deact,max | 3 | Max(TDRX, measCycleSCell)≤640 |
|  |  |  | 3 NOTE 1 | 640 < Max(TDRX, measCycleSCell) |
| Measurement, no gaps | 5 | Lmeas,max | 15 | Max(TDRX, TSMTC)≤20 |
|  |  |  | 5 | 20< Max(TDRX, TSMTC)≤640 |
|  |  |  | 5 NOTE 1 | 640< Max(TDRX, TSMTC)≤1280 |
| Measurement on deactivated SCell, no gaps | 5 | Lmeas,deact,max | 5 | Max(TDRX, measCycleSCell)≤640 |
|  |  |  | 5 NOTE 1 | 640 < Max(TDRX, measCycleSCell) |
| PSS/SSS detection, with gaps | 5 | LPSS/SSS,max | 20 | Max(TDRX, MGRP, TSMTC)≤40 |
|  |  |  | 10 | 40<Max(TDRX, MGRP, TSMTC)≤80 |
|  |  |  | 5 | 80<Max(TDRX, MGRP, TSMTC)≤640 |
|  |  |  | 5 NOTE 1 | TDRX>640 |
| PSS/SSS detection for deactivated SCell, with gaps | 5 | LPSS/SSS,deact,max | 5 | Max(TDRX, MGRP, measCycleSCell)≤640 |
|  |  |  | 5 NOTE 1 | 640 < Max(TDRX, MGRP, measCycleSCell) |
| Time index detection, with gaps | 3 | Lind,max | 9 | Max(TDRX, MGRP, TSMTC)≤40 |
|  |  |  | 3 | 40< Max(TDRX, MGRP, TSMTC)≤1280 |
|  |  |  | 3 NOTE 1 | TDRX>1280 |
| NOTE 1: If the LBT failures are contiguous, the acceptable number of LBT failures is selected as the maximum value of failures within a X seconds period. |  |  |  |  |

3 Conclusions

- In NR-U, in each DRS transmission window, multiple occasions for QCLed beams can be configured. In this case, an LBT failure would be result of failure in all these opportunities.

- In NR-U, the UE shall monitor the DRS transmission window until it detects at least one SSB. Only when the entire DRS transmission window is monitored, and no SSB is detected, the UE can determine that the SSB is not available.

By defining a short extension of the measurement / cell identification periods, the UE might not be able to complete the measurement due to LBT failure.

By defining a large extension of the measurement / cell identification periods, in the worst case, the LBT failures can be consecutive, leading to a long time between two valid measurements.

In order to avoid a very long time between valid SSBs, while maximizing the probability that the UE gets valid samples within the measurement period even under high LBT failure probability, there are two options:

- Limiting the extension so that even in the worst case the time between valid SSBs is within reasonable limits.

- Introduce an additional condition, to limit the number of consecutive failures.

Introduce a new criterion to limit the maximum acceptable period between two available SSBs within the measurement or cell identification period.

For longer DRX cycles > 640 ms, an additional criterion applies, to limit the acceptable period of consecutive LBT failures to [X] seconds.

Define the extensions for intra-frequency measurement as:

| Procedure | Rel-15 samples | Maximum number of DL LBT failures |  |  |
| --- | --- | --- | --- | --- |
|  |  | Parameter name | Parameter value | Condition |
| PSS/SSS detection, no gaps | 5 | LPSS/SSS,max | 20 | Max(TDRX,TSMTC)≤40 |
|  |  |  | 10 | 40<Max(TDRX,TSMTC)≤80 |
|  |  |  | 5 | 80<Max(TDRX,TSMTC)≤640 |
|  |  |  | 5NOTE 1 | TDRX>640 |
| PSS/SSS detection for deactivated SCell, no gaps | 5 | LPSS/SSS,deact,max | 5 | Max(TDRX, measCycleSCell)≤640 |
|  |  |  | 5 NOTE 1 | 640 < Max(TDRX, measCycleSCell) |
| Time index detection, no gaps | 3 | Lind,max | 9 | Max(TDRX, TSMTC)≤40 |
|  |  |  | 3 | 40< Max(TDRX, TSMTC)≤1280 |
|  |  |  | 3 NOTE 1 | TDRX>1280 |
| Time index detection for deactivated SCell, no gaps | 3 | Lind,deact,max | 3 | Max(TDRX, measCycleSCell)≤640 |
|  |  |  | 3 NOTE 1 | 640 < Max(TDRX, measCycleSCell) |
| Measurement, no gaps | 5 | Lmeas,max | 15 | Max(TDRX, TSMTC)≤20 |
|  |  |  | 5 | 20< Max(TDRX, TSMTC)≤640 |
|  |  |  | 5 NOTE 1 | 640< Max(TDRX, TSMTC)≤1280 |
| Measurement on deactivated SCell, no gaps | 5 | Lmeas,deact,max | 5 | Max(TDRX, measCycleSCell)≤640 |
|  |  |  | 5 NOTE 1 | 640 < Max(TDRX, measCycleSCell) |
| PSS/SSS detection, with gaps | 5 | LPSS/SSS,max | 20 | Max(TDRX, MGRP, TSMTC)≤40 |
|  |  |  | 10 | 40<Max(TDRX, MGRP, TSMTC)≤80 |
|  |  |  | 5 | 80<Max(TDRX, MGRP, TSMTC)≤640 |
|  |  |  | 5 NOTE 1 | TDRX>640 |
| PSS/SSS detection for deactivated SCell, with gaps | 5 | LPSS/SSS,deact,max | 5 | Max(TDRX, MGRP, measCycleSCell)≤640 |
|  |  |  | 5 NOTE 1 | 640 < Max(TDRX, MGRP, measCycleSCell) |
| Time index detection, with gaps | 3 | Lind,max | 9 | Max(TDRX, MGRP, TSMTC)≤40 |
|  |  |  | 3 | 40< Max(TDRX, MGRP, TSMTC)≤1280 |
|  |  |  | 3 NOTE 1 | TDRX>1280 |
| NOTE 1: If the LBT failures are contiguous, the acceptable number of LBT failures is selected as the maximum value of failures within a [X] seconds period. |  |  |  |  |

References

- R4-1912851, “WF on RRM Requirements for NR-U”, Ericsson, October 2019.

- R4-79AH-0007. “On preventing infinite measurement and cell identification time in LAA”, Nokia

- R1-1910614, “On Enhancements to Initial Access Procedures for NR-U”, Nokia, Nokia Shanghai Bell.

- R4-165299, “Known cell requirements for LAA Rel-14”, Ericsson

Appendix 1: Examples of maximum cell identification and measurement times

Table 2 – Examples of maximum cell identification time for deactivated SCell, considering CSSF = 1.

|  | max( measCycleScell, DRX) | LPSS/SSS = 0 | LPSS/SSS = 2 | LPSS/SSS = 5 | LPSS/SSS = 10 | LPSS/SSS = 15 | LPSS/SSS = 20 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No DRX | 160 ms | 0.8 | 1.12 | 1.6 | 2.4 | 3.2 | 4 |
|  | 256 ms | 1.28 | 1.792 | 2.56 | 3.84 | 5.12 | 6.4 |
|  | 320 ms | 1.6 | 2.24 | 3.2 | 4.8 | 6.4 | 8 |
|  | 512 ms | 2.56 | 3.584 | 5.12 | 7.68 | 10.24 | 12.8 |
|  | 640 ms | 3.2 | 4.48 | 6.4 | 9.6 | 12.8 | 16 |
|  | 1280 ms | 6.4 | 8.96 | 12.8 | 19.2 | 25.6 | 32 |
| DRX | 160 ms | 1.2 | 1.68 | 2.4 | 3.6 | 4.8 | 6 |
|  | 256 ms | 1.92 | 2.688 | 3.84 | 5.76 | 7.68 | 9.6 |
|  | 320 ms | 2.4 | 3.36 | 4.8 | 7.2 | 9.6 | 12 |
|  | 640 ms | 3.2 | 4.48 | 6.4 | 9.6 | 12.8 | 16 |
|  | 1.28 s | 6.4 | 8.96 | 12.8 | 19.2 | 25.6 | 32 |
|  | 2.56 s | 12.8 | 17.92 | 25.6 | 38.4 | 51.2 | 64 |

| Top of Form Maximum TPSS/SSS_sync_intra_NR_U [s] Bottom of Form |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No DRX |  |  |  |  |  |  |
| measCycleSCell | LPSS/SSS = 0 | LPSS/SSS = 1 | LPSS/SSS = 2 | LPSS/SSS = 5 | LPSS/SSS = 12 | LPSS/SSS = 15 |
| 160 ms | 0.8 | 0.96 | 1.12 | 1.6 | 2.72 | 3.2 |
| 256 ms | 1.28 | 1.536 | 1.792 | 2.56 | 4.352 | 5.12 |
| 320 ms | 1.6 | 1.92 | 2.24 | 3.2 | 5.44 | 6.4 |
| 512 ms | 2.56 | 3.072 | 3.584 | 5.12 | 8.704 | 10.24 |
| 640 ms | 3.2 | 3.84 | 4.48 | 6.4 | 10.88 | 12.8 |
| 1280 ms | 6.4 | 7.68 | 8.96 | 12.8 | 21.76 | 25.6 |
| DRX cycle= 320ms |  |  |  |  |  |  |
| ≤ 320 ms | 1.6 | 1.92 | 2.24 | 3.2 | 5.44 | 6.4 |
| 512 ms | 4.096 | 4.608 | 5.632 | 7.68 | 13.312 | 15.36 |
| 640 ms | 5.12 | 5.76 | 7.04 | 9.6 | 16.64 | 19.2 |
| 1280 ms | 10.24 | 11.52 | 14.08 | 19.2 | 33.28 | 38.4 |
| DRX cycle= 2560 ms |  |  |  |  |  |  |
| all | 12.8 | 15.36 | 17.92 | 25.6 | 43.5 | 51.2 Bottom of Form |

Table 3 – Examples of maximum T SSB_measurement_period_intra_NR_U  considering CSSF = 1.

| Top of Form Maximum T SSB_measurement_period_intra_NR_U [s] Bottom of Form |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No DRX |  |  |  |  |  |  |
| DRS periodicity | Lmeas = 0 baseline | Lmeas = 1 | Lmeas = 2 | Lmeas = 5 | Lmeas = 12 | Lmeas = 15 |
| 20 ms | 0.2 | 0.2 | 0.2 | 0.2 | 0.34 | 0.4 |
| 40 ms | 0.2 | 0.24 | 0.28 | 0.4 | 0.68 | 0.8 |
| 80 ms | 0.4 | 0.48 | 0.56 | 0.8 | 1.36 | 1.6 |
| 160 ms | 0.8 | 0.96 | 1.12 | 1.6 | 2.72 | 3.2 |
| DRX cycle= 320ms |  |  |  |  |  |  |
| all | 2.56 | 2.88 | 3.52 | 4.8 | 8.32 | 9.6 |
| DRX cycle= 2560 ms |  |  |  |  |  |  |
| all | 12.8 | 15.36 | 17.92 | 25.6 | 43.5 | 51.2 Bottom of Form |