3GPP TSG-RAN WG4 Meeting #93                	R4- 1914178

Reno, USA, November 18th – 22nd, 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	Inter-frequency measurement requirements in NR-U

Agenda item:	9.1.4.9

Document for:	Discussion

# 1	Introduction

The following was agreed in RAN4 #92bis:

- The following L-max values are to be decided in RAN4#93 for inter-frequency measurement requirements

- Other DRX ranges are not precluded

In this contribution we discuss the maximum extension of the inter-frequency cell identification, time index detection and measurement periods for NR-U.

2	Discussion

- One important difference between NR-U and LAA is that for NR-U the cell detection and measurement period also apply for the serving PCell and PSCell, while for LAA the requirements only apply for SCells: for NR-U the latencies are more important and will have direct impact on the UE connectivity and mobility. On the other hand, in NR-U, in each DRS transmission window, multiple occasions for QCLed beams can be configured. In this case, an LBT failure would be result of failure in all these opportunities. Therefore, a complete LBT failure is less probable, if the multiple transmission occasions within the DRS TX window are considered.

In NR-U, in each DRS transmission window, multiple occasions for QCLed beams can be configured. In this case, an LBT failure would be result of failure in all these opportunities.

- In NR-U, the UE shall monitor the DRS transmission window until it detects at least one SSB. Only when the entire DRS transmission window is monitored, and no SSB is detected, the UE can determine that the SSB is not available.

The approach defined in last two RAN4 meetings to deal with missed LBT samples in NR-U follows the same approach defined for LTE-LAA:

- Extending the measurement or the cell identification periods.

As agreed in last meeting, a maximum extension of the cell identification or measurement periods needs to be defined, since the network needs to have an estimate of when to expect the UE to report measurements, while not decreasing the probability of having valid measurements even under LBT failure.

By defining a short extension of the measurement / cell identification periods, the UE might not be able to complete the measurement due to LBT failure.

By defining a large extension of the measurement / cell identification periods, in the worst case, the LBT failures can be consecutive, leading to a long time between two valid measurements.

As agreed in last meeting, the extension might be different for different SMTC periodicities, or DRX cycles. In order to evaluate the extension of the cell identification time in each case, Table 1 shows a set of examples considering L PSS/SSS max = 0, 3, 8, 16, 24 and 32. For short a SMTC period, such as 20 ms, for example,  the cell identification time does not vary significantly disregarding the chosen maximum value for LPSS/SSS, therefore, larger values of L PSS/SSS max are still acceptable. As the SMTC/DRX/MGRP period increases, the measurement time increases, as well as the period between valid samples, in case the LBT failures are consecutive, as observed in [2].

Table 1- Maximum cell identification time considering different values of L PSS/SSS max

|  | max(DRX,SMTC) ms | LPSS/SSS = 0 | LPSS/SSS = 3 | LPSS/SSS = 8 | LPSS/SSS = 16 | LPSS/SSS = 24 | LPSS/SSS = 32 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No DRX | 20 | 0.6 | 0.6 | 0.6 | 0.6 | 0.64 | 0.8 |
|  | 40 | 0.6 | 0.6 | 0.64 | 0.96 | 1.28 | 1.6 |
|  | 80 | 0.64 | 0.88 | 1.28 | 1.92 | 2.56 | 3.2 |
|  | 160 | 1.28 | 1.76 | 2.56 | 3.84 | 5.12 | 6.4 |
| DRX | 40 | 0.6 | 0.66 | 0.96 | 1.44 | 1.92 | 2.4 |
|  | 80 | 0.96 | 1.32 | 1.92 | 2.88 | 3.84 | 4.8 |
|  | 160 | 1.92 | 2.64 | 3.84 | 5.76 | 7.68 | 9.6 |
|  | 320 | 3.84 | 5.28 | 7.68 | 11.52 | 15.36 | 19.2 |
|  | 640 | 5.12 | 7.04 | 10.24 | 15.36 | 20.48 | 25.6 |
|  | 1280 | 10.24 | 14.08 | 20.48 | 30.72 | 40.96 | 51.2 |
|  | 2560 | 20.48 | 28.16 | 40.96 | 61.44 | 81.92 | 102.4 |

To avoid to unnecessarily limiting the extension of the cell identification or measurement times by the number of consecutive LBT failures, it is proposed to:

Introduce a new criterion to limit the maximum acceptable period between two available SSBs within the measurement or cell identification period.

For longer DRX cycles > 640 ms, an additional criterion applies, to limit the acceptable period of consecutive LBT failures to [X] seconds.

- Additionally, to maximize the probability of receiving SSBs within the measurement period even under high probability of LBT failure, while keeping the measurement period within reasonable limits, it is proposed that the maximum extension of the measurement / cell identification period is defined as the one leading to, at most, two times the baseline period.

Define the extensions for inter-frequency measurement as:

| Procedure | Rel-15 samples | Maximum number of DL LBT failures |  |  |
| --- | --- | --- | --- | --- |
|  |  | Parameter name | Parameter value | Condition |
| PSS/SSS detection, gaps | 8 | LPSS/SSS,gaps,max | 32 | Max(TSMTC,MGRP)=20, no DRX |
|  |  |  | 16 | Max(TSMTC,MGRP)=40, no DRX |
|  |  |  | 8 NOTE 1 | Max(TDRX,TSMTC,MGRP)>20 |
| Time index detection, gaps | 3 | Lind,gaps,max | 9 | Max(TDRX, TSMTC,MGRPG)≤40 |
|  |  |  | 3 NOTE 1 | Max(TDRX, TSMTC,MGRPG)>40 |
| Measurement, gaps | 8 | Lmeas,gaps max | 8 NOTE 1 | Max(TDRX, TSMTC,MGRPG)>20 |
| NOTE 1: If the LBT failures are contiguous, the acceptable number of LBT failures is selected as the maximum value of failures within a X seconds period. |  |  |  |  |

3 Conclusion

- In NR-U, in each DRS transmission window, multiple occasions for QCLed beams can be configured. In this case, an LBT failure would be result of failure in all these opportunities.

- In NR-U, the UE shall monitor the DRS transmission window until it detects at least one SSB. Only when the entire DRS transmission window is monitored, and no SSB is detected, the UE can determine that the SSB is not available.

- Introduce a new criterion to limit the maximum acceptable period between two available SSBs within the measurement or cell identification period.

- For longer DRX cycles > 640 ms, an additional criterion applies, to limit the acceptable period of consecutive LBT failures to [X] seconds.

- Define the extensions for inter-frequency measurement as:

| Procedure | Rel-15 samples | Maximum number of DL LBT failures |  |  |
| --- | --- | --- | --- | --- |
|  |  | Parameter name | Parameter value | Condition |
| PSS/SSS detection, gaps | 8 | LPSS/SSS,gaps,max | 32 | Max(TSMTC,MGRP)=20, no DRX |
|  |  |  | 16 | Max(TSMTC,MGRP)=40, no DRX |
|  |  |  | 8 NOTE 1 | Max(TDRX,TSMTC,MGRP)>20 |
| Time index detection, gaps | 3 | Lind,gaps,max | 9 | Max(TDRX, TSMTC,MGRPG)≤40 |
|  |  |  | 3 NOTE 1 | Max(TDRX, TSMTC,MGRPG)>40 |
| Measurement, gaps | 8 | Lmeas,gaps max | 8 NOTE 1 | Max(TDRX, TSMTC,MGRPG)>20 |
| NOTE 1: If the LBT failures are contiguous, the acceptable number of LBT failures is selected as the maximum value of failures within a X seconds period. |  |  |  |  |

References

- R4-1912851, “WF on RRM Requirements for NR-U”, Ericsson, October 2019.

- R4-XXXXXX, “Intra-frequency measurement requirements in NR-U”, Nokia, Nokia Shanghai Bell.

Appendix 1: Examples of maximum cell identification and measurement times

Table 2 – Examples of maximum time index detection time.

|  |  | Lind,gaps,max |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | max(DRX,SMTC, gaps) ms | 0 | 1 | 3 | 6 | 9 | 12 |
| No DRX | 20 | 0.12 | 0.12 | 0.12 | 0.18 | 0.24 | 0.3 |
|  | 40 | 0.12 | 0.16 | 0.24 | 0.36 | 0.48 | 0.6 |
|  | 80 | 0.24 | 0.32 | 0.48 | 0.72 | 0.96 | 1.2 |
|  | 160 | 0.48 | 0.64 | 0.96 | 1.44 | 1.92 | 2.4 |
| DRX | 40 | 0.18 | 0.24 | 0.36 | 0.54 | 0.72 | 0.9 |
|  | 80 | 0.36 | 0.48 | 0.72 | 1.08 | 1.44 | 1.8 |
|  | 160 | 0.72 | 0.96 | 1.44 | 2.16 | 2.88 | 3.6 |
|  | 320 | 1.44 | 1.92 | 2.88 | 4.32 | 5.76 | 7.2 |
|  | 640 | 1.92 | 2.56 | 3.84 | 5.76 | 7.68 | 9.6 |
|  | 1280 | 3.84 | 5.12 | 7.68 | 11.52 | 15.36 | 19.2 |
|  | 2560 | 7.68 | 10.24 | 15.36 | 23.04 | 30.72 | 38.4 |

Table 3 – Examples of maximum T SSB_measurement_period_inter_NR_U  considering CSSF = 1.

|  |  | Lmeas,gaps,max |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | max(DRX,SMTC, gaps) ms | 0 | 3 | 8 | 16 | 24 | 32 |
| No DRX | 20 | 0.2 | 0.22 | 0.32 | 0.48 | 0.64 | 0.8 |
|  | 40 | 0.32 | 0.44 | 0.64 | 0.96 | 1.28 | 1.6 |
|  | 80 | 0.64 | 0.88 | 1.28 | 1.92 | 2.56 | 3.2 |
|  | 160 | 1.28 | 1.76 | 2.56 | 3.84 | 5.12 | 6.4 |
| DRX | 40 | 0.48 | 0.66 | 0.96 | 1.44 | 1.92 | 2.4 |
|  | 80 | 0.96 | 1.32 | 1.92 | 2.88 | 3.84 | 4.8 |
|  | 160 | 1.92 | 2.64 | 3.84 | 5.76 | 7.68 | 9.6 |
|  | 320 | 3.84 | 5.28 | 7.68 | 11.52 | 15.36 | 19.2 |
|  | 640 | 5.12 | 7.04 | 10.24 | 15.36 | 20.48 | 25.6 |
|  | 1280 | 10.24 | 14.08 | 20.48 | 30.72 | 40.96 | 51.2 |
|  | 2560 | 20.48 | 28.16 | 40.96 | 61.44 | 81.92 | 102.4 |