3GPP TSG-RAN WG4 Meeting #93	R4-1914343

Reno, NV, USA, 18 – 22 November 2019

Agenda Item:	8.10.3.3

Source:	Ericsson

Title:	RLM for enhanced MPDCCH

Document for:	Discussion

# 1	Introduction

RAN4#92bis agreed with the way forward on MPDCCH performance improvement for RLM in Rel-16 eMTC [1]:

- The evaluation period of DL radio link quality measurement is same as evaluation period for Rel-13 eMTC

We discuss the simulation assumption for improved MPDCCH based RLM according to the way forward.

# 2	Discussion

## 2.1	MPDCCH parameters

Table 1 shows the MPDCCH parameters for Rel-13 RLM (TS36.133 7.19.2 for CE Mode A and 7.19.4 for CE Mode B). With the improved MPDCCH, network configures MPDCCH uses DMRS only or DMRS+CRS. Since this configuration will shift SNR levels of Qin/Qout only, we don’t think RAN4 RLM core part is impacted by the improved MPDCCH.

Proposal 1: RLM core requirements, i.e., TS36.133 7.19, can be applicable for UE category M1 configured with improved MPDCCH.

Proposal 2: RAN4 discuss the SNR test points of RLM Qin/Qout with improved PDCCH in RRM performance part.

Table 1	MPDCCH parameters for Rel-13 RLM for CE Mode A.

| Attribute | Out-of-sync | In-sync |
| --- | --- | --- |
| DCI format | 6-1A | 6-1A |
| Starting OFDM symbols | 2; Bandwidth >= 10MHz 3; 3MHz <= Bandwidth < 10MHz 4; Bandwidth = 1.4MHz | 2; Bandwidth >= 10MHz 3; 3MHz <= Bandwidth < 10MHz 4; Bandwidth = 1.4MHz |
| Maximum M-PDCCH repetition level | Rmax Note1 | Rmax /2 Note1 |
| Aggregation level (ECCE) | L’max Note2 | L’max-2Note2 |
| M-PDCCH Transmission type | Distributed | Distributed |
| NOTE 1: Rmax is determined by the configurable parameter mPDCCH-NumRepetition defined in 36.331 and Rmax>1. NOTE 2: L’max and L’max-2 is derived from the configurable parameter numberPRB-Pairs defined in 36.331. L’max is 24, 16 and 8, if numberPRB-Pairs is 6, 4 and 2, respectively. L’max-2 is the aggregation level two levels below L’max, and L’max-2 is 8, 4 and 2, if numberPRB-Pairs is 6, 4 and 2, respectively. |  |  |

Table 2	MPDCCH parameters for Rel-13 RLM for CE Mode B.

| Attribute | Out-of-sync | In-sync |
| --- | --- | --- |
| DCI format | 6-1B | 6-1B |
| Starting OFDM symbols | 2; Bandwidth >= 10MHz 3; 3MHz <= Bandwidth < 10MHz 4; Bandwidth = 1.4MHz | 2; Bandwidth >= 10MHz 3; 3MHz <= Bandwidth < 10MHz 4; Bandwidth = 1.4MHz |
| Maximum M-PDCCH repetition level | RmaxNote1 | Rmax/2Note1 |
| Aggregation level (ECCE) | L’max Note2 | L’max-2 Note2 |
| M-PDCCH Transmission type | Distributed | Distributed |
| NOTE 1: Rmax is determined by the configurable parameter mPDCCH-NumRepetition defined in 36.331 and Rmax>1. NOTE 2: L’max and L’max-2 is derived from the configurable parameter numberPRB-Pairs defined in 36.331. L’max is 24, 16 and 8, if numberPRB-Pairs is 6, 4 and 2, respectively. L’max-2 is the aggregation levels two levels below L’max, and L’max-2 is 8, 4 and 2, if numberPRB-Pairs is 6, 4 and 2, respectively. |  |  |

In Rel-16 MPDCCH improvement, MPDCCH uses CRS as well as DMRS for reference signals. If UE will use CRS as the demodulation reference signals for MPDCCH on top of DMRS, UE needs to know the precoder and power offset between CRS and MPDCCH DMRS. For this issue RAN1 agreed the mapping between CRS ports and MPDCCH ports is based on 1) CSI-based precoder and 2) precoder cycling across time and frequency domain.

Regarding the precoding information, considering the low operation point for out-of-synch, we propose to use the cyclic precoder instead of the CSI-based precoding. Regarding the cyclic precoder, RAN1 is still discussing the used codebook in frequency domain and it is supposed to be complete in RAN1#98bis. RAN4 need to wait further for the RAN1 conclusion.

Table 3 is our proposal for MPDCCH parameters for RLM with improved MPDCCH. We reuse the sets of repetition level (RL) and aggregation level (AL) used for Rel-13 eMTC RLM test cases. For set 1 it is set to (RL, AL) = (4, 16) for OOS and (2, 4) of IS. For set 2, it is set to (RL, AL) = (8, 24) for OOS and (4, 8) for IS. However we still need to wait for RAN1 decision on precoder design.

Table 3	Proposed MPDCCH parameters for RLM with improved MPDCCH.

| Parameters | Values |
| --- | --- |
| DCI format | 6-1A (CE Mode A) 6-1B (CE Mode B) |
| System BW | 10MHz |
| Starting OFDM symbols | 2 |
| (RL, AL) | Set 1: (4, 16) for Out-of-synch and (2, 4) for In-synch Set 2: (8, 24) for Out-of-synch and (4, 8) for In-synch |
| Transmission type | Distributed |
| Channel model | AWGN, EPA5, ETU30 for CE Mode A AWGN, ETU1, EPA1 for CE Mode B |
| Antenna configuration | 2x1, 4x1 |
| Metric | In-synch: MPDCCH BLER of 2% Out-ot-synch: MPDCCH BLER of 10% |
| Frequency hopping | ON |
| Reference signal | DMRS + CRS |
| Power offset between DMRS and CRS | 0dB |
| DMRS/CRS precoding | Cyclic precoder: Precoder granularity in frequency domain: 1PRB Precoder granularity in time domain: [4] ms - Pending RAN1 agreements for the detailed design Codebook: Pending RAN1 agreements |

# 3	Summary

Proposal 1: RLM core requirements, i.e., TS36.133 7.19, can be applicable for UE category M1 configured with improved MPDCCH.

Proposal 2: RAN4 discuss the SNR test points of RLM Qin/Qout with improved PDCCH in RRM performance part.

# References

- R4-1912735, “Way forward on Rel-16 MTC RRM enhancement”, Ericsson, Huawei, HiSilicon, Qualcomm Incorporated, Nokia, Nokia Shanghai Bell.

# Appendix

## RAN1 agreements

RAN1#94 Agreement CRS for improving channel estimation on USS MPDCCH is supported Agreement The relation between the MPDCCH DMRS and CRS is defined for enabling improved channel estimation. FFS the details on specifying the relation (e.g., whether the ‘definition’ means predefined in specification or done by signalling, relation between the CRS ports and DMRS ports, power offset between DMRS-CRS). Agreement eNB enables/disables “CRS for improving channel estimation on MPDCCH” via dedicated or broadcasted RRC signaling. FFS the details on the signaling. Agreement CRS for improving channel estimation on Type0-MPDCCH CSS MPDCCH is supported. RAN1#94bis Agreement CRS for improving channel estimation on MPDCCH in idle mode is supported. FFS the details. Agreement For localized MPDCCH, the following mappings are supported between CRS and DMRS: - Predefined mapping - Mapping based on CSI report FFS the details. Agreement For distributed MPDCCH, mapping between CRS and DMRS is defined in specifications. - FFS the details (e.g. number of PRBs over which mapping applies, fixed or configurable) Agreement Power offset between CRS and DMRS antenna ports of MPDCCH is signaled by eNB. FFS the signaling details (e.g. provisioning for default power offset value). RAN1#95 Agreement For distributed MPDCCH, predefined mapping between CRS ports and MPDCCH DMRS ports is supported. Agreement For localized/distributed MPDCCH, the predefined mapping between CRS ports and MPDCCH ports is based on precoder cycling across time and frequency domain. FFS the details. Agreement Reuse existing LTE Rel-8 codebooks for precoding of MPDCCH is the starting point. - Unless performance gains are idenfied further optimization on top of Rel-8 codebook will not be introduced Agreement For UE in Connected/Idle mode, the power offset between CRS and DMRS antenna ports of MPDCCH is indicated by SIB. Agreement - For UE in Idle mode, the configuration of CRS for MPDCCH performance improvement is indicated via SIB/SI. FFS the details of configuration. - For UE in Connected mode, the configuration of CRS for MPDCCH performance improvement is indicated via UE-specific RRC signalling. FFS the details of configuration. RAN1#96 Agreement Precoding cycling in time domain with a granularity of YCH is supported. FFS whether futher values are supported in which case the granularity is configurable. Agreement For precoding cycling, the precoder set consists of [2 or 4] existing precoders for 2Tx antennas and 4 existing precoders for 4Tx antennas. FFS: The precoder set is configured or not. For further study - The order and pattern of the used precoder in time and frequency domain for precoder cycling RAN1#96bis Agreement When CSI-based precoding is configured: - The UE monitors a set of MPDCCH candidates assuming the mapping between DMRS and CRS ports is based on a previously reported PMI. - FFS details, including delay for applicability of PMI. - A fallback mechanism is supported. The details of the fallback mechanism are FFS. Agreement For precoding cycling for 2Tx antennas, predefine one precoder set which consists of 2 existing precoders. FFS: Which of the two precoder among the existing 2TX precoders are used. Agreement For precoding cycling for 4Tx antennas, predefine one precoder set which consists of 4 existing precoders. FFS: Which of the four precoder among the existing 4TX precoders are used. Agreement The precoding cycling in time domain has a granularity of YCH (granularity of frequency hopping) only. - Precoder cycling can be used even in case frequency hopping is not used as long as YCH is configured RAN1#97 Agreement When CSI-based precoding is configured, use predefined precoding cycling as the fallback mechanisms. - FFS: Details on fallback mechanism Agreement For precoder cycling for distributed MPDCCH, select one options for precoding in RAN1#98 - Option 1: Based on rank-1 precoders - Option 2: Based on rank-2 precoders Companies are encouraged to submit simulation results. Agreement Precoders with indices 0, 1 consist of the precoder set for 2Tx Agreement If n is the last subframe in which PMI is reported, the reported precoder is applied to a set of MPDCCH candidates for mapping between CRS port and MPDCCH DMRS port in subframe n+4 Agreement For precoder cycling, at least the granularity of 1 PRB in frequency domain is supported. - FFS: whether other granularities are supported or not. RAN1#98 Agreement A fixed subset of the candidates in the MPDCCH search space(s) are reserved for the precoder cycling fallback. The subset of candidates is designed in such a way that the UE complexity is taken into account for the following aspects: - Number of channel estimations - LLR storage - Blind decodes are not increased For comparable performance, the alternative with the smaller complexity will be selected. Agreement Rank-1 precoder is used for precoder cycling in distributed MPDCCH. - FFS whether and how predefined pairs of Rank-1 precoders specified. Agreement The precoder set for 4Tx is the set of rank-1 precoders with indices 12, 13, 14, and 15 Agreement For both distributed and localized MPDCCH, the granularity of precoding in the frequency domain is only 1 PRB. Agreement Select one of the following alternatives (to be done in RAN1#98bis): Precoder cycling in time domain is done - Alt1: In a pseudo-random manner - Alt2: Sequentially and in a cyclic manner RAN1#98bis Agreement Precoder cycling in time domain is done sequentially and in a cyclic manner - FFS whether it is combined with periodic pseudo random initialization Agreement Select one of the following in RAN1#99 - Alt1: For 2 Tx in the distributed transmission, fixed two precoders (0,1) are paired for every subframe and PRB. - Alt2: For 2 Tx in the distributed transmission, precoder set { (0,1), (1,0) } is used for precoder cycling Agreement Select one of the following in RAN1#99 - Alt1: The predefined pairs of rank-1 precoders {12,13} and {14,15} can be cyclically used in the frequency domain - Alt2: The predefined pairs of rank-1 precoders (12,13), (14,15), (15,12), (13,14) are used for precoder cycling in the frequency domain. Email discussion on PDCCH candidates for precoding cycling fallback by Nov 1st to be coordinated by Zheng (Huawei) - Including consideration on UE complexity