3GPP TSG-RAN WG4 Meeting #93	R4-1914488

Reno, USA, 18th – 22nd November, 2019

Title:	Draft LS on introduction of carrier specific thresholds for UE Power Saving schemes

Response to:	-

Release:	Release 16

Work Item:	NR_UE_pow_sav

Source:	Nokia  (TSG RAN WG4)

To:	TSG RAN WG2

Cc:

Contact Person:

#### Name:	Lars Dalsgaard

E-mail Address:	lars.dalsgaard@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN4 has been discussing UE power saving opportunities related to number of carriers to monitor in idle and inactive mode. Current inter-frequency and inter-RAT search thresholds (Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ) are defined such that they apply for all configured inter-frequency and inter-RAT carrier.

RAN4 has identified that such coarse applicability of the inter-frequency/RAT search thresholds have drawbacks in terms of effectively reducing the usage of the search thresholds and thereby reducing the potential UE power saving opportunities. RAN4 agrees that by having search thresholds per carrier, the network can ensure that the UE is measuring the correct inter-frequency carriers based on serving cell thresholds, thereby reducing the overall number of carriers to be measured by the UE, which enables improved UE power savings.

RAN4 kindly requests RAN2 to evaluate feasibility of introducing carrier specific inter-frequency/RAT measurement search thresholds for improved UE power saving.

2. Actions:

To RAN2 group.

ACTION: 	RAN4 respectfully asks RAN2 to evaluate feasibility of introducing carrier specific inter-frequency/RAT measurement search thresholds for improved UE power saving.

3. Date of Next TSG-RAN WG4 Meetings:

3GPP RAN4#94  	24 – 28 Feb 2020      Athens, Greece

3GPP RAN4#94bis  	20 – 24 Apr 2020      China