TSG-RAN Working Group 4 (Radio) meeting #93	R4-1914531

Reno, US, 18th – 22nd November 2019

Source:	Ericsson

Title:	TP to TR 38.820: Adding information in TX requirement overview tables in subclause 7.4.1.1

Agenda item:	11.4.8.3

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#90-bis in Chongqing) technical background for FR1 and FR2 as well as considerations for the frequency range 7 to 24 GHz was captured in TR 38.820, subclause 7.4 [1].

A text proposal to update the summary tables have been created. The text proposal adds findings and conclusions in the transmitter requirement overview tables. In this contribution a text proposal is attached for approval.

# Discussion

Summary of additions;

- Information from [2] is divided and moved into the conducted requirement table in Table 7.4.1.1-1 and the OTA requirements table in Table 7.4.1.1-2.

- Information captured in subclause 7.4 is summarized in Table 7.4.1.1-1 and Table 7.4.1.1-2.

- For table cell with no information found the following text was added; Not discussed in SI.

# Conclusion

At the end of this contribution a text proposal is attached for approval.

# References

[1]	R4-1913080, “DRAFT TR 38.820 V0.3.0”, Huawei

[2]	R4-1911844, “TP to TR 38.820: Requirement approach in subclause 7.4.1 and subclause 7.4.2”, Ericsson

TEXT PROPOSAL:

#### 7.4.1.1	Tx requirements overview

Summary of the conducted and radiated Tx requirements specified in Rel-15 for the NR BS is presented in this subclause. More detailed elaboration on the motivation on selected requirements is provided in dedicated subclauses below.

All the findings captured for the conducted requirements in table 7.4.1.1-1 and related subclauses below are considered to be applicable to the (sub)-range of the 7 – 24 GHz for which the conducted requirements will be found to be feasible during related WI.

While radiated requirements are considered to be applicable to the whole 7 – 24 GHz range, their definitions, values and levels may differ across the 7 – 24 GHz range.

Table 7.4.1.1-1: Overview of conducted Tx requirements for NR BS in 7 – 24 GHz range

| Tx requirement |  | Conclusions from SI | Items to be completed in related WI |
| --- | --- | --- | --- |
| Base station output power |  | Some indicative PA output power levels are discussed but BS output power depending on the intended deployment scenario is not investigated as no specific frequency band is handled. | Define the requirement level for a specific band. |
| Output power dynamics | RE power control dynamic range | If unwanted emission limits are not increased compared to FR1 limits, then the RE power control dynamic range requirements for QPSK and 16 QAM can be reused for the applicable 7-24 GHz (sub)-range. No requirement for 64QAM. | For 10 GHz possible FR1 approach can be used. If unwanted emission limits are increased compared to FR1 limits, then the RE power control dynamic range may require to be revisited. |
|  | Total power dynamic range | Same requirement derivation methodology as and FR1 and FR2, based on the set of supported channel bandwidths and SCS. | For 10 GHz possible FR1 approach can be used. |
| Transmit ON/OFF power | Transmitter OFF power | There are no existing conducted requirements, so there is no need to maintain equivalence to anything. The scenario driving this requirement is different for FR1 and FR2. | For 10 GHz possible FR1 approach can be used. Determine appropriate antenna port isolation for specific band. |
|  | Transmitter transient period | The requirement depends on scenario and SCS. | For 10 GHz possible FR1 approach can be used. |
| Transmitted signal quality | Frequency error | Not discussed in SI. | For 10 GHz possible FR1 approach can be used. |
|  | Modulation quality | It would be reasonable that same EVM requirement levels which is common between FR1 and FR2 should apply. Phase noise performance and parameterized phase noise model was discussed in SI. | For 10 GHz possible FR1 approach can be used. |
|  | Time alignment error | Not discussed in SI. | For 10 GHz possible FR1 approach can be used. |
| Occupied bandwidth |  | Single carrier and CA requirements can be reused from FR1. | For 10 GHz possible FR1 approach can be used. |
| ACLR |  | As such it is important that co-existence simulations are carried out on specific bands (or ranges) as they are identified. The 7 to 24GHz region covers almost 3 octaves so there may be more than 1 ACIR (and hence BS ACLR) requirements over the 7 to 24GHz range. | The ACLR requirements should be derived from a co-existence study. |
| Operating band unwanted emissions |  | Discussed in SI but no conclusion around the OBUE domain and possible mask shape. Higher ΔfOBUE, than in FR1 needs to be considered, probably with an approach similar to FR2. Existing mask shapes could also be a starting point. | To be completed in the WI when specific frequency bands are specified. Also consider the OBUE mask. |
| Transmitter spurious emissions | General transmitter spurious emissions | Regulation defines Category A and B limits, but Category B limits for AAS BS operating in 6 to 24.25 GHz are presently not defined. A starting point for those limits could be the present limits above 24.25 GHz, but the work defining them will take place in ECC. Feasibility of filters based on LTCC technology was discussed during WI where based on attenuation assumptions, insertion loss and needed guard has been investigated. | To be completed in the WI. |
|  | Protection of the BS receiver of own or different BS | No discussion during SI. | Determine appropriate antenna port isolation for specific band. |
|  | Additional spurious emissions requirements | No discussion during SI. | For specific band check relevant regulation. |
|  | Co-location with other base stations | Discussed during SI, the port to port isolation needs to be further considered for the frequency range. | Determine appropriate antenna port isolation for specific band. |
| Transmitter intermodulation |  | The necessity of the TX IMD requirement can be analyzed by understanding the isolation, the aggressor Pout and the emissions requirements (notably ACLR), as frequency increases the ACLR requirement is smaller, achievable Pout is lower and isolation increases. As none of these parameters will be decided for the 7 to 24GHz region will be agreed in the SI the need for TX IMD cannot be accurately analyzed however it is clear as the frequency increases the needed for a TX IMD requirement reduces. | Determine appropriate antenna port isolation for specific band. |

Table 7.4.1.1-2: Overview of radiated Tx requirements for NR BS in 7 – 24 GHz range

| OTA Tx requirement |  | Conclusions from SI | Items to be completed in related WI |
| --- | --- | --- | --- |
| Radiated transmit power |  | Some indicative PA output power levels are discussed but BS output power depending on the array size is not investigated as no specific frequency band is handled. | Define the requirement level for a specific band. |
| OTA base station output power |  | Some indicative PA output power levels are discussed but BS output power depending on the power capability is not investigated as no specific frequency band is handled. | Define the requirement level for a specific band. |
| OTA output power dynamics | RE power control dynamic range | For “FR1-like” sub-range of the 7 – 24 GHz range, the requirement for QPSK and 16QAM is the same as conduced requirement for 7 – 24 GHz. For “FR2-like” sub-range of the 7 – 24 GHz range, there is no requirement defined. No requirement for 64QAM. | If unwanted emission limits are increased compared to FR1 limits, then the RE power control dynamic range may require to be revisited. |
|  | Total power dynamic range | Same requirement derivation methodology as FR1 and FR2, based on the set of supported channel bandwidths and SCS. | For 10 GHz possible FR1 approach can be used while for 15 and 20 GHz, the FR2 approach should be adopted. |
| OTA transmit ON/OFF power | Transmitter OFF power | The background assumptions for FR1 and FR2 are significantly different. | Define appropriate assumption for 7GHz to 24 GHz. |
|  | Transmitter transient period | Not discussed in SI. | For 10 GHz possible FR1 approach can be used while for 15 and 20 GHz, the FR2 approach should be adopted. This would depend on chosen SCS. |
| OTA transmitted signal quality | Frequency error | Not discussed in SI. | For 10 GHz possible FR1 approach can be used while for 15 and 20 GHz, the FR2 approach should be adopted. This would depend on chosen SCS. |
|  | Modulation quality | Not discussed in SI but we foresee that same EVM requirement levels which is common between FR1 and FR2 should apply. Phase noise performance and parameterized phase noise model was discussed in SI. | For 10 GHz possible FR1 approach can be used while for 15 and 20 GHz, the FR2 approach should be adopted. |
|  | Time alignment error | Not discussed in SI. | For 10 GHz possible FR1 approach can be used while for 15 and 20 GHz, the FR2 approach should be adopted. |
| OTA occupied bandwidth |  | Single carrier and CA requirements can be reused from FR1/FR2. | For 10 GHz possible FR1 approach can be used while for 15 and 20 GHz, the FR2 approach should be adopted. |
| OTA ACLR |  | As such it is important that co-existence simulations are carried out on specific bands (or ranges) as they are identified. The 7 to 24GHz region covers almost 3 octaves so there may be more than 1 ACIR (and hence BS ACLR) requirements over the 7 to 24GHz range. | The ACLR requirements should be derived from a co-existence study. |
| OTA operating band unwanted emissions |  | Discussed in SI but no conclusion around the OBUE domain and possible mask shape. Higher ΔfOBUE, than in FR1 needs to be considered, probably with an approach similar to FR2. Existing mask shapes could also be a starting point. | To be completed in the WI when specific frequency bands are specified. Also consider the OBUE mask. |
| OTA transmitter spurious emissions | General transmitter spurious emissions | Regulation defines Category A and B limits, but Category B limits for AAS BS operating in 6 to 24.25 GHz are presently not defined. A starting point for those limits could be the present limits above 24.25 GHz, but the work defining them will take place in ECC. Feasibility of filters based on LTCC technology was discussed during WI where based on attenuation assumptions, insertion loss and needed guard has been investigated. | To be completed in the WI. |
|  | Protection of the BS receiver of own or different BS | Not discussed in the SI. | Determine appropriate antenna port isolation for specific band. If a requirement is needed in the 7 - 24 GHz region then it will be a co-location requirement, co-location in the 7 - 24 GHz region has a number of implementation issues, see subclause 7.4.1.10, which requires a new concept. |
|  | Additional spurious emissions requirements | Not discussed in the SI. | For specific band check relevant regulation. |
|  | Co-location with other base stations | A new solution based on measurements of DUT based on field strength instead of using measurement antennas. | Determine appropriate antenna port isolation for specific band. If a requirement is needed in the 7 - 24 GHz region then it will be a co-location requirement, co-location in the 7 - 24 GHz region has a number of implementation issues, see subclause 7.4.1.10, which requires a new concept. |
| OTA transmitter intermodulation |  | The necessity of the TX IMD requirement can be analyzed by understanding the isolation, the aggressor Pout and the emissions requirements (notably ACLR), as frequency increases the ACLR requirement is smaller, achievable Pout is lower and isolation increases. As none of these parameters will be decided for the 7 to 24GHz region will be agreed in the SI the need for TX IMD cannot be accurately analyzed however it is clear as the frequency increases the needed for a TX IMD requirement reduces. | Consider not to define a requirement for OTA TX IMD, since the coupling effects is captured by OTA unwanted emission requirement due to intra-system coupling effects. |