3GPP TSG-RAN4 Meeting #93	R4-1914562

# Reno, Nevada, USA, November 18th -22nd, 2019.

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | 0062 | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.141-2: Random data content for NR BS Test Models |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-11-18 |
| Category: | A |  |  |  |  |  | Release: |  |  | Release 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Corresponding discussion paper describing the necessary change R4-1914558 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update data content of ‘all zero data’ to random data |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Update NR Test Model data content of physical channels with “random” data in place of "all zero" data to resolve undesirable signal qualities for BS conformance testing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 4.9.2.3	Data content of physical channels and signals for NR-FR2-TM

Randomisation of the data content is obtained by utilizing the length-31 Gold sequence scrambling of TS 38.211 [20], subclause 5.2.1 which is invoked by all physical channels prior to modulation and mapping to the RE grid. An appropriate number of '0' bits shall be generated prior to the scrambling.

Initialization of the scrambler and RE-mappers as defined in TS 38.211 [20] use the following additional parameters:

-	NIDcell=1 for the lowest configured carrier, NIDcell=2 for the 2nd lowest configured carrier, … NIDcell=n for the nth configured carrier

-	Antenna ports starting with 1000 for PDSCH

-	Antenna ports starting with 2000 for PDCCH

-	q = 0 (single code word)

-	Rank 1, single layer (except for TAE requirement of 2 layer MIMO transmission)

##### 4.9.2.3.1	PDCCH

-	NsymbCORESET= 2

-	PDCCH modulation to be QPSK as described in TS 38.211 [20], subclause 5.1.3.

-	For each slot the required amount of bits for all PDCCHs is as follows: 1(# of PDCCH) * 1(# of CCE per PDCCH) * 6(REG per CCE) * 9(data RE per REG) * 2(bits per RE) with these parameters according to the NR-FR2-TM definitions in subclause 4.4.9.2.2.

-	Generate this amount of bits according to random data using uniform distribution with starting seed [xx].

-	1 CCE shall be according to TS 38.211 [20], subclause 7.3.2. PDCCH using non-interleaved CCE-to-REG mapping. PDCCH occupies the first two symbols for 6 resource-element groups, where a resource element group equals one resource block during one OFDM symbol.

-	Perform PDCCH scrambling according to TS 38.211 [20], subclause 7.3.2.3.

-	NID=NIDcell in DM-RS sequence generation in TS 38.211 [20], subclause 7.4.1.3.

-	nRNTI=0 in scrambling sequence generation in TS 38.211 [20], subclause 7.3.2.3.

-	Perform mapping to REs according to TS 38.211 [20], subclause 7.3.2.5.

##### 4.9.2.3.2	PDSCH

-	For each slot generate the required amount of bits for all PRBs according to  random data using uniform distribution with starting seed [xx].

-	NR-FR2-TMs utilize 1 or 2 user PDSCH transmissions distinguished by nRNTI. For each NR-FR2-TM, PRBs are mapped to user (nRNTI) as follows:

Table 4.9.2.3.2-1: Mapping of PRBs to  for NR-FR2-TM

| Test model |  | Number of users |
| --- | --- | --- |
| NR-FR2-TM1.1 | 2 for PRBs located in PRB#0-2 0 for remaining PRBs | 2 |
| NR-FR2-TM2 | 2 for all PRBs | 1 |
| NR-FR2-TM3.1 | 2 for PRBs located in PRB#0-2 0 for remaining PRBs | 2 |

-	Perform user specific scrambling according to TS 38.211 [20], subclause 7.3.1.1.

-	nID=NIDcell

-	Perform modulation of the scrambled bits with the modulation scheme defined for each user according to TS 38.211 [20], subclause7.3.1.2.

-	Perform mapping of the complex-valued symbols to layer according to TS 38.211 [20], subclause 7.3.1.3.   Complex-valued modulation symbols  for codeword q shall be mapped onto the layers ,  where υ is equal to 1.

-	Perform PDSCH mapping type A according to TS 38.211 [20].

-	PDSCH resource allocation according to TS 38.214 [21] as following;

-	NR-FR2-TM1.1, NR-FR2-TM3.1: type 1 for PDSCH with nRNTI = 0, nRNTI = 2

-	NR-FR2-TM2: type 1 for PDSCH with nRNTI = 2

-	DM-RS sequence generation according to TS 38.211 [20], subclause 7.4.1.1.1 where l is the OFDM symbol number within the slot with symbols indicated by table 4.9.2.2-3.

-	NIDnSCID=NIDcell

-	nSCID=0

-	DM-RS mapping according to TS 38.211 [20], subclause 7.4.1.1.2 with parameters listed in table 4.9.2.2-3.

-	For NR-FR2-TM PT-RS sequence generation according to TS 38.211 [20], subclause 7.4.1.2.1, with parameters listed in table 4.9.2.2-3.

-	For NR-FR2-TM PT-RS mapping according to TS 38.211 [20], subclause 7.4.1.2.2, with parameters listed in table 4.9.2.2-3.

##### [End of changes]