3GPP TSG-RAN WG4 #93	R4-1914640

Reno, USA, 14th – 18th November, 2019

Title: 					Simulation results for measurements in high speed

Source: 				Ericsson

Agenda item:	9.17.1

Document for:	Discussion

# Introduction

In this contribution we provide simulation results for NR RSRP and RSRQ measurement accuracy in high speed according to simulation assumptions in [1].

# Discussion

Simulations were performed according to the parameters in table 1 and table 2

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | 3.6 GHz |
| DRX | No |
| BS transmit antennas for SS blocks | 1 tx or single layer transmissions |
| UE receive antennas | 2 rx |
| Data and control channel subcarrier spacing | The same as SS block subcarrier spacing |
| Measurement period (in number of measurement samples) | 1,3,5 |
| - Subcarrier spacing | 15 kHz; 30 kHz |
| - Number of SS blocks per SS burst set, K | 1 |
| - SS burst periodicity | 5ms, 10ms, 20ms |

Table 1: General parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| E-UTRA RF Channel number | - | Channel 1 | Channel 1 |
| PBCH and DMRS power offset with respect to NR-PSS and NR-SSS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS and NR-SSS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | Ms | 0 | 3 ms |
| Es/IoT | dB | 6.4 | 1.25; FSS: -0.75, 0.25; (SINR: -6dB; FFS: -8dB, -7dB) |
| Propagation conditions | - | AWGN with 2408Hz offset between desired and interfering cells; AWGN with 3334Hz offset between desired and interfering cells |  |

Table 2: Cell-specific parameters

Some typical CDF results for RSRP and RSRQ accuracy are shown in figure 1 and figure 2.

Figure 1 : RSRP accuracy CDF for AWGN2408Hz, SSB period=5ms, SCS=15kHz, SNR=-6dB

Figure 2 : RSRQ accuracy CDF for AWGN2408Hz, SSB period=5ms, SCS=15kHz, SNR=-6dB

Since there are a very large number of simulation points, we have extracted data from CDFs in annex A (SS-RSRP) and annex B (SS-RSRQ). We further provide statistics at SINR=-6dB for RSRP and RSRQ accuracy and bias at the 50th percentile extracted from all the results in annex A and annex B for async and sync cases respectively in tables 1,2,3 and 4.

## SS-RSRP results summary

| Asynchronous case |  |  |
| --- | --- | --- |
| Sample number | Accuracy (5th percentile, 95th percentile) | Mean 50th percentile |
| 1 | (-2.45,1.95) | -0.13 |
| 3 | (-1.35,1.13) | 0.00046 |
| 5 | (-0.97,0.91) | 0.03 |
| Synchronous case |  |  |
| Sample number | Accuracy (5th percentile, 95th percentile) | Mean 50th percentile |
| 1 | (-0.07,1.49) | 0.74 |
| 3 | (0.28,1.17) | 0.73 |
| 5 | (0.40,1.05) | 0.73 |

## SS-RSRQ results summary

| Asynchronous case |  |  |
| --- | --- | --- |
| Sample number | Accuracy (5th percentile, 95th percentile) | Mean 50th percentile |
| 1 | (-2.43,1.93) | -0.11 |
| 3 | (-1.33,1.15) | 0.02 |
| 5 | (-0.95,0.93) | 0.05 |
| Synchronous case |  |  |
| Sample number | Accuracy (5th percentile, 95th percentile) | Mean 50th percentile |
| 1 | (-0.04,1.51) | 0.76 |
| 3 | (0.30,1.19) | 0.74 |
| 5 | (0.42,1.06) | 0.75 |

Based on the results we can observe that at -6dB SINR:

Observation 1 : Compared with 5 measurement samples, 3 measurement samples gives only a minor degradation in both SS-RSRP and SS-RSRQ measurement accuracy in HST conditions (degradation of less than (-0.4,+0.25)dB in all cases

Observation 2 : In synchronous cases, a bias of approximately 0.75dB is observed

Due to the need to reduce measurement delays in high speed operation, and given the small difference between using 3 and 5 samples we propose

Proposal : A measurement period of 3 samples may be used to support high speed scenarios

The proposal is aligned with LTE DRX measurement requirements for high speed, where 3 samples is assumed

# Conclusions

In this contribution we provide results for high speed measurement accuracy. Based on the results we can observe that at -6dB SNR:

Observation 1 : Compared with 5 measurement samples, 3 measurement samples gives only a minor degradation in both SS-RSRP and SS-RSRQ measurement accuracy in HST conditions (degradation of less than (-0.4,+0.25)dB in all cases

Observation 2 : In synchronous cases, a bias of approximately 0.75dB is observed

Due to the need to reduce measurement delays in high speed operation, and given the small difference between using 3 and 5 samples we propose

Proposal : A measurement period of 3 samples may be used to support high speed scenarios

# References

- R4-1912842	 “Link level simulation assumption on measurement accuracy in NR HST”, Huawei, HiSilicon

# Annex A : Summary of SS-RSRP CDFs

| Numerology | Interferer Channel | is_sync | SINR | SSB Periodicity ms | #rsrp samples | 5% | 50% | 95% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 5 | 1 | -0.069779 | 0.74822 | 1.4867 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 5 | 3 | 0.28283 | 0.72921 | 1.1688 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 5 | 5 | 0.4017 | 0.73624 | 1.0453 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 10 | 1 | -0.069779 | 0.74822 | 1.4867 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 10 | 3 | 0.28283 | 0.72921 | 1.1688 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 10 | 5 | 0.4017 | 0.73624 | 1.0453 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 20 | 1 | -0.069779 | 0.74822 | 1.4867 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 20 | 3 | 0.28283 | 0.72921 | 1.1688 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 20 | 5 | 0.4017 | 0.73624 | 1.0453 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 5 | 1 | 0.0058153 | 1.0687 | 1.9826 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 5 | 3 | 0.4757 | 1.0617 | 1.6151 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 5 | 5 | 0.64007 | 1.066 | 1.4597 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 10 | 1 | 0.0058153 | 1.0687 | 1.9826 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 10 | 3 | 0.4757 | 1.0617 | 1.6151 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 10 | 5 | 0.64007 | 1.066 | 1.4597 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 20 | 1 | 0.0058153 | 1.0687 | 1.9826 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 20 | 3 | 0.4757 | 1.0617 | 1.6151 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 20 | 5 | 0.64007 | 1.066 | 1.4597 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 5 | 1 | -0.036101 | 0.88653 | 1.714 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 5 | 3 | 0.36048 | 0.87761 | 1.3735 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 5 | 5 | 0.50714 | 0.8837 | 1.2328 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 10 | 1 | -0.036101 | 0.88653 | 1.714 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 10 | 3 | 0.36048 | 0.87761 | 1.3735 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 10 | 5 | 0.50714 | 0.8837 | 1.2328 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 20 | 1 | -0.036101 | 0.88653 | 1.714 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 20 | 3 | 0.36048 | 0.87761 | 1.3735 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 20 | 5 | 0.50714 | 0.8837 | 1.2328 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 5 | 1 | -2.4554 | -0.19984 | 1.9051 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 5 | 3 | -1.3456 | -0.014377 | 1.1324 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 5 | 5 | -0.97391 | 0.020358 | 0.90836 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 10 | 1 | -2.4554 | -0.19984 | 1.9051 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 10 | 3 | -1.3456 | -0.014377 | 1.1324 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 10 | 5 | -0.97391 | 0.020358 | 0.90836 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 20 | 1 | -2.4554 | -0.19984 | 1.9051 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 20 | 3 | -1.3456 | -0.014377 | 1.1324 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 20 | 5 | -0.97391 | 0.020358 | 0.90836 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 5 | 1 | -3.082 | -0.19991 | 2.4085 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 5 | 3 | -1.6769 | 0.098191 | 1.478 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 5 | 5 | -1.127 | 0.11068 | 1.2341 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 10 | 1 | -3.082 | -0.19991 | 2.4085 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 10 | 3 | -1.6769 | 0.098191 | 1.478 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 10 | 5 | -1.127 | 0.11068 | 1.2341 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 20 | 1 | -3.082 | -0.19991 | 2.4085 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 20 | 3 | -1.6769 | 0.098191 | 1.478 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 20 | 5 | -1.127 | 0.11068 | 1.2341 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 5 | 1 | -2.7767 | -0.218 | 2.1525 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 5 | 3 | -1.5144 | 0.03171 | 1.2798 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 5 | 5 | -1.0638 | 0.053948 | 1.0657 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 10 | 1 | -2.7767 | -0.218 | 2.1525 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 10 | 3 | -1.5144 | 0.03171 | 1.2798 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 10 | 5 | -1.0638 | 0.053948 | 1.0657 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 20 | 1 | -2.7767 | -0.218 | 2.1525 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 20 | 3 | -1.5144 | 0.03171 | 1.2798 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 20 | 5 | -1.0638 | 0.053948 | 1.0657 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 5 | 1 | -0.069779 | 0.74822 | 1.4867 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 5 | 3 | 0.28283 | 0.72921 | 1.1688 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 5 | 5 | 0.4017 | 0.73624 | 1.0453 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 10 | 1 | -0.069779 | 0.74822 | 1.4867 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 10 | 3 | 0.28283 | 0.72921 | 1.1688 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 10 | 5 | 0.4017 | 0.73624 | 1.0453 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 20 | 1 | -0.069779 | 0.74822 | 1.4867 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 20 | 3 | 0.28283 | 0.72921 | 1.1688 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 20 | 5 | 0.4017 | 0.73624 | 1.0453 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 5 | 1 | 0.0058153 | 1.0687 | 1.9826 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 5 | 3 | 0.4757 | 1.0617 | 1.6151 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 5 | 5 | 0.64007 | 1.066 | 1.4597 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 10 | 1 | 0.0058153 | 1.0687 | 1.9826 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 10 | 3 | 0.4757 | 1.0617 | 1.6151 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 10 | 5 | 0.64007 | 1.066 | 1.4597 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 20 | 1 | 0.0058153 | 1.0687 | 1.9826 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 20 | 3 | 0.4757 | 1.0617 | 1.6151 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 20 | 5 | 0.64007 | 1.066 | 1.4597 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 5 | 1 | -0.036101 | 0.88653 | 1.714 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 5 | 3 | 0.36048 | 0.87761 | 1.3735 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 5 | 5 | 0.50714 | 0.8837 | 1.2328 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 10 | 1 | -0.036101 | 0.88653 | 1.714 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 10 | 3 | 0.36048 | 0.87761 | 1.3735 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 10 | 5 | 0.50714 | 0.8837 | 1.2328 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 20 | 1 | -0.036101 | 0.88653 | 1.714 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 20 | 3 | 0.36048 | 0.87761 | 1.3735 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 20 | 5 | 0.50714 | 0.8837 | 1.2328 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 5 | 1 | -2.4554 | -0.19984 | 1.9051 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 5 | 3 | -1.3456 | -0.014377 | 1.1324 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 5 | 5 | -0.97391 | 0.020358 | 0.90836 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 10 | 1 | -2.4554 | -0.19984 | 1.9051 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 10 | 3 | -1.3456 | -0.014377 | 1.1324 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 10 | 5 | -0.97391 | 0.020358 | 0.90836 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 20 | 1 | -2.4554 | -0.19984 | 1.9051 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 20 | 3 | -1.3456 | -0.014377 | 1.1324 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 20 | 5 | -0.97391 | 0.020358 | 0.90836 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 5 | 1 | -3.082 | -0.19991 | 2.4085 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 5 | 3 | -1.6769 | 0.098191 | 1.478 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 5 | 5 | -1.127 | 0.11068 | 1.2341 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 10 | 1 | -3.082 | -0.19991 | 2.4085 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 10 | 3 | -1.6769 | 0.098191 | 1.478 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 10 | 5 | -1.127 | 0.11068 | 1.2341 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 20 | 1 | -3.082 | -0.19991 | 2.4085 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 20 | 3 | -1.6769 | 0.098191 | 1.478 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 20 | 5 | -1.127 | 0.11068 | 1.2341 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 5 | 1 | -2.7767 | -0.218 | 2.1525 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 5 | 3 | -1.5144 | 0.03171 | 1.2798 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 5 | 5 | -1.0638 | 0.053948 | 1.0657 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 10 | 1 | -2.7767 | -0.218 | 2.1525 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 10 | 3 | -1.5144 | 0.03171 | 1.2798 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 10 | 5 | -1.0638 | 0.053948 | 1.0657 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 20 | 1 | -2.7767 | -0.218 | 2.1525 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 20 | 3 | -1.5144 | 0.03171 | 1.2798 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 20 | 5 | -1.0638 | 0.053948 | 1.0657 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 5 | 1 | 0.18929 | 0.7381 | 1.2889 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 5 | 3 | 0.40738 | 0.72528 | 1.0127 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 5 | 5 | 0.51314 | 0.72584 | 0.96054 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 10 | 1 | 0.18929 | 0.7381 | 1.2889 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 10 | 3 | 0.40738 | 0.72528 | 1.0127 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 10 | 5 | 0.51314 | 0.72584 | 0.96054 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 20 | 1 | 0.18929 | 0.7381 | 1.2889 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 20 | 3 | 0.40738 | 0.72528 | 1.0127 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 20 | 5 | 0.51314 | 0.72584 | 0.96054 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 5 | 1 | 0.33578 | 1.0594 | 1.7666 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 5 | 3 | 0.64955 | 1.0542 | 1.4216 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 5 | 5 | 0.77596 | 1.0568 | 1.3601 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 10 | 1 | 0.33578 | 1.0594 | 1.7666 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 10 | 3 | 0.64955 | 1.0542 | 1.4216 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 10 | 5 | 0.77596 | 1.0568 | 1.3601 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 20 | 1 | 0.33578 | 1.0594 | 1.7666 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 20 | 3 | 0.64955 | 1.0542 | 1.4216 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 20 | 5 | 0.77596 | 1.0568 | 1.3601 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 5 | 1 | 0.25443 | 0.87888 | 1.498 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 5 | 3 | 0.51516 | 0.87204 | 1.1984 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 5 | 5 | 0.62568 | 0.87226 | 1.1414 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 10 | 1 | 0.25443 | 0.87888 | 1.498 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 10 | 3 | 0.51516 | 0.87204 | 1.1984 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 10 | 5 | 0.62568 | 0.87226 | 1.1414 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 20 | 1 | 0.25443 | 0.87888 | 1.498 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 20 | 3 | 0.51516 | 0.87204 | 1.1984 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 20 | 5 | 0.62568 | 0.87226 | 1.1414 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 5 | 1 | -1.7077 | -0.063692 | 1.4116 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 5 | 3 | -0.88249 | 0.018264 | 0.81174 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 5 | 5 | -0.66175 | 0.047482 | 0.65258 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 10 | 1 | -1.7077 | -0.063692 | 1.4116 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 10 | 3 | -0.88249 | 0.018264 | 0.81174 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 10 | 5 | -0.66175 | 0.047482 | 0.65258 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 20 | 1 | -1.7077 | -0.063692 | 1.4116 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 20 | 3 | -0.88249 | 0.018264 | 0.81174 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 20 | 5 | -0.66175 | 0.047482 | 0.65258 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 5 | 1 | -2.0442 | -0.0064862 | 1.8052 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 5 | 3 | -1.0065 | 0.12017 | 1.0739 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 5 | 5 | -0.74573 | 0.1643 | 0.9219 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 10 | 1 | -2.0442 | -0.0064862 | 1.8052 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 10 | 3 | -1.0065 | 0.12017 | 1.0739 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 10 | 5 | -0.74573 | 0.1643 | 0.9219 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 20 | 1 | -2.0442 | -0.0064862 | 1.8052 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 20 | 3 | -1.0065 | 0.12017 | 1.0739 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 20 | 5 | -0.74573 | 0.1643 | 0.9219 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 5 | 1 | -1.8705 | -0.035626 | 1.5884 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 5 | 3 | -0.96279 | 0.066582 | 0.93356 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 5 | 5 | -0.71444 | 0.089437 | 0.77304 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 10 | 1 | -1.8705 | -0.035626 | 1.5884 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 10 | 3 | -0.96279 | 0.066582 | 0.93356 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 10 | 5 | -0.71444 | 0.089437 | 0.77304 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 20 | 1 | -1.8705 | -0.035626 | 1.5884 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 20 | 3 | -0.96279 | 0.066582 | 0.93356 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 20 | 5 | -0.71444 | 0.089437 | 0.77304 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 5 | 1 | 0.18929 | 0.7381 | 1.2889 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 5 | 3 | 0.40738 | 0.72528 | 1.0127 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 5 | 5 | 0.51314 | 0.72584 | 0.96054 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 10 | 1 | 0.18929 | 0.7381 | 1.2889 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 10 | 3 | 0.40738 | 0.72528 | 1.0127 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 10 | 5 | 0.51314 | 0.72584 | 0.96054 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 20 | 1 | 0.18929 | 0.7381 | 1.2889 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 20 | 3 | 0.40738 | 0.72528 | 1.0127 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 20 | 5 | 0.51314 | 0.72584 | 0.96054 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 5 | 1 | 0.33578 | 1.0594 | 1.7666 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 5 | 3 | 0.64955 | 1.0542 | 1.4216 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 5 | 5 | 0.77596 | 1.0568 | 1.3601 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 10 | 1 | 0.33578 | 1.0594 | 1.7666 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 10 | 3 | 0.64955 | 1.0542 | 1.4216 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 10 | 5 | 0.77596 | 1.0568 | 1.3601 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 20 | 1 | 0.33578 | 1.0594 | 1.7666 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 20 | 3 | 0.64955 | 1.0542 | 1.4216 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 20 | 5 | 0.77596 | 1.0568 | 1.3601 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 5 | 1 | 0.25443 | 0.87888 | 1.498 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 5 | 3 | 0.51516 | 0.87204 | 1.1984 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 5 | 5 | 0.62568 | 0.87226 | 1.1414 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 10 | 1 | 0.25443 | 0.87888 | 1.498 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 10 | 3 | 0.51516 | 0.87204 | 1.1984 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 10 | 5 | 0.62568 | 0.87226 | 1.1414 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 20 | 1 | 0.25443 | 0.87888 | 1.498 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 20 | 3 | 0.51516 | 0.87204 | 1.1984 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 20 | 5 | 0.62568 | 0.87226 | 1.1414 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 5 | 1 | -1.7077 | -0.063692 | 1.4116 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 5 | 5 | -0.66175 | 0.047482 | 0.65258 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 10 | 1 | -1.7077 | -0.063692 | 1.4116 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 10 | 3 | -0.88249 | 0.018264 | 0.81174 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 10 | 5 | -0.66175 | 0.047482 | 0.65258 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 20 | 1 | -1.7077 | -0.063692 | 1.4116 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 20 | 3 | -0.88249 | 0.018264 | 0.81174 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 20 | 5 | -0.66175 | 0.047482 | 0.65258 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 5 | 1 | -2.0442 | -0.0064862 | 1.8052 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 5 | 3 | -1.0065 | 0.12017 | 1.0739 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 5 | 5 | -0.74573 | 0.1643 | 0.9219 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 10 | 1 | -2.0442 | -0.0064862 | 1.8052 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 10 | 3 | -1.0065 | 0.12017 | 1.0739 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 10 | 5 | -0.74573 | 0.1643 | 0.9219 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 20 | 1 | -2.0442 | -0.0064862 | 1.8052 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 20 | 3 | -1.0065 | 0.12017 | 1.0739 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 20 | 5 | -0.74573 | 0.1643 | 0.9219 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 5 | 1 | -1.8705 | -0.035626 | 1.5884 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 5 | 3 | -0.96279 | 0.066582 | 0.93356 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 5 | 5 | -0.71444 | 0.089437 | 0.77304 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 10 | 1 | -1.8705 | -0.035626 | 1.5884 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 10 | 3 | -0.96279 | 0.066582 | 0.93356 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 10 | 5 | -0.71444 | 0.089437 | 0.77304 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 20 | 1 | -1.8705 | -0.035626 | 1.5884 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 20 | 3 | -0.96279 | 0.066582 | 0.93356 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 20 | 5 | -0.71444 | 0.089437 | 0.77304 |

# Annex B : Summary of SS-RSRQ CDFs

| Numerology | Interferer Channel | is_sync | SINR | SSB Periodicity ms | #rsrq samples | 5% | 50% | 95% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 5 | 1 | -0.049314 | 0.76868 | 1.5071 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 5 | 3 | 0.3033 | 0.74968 | 1.1893 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 5 | 5 | 0.42217 | 0.7567 | 1.0658 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 10 | 1 | -0.049314 | 0.76868 | 1.5071 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 10 | 3 | 0.3033 | 0.74968 | 1.1893 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 10 | 5 | 0.42217 | 0.7567 | 1.0658 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 20 | 1 | -0.049314 | 0.76868 | 1.5071 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 20 | 3 | 0.3033 | 0.74968 | 1.1893 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 20 | 5 | 0.42217 | 0.7567 | 1.0658 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 5 | 1 | 0.02628 | 1.0892 | 2.003 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 5 | 3 | 0.49617 | 1.0822 | 1.6356 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 5 | 5 | 0.66054 | 1.0865 | 1.4802 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 10 | 1 | 0.02628 | 1.0892 | 2.003 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 10 | 3 | 0.49617 | 1.0822 | 1.6356 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 10 | 5 | 0.66054 | 1.0865 | 1.4802 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 20 | 1 | 0.02628 | 1.0892 | 2.003 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 20 | 3 | 0.49617 | 1.0822 | 1.6356 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 20 | 5 | 0.66054 | 1.0865 | 1.4802 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 5 | 1 | -0.015636 | 0.907 | 1.7345 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 5 | 3 | 0.38095 | 0.89807 | 1.3939 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 5 | 5 | 0.5276 | 0.90417 | 1.2532 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 10 | 1 | -0.015636 | 0.907 | 1.7345 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 10 | 3 | 0.38095 | 0.89807 | 1.3939 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 10 | 5 | 0.5276 | 0.90417 | 1.2532 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 20 | 1 | -0.015636 | 0.907 | 1.7345 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 20 | 3 | 0.38095 | 0.89807 | 1.3939 |
| 15000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 20 | 5 | 0.5276 | 0.90417 | 1.2532 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 5 | 1 | -2.435 | -0.17937 | 1.9255 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 5 | 3 | -1.3251 | 0.0060883 | 1.1528 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 5 | 5 | -0.95344 | 0.040823 | 0.92883 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 10 | 1 | -2.435 | -0.17937 | 1.9255 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 10 | 3 | -1.3251 | 0.0060883 | 1.1528 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 10 | 5 | -0.95344 | 0.040823 | 0.92883 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 20 | 1 | -2.435 | -0.17937 | 1.9255 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 20 | 3 | -1.3251 | 0.0060883 | 1.1528 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 20 | 5 | -0.95344 | 0.040823 | 0.92883 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 5 | 1 | -3.0615 | -0.17945 | 2.4289 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 5 | 3 | -1.6564 | 0.11866 | 1.4984 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 5 | 5 | -1.1065 | 0.13115 | 1.2546 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 10 | 1 | -3.0615 | -0.17945 | 2.4289 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 10 | 3 | -1.6564 | 0.11866 | 1.4984 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 10 | 5 | -1.1065 | 0.13115 | 1.2546 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 20 | 1 | -3.0615 | -0.17945 | 2.4289 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 20 | 3 | -1.6564 | 0.11866 | 1.4984 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 20 | 5 | -1.1065 | 0.13115 | 1.2546 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 5 | 1 | -2.7563 | -0.19753 | 2.1729 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 5 | 3 | -1.4939 | 0.052175 | 1.3003 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 5 | 5 | -1.0433 | 0.074413 | 1.0862 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 10 | 1 | -2.7563 | -0.19753 | 2.1729 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 10 | 3 | -1.4939 | 0.052175 | 1.3003 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 10 | 5 | -1.0433 | 0.074413 | 1.0862 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 20 | 1 | -2.7563 | -0.19753 | 2.1729 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 20 | 3 | -1.4939 | 0.052175 | 1.3003 |
| 15000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 20 | 5 | -1.0433 | 0.074413 | 1.0862 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 5 | 1 | -0.049314 | 0.76868 | 1.5071 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 5 | 3 | 0.3033 | 0.74968 | 1.1893 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 5 | 5 | 0.42217 | 0.7567 | 1.0658 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 10 | 1 | -0.049314 | 0.76868 | 1.5071 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 10 | 3 | 0.3033 | 0.74968 | 1.1893 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 10 | 5 | 0.42217 | 0.7567 | 1.0658 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 20 | 1 | -0.049314 | 0.76868 | 1.5071 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 20 | 3 | 0.3033 | 0.74968 | 1.1893 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 20 | 5 | 0.42217 | 0.7567 | 1.0658 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 5 | 1 | 0.02628 | 1.0892 | 2.003 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 5 | 3 | 0.49617 | 1.0822 | 1.6356 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 5 | 5 | 0.66054 | 1.0865 | 1.4802 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 10 | 1 | 0.02628 | 1.0892 | 2.003 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 10 | 3 | 0.49617 | 1.0822 | 1.6356 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 10 | 5 | 0.66054 | 1.0865 | 1.4802 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 20 | 1 | 0.02628 | 1.0892 | 2.003 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 20 | 3 | 0.49617 | 1.0822 | 1.6356 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 20 | 5 | 0.66054 | 1.0865 | 1.4802 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 5 | 1 | -0.015636 | 0.907 | 1.7345 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 5 | 3 | 0.38095 | 0.89807 | 1.3939 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 5 | 5 | 0.5276 | 0.90417 | 1.2532 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 10 | 1 | -0.015636 | 0.907 | 1.7345 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 10 | 3 | 0.38095 | 0.89807 | 1.3939 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 10 | 5 | 0.5276 | 0.90417 | 1.2532 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 20 | 1 | -0.015636 | 0.907 | 1.7345 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 20 | 3 | 0.38095 | 0.89807 | 1.3939 |
| 15000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 20 | 5 | 0.5276 | 0.90417 | 1.2532 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 5 | 1 | -2.435 | -0.17937 | 1.9255 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 5 | 3 | -1.3251 | 0.0060883 | 1.1528 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 5 | 5 | -0.95344 | 0.040823 | 0.92883 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 10 | 1 | -2.435 | -0.17937 | 1.9255 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 10 | 3 | -1.3251 | 0.0060883 | 1.1528 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 10 | 5 | -0.95344 | 0.040823 | 0.92883 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 20 | 1 | -2.435 | -0.17937 | 1.9255 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 20 | 3 | -1.3251 | 0.0060883 | 1.1528 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 20 | 5 | -0.95344 | 0.040823 | 0.92883 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 5 | 1 | -3.0615 | -0.17945 | 2.4289 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 5 | 3 | -1.6564 | 0.11866 | 1.4984 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 5 | 5 | -1.1065 | 0.13115 | 1.2546 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 10 | 1 | -3.0615 | -0.17945 | 2.4289 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 10 | 3 | -1.6564 | 0.11866 | 1.4984 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 10 | 5 | -1.1065 | 0.13115 | 1.2546 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 20 | 1 | -3.0615 | -0.17945 | 2.4289 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 20 | 3 | -1.6564 | 0.11866 | 1.4984 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 20 | 5 | -1.1065 | 0.13115 | 1.2546 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 5 | 1 | -2.7563 | -0.19753 | 2.1729 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 5 | 3 | -1.4939 | 0.052175 | 1.3003 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 5 | 5 | -1.0433 | 0.074413 | 1.0862 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 10 | 1 | -2.7563 | -0.19753 | 2.1729 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 10 | 3 | -1.4939 | 0.052175 | 1.3003 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 10 | 5 | -1.0433 | 0.074413 | 1.0862 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 20 | 1 | -2.7563 | -0.19753 | 2.1729 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 20 | 3 | -1.4939 | 0.052175 | 1.3003 |
| 15000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 20 | 5 | -1.0433 | 0.074413 | 1.0862 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 5 | 1 | 0.20975 | 0.75856 | 1.3094 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 5 | 3 | 0.42784 | 0.74575 | 1.0332 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 5 | 5 | 0.5336 | 0.74631 | 0.981 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 10 | 1 | 0.20975 | 0.75856 | 1.3094 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 10 | 3 | 0.42784 | 0.74575 | 1.0332 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 10 | 5 | 0.5336 | 0.74631 | 0.981 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 20 | 1 | 0.20975 | 0.75856 | 1.3094 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 20 | 3 | 0.42784 | 0.74575 | 1.0332 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -6 | 20 | 5 | 0.5336 | 0.74631 | 0.981 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 5 | 1 | 0.35624 | 1.0798 | 1.787 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 5 | 3 | 0.67002 | 1.0746 | 1.4421 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 5 | 5 | 0.79642 | 1.0772 | 1.3806 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 10 | 1 | 0.35624 | 1.0798 | 1.787 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 10 | 3 | 0.67002 | 1.0746 | 1.4421 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 10 | 5 | 0.79642 | 1.0772 | 1.3806 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 20 | 1 | 0.35624 | 1.0798 | 1.787 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 20 | 3 | 0.67002 | 1.0746 | 1.4421 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -8 | 20 | 5 | 0.79642 | 1.0772 | 1.3806 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 5 | 1 | 0.27489 | 0.89935 | 1.5185 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 5 | 3 | 0.53563 | 0.89251 | 1.2189 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 5 | 5 | 0.64614 | 0.89272 | 1.1619 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 10 | 1 | 0.27489 | 0.89935 | 1.5185 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 10 | 3 | 0.53563 | 0.89251 | 1.2189 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 10 | 5 | 0.64614 | 0.89272 | 1.1619 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 20 | 1 | 0.27489 | 0.89935 | 1.5185 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 20 | 3 | 0.53563 | 0.89251 | 1.2189 |
| 30000 | AWGN_2408DOPPLER_HZ | 1 | -7 | 20 | 5 | 0.64614 | 0.89272 | 1.1619 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 5 | 1 | -1.6872 | -0.043227 | 1.4321 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 5 | 3 | -0.86203 | 0.038729 | 0.83221 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 5 | 5 | -0.64129 | 0.067947 | 0.67304 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 10 | 1 | -1.6872 | -0.043227 | 1.4321 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 10 | 3 | -0.86203 | 0.038729 | 0.83221 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 10 | 5 | -0.64129 | 0.067947 | 0.67304 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 20 | 1 | -1.6872 | -0.043227 | 1.4321 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 20 | 3 | -0.86203 | 0.038729 | 0.83221 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -6 | 20 | 5 | -0.64129 | 0.067947 | 0.67304 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 5 | 1 | -2.0237 | 0.013979 | 1.8257 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 5 | 3 | -0.98599 | 0.14063 | 1.0944 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 5 | 5 | -0.72527 | 0.18477 | 0.94237 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 10 | 1 | -2.0237 | 0.013979 | 1.8257 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 10 | 3 | -0.98599 | 0.14063 | 1.0944 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 10 | 5 | -0.72527 | 0.18477 | 0.94237 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 20 | 1 | -2.0237 | 0.013979 | 1.8257 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 20 | 3 | -0.98599 | 0.14063 | 1.0944 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -8 | 20 | 5 | -0.72527 | 0.18477 | 0.94237 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 5 | 1 | -1.8501 | -0.015161 | 1.6088 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 5 | 3 | -0.94233 | 0.087047 | 0.95403 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 5 | 5 | -0.69397 | 0.1099 | 0.79351 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 10 | 1 | -1.8501 | -0.015161 | 1.6088 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 10 | 3 | -0.94233 | 0.087047 | 0.95403 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 10 | 5 | -0.69397 | 0.1099 | 0.79351 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 20 | 1 | -1.8501 | -0.015161 | 1.6088 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 20 | 3 | -0.94233 | 0.087047 | 0.95403 |
| 30000 | AWGN_2408DOPPLER_HZ | 0 | -7 | 20 | 5 | -0.69397 | 0.1099 | 0.79351 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 5 | 1 | 0.20975 | 0.75856 | 1.3094 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 5 | 3 | 0.42784 | 0.74575 | 1.0332 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 5 | 5 | 0.5336 | 0.74631 | 0.981 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 10 | 1 | 0.20975 | 0.75856 | 1.3094 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 10 | 3 | 0.42784 | 0.74575 | 1.0332 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 10 | 5 | 0.5336 | 0.74631 | 0.981 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 20 | 1 | 0.20975 | 0.75856 | 1.3094 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 20 | 3 | 0.42784 | 0.74575 | 1.0332 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -6 | 20 | 5 | 0.5336 | 0.74631 | 0.981 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 5 | 1 | 0.35624 | 1.0798 | 1.787 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 5 | 3 | 0.67002 | 1.0746 | 1.4421 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 5 | 5 | 0.79642 | 1.0772 | 1.3806 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 10 | 1 | 0.35624 | 1.0798 | 1.787 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 10 | 3 | 0.67002 | 1.0746 | 1.4421 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 10 | 5 | 0.79642 | 1.0772 | 1.3806 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 20 | 1 | 0.35624 | 1.0798 | 1.787 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 20 | 3 | 0.67002 | 1.0746 | 1.4421 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -8 | 20 | 5 | 0.79642 | 1.0772 | 1.3806 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 5 | 1 | 0.27489 | 0.89935 | 1.5185 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 5 | 3 | 0.53563 | 0.89251 | 1.2189 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 5 | 5 | 0.64614 | 0.89272 | 1.1619 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 10 | 1 | 0.27489 | 0.89935 | 1.5185 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 10 | 3 | 0.53563 | 0.89251 | 1.2189 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 10 | 5 | 0.64614 | 0.89272 | 1.1619 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 20 | 1 | 0.27489 | 0.89935 | 1.5185 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 20 | 3 | 0.53563 | 0.89251 | 1.2189 |
| 30000 | AWGN_3334DOPPLER_HZ | 1 | -7 | 20 | 5 | 0.64614 | 0.89272 | 1.1619 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 5 | 1 | -1.6872 | -0.043227 | 1.4321 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 5 | 5 | -0.64129 | 0.067947 | 0.67304 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 10 | 1 | -1.6872 | -0.043227 | 1.4321 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 10 | 3 | -0.86203 | 0.038729 | 0.83221 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 10 | 5 | -0.64129 | 0.067947 | 0.67304 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 20 | 1 | -1.6872 | -0.043227 | 1.4321 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 20 | 3 | -0.86203 | 0.038729 | 0.83221 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -6 | 20 | 5 | -0.64129 | 0.067947 | 0.67304 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 5 | 1 | -2.0237 | 0.013979 | 1.8257 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 5 | 3 | -0.98599 | 0.14063 | 1.0944 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 5 | 5 | -0.72527 | 0.18477 | 0.94237 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 10 | 1 | -2.0237 | 0.013979 | 1.8257 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 10 | 3 | -0.98599 | 0.14063 | 1.0944 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 10 | 5 | -0.72527 | 0.18477 | 0.94237 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 20 | 1 | -2.0237 | 0.013979 | 1.8257 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 20 | 3 | -0.98599 | 0.14063 | 1.0944 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -8 | 20 | 5 | -0.72527 | 0.18477 | 0.94237 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 5 | 1 | -1.8501 | -0.015161 | 1.6088 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 5 | 3 | -0.94233 | 0.087047 | 0.95403 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 5 | 5 | -0.69397 | 0.1099 | 0.79351 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 10 | 1 | -1.8501 | -0.015161 | 1.6088 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 10 | 3 | -0.94233 | 0.087047 | 0.95403 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 10 | 5 | -0.69397 | 0.1099 | 0.79351 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 20 | 1 | -1.8501 | -0.015161 | 1.6088 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 20 | 3 | -0.94233 | 0.087047 | 0.95403 |
| 30000 | AWGN_3334DOPPLER_HZ | 0 | -7 | 20 | 5 | -0.69397 | 0.1099 | 0.79351 |