3GPP TSG RAN WG4 Meeting #93	R4-1914728

Reno, USA, November 18 - 22, 2019

Agenda Item:	8.10.3.2

Source: 	Ericsson

Title:	Rel-16 MTC WUS simulation results

Document for:	Discussions

# Introduction

At previous RAN4 (#92bis) meeting, RAN4 discussed WUS reception requirements and reached following agreement [1]:

- Based on the Rel-16 WUS contributions provided to RAN4#92bis meeting, following options are possible: - Option 1: : Rel-15 WUS reception requirements apply for Rel-16 UE. - Option 2: Required number of repetitions for WUS reception with 1 tx antenna when DRX cycle length is ≤ 1.28 s is increased from 32 to 64. Others can be kept unchanged. - Companies are expected to further study the WUS detection performance of each WUS sequence assuming UE monitors two sequences based on Rel-15 WUS framework in R4-1816334 - RAN4 shall reach agreement at RAN4#93 meeting.

In this contribution we present the simulation result based on agreed assumptions.

# Discussion

Table 1 WUS simulation results with 1 Tx

| SNR (dB) | DRX Cycle (s) | EPA1 (no coherent comb) | ETU1 (no coherent comb) | EPA5 (no coherent comb) | ETU30 (no coherent comb) | EPA1 (2sf coherent comb) | ETU1 (2sf coherent comb) | EPA5 (2sf coherent comb) | ETU30 (2sf coherent comb) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -6 | 0.32 |  | 2 | 32 | 2 |  | 2 | 32 | 2 |
|  | 0.64 |  | 2 | 32 | 2 |  | 2 | 32 | 2 |
|  | 1.28 |  | 2 | 32 | 2 |  | 2 | 32 | 2 |
|  | 2.56 |  |  | 32 | 4 |  |  |  | 8 |
| -12 | 0.32 |  | 8 | 32 | 8 |  | 8 | 32 | 8 |
|  | 0.64 |  | 8 | 32 | 8 |  | 8 | 32 | 8 |
|  | 1.28 |  | 8 | 32 | 8 |  | 16 | 32 | 8 |
|  | 2.56 |  |  | 32 | 8 |  |  |  | 16 |

Table 2 WUS simulation results with 2 Tx

| SNR (dB) | DRX Cycle (s) | EPA1 (no coherent comb) | ETU1 (no coherent comb) | EPA5 (no coherent comb) | ETU30 (no coherent comb) | EPA1 (2sf coherent comb) | ETU1 (2sf coherent comb) | EPA5 (2sf coherent comb) | ETU30 (2sf coherent comb) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -6 | 0.32 | 4 | 2 | 4 | 2 | 4 | 2 | 4 | 2 |
|  | 0.64 | 4 | 2 | 4 | 2 | 4 | 2 | 4 | 2 |
|  | 1.28 | 4 | 2 | 4 | 2 | 4 | 2 | 4 | 2 |
|  | 2.56 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| -12 | 0.32 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
|  | 0.64 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
|  | 1.28 | 8 | 4 | 8 | 4 | 8 | 4 | 4 | 4 |
|  | 2.56 | 8 | 4 | 8 | 4 | 32 | 16 | 32 | 16 |

Based on the results in Table 1 and 2, the existing UE requirements in [2] can be modified follows shown in Table 3 and 4. These results show there is no impact on required number of repetitions for the case highlighted in the agreed way forward, i.e. when DRX cycle ≤ 1.28 s when compared to legacy UE requirements. Hence, we support option 1.

Table 3: Conditions for WUS reception for UE normal coverage level

| DRX cycle length [s] | Required number of repetition of WUS signal with 1 transmit antenna | Required number of repetition of WUS signal with 2 transmit antennas |
| --- | --- | --- |
| ≤ 1.28 | 32 | 4 |
| > 1.28 | 8 for ETU and EPA results n/a | 4 |

Table 4: Conditions for WUS reception for UE enhanced coverage level

| DRX cycle length [s] | Required number of repetition of WUS signal with 1 transmit antenna | Required number of repetition of WUS signal with 2 transmit antennas |
| --- | --- | --- |
| ≤ 1.28 | 16 for ETU, EPA results n/a | 8 |
| > 1.28 | n/a | 32 |

- Proposal: Rel-15 WUS reception requirements apply for Rel-16 UE.

# Summary

We have in this contribution presented simulation results for release 16 WUS reception performance to down select between option 1 and option 2 as agreed in [1]. It is observed from these results that there is no impact for case highlighted in option 2. Hence, we propose to reuse the release 15 WUS requirements.

- Proposal: Rel-15 WUS reception requirements apply for Rel-16 UE.

# References

- R4-1912735, “Way forward on Rel-16 MTC RRM enhancement”, Ericsson, Huawei, HiSilicon, Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, October 2019

- TS 36.133 V15.7.0