3GPP TSG-RAN WG4 Meeting #93	R4-1914789

Reno, US, 18 November – 22 November 2019

Source:	ZTE

Title:	Discussion on definition of CSI-RS based RRM measurements

Agenda item:	9.16.1.3

Document for:	Discussion

1. Introduction

In the RAN4#92bis meeting, there were initial discussions on CSI-RS based measurements. The discussions were mainly focused on definition of intra/inter frequency measurements and progress was captured in way forward [1].

In this contribution we further provide our views on UE measurement capability of CSI-RS based measurements.

2. Discussion

The UE measurement capability of monitoring multiple layers using gaps are specified for SSB based measurement. For example the UE measurement capability for SA in TS38.133 are as follows.

When monitoring of multiple inter-RAT E-UTRAN carriers and inter-frequency NR carriers using gaps (or without using gaps provided the UE supports such capability) is configured by PCell, the UE shall be capable of performing one measurement of the configured measurement type (SS-RSRP, SS-RSRQ, SS-SINR, E-UTRAN RSRP, E-UTRAN RSRQ, E-UTRAN RS-SINR measurements, etc.) of detected cells on all the layers. …… If a UE is configured with SA NR operation mode, the UE shall be capable of monitoring at least: - Depending on UE capability, 7 NR inter-frequency carriers configured by PCell, and

When CSI-RS based RRM is also configured, new UE capabilities of monitoring multiple layers using gaps should be defined. As CSI-RS based measurement object could be different from SSB based measurement object in terms of frequency layer. Sharing UE capability of monitoring multiple layers using gaps by SSB based measurement and CSI-RS based measurement will degrade UE mobility performance as less number of layers for SSB based measurement can be configured.

Proposal 1. Separated UE capability of monitoring multiple layers using gaps for CSI-RS based measurement for L3 mobility should be specified.

It may be reasonable to reuse the same capability for SSB based measurement, i.e. 7 inter frequency layers for CSI-RS based measurement.

Proposal 2. UE shall be capable of monitoring at least 7 NR inter frequency layers for CSI-RS based measurement for L3 mobility

The UE capability of number of cells and number of SSBs that the UE shall be capable of performing SSB based measurement are also specified for intra frequency measurement and inter frequency measurement in FR1 and FR2 respectively. Since it is clearly specified for SSB based measurement, new UE capability of number of cells and number of CSI-RS resources should be specified for CSI-RS based measurements.

Proposal 3. UE capability of number of cells and number of CSI-RS resources that the UE shall be capable of performing CSI-RS based measurement for L3 mobility should be specified.

Regarding the number of cells it is reasonable that UE is capable of performing CSI-RS based intra frequency measurements and inter frequency measurements with the same capability as of performing SSB based intra frequency measurements and inter frequency measurements. It is proposed to use the same number of cells for SSB..

Proposal 4. UE shall be capable of performing CSI-RS based measurements for at least [8] identified cells in FR1 for intra frequency measurement and at least [4] identified cells in FR1 for inter frequency measurement.

Proposal 5. UE shall be capable of performing CSI-RS based measurements for at least [6] identified cells in FR2 for intra frequency measurement and at least [4] identified cells in FR2 for inter frequency measurement.

Regarding the number of CSI-RS resources that UE shall be capable of performing measurement over, maybe more studies are needed. In general, we think larger number of CSI-RS resources than number of SSBs should be specified as CSI-RS resources are supposed be finer beams than SSBs. The configurable CSI-RS resources for a cell could be 96 if MO is configured with associatedSSB, or 64 if not all the CSI-RS resources in the MO is configured with associatedSSB.

In TS 38.133, the UE measurement capability on number of SSBs to be monitored during L1 measurement period is as follows.

For intra frequency measurements in FR1,

-	14 SSBs with different SSB index and/or PCI on the intra-frequency layer, where the number of SSBs in the serving cell (except for the SCell) is no smaller than the number of configured RLM-RS SSB resources.

For intra frequency measurements in FR2,

-	24 SSBs with different SSB index and/or PCI,

For inter frequency measurements in FR1,

-	7 SSBs with different SSB index and/or PCI on the inter-frequency layer.

For inter frequency measurements in FR2,

-	10 SSBs with different SSB index and/or PCI on the inter-frequency layer, and

Since CSI-RS beams are finer, more beams can be measured by UE. NW can get more useful information.

Proposal 6. Number of CSI-RS resources shall be monitored by UE,

-	[24] SSBs for intra frequency measurements in FR1

-	[48] SSBs for intra frequency measurements in FR2,

-	[16] SSBs for inter frequency measurements in FR1,

-	[24] SSBs for inter frequency measurements in FR2,

3. Conclusion

In this contribution, we provided our views on on aspects of RRM requirements for CSI-RS based L3 measurements. Based on the observations following proposals are present.

Proposal 1. Separated UE capability of monitoring multiple layers using gaps for CSI-RS based measurement for L3 mobility should be specified.

Proposal 2. UE shall be capable of monitoring at least 7 NR inter frequency layers for CSI-RS based measurement for L3 mobility

Proposal 3. UE capability of number of cells and number of CSI-RS resources that the UE shall be capable of performing CSI-RS based measurement for L3 mobility should be specified.

Proposal 4. UE shall be capable of performing CSI-RS based measurements for at least [8] identified cells in FR1 for intra frequency measurement and at least [4] identified cells in FR1 for inter frequency measurement.

Proposal 5. UE shall be capable of performing CSI-RS based measurements for at least [6] identified cells in FR2 for intra frequency measurement and at least [4] identified cells in FR2 for inter frequency measurement.

Proposal 6. Number of CSI-RS resources shall be monitored by UE,

-	[24] SSBs for intra frequency measurements in FR1

-	[48] SSBs for intra frequency measurements in FR2,

-	[16] SSBs for inter frequency measurements in FR1,

-	[24] SSBs for inter frequency measurements in FR2,

4. References

- R4-1910956	WF on CSI-RS based RRM measurements requirements, CATT

- R4-1914790	Further discussion on definition of CSI-RS based RRM measurements, ZTE