3GPP TSG-RAN4 Meeting #93	R4-1914922

# Reno, US, 18th – 22nd November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0327 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FR2 CSI-RS RLM test OOS/IS non-DRX for EN-DC (section A.5.5.1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-09-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Endorsed in R4-1911966 in RAN4#92bis. New change is that CCR.3.2 which is optional for UE to support is changed to CCR.3.3 which all UE can support. In RLM test, BFD-RS should be configured to a different RS than RLM-RS to avoid beam failure in the test. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update CSI-RS RLM test case for FR2 for above issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CSI-RS RLM test case for FR2 are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.5.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode

##### A.5.5.1.5.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PSCell when no DRX is used. This test will partly verify the FR2 TDD PSCell CSI-RS Out-of-sync radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.5.5.1.5.1-1, A.5.5.1.5.1-2, A.5.5.1.5.1-3 and A.5.5.1.5.1-3A below. There are two cells, cell 1 is the E-UTRAN PCell, and cell 2 is the PSCell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.5.5.1.5.1-1 shows the variation of the downlink SNR in the E-UTRAN PCell and the PSCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity defined in CSI-RS configuration. In the test, DRX configuration is not enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms). In the test, SSB0 and SSB1 are configured as BFD-RS.

Table A.5.5.1.5.1-1: Supported test configurations for FR2 PSCell

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR2 |  |

Table A.5.5.1.5.1-2: General test parameters for FR2 PSCell for CSI-RS out-of-sync testing in non-DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active E-UTRA PCell |  |  | Cell 1 |
| E-UTRA RF Channel Number |  |  | 1 |
| Active PSCell |  |  | Cell 2 |
| RF Channel Number |  |  | 2 |
| Duplex Mode |  |  | TDD |
| TDD Configuration | Config 1 |  | TDDConf.3.1 |
|  | Config 2 |  | TDDConf.3.1 |
| DL initial BWP configuration | Config 1, 2 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1, 2 |  | DLBWP.1.1 |
| UL initial BWP configuration | Config 1, 2 |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1, 2 |  | ULBWP.1.1 |
| RMC CORESET Reference Channel | Config 1 |  | CCR.3.1 TDD CCR.3.3CCR.3.3CCR.3.3 TDD |
|  | Config 2 |  | CCR.3.1 TDD CCR.3.3CCR.3.3CCR.3.3 TDD |
| SSB Configuration | Config 1 |  | SSB.1 FR2 |
|  | Config 2 |  | SSB.1 FR2 |
| SMTC Configuration | Config 1 |  | SMTC.1 |
|  | Config 2 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1 |  | 120 KHz |
|  | Config 2 |  | 120 KHz |
| CSI-RS for RLM | Config 1, 2 |  | Resource #4 in TRS.2.1 TDD Resource #4 in TRS.2.2 TDD |
| TRS configuration |  |  | TRS.2.1 TDD TRS.2.2 TDD |
| TCI configuration for PDCCH#1/PDSCH |  |  | TCI.State.2 |
| TCI configuration for PDCCH#2 |  |  | TCI.State.3 |
| OCNG parameters |  |  | OP.1 |
| CP length |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | OFF |
| Gap pattern ID |  |  | gp0 |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 0 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS for CSI reporting | Config 1 |  | CSI-RS.3.1 TDD |
|  | Config 2 |  | CSI-RS.3.1 TDD |
| T1 |  | s | 10.2 |
| T2 |  | s | 00.35 |
| T3 |  | s | 00.35 |
| D1 |  | s | 00.31 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: E-UTRAN is in non-DRX mode under test. |  |  |  |

Table A.5.5.1.5.1-3: Cell specific test parameters for FR2 for CSI-RS out-of-sync radio link monitoring in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| PDCCH_beta |  | dB | 4 |  |  |
| PDCCH_DMRS_beta |  | dB | 4 |  |  |
| PBCH_beta |  | dB | 0 |  |  |
| PSS_beta |  | dB |  |  |  |
| SSS_beta |  | dB |  |  |  |
| PDSCH_beta |  | dB |  |  |  |
| OCNG_beta |  | dB |  |  |  |
| SNR on RLM-RS1 | Config 1,2 | dB | [2 | [-6 | [-15 |
| SNR on RLM-RS2 | Config 1,2 |  | [2 | [-14 | [-15 |
| SNR on other channels and signals | Config 1, 2 | dB | 12 |  |  |
|  | Config 1 | dBm/15KHz | TBD T |  |  |
|  | Config 2 |  |  |  |  |
| Propagation condition |  |  | [TDL-A 30ns 75Hz] |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.5.5.1.5.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is [A.3.6]. |  |  |  |  |  |

Table A.5.5.1.5.1-3A: Measurement gap configuration for FR2 CSI-RS out-of-sync radio link monitoring in non-DRX mode

| Field | Test 1 |
| --- | --- |
|  | Value |
| gapOffset | 0 |
| Note 1: E-UTRAN PCell and PSCell are SFN-synchronous and frame boundary aligned. (Ensure that RLM RS is partially overlapped with measurement gap) |  |

Table A.5.5.1.5.1-4: Void

Figure A.5.5.1.5.1-1: SNR variation for CSI-RS out-of-sync testing

##### A.5.5.1.5.2	Test Requirements

The UE behaviour during time durations T1, T2, and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 2 (PSCell) at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 2.

The UE shall stop transmitting uplink signal in Cell 2 (PSCell) no later than time point C (D1 after the start of the time duration T3) on the PSCell.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode

##### A.5.5.1.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the in sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PSCell when no DRX is used. This test will partly verify the FR2 TDD PSCell CSI-RS In-sync radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.5.5.1.6.1-1, A.5.5.1.6.1-2, and A.5.5.1.6.1-3 below. There are two cells, cell 1which is the E-UTRAN PCell, and cell 2 is the PSCell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.5.5.1.6.1-1 shows the variation of the downlink SNR in the PSCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity defined in CSI-RS configuration. In the test, DRX configuration is not enabled. In the test, SSB0 and SSB1 are configured as BFD-RS.

Table A.5.5.1.6.1-1: Supported test configurations for FR2 PSCell

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR2 |  |

Table A.5.5.1.6.1-2: General test parameters for FR2 PSCell for CSI-RS in-sync testing in non-DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active E-UTRA PCell |  |  | Cell 1 |
| E-UTRA RF Channel Number |  |  | 1 |
| Active PSCell |  |  | Cell 2 |
| RF Channel Number |  |  | 2 |
| Duplex Mode |  |  | TDD |
| TDD Configuration | Config 1 |  | TDDConf.3.1 |
|  | Config 2 |  | TDDConf.3.1 |
| DL initial BWP configuration | Config 1, 2 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1, 2 |  | DLBWP.1.1 |
| UL initial BWP configuration | Config 1, 2 |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1, 2 |  | ULBWP.1.1 |
| RMC CORESET Reference Channel | Config 1 |  | CCR.3.1 TDD CCR.3.3CCR.3.3CCR.3.3 TDD |
|  | Config 2 |  | CCR.3.1 TDD CCR.3.3CCR.3.3CCR.3.3 TDD |
| SSB Configuration | Config 1 |  | SSB.1 FR2 |
|  | Config 2 |  | SSB.1 FR2 |
| SMTC Configuration | Config 1 |  | SMTC.1 |
|  | Config 2 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1 |  | 120 KHz |
|  | Config 2 |  | 120 KHz |
| CSI-RS for RLM | Config 1, 2 |  | Resource #4 in TRS.2.1 TDD Resource #4 in TRS.2.2 TDD |
| OCNG parameters |  |  | OP.1 |
| TRS configuration |  |  | TRS.2.1 TDD TRS.2.2 TDD |
| TCI configuration for PDCCH#1/PDSCH |  |  | TCI.State.2 |
| TCI configuration for PDCCH#2 |  |  | TCI.State.3 |
| CP length |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| In sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 0 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | OFF |
| Gap pattern ID |  |  | N.A. |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 01000 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS for CSI reporting | Config 1 |  | CSI-RS.3.1 TDD |
|  | Config 2 |  | CSI-RS.3.1 TDD |
| T1 |  | s | 10.2 |
| T2 |  | s | 00.2 |
| T3 |  | s | [0.24 |
| T4 |  | s | 00.2 |
| T5 |  | s | 10.88 |
| D1 |  | s | [0.84 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: E-UTRAN is in non-DRX mode under test. |  |  |  |

Table A.5.5.1.6.1-3: Cell specific test parameters for FR2 for CSI-RS in-sync radio link monitoring in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| PDCCH_beta |  | dB | 4 |  |  |  |  |
| PDCCH_DMRS_beta |  | dB | 4 |  |  |  |  |
| PBCH_beta |  | dB | 0 |  |  |  |  |
| PSS_beta |  | dB |  |  |  |  |  |
| SSS_beta |  | dB |  |  |  |  |  |
| PDSCH_beta |  | dB |  |  |  |  |  |
| OCNG_beta |  | dB |  |  |  |  |  |
| SNR on RLM-RS1 | Config 1, 2 | dB | [2 | [-6 | [-15 | [-4.5 | [2 |
| SNR on RLM-RS2 | Config 1, 2 | dB | [2 | [-14 | [-15] | [-15] | [-14 |
| SNR on other channels and signals | Config 1, 2 | dB | 12 |  |  |  |  |
|  | Config 1, 2 | dBm/15KHz | TBD |  |  |  |  |
| Propagation condition |  |  | [TDL-A 30ns 75Hz] |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.5.5.1.6.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is [A.3.6]. |  |  |  |  |  |  |  |

Table A.5.5.1.6.1-3A: Void

Table A.5.5.1.6.1-4: Void

Figure A.5.5.1.6.1-1: SNR variation for CSI-RS in-sync testing

##### A.5.5.1.6.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting on the PSCell.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode

##### A.5.5.1.7.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PSCell when no DRX is used. This test will partly verify the FR2 TDD PSCell CSI-RS Out-of-sync radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.5.5.1.7.1-1, A.5.5.1.7.1-2, and A.5.5.1.7.1-3 below. There are two cells, cell 1 is the E-UTRAN PCell, and cell 2 is the PSCell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.5.5.1.7.1-1 shows the variation of the downlink SNR in the E-UTRAN PCell and the PSCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity defined in CSI-RS configuration. In the test, DRX configuration is enabled in PSCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test. In the test, SSB0 and SSB1 are configured as BFD-RS.

Table A.5.5.1.7.1-1: Supported test configurations for FR2 PSCell

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR2 |  |

Table A.5.5.1.7.1-2: General test parameters for FR2 PSCell for CSI-RS out-of-sync testing in DRX mode

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active E-UTRA PCell |  |  |  | Cell 1 |
| E-UTRA RF Channel Number |  |  |  | 1 |
| Active PSCell |  |  |  | Cell 2 |
| RF Channel Number |  |  |  | 2 |
| Duplex Mode |  |  |  | TDD |
| TDD Configuration |  | Config 1 |  | TDDConf.3.1 |
|  |  | Config 2 |  | TDDConf.3.1 |
| DL initial BWP configuration |  | Config 1, 2 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1, 2 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1, 2 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1, 2 |  | ULBWP.1.1 |
| RMC CORESET Reference Channel |  | Config 1 |  | CCR. 3.1 TDD CCR.3.3CCR.3.3CCR.3.3 TDD |
|  |  | Config 2 |  | CCR. 3.1 TDD CCR.3.3CCR.3.3CCR.3.3 TDD |
| SSB Configuration |  | Config 1 |  | SSB.1 FR2 |
|  |  | Config 2 |  | SSB.1 FR2 |
| SMTC Configuration |  | Config 1 |  | SMTC.1 |
|  |  | Config 2 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 |  | 120 KHz |
|  |  | Config 2 |  | 120 KHz |
| CSI-RS for RLM |  | Config 1, 2 |  | Resource #4 in TRS.2.1 TDD Resource #4 in TRS.2.2 TDD |
| TRS configuration |  |  |  | TRS.2.1 TDD TRS.2.2 TDD |
| TCI configuration for PDCCH#1/PDSCH |  |  |  | TCI.State.2 |
| TCI configuration for PDCCH#2 |  |  |  | TCI.State.3 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX |  |  |  | DRX.3 |
| Gap pattern ID |  |  |  | N.A. |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 0 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS for CSI reporting | Config 1 |  |  | CSI-RS.3.1 TDD |
|  | Config 2 |  |  | CSI-RS.3.1 TDD |
| T1 |  |  | s | 10.2 |
| T2 |  |  | s | 01.28 |
| T3 |  |  | s | [1.28 |
| D1 |  |  | s | [1.24 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: E-UTRAN is in non-DRX mode under test. |  |  |  |  |

Table A.5.5.1.7.1-3: Cell specific test parameters for FR2 for CSI-RS out-of-sync radio link monitoring in DRX mode

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| PDCCH_beta |  | dB | 4 |  |  |
| PDCCH_DMRS_beta |  | dB | 4 |  |  |
| PBCH_beta |  | dB | 0 |  |  |
| PSS_beta |  | dB |  |  |  |
| SSS_beta |  | dB |  |  |  |
| PDSCH_beta |  | dB |  |  |  |
| OCNG_beta |  | dB |  |  |  |
| SNR on RLM-RS1 | Config 1, 2 | dB | [2 | [-6 | [-15] |
| SNR on RLM-RS2 | Config 1, 2 |  | [2 | [-14 | [-15] |
| SNR on other channels and signals | Config 1, 2 | dB | 12 |  |  |
|  | Config 1 | dBm/15KHz | T-104.7 |  |  |
|  | Config 2 |  | T-104.7 |  |  |
| Propagation condition |  |  | [TDL-A 30ns 75Hz] |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.5.5.1.7.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is [A.3.6]. |  |  |  |  |  |

Table A.5.5.1.7.1-3A: Void

Table A.5.5.1.7.1-4: Void

Table A.5.5.1.7.1-5: Void

Table A.5.5.1.7.1-6: Void

Figure A.5.5.1.7.1-1: SNR variation for CSI-RS out-of-sync testing

##### A.5.5.1.7.2	Test Requirements

The UE behaviour during time durations T1, T2, and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 2 (PSCell) at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 2.

The UE shall stop transmitting uplink signal in Cell 2 (PSCell) no later than time point C (D1 after the start of the time duration T3) on the PSCell.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode

##### A.5.5.1.8.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the in sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PSCell when no DRX is used. This test will partly verify the FR2 TDD PSCell CSI-RS In-sync radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.5.5.1.8.1-1, A.5.5.1.8.1-2, A.5.5.1.8.1-3 and A.5.5.1.8.1-3A below. There are two cells, cell 1which is the E-UTRAN PCell, and cell 2 is the NR PSCell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.5.5.1.8.1-1 shows the variation of the downlink SNR in the PSCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity defined in CSI-RS configuration. In the test, DRX configuration is not enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms). In the test, SSB0 and SSB1 are configured as BFD-RS.

Table A.5.5.1.8.1-1: Supported test configurations for FR2 PSCell

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR2 |  |

Table A.5.5.1.8.1-2: General test parameters for FR2 PSCell for CSI-RS in-sync testing in non-DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active E-UTRA PCell |  |  | Cell 1 |
| E-UTRA RF Channel Number |  |  | 1 |
| Active PSCell |  |  | Cell 2 |
| RF Channel Number |  |  | 2 |
| Duplex Mode |  |  | TDD |
| TDD Configuration | Config 1 |  | TDDConf.3.1 |
|  | Config 2 |  | TDDConf.3.1 |
| DL initial BWP configuration | Config 1, 2 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1, 2 |  | DLBWP.1.1 |
| UL initial BWP configuration | Config 1, 2 |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1, 2 |  | ULBWP.1.1 |
| RMCCORESET Reference Channel | Config 1 |  | CCR.3.1 TDD CCR.3.3CCR.3.3CCR.3.3 TDD |
|  | Config 2 |  | CCR.3.1 TDD CCR.3.3CCR.3.3CCR.3.3 TDD |
| SSB Configuration | Config 1 |  | SSB.1 FR2 |
|  | Config 2 |  | SSB.1 FR2 |
| SMTC Configuration | Config 1 |  | SMTC.1 |
|  | Config 2 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1 |  | 120 KHz |
|  | Config 2 |  | 120 KHz |
| CSI-RS for RLM | Config 1, 2 |  | Resource #4 in TRS.2.1 TDD Resource #4 in TRS.2.2 TDD |
| TRS configuration |  |  | TRS.2.1 TDD TRS.2.2 TDD |
| TCI configuration for PDCCH#1/PDSCH |  |  | TCI.State.2 |
| TCI configuration for PDCCH#2 |  |  | TCI.State.3 |
| OCNG parameters |  |  | OP.1 |
| CP length |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| In sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 0 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | DRX.3 |
| Gap pattern ID |  |  | gp0 |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 02000 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS for CSI reporting | Config 1 |  | CSI-RS.3.1 TDD |
|  | Config 2 |  | CSI-RS.3.1 TDD |
| T1 |  | s | 10.2 |
| T2 |  | s | 00.2 |
| T3 |  | s | [1.64 |
| T4 |  | s | 00.2 |
| T5 |  | s | 41.88 |
| D1 |  | s | [1.84 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: E-UTRAN is in non-DRX mode under test. |  |  |  |

Table A.5.5.1.8.1-3: Cell specific test parameters for FR2 for CSI-RS in-sync radio link monitoring in DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| PDCCH_beta |  | dB | 4 |  |  |  |  |
| PDCCH_DMRS_beta |  | dB | 4 |  |  |  |  |
| PBCH_beta |  | dB | 0 |  |  |  |  |
| PSS_beta |  | dB |  |  |  |  |  |
| SSS_beta |  | dB |  |  |  |  |  |
| PDSCH_beta |  | dB |  |  |  |  |  |
| OCNG_beta |  | dB |  |  |  |  |  |
| SNR on RLM-RS1 | Config 1, 2 | dB | [2 | [-6 | [-15] | [-4.5] | [2 |
| SNR on RLM-RS2 | Config 1, 2 | dB | [2 | [-14 | [-15] | [-15] | [-14 |
| SNR on other channels and signals | Config 1, 2 | dB | 12 |  |  |  |  |
|  | Config 1, 2 | dBm/15KHz | T-104.7 |  |  |  |  |
| Propagation condition |  |  | [TDL-A 30ns 75Hz] |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.5.5.1.8.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is [A.3.6]. |  |  |  |  |  |  |  |

Table A.5.5.1.8.1-3A: Measurement gap configuration for FR2 CSI-RS in-sync radio link monitoring in DRX mode

| Field | Test 1 |
| --- | --- |
|  | Value |
| gapOffset | 0 |
| Note 1: E-UTRAN PCell and PSCell are SFN-synchronous and frame boundary aligned. (Ensure that RLM RS is partially overlapped with measurement gap) |  |

Table A.5.5.1.8.1-4: Void

Table A.5.5.1.8.1-5: Void

Table A.5.5.1.8.1-6: Void

Figure A.5.5.1.8.1-1: SNR variation for CSI-RS in-sync testing

##### A.5.5.1.8.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting on the PSCell.

The rate of correct events observed during repeated tests shall be at least 90%.

<End of Change 1>