#### 3GPP TSG RAN WG4 Meeting #93 				R4- 1915048

# Reno, USA, 18 - 22 November, 2019

Agenda Item:	11.2.3.1

Source:	Spirent Communications

Title:	Channel Model Validation for MIMO OTA FR1 Testing

Document for:	Approval

# 1	Introduction

The channel models for MIMO OTA FR1 and FR2 are presented in [1]. This contribution outlines the channel model parameters to be validated, and the methods to be followed to perform such validation for FR1.

# 2	Details

In [1] the following parameters are to be validated:

- Power Delay Profile (PDP)

- Doppler/Temporal correlation

- Spatial correlation

- Cross-polarization

- Power validation

### Channel Model Validation

Proposal 1: Reuse the methodology in [2] to validate parameters 1, and 2 above.

Proposal 2: Introduce a new technique based on time-domain analysis to validate Spatial Correlation.

Signal

Analyzer

Figure 1. Spatial Correlation Validation setup.

A CW Signal Generator (SG) feeds one input of the Channel Emulator (CE). At the other end, collect the signal and feed it to a Signal Analyzer).

The procedure consists on recording time domain traces for each desired position in space (see proposals in [3]). After repositioning the antenna to the new point in space, the fading sequence is rewound before recording the new trace.

After that, regular crosscorrelation calculation procedures are performed as described in [2].

The suggested settings for the SG are in Table 1 below.

Table 1: SG settings.

| Item | Unit | Value |
| --- | --- | --- |
| Center frequency | MHz | Downlink centre frequency in 3GPP as required per band |
| Output power | dBm | Function of the CE. Sufficiently above Noise Floor |

The suggested settings for the CE are in Table 2 below.

Table 2: CE settings.

| Item | Unit | Value |
| --- | --- | --- |
| Center frequency | MHz | Downlink centre frequency in 3GPP as required per band |
| Output power | dBm | Sufficiently above Noise Floor |
| Input power | dBm | Matched to output power of SG |
| Channel |  | As per required CDL |
| Mobile Speed | Km/hr | As per required CDL |
| Doppler Spread |  | As per required CDL |

The suggested settings for the Signal Analyzer are in Table 3 below.

Table 3: Signal Analyzer settings.

| Item | Unit | Value |
| --- | --- | --- |
| Center frequency | MHz | Downlink centre frequency in 3GPP as required per band |
| Sampling | Hz | At least 8 times bigger than the max Doppler spread |
| Observation time | s | TBD |

Proposal 3: Use the same procedure for spatial correlation to validate the Vertical to Horizontal ratio (an indirect measure of the crosspolarization ratio). For this purpose, a Vertical dipole is placed in the center of the test volume, and a single time-domain trace is taken (this is the power in the Vertical Polarization). Then, the Vertical dipole is replaced by a horizontal loop, and a single time-domain trace is taken (this is the power in the Horizontal Polarization). If a horizontal loop is not available, a horizontally polarized dipole can be used with the following modification:

- Time-domain traces are taken for each probe angle by disabling all other probes. This is repeated 16 times. Then the power for each of the 16 traces is added (this is the power in the Horizontal Polarization).

Proposal 4: Reuse the data taken for V/H as power validation.

# 3	Conclusions

This contribution has presented a way to validate channel models given in 1 [1].

Proposal 1: Reuse the methodology in [2] to validate parameters 1, and 2 above.

Proposal 2: Introduce a new technique based on time-domain analysis to validate Spatial Correlation.

Proposal 3: Use the same procedure for spatial correlation to validate the Vertical to Horizontal ratio (an indirect measure of the crosspolarization ratio).

Proposal 4: Reuse the data taken for V/H as power validation.

Instrument settings not specified in this contribution are FFS.

# 4	References

[1] 3GPP TR 38.827, “Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE), (Release 16),” 3GPP, October 2019.

[2] TR 37.977, “Verification of radiated multi-antenna reception performance of User Equipment (UE) (Release 14),” 3GPP, September 2017.

[3] R4-1911637, WF on NR MIMO OTA, CAICT, Spirent, CMCC, SGS Wireless, 3GPP TSG-RAN WG4 Meeting #92Bis, Chongqing, China, October 14-18, 2019.