3GPP TSG-RAN WG4 #93		R4-1915079
Reno, NV, USA, 18 – 22 November 2019

Agenda item:	11.2.3.2

Source:	Keysight Technologies

Title:	System Implementation of FR2 3D MPAC Systems

Document for:	Approval

# Introduction

Investigations have been carried out on designing 3D MPAC systems for NR FR2 MIMO OTA testing [1] and to evaluate the impact of key design parameters (e.g., the number of probes, range length, DUT-array offset, channel model, etc.) on the OTA system performance [2], [3], and [4]. In the WF from RAN4#92 [5], it was agreed to decide on limits for PSP (i.e., the validation metric used in FR2 MIMO OTA testing) for the down selected channel models (InO CDL-A and UMi CDL-C). To this end, in our previous study [6], we have provided the initial results to illustrate the behaviour of PSP with the DUT offset for the two selected channel models in the WF.

In this contribution, we further extend our investigations on suitable system implementation of FR2 3D MPAC systems with the goal to defining the limits of PSP, the number of probes, and the range length.  Specifically, we evaluate the PSP by sampling a large number of DUT offsets in 3D within the spherical DUT test zone, for InO CDL-A and UMi CDL-C models with range lengths = 1 m and 0.75 m.

# Simulation Model and Results

In the simulations, the BS antenna array is an 8×16 uniform rectangular array (URA) with half wavelength inter-element spacing. In total, 128 fixed beams are constructed from a grid of 8 elevation angles from –25 to +25 with ~7.1 step size and 16 azimuth angles from –60 to +60 with 8 step size [8]. For the study, the strongest beam is selected. A virtual 4×4 rectangular array with half wavelength inter-element spacing is used to observe PAS at the DUT-array and to determine the PSP. Then, for a predefined range length we evaluate the PSP at each feasible point in the DUT test zone.

To model the spherical DUT test zone, we first define a cube with base = 10 cm, see Figure 1. Then, we sample the cube such that the distance between two sampling points is 2 cm. To construct the spherical DUT test zone, we pick all the points from the sampled cube which satisfy the following condition;

(x0- xi)2+ (y0- yi)2+(z0- zi)2  ≤r,

where (x0,y0,z0) are the coordinates of the centre of the test zone, and  (xi,yi,zi) are the coordinates of any ith point in the cubic region, and r is the radius of the spherical test zone. Parameter r is defined in a way that the considered DUT array lies fully inside the DUT test zone. Based on the dimensions of the virtual 4x4 DUT array, we set r≈ 9.2 cm. Once the DUT test zone is defined, the PSP is evaluated at each point of the test zone.

Figure 1: Illustration of DUT test zone sampling with inter-sampling point distance = 2 cm.

In this study, we have reoptimized the probe locations (compared to the probe locations we have used in our previous studies [2], [3] and [4]) to be matched with the spatial characteristics of InO CDL-A and UMi CDL-C channel models. The selected probe locations support both these channel models without moving or switching off the probes. In the probe location optimization phase, a minimum angular separation of 3° between two probes is imposed [7]. The simulations are carried out for 28 GHz frequency band, and we have used the agreed NR MIMO OTA channel model implementation in [8].

Unfortunately, the simulations did not complete on time prior to the submission deadline and will be provided in a revised (late) contribution.

# Conclusion

The following observations and proposals were made in this contribution

Pending

# References

- R4-1814835 “On Feasibility of MPAC Systems for FR2”, Keysight Technologies, 3GPP TSG-RAN WG4 Meeting #89, November 2018

- R4-1900499, “3D MPAC System Proposal for FR2 NR MIMO OTA Testing”, Keysight Technologies, 3GPP TSG-RAN WG4 Meeting #90, February 2019

- R4-1906830, “DUT test zone size evaluation for UMi and InO at FR 2”, Keysight Technologies, 3GPP TSG-RAN WG4 Meeting #91, May 2019

- R4-1909729, “System Implementation of FR2 3D MPAC”, Keysight Technologies, 3GPP TSG-RAN WG4 Meeting #92, Aug 2019

- R4-1910609, “WF on NR MIMO OTA”, CAICT, Spirent, Keysight Technologies, Qualcomm, ETS-Lindgren, MVG, 3GPP TSG-RAN WG4 Meeting #92, Ljubljana, Aug 2019

- R4-1912105, “System Implementation of FR2 3D MPAC”, Keysight Technologies, 3GPP TSG-RAN WG4 Meeting #92bis, Oct 2019

- R4-1904586, MPAC setup -Feasibility study, MVG Industries, 3GPP TSG-RAN WG4 #90-bis, April 2019

- TR 38.827, Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE), V0.4.0 (2019-10)