3GPP TSG-RAN WG4 Meeting #93	R4-1915111

# Reno, Nevada, USA, 18 - 22 November, 2019

Title: 	Simulation results on NR FR1 normal CA UE performance requirements for TDD

Source: 	Huawei, HiSilicon

Agenda item:	9.18.1.1

Document for:	Discussion

Background

During RAN4#92bis meeting, simulation assumption for NR FR1 normal CA was approved, and the simulation assumption is extracted as shown below:

Single carrier SCS - FDD: 15kHz - TDD: 30kHz Single carrier channel bandwidth - FDD 15 kHz: 5, 10, 15, 20, 25, 30, 40, 50 MHz - TDD 30 kHz: 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 MHz TDD UL-DL pattern: 7DS2U with S=6:4:4 SSB configuration: Periodicity 20 ms, Allocated in first slot within 20ms PDSCH configuration - FDD: Type A, Start symbol 2, Duration 12 - TDD 30 kHz: - Full DL slots: Type A, Start symbol 2, Duration 12 - Special slots: Type A, Start symbol 2, Duration 4 DMRS configuration for FR1 and FR2 - Type 1, 1 additional DMRS, Single symbol PTRS configuration for FR2: Frequency density 2, Time density 1 TRS configuration for FR1 and FR2: 20 ms periodicity, 2 slots, Offset 10 ms HARQ process number: - FDD: 4 - TDD with 30 kHz SCS: 8 Rank: Rank 2 for 2Rx and 4Rx MCS: MCS 13 (16QAM, CR 1/2) Receiver type: MMSE-IRC Propagation condition: TDLA30-10 Antenna correlation for MMSE-IRC: ULA Low Throughput test point: 70% of max TP

In this contribution, we provide our simulation results on NR FR1 normal CA for UE performance requirement for TDD.

Simulations

Table 2-1: Ideal Simulation results for NR FR1 normal CA with TDD cases

| Case Number | Reference channel | Antenna configuration | SNR@70% Max TP |
| --- | --- | --- | --- |
| 1 | R.TDD 5 MHz | 2Tx2Rx | 10.99 |
| 2 | R.TDD 5 MHz | 2Tx4Rx | 5.41 |
| 3 | R.TDD 10 MHz | 2Tx2Rx | 11.4 |
| 4 | R.TDD 10 MHz | 2Tx4Rx | 5.55 |
| 5 | R.TDD 15 MHz | 2Tx2Rx | 11.54 |
| 6 | R.TDD 15 MHz | 2Tx4Rx | 5.76 |
| 7 | R.TDD 20 MHz | 2Tx2Rx | 11.26 |
| 8 | R.TDD 20 MHz | 2Tx4Rx | 5.57 |
| 9 | R.TDD 25 MHz | 2Tx2Rx | 11.26 |
| 10 | R.TDD 25 MHz | 2Tx4Rx | 5.57 |
| 11 | R.TDD 30 MHz | 2Tx2Rx | 11.42 |
| 12 | R.TDD 30 MHz | 2Tx4Rx | 5.72 |
| 13 | R.TDD 40 MHz | 2Tx2Rx | 11.44 |
| 14 | R.TDD 40 MHz | 2Tx4Rx | 5.65 |
| 15 | R.TDD 50 MHz | 2Tx2Rx | 11.72 |
| 16 | R.TDD 50 MHz | 2Tx4Rx | 5.73 |
| 17 | R.TDD 60 MHz | 2Tx2Rx | 11.56 |
| 18 | R.TDD 60 MHz | 2Tx4Rx | 5.75 |
| 19 | R.TDD 80 MHz | 2Tx2Rx | 11.89 |
| 20 | R.TDD 80 MHz | 2Tx4Rx | 5.94 |
| 21 | R.TDD 90 MHz | 2Tx2Rx | 12.06 |
| 22 | R.TDD 90 MHz | 2Tx4Rx | 6.01 |
| 23 | R.TDD 100 MHz | 2Tx2Rx | 12.2 |
| 24 | R.TDD 100 MHz | 2Tx4Rx | 6.21 |

Figure 2-1: Ideal Simulation results for NR FR1 normal CA with TDD cases

Observation: For NR FR1 normal CA with TDD cases, there is almostly the same for different channel bandwidth.

Conclusion

In this contribution, we give our simulation results on NR FR1 normal CA with TDD cases for UE performance requirement. Our observations and proposals are:

Observation: There is almostly the same for different channel bandwidth for NR FR1 normal CA for TDD.

Reference

- R4-1912810, Simulation assumptions for NR FR1 normal CA UE performance requirements, RAN4#92Bis, Intel Corporation