3GPP TSG-RAN WG4 Meeting #93	R4-1915119

# Reno, Nevada, USA, 18 - 22 November, 2019

Title: 	Discussion and simulation results on NR UE HST performance requirements under SFN

Source: 	Huawei, HiSilicon

Agenda item:	9.17.2.2.2

Document for:	Discussion

Background

During RAN4#92bis meeting, way forward [1] for NR Rel-16 UE demodulation was approved. In this contribution, we share our views about the UE demodulation requirements for NR Rel-16 HST under SFN channel model.

Discussion

Maximum Doppler shift

- For TDD 30KHz - For HST-SFN, maximum Doppler is - Option 1: 1500Hz - Option 2: 1667Hz - Other values between option 1 and option 2 are not precluded - For FDD 15KHz - HST-SFN, maximum Doppler is - Option 1: 712Hz - Option 2: 875Hz - Other values between option 1 and option 2 are not precluded

Option 1 is proposed considering FTL error, which can see detail in [2].

The following figure shows the Doppler shift and power trajectories for FDD for different RRHs with maximum Doppler shift 875Hz.

Figure 2.1-1: Doppler shift and power trajectories for FDD

For one path, frequency tracking compensation value should be near maximum Doppler shift (875Hz or -875Hz). For two path or more, frequency tracking compensation value should be in somewhere (blue line in Figure 2.1-1) between [-875Hz, 875Hz] based on power of different path.

Doppler spread is about 2 times maximum Doppler shift. Considering the most extreme cases, in above figure, the value of dotted lines which are with FTL error (±0.1ppm) is in range of about [-600Hz, 600Hz], and frequency shift for all paths are not greater than maximum frequency tracking capability. It can be calculatored by same method for TDD.

Observation 1: For SFN, maximum frequency tracking capability is not affected by FTL error no matter where UE is.

As per simulation results in Section 3, we can get the following proposal:

Observation 2: For FDD, both 712Hz and 875Hz are feasible, it is preferable to set maximum Doppler shift 875Hz. For TDD, both 1500Hz and 1667Hz are feasible, it is preferable to set maximum Doppler shift 1667Hz.

Proposal 1: It is proposed to adopt maximum Doppler shift 875Hz for FDD, 1667 for TDD.

MCS

- For all the channel models in this way forward, FFS on the MCS - Option 1: MCS4 - Option 2: MCS13 - Option 3: MCS17 - Other MCS are not precluded - MCS should be decided based on whether the maximum throughput can be achieved

As per simulation results in Section 3, we can get the following proposal:

Observation 3: For SFN, only MCS 4 and 13 are feasible. MCS 13 has better performance considering balance between throughput and SNR.

Proposal 2: It is proposed to adopt MCS 13 for SFN.

Simulations

- Updated simulation assumption for HST-SFN Parameter; Value. FDD 15KHz SCS; TDD 30KHz SCS. Antenna configuration; 2×2; 2×4. DMRS type; type 1. Number of DMRS symbols; DMRS 1+1+1. TDD pattern; 7D1S2U, S: 6D 4G 4U. MCS; MCS 4; MCS 13; MCS 17 based on 64QAM table. Propagation condition; HST-SFN. TRS periodicity; 10ms, 2slot pattern. PDSCH mapping; Type A, Start symbol 2, Duration 12. Ds and Dmin; Ds=700m, Dmin=150m. Rank; Rank = 2. BW; 10MHz; 40MHz. Maximum Doppler shift; Option 1: 875Hz Option 2: 712Hz; Option 1: 1667Hz Option 2: 1500Hz. Testing metric; SNR @70% of maximum throughput.

We provide our simulations as per the simulation assumptions in [1] for HST SFN as following:

Table 3-1: Ideal Simulation results for NR PDSCH under HST-SFN

| Case Number | CHBW/ SCS | Max Doppler shift fd (Hz) | MIMO | MCS | SNR@70% Max TP |
| --- | --- | --- | --- | --- | --- |
| 1 | 10MHz/15kHz | 712 | 2Tx 2Rx ULA Low | 4 | 0.41 |
| 2 |  |  |  | 13 | 9.58 |
| 3 |  |  |  | 17 | 15.56 |
| 4 |  |  | 2Tx 4Rx ULA Low | 4 | -2.18 |
| 5 |  |  |  | 13 | 7.27 |
| 6 |  |  |  | 17 | 13.11 |
| 7 |  | 875 | 2Tx 2Rx ULA Low | 4 | 0.66 |
| 8 |  |  |  | 13 | 10.23 |
| 9 |  |  |  | 17 | N/A |
| 10 |  |  | 2Tx 4Rx ULA Low | 4 | -1.95 |
| 11 |  |  |  | 13 | 7.60 |
| 12 |  |  |  | 17 | 17.79 |
| 13 | 40MHz/30kHz | 1500 | 2Tx 2Rx ULA Low | 4 | 0.87 |
| 14 |  |  |  | 13 | 11.00 |
| 15 |  |  |  | 17 | 18.77 |
| 16 |  |  | 2Tx 4Rx ULA Low | 4 | -1.67 |
| 17 |  |  |  | 13 | 8.57 |
| 18 |  |  |  | 17 | 15.79 |
| 19 |  | 1667 | 2Tx 2Rx ULA Low | 4 | 1.01 |
| 20 |  |  |  | 13 | 11.28 |
| 21 |  |  |  | 17 | N/A |
| 22 |  |  | 2Tx 4Rx ULA Low | 4 | -1.50 |
| 23 |  |  |  | 13 | 8.71 |
| 24 |  |  |  | 17 | N/A |

Figure 3-1: Ideal Simulation results for NR PDSCH under HST SFN

Proposals

In this contribution, we discuss and provide our simulation results on NR UE HST performance requirements under SFN channel model. Our observations and proposals are:

Observation 1: For SFN, maximum frequency tracking capability is not affected by FTL error no matter where UE is.

Observation 2: For FDD, both 712Hz and 875Hz are feasible, it is preferable to set maximum Doppler shift 875Hz. For TDD, both 1500Hz and 1667Hz are feasible, it is preferable to set maximum Doppler shift 1667Hz.

Observation 3: For SFN, only MCS 4 and 13 are all feasible. MCS 13 has better performance considering balance between throughput and SNR.

Proposal 1: It is proposed to adopt maximum Doppler shift 875Hz for FDD, 1667 for TDD.

Proposal 2: It is proposed to adopt MCS 13 for SFN.

Reference

- R4-1912808, WF on UE demodulation for NR HST, RAN4#92Bis, CMCC

- R4-1912423, Views on Tests for High Speed Train Scenarios, RAN4#92Bis, Qualcomm Incorporated