3GPP TSG-RAN WG4 Meeting #93	R4-1915228

Reno, Nevada, USA, 18 – 22 November 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on TRP MU discrepancies for CATR

Agenda item:	7.8.5.1

Document for:	Discussion

# 1	Introduction

A discrepancy has been identified in the “Misalignment DUT & pointing error” MU value for CATR for the following TRP requirements:

- OTA BS maximum output power

- OTA ACLR

- OTA spectrum emission mask

- OTA operating band unwanted emissions

- OTA TX spurious emissions

This document examines and attempts to resolve the discrepancy without changing the final MU value for each of the TRP requirements.

# 2	Discussion

## 2.1	TR 38.817-02

Referring to Section 12.6 in TR 38.817-02, MU tables for each of the TRP requirements are listed below.

OTA BS maximum output power:

Table 12.6.1.2.2.1-1: Compact antenna test range uncertainty assessment for EIRP measurements for BS output power

| UID (Note 1) | Uncertainty Source | Uncertainty value 24.25<f <29.5GHz | Uncertainty value 37<f <40GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB)(dB) 24.25<f <29.5GHz | Standard uncertainty ui (dB)(dB) 37<f <40GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| E2-1 | Misalignment DUT & pointing error | 0.35 | 0.35 | Exp. normal | 2 | 1 | 0,174 | 0,174 |

Observation 1: Uncertainty value = 0.35 and divisor based on distribution shape = 2.

OTA ACLR:

Table 12.6.2.2.2.1-1	CATR uncertainty assessment for relative ACLR measurement

| Relative ACLR uncertainty budget |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID (Note 1) | Uncertainty Source | Uncertainty value 24.25<f <29.5GHz | Uncertainty value 37<f <40GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB)(dB) 24.25<f <29.5GHz | Standard uncertainty ui (dB)(dB) 37<f <40GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| E2-1 | Misalignment DUT & pointing error | 0,3 | 0,3 | Exp. normal | 2 | 1 | 0,15 | 0,15 |

Observation 2: Uncertainty value = 0.3 and divisor based on distribution shape = 2.

OTA operating band unwanted emissions:

Table 12.6.3.2.2.1-1: Compact antenna test range uncertainty assessment for EIRP measurements for occupied bandwidth unwanted emissions

| UID (Note 1) | Uncertainty Source | Uncertainty value 24.25<f <29.5GHz | Uncertainty value 37<f <40GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB)(dB) 24.25<f <29.5GHz | Standard uncertainty ui (dB)(dB) 37<f <40GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| E2-1 | Misalignment DUT & pointing error | 0.3 | 0.3 | Exp. normal | 2 | 1 | 0,15 | 0,15 |

Observation 3: Uncertainty value = 0.3 and divisor based on distribution shape = 2.

OTA TX spurious emissions:

Table 12.7.1.2.2.2-1: Compact antenna test range uncertainty assessment for spurious emissions

| UID (Note) | Uncertainty source | 40 GHz < f ≤ 60 GHz | Distribution of the probability | Divisor based on distribution shape | ci | 40 GHz < f ≤ 60 GHz |
| --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |
| E2-1 (Note 1) | Misalignment DUT & pointing error | 0.3 | Rect | √3 | 1 | 0.173 |

Observation 4: Uncertainty value = 0.3 and divisor based on distribution shape = 3.

## 2.2	TR 37.843

Referring Section 10.4 in TR 37.843, MU tables for each of the TRP requirements are listed below.

OTA BS maximum output power:

Table 10.4.1.3.4.2-1: Compact antenna test range uncertainty assessment for OTA Base station output power measurement

| UID | Uncertainty Source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | [0.3] | [0.3] | Rectangular | √3 | 1 | [0.174] | [0.174] |

Observation 5: Uncertainty value = 0.3 and divisor based on distribution shape = 3.

OTA ACLR:

Table 10.4.2.3.4.2-1: Compact antenna test range uncertainty assessment for absolute ACLR measurement

| UID | Uncertainty Source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | [0.3] | [0.3] | Rectangular | √3 | 1 | [0.174] | [0.174] |

Observation 6: The MU is consistent with OTA BS maximum output power.

OTA operating band unwanted emissions:

Table 10.4.3.3.4.2-1: Compact antenna test range uncertainty assessment for AAS BS OTA operating band unwanted emissions measurement

| UID | Uncertainty Source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | [0.3] | [0.3] | Rectangular | √3 | 1 | [0.174] | [0.174] |

Observation 7: The MU is consistent with OTA BS maximum output power.

OTA Spectrum emissions mask:

Table 10.4.4.3.4.2-1: Compact antenna test range uncertainty assessment for AAS BS OTA SEM

| UID | Uncertainty Source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | [0.3] | [0.3] | Rectangular | √3 | 1 | [0.174] | [0.174] |

Observation 7: The MU is consistent with OTA BS maximum output power.

OTA TX spurious emissions:

Table 10.5.2.2.4.2-1: General  Chamber uncertainty assessment
Tx spurious emissions

| UID | Uncertainty source | Uncertainty value |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 30MHz<f≤6 GHz | 6GHz<f ≤19GHz |  |  |  | 30MHz<f≤6 GHz | 6GHz<f ≤19GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | Rectangular | √3 | 1 | 0.02 | 0.02 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 |

Observation 8: The MU is not consistent with OTA BS maximum output power and also with the corresponding MU for NR OTA TX spurious emissions. However, this MU is for general chamber rather than for CATR.

# 3	Conclusions

This document has identified that there exists a discrepancy in the “Misalignment DUT & pointing error” MU value for CATR for the following NR BS TRP requirements in TR 38.817-02:

- NR OTA BS maximum output power

- NR OTA ACLR

- NR OTA spectrum emission mask

- NR OTA operating band unwanted emissions

Proposal 1: The final MU for Misalignment DUT & pointing error should remain unchanged, i.e., 0.17 dB.

Option 1: Adopt Misalignment DUT & pointing error = 0.3 dB, distribution of probability = Rectangular and divisor based on distribution shape = 3.

Option 2: Adopt Misalignment DUT & pointing error = 0.34 dB, distribution of probability = Exp. normal and divisor based on distribution shape = 2.

Either Option 1 or 2 is feasible. Thus, Option 1 is chosen.

Proposal 2: Remove square brackets for “Misalignment DUT & pointing error” MU in TR 37.843.

The proposals are implemented in CRs [1] and [2], respectively.

# References

- R4-1915213, CR to TR 38.817-02: Aligning TRP MU for CATR, Nokia, Nokia Shanghai Bell

- R4-1915291, CR to TR 37.843: Removing square brackets in TRP MU for CATR, Nokia, Nokia Shanghai Bell