3GPP TSG-RAN WG4 Meeting #93	                                                         R4-1915317

Reno, USA, 18th - 22th Nov 2019

Agenda item:	9.4.2.1

Source:	Qualcomm Inc.

Title:	On UE to UE Coexistence study of NR V2X at 5.9GHz

Document for:	Discussion

# Introduction

We present in this paper our simulation results for RF Coexistence evaluation in 5.9GHz frequency between LTE V2X and NR V2X in the urban scenario.

The simulation assumptions are almost identical to those agreed in the email discussion. The result will be further updated to resolve those difference.

# Discussion

## Prioritization of Coexistence Scenarios

At 5.9GHz, 4 coexistence scenarios have been identified by RAN4, namely

- DSRC (aggressor)-> NR V2X (victim)

- NR V2X (aggressor)-> DSRC (victim)

- LTE V2X (aggressor)->NR V2X(victim)

- NR V2X (aggressor)->LTE V2X(victim)

Given the recent 5GAA filing to FCC for spectrum allocation [6], we believe that cases 3 and 4 should be treated at the highest priority.

For cases 1 and 2, assuming NR V2X and LTE V2X have the same ACLR and ACS, all the simulation assumptions seem to be similar. The only difference here is that NR V2X traffic can be both aperiodic and periodic, while LTE V2X traffic is strictly periodic. Given that, we may be able to reuse DSRC/LTE V2X coexistence study result for DSRC/NR V2X but it is also worth redoing the simulations to verify that the change in traffic pattern indeed does not affect anything. However, such study should be treated as second priority.

## UE Drop and Pathloss Model

According to the agreed WF [4]

- Assume all devices will support both LTE V2X and NR V2X

- Assume from a device perspective Tx between LTE V2X and NR V2X is TDM while rx of LTE V2X and NR V2X is simultaneous (as reflected in figures in slide 7)

- Case3: NR V2X UE-to-LTE V2X UE.

Each vehicle has to be associated with 2 radios, 1 LTE V2X and 1 NR V2X. As the 2 radios will be in the same chipset, we assume that the coupling loss between them is 0dB (as a pessimistic estimation).

Further details on simulation setting as per email discussion.

Layout model

Figure 2.2.1 Network layout model for NR V2X coexistence simulation in ITS spectrum

Table 2.2.1: Simulation layout model in ITS spectrum

| Parameter | Value |
| --- | --- |
| carrier frequency | 5.9GHz |
| Deployment scenarios | Urban Mahathan grid model, Freeway model |
| Simulation Block Size | 3*433m, 3*250m |
| Velocity of vehicles | 60km/h Note1: Fixed location will be considered for adjacent coexistence evaluation. Note2: Velocity only used to decide the UE density. |
| Pathloss model for - DSRC UE-to-DSRC UE - LTE V2X UE-to-LTE V2X UE - NR V2X UE-to-DSRC UE - DSRC UE-to-NR V2X UE - NR V2X UE-to-LTE V2X UE - LTE V2X UE-to-NR V2X UE | LOS/NLOS/NLOSv; Pathloss [dB]; Shadow fading std [dB]. LOS, NLOSv; For Highway case, PL = 32.4 + 20 log10(d3D) + 20 log10(fc) For Urban case, PL= 38.77 + 16.7 log10(d3D) + 18.2 log10(fc); σSF = 3. NLOS; PL= 36.85 + 30 log10(d3D) + 18.9 log10(fc); σSF = 4. Note 1: fc denotes the center frequency in GHz and d3D denotes the Euclidean distance between TX and RX in 3D space in meters. Note 2: The model for spatial correlation of shadow fading defined in TR 36.885 applies.. |
| Dropping vehicles (based on clause 6.1.2 from TR 37.885) | Vehicles are dropped according to the following process: - The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec}. - All the vehicles in the same lane have the same speed. - Vehicle type distribution is not dependent of the lane. |
| Total number of vehicles | It depends on vehicle UE density and block size. |
| vehicle UE activation rate for LTE V2X | 1% of total number of UEs |
| vehicle type (based on clause 6.1.2 from TR 37.885) | - Type 2 (passenger vehicle with higher antenna position): length 5 meters, width 2.0 meters, height 1.6 meters, antenna height 1.6 meters |

#### Simulation parameters

Table 2.2.2: Simulation parameters in ITS spectrum

| Parameter | Value |  |  |
| --- | --- | --- | --- |
|  | DSRC UE | LTE V2X UE | NR V2X UE |
| Tx power | 23dBm | 23dBm | 23dBm |
| Channel Bandwidth | 10MHz | 20MHz | 40MHz |
| Packet size | 1) 190 byte packet 2) 300 byte packet | 1)18 PRB (190 byte packet) 2)18 PRB (300 byte packet) | 70 PRB(1100 byte packet) for 15kHz SCS |
| Traffic model | 1 transmission every 100ms 100ms message generation period Time instance of message generation is randomized among vehicles |  | 1 transmission every 50ms 50ms message generation period 50% vehicles beign active Time instance of message generation is randomized among vehicles |
| Noise figure | 10dB | 9dB | 9dB |
| Antenna pattern | Horizontally Omni-directional with gain of 3 dBi |  |  |
| Sidelink Power control | In ITS spectrum, no power control is used |  |  |
| SINR-to-BLER mapping | As per link level performance model in TR 36.785 Table A-4 for 5.9GHz | As per link level performance model in TR 36.785 Table A-3 for 5.9GHz | Reuse LTE V2X link level performance for 300 bytes As per link level performance model in TR 36.785 Table A-3 for 5.9GHz |

In Table 2.2.2, item that are different from what agreed in the simulation assumption is highlighted in bold font. We will discuss them here one by one:

- LTE V2X resource allocation

According to RAN1/RAN2 spec, if we use only a single SPS flow, the same number of RB is used to carry the packet, regardless of packet size. Furthermore, if 20RB (e.g. 2 sub-channels, each sub channel is 10RB) allocation is used, the first 2RB is dedicated to PSCCH, hence PSSCH only has 18RB. Given that this reflect the real deployment scenarios, we update our LTE V2X link curve accordingly.

- NR V2X resource allocation and traffic.

According to Table 5.2.1.2-1 in R4-1910400-chapter 5.2.1 coexistence simulation parameters in ITS spectrum, 1100 bytes packet size with 70 PRBs for 15kHz SCS is adopted. Also, LTE V2X link level performance for 300 bytes is reused in NR V2X SINR-to-BLER mapping, as suggested in the same table.

- Antenna pattern.

In RAN1, it is agreed that for LTE V2X, when there is a single antenna panel on the rooftop of a vehicle, the antenna pattern should be horizontal omni-directional. The detail of model is as in Table 2.2.3

Table 2.2.3: Antenna element pattern for vehicle UE in Option 1

| For 6 GHz |  |
| --- | --- |
| Antenna element gain vertical pattern |  |
| Antenna element gain horizontal pattern | Vehicle Type 2: Vehicle Type 1 and Type 3: |
| Pattern combining method for 3D |  |
| Max direct. gain of the antenna element | 3 dBi |

This is a reasonable assumption for vehicular communication, and we think that it should be carried over to RAN4 RF coexistence evaluation.

## ACLR and ACS

We will follow the 2 step ACLR and ACS model. The details are as follow

- Option 2: 2 steps ACLR/ACS model.

  - ACLR:

    - If the frequency offset between transmitting RB (aggressor) and receiving RB (victim) is smaller than the transmitter (aggressor) allocated transmission bandwidth bandwidth then ACLR1 applies.

    - If the frequency offset between transmitting RB (aggressor) and receiving RB (victim) is larger than the transmitter (aggressor) allocated transmission bandwidth bandwidth then ACLR2 applies

  - ACS

    - If the frequency offset between transmitting RB (aggressor) and receiving channel edge (victim) is smaller than 10MHz then ACS1 applies.

    - If the frequency offset between transmitting RB (aggressor) and receiving channel edge (victim) is larger than 10MHz then ACS2 applies.

NOTE: 10MHz offset value is derived from LTE V2X definition of ACS and in band blocking requirement, as will as DSRC adjacent and alternate adjacent channel rejection requirement definition

Table 2.3.1: ACLR1/2 and ACS1/2 in ITS spectrum

| Parameter | Value |  |  |
| --- | --- | --- | --- |
|  | DSRC UE | LTE V2X UE | NR V2X UE |
| ACLR1 | 26dB | 30dB | 30 dB |
| ACLR2 | [39]dB | [43]dB | [43] dB |
| ACS1 | Use three candidate ACS level for DSRC : - 22/25/29dB | 27dB | 24 dB |
| ACS2 | ACS1 + 16 (NOTE 1) | 43dB (NOTE 2) | [40]dB (NOTE 3) |
| NOTE 1: following definition of alternate adjacent channel rejection requirement of DSRC [EN 302 571] NOTE 2: following definition of in band blocking requirement, case 1 of LTE CV2X [TS 36.101] NOTE 3: following definition of in band blocking requirement, case 2 of NR and apply same tightening as for LTE V2X. |  |  |  |

# Simulation Result

Figure 3.1 is the performance of LTE V2X Packet Reception Ratio, while figure 3.2 is the performance of NR V2X. We can see that under the simulation assumptions above, the degradation coming from adjacent channel aggressor is quite acceptable to both systems.

Figure 3.1. LTE V2X PRR

Figure 3.2 NR V2X PRR

In figure 3.1, we compare the PRR performance of LTE V2X under 2 different scenarios.

- Single system: only LTE V2X operates in a 20MHz channel.

- NR V2X aggressor, X = 0: LTE V2X operates in a 20MHz channel. NR V2X operates in the adjacent 40MHz channel. The ACLR of NR V2X is 30dB in first step and 43 dB in the second step.

In figure 3.2, we compare the PRR performance of NR V2X under 2 different scenarios.

- Single system: only NR V2X operates in a 40MHz channel.

- LTE V2X aggressor, X = 0: NR V2X operates in a 40MHz channel. LTE V2X operates in the adjacent 20MHz channel. The ACLR of LTE V2X is 30dB in first step and 43 dB in the second step.

Based on the results, the performance degradation inflicted to LTE V2X system is negligible, even without any tightening to the existing NR Uu ACLR requirement.

# Conclusions

We present in this paper our initial simulation result for RF Coexistence evaluation in 5.9GHz frequency between LTE V2X and NR V2X in the urban scenario.

The initial assessment is that LTE V2X can coexist with NR V2X aggressor, assuming NR V2X also meet the current NR Uu ACLR requirement.

# References

- TR 36.885 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on LTE-based V2X Services;

- TR 37.885 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR;

- QC R1 paper, TBD

- R4-1905098 WF on coexistence scenarios for NR V2X. Qualcomm, LG Electronics

- 5GAA Petition for Waiver to Allow Deployment of Cellular Vehicle-to-Everything (C-V2X) Technology in the 5.9 GHz Band; GN Docket No. 18-357. https://ecfsapi.fcc.gov/file/104030451515194/5GAA%20Band%20Plan%20Ex%20Parte%20-%20FINAL.pdf