3GPPRAN4#93	R4-1915404

Reno, USA, Nov 18th – 22nd , 2019

Title: 	Discussion on interruptions during SRS carrier switching

Source: 	Qualcomm Incorporated

Agenda item:	9.15.1.1	SRS carrier switching requirements 	[NR_RRM_Enh_Core]

Document for:	Discussion

# Introduction

In RAN4-92Bis, there was a WF [1] agreed on the SRS carrier switching requirements. The WF listed quite a few open issues. In this paper, we provide our view on interruptions.

# Discussion

One of the open issues in defining the interruption requirements for SRS carrier switching was if we should define the requirements for the worst case UE capability or define multiple sets. AS listed in the WF

Interruption requirements

Option 1: define requirements for 900us NR SRS carrier switching time

Option 2: define requirements for 900us and [TBD] NR SRS carrier switching time respectively

Option 3: define requirements for any NR SRS carrier switching time if it is justified

Option 4: Other option is not precluded.

Note: Further check on SRS carrier switching time definition

Whether the interruption depends on SRS switching time or pure RF tuning time

It our view that to keep the specification simple we should only define the interruptions for the worst-case scenario. We can just define the interruptions for 900us RF switching time. Since in any scenario it is in UE’s best interest to take interruptions no longer than needed, a UE would always take the shortest possible interruptions. Defining the requirements for the worst case gives an upper bound to the network.

Proposal 1: Define only one set of requirements for the worst case of RF switching time.

The SRS carrier switching procedure includes the UE switching to target frequency, transmitting up to 6 SRS symbols and the tuning back. Depending on the source cell SCS it would take up to the following in terms of completing the procedure.

| SCS | 15 kHz | 30 kHz | 60 kHz | 120 kHz |
| --- | --- | --- | --- | --- |
| Time (in us) | 2228.5 | 2014.2 | 1907.1 | 1853 |

Thus, in sync scenarios without accounting for any TA command the above would translate to the following in terms of interruptions (in slots)

Table 2-1: Interruptions in sync scenario without accounting for TA

| Victim cell SCS (in kHz) | Aggressor cell SCS (in kHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 15 | 30 | 60 | 120 |
| 15 | 3 | 3 | 2 | 2 |
| 30 | 5 | 5 | 4 | 4 |
| 60 | 9 | 9 | 8 | 8 |
| 120 | 18 | 17 | 16 | 15 |

In addition to the above, the UE would have some amount of TA which would cause one more slot of interruption. Also note that including the TA also covers the maximum number of slots that can be interrupted under async scenario too.

Thus, the interruptions for SCS carrier switching translate to

Table 2-2: Interruptions for SRS carrier switching with TA or in async scenario

| Victim cell SCS (in kHz) | Aggressor cell SCS (in kHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 15 | 30 | 60 | 120 |
| 15 | 4 | 4 | 3 | 3 |
| 30 | 6 | 6 | 5 | 5 |
| 60 | 11 | 10 | 9 | 9 |
| 120 | 20 | 18 | 17 | 16 |

Proposal 2: Interruptions for SRS switching should be as defined in Table 2-2.

# Conclusions

Proposal 1: Define only one set of requirements for the worst case of RF switching time.

Proposal 2: Interruptions for SRS switching should be as defined in Table 2-2 (reproduced below).

| Victim cell SCS (in kHz) | Aggressor cell SCS (in kHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 15 | 30 | 60 | 120 |
| 15 | 4 | 4 | 3 | 3 |
| 30 | 6 | 6 | 5 | 5 |
| 60 | 11 | 10 | 9 | 9 |
| 120 | 20 | 18 | 17 | 16 |

# References

[1] R4-1912838 Way forward on SRS carrier switching RRM requirements