3GPP TSG-RAN WG4 Meeting #93                                      R4-1915850

Reno, NV, USA, 18th – 22nd Nov, 2019

Agenda item:		9.11.2

Source:		Samsung

Title:	Way forward on NR eMIMO RRM

Document for:		Approval

# 1	Background

Rel-16 NR eMIMO WI (i.e., Enhancements on MIMO for NR) is a RAN1 leading WI with below major enhancement in RAN1 area:

- Enhancements on MU-MIMO support

- Enhancements on multi-TRP/panel

- Enhancements on multi-beam operation

  - DL/UL beam indication with reduced latency and overhead

  - Beam failure recovery for SCell

  - L1-SINR measurement

- Enhancement on low PAPR RS

- Enhancement on full Tx power uplink transmission

Some of enhancements in Rel-16 NR eMIMO WI need introducing corresponding RAN4 RRM core requirement.

# 2	Agreement from RAN4#93 RRM session (Wednesday)

2.1  L1-SINR Measurement

Issue-2(a): Scenarios down-selection for L1-SINR measurement, with below scenarios listed as reference from WF (R4-1912742):

Agreement from RRM Session (Wednesday) Scenarios for RRM Core requirements definition Scenario 1: L1-SINR measured on CMR only (when dedicated IMR is not configured) Scenario 1A: CSI-RS based CMR (only with density 3 Res/RB) Scenario 2: L1-SINR measured on CMR+IMR (when dedicated IMR is configured) Scenario 2A: SSB based CMR + ZP-IMR Scenario 2B: CSI-RS based CMR + ZP-IMR Scenario 2C: SSB based CMR + NZP-IMR (only with density 3 Res/RB) Scenario 2D: CSI-RS based CMR + NZP-IMR (only with density 3 Res/RB)

Issue-2(b): RRM requirement structure (separate requirement subsection for different scenarios):

Agreement from RRM Session (Wednesday) RRM Core requirement structure Separate requirement for CSI-RS-based and SSB-based L1-SINR Separate requirement for CMR-only and CMR+IMR Same requirement for ZP-IMR and NZP-IMR Define L1-SINR requirement based on following three generalized scenarios: L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured L1-SINR reporting with SSB based CMR and dedicated IMR configured L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured

Issue-3(b): Measurement period: For L1-SINR measurement report for CMR only scenario

Agreement from RRM Session (Wednesday) Measurement period: For L1-SINR measurement report for CMR only scenario follow below two tables, similar to L1-RSRP - Table x: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_Only for FR1 Configuration; TL1-SINR_Measurement_Period_CSI-RS_CMR_Only (ms). non-DRX; max(TReport, ceil(M*P)*TCSI-RS). DRX cycle ≤ 320ms; max(TReport, ceil(1.5*M*P)*max(TDRX,TCSI-RS)). DRX cycle > 320ms; ceil(M*P)*TDRX. Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3.. - Table y: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_Only for FR2 Configuration; TL1-SINR_Measurement_Period_CSI-RS_CMR_Only (ms). non-DRX; max(TReport, ceil(M*P*N)*TCSI-RS). DRX cycle ≤ 320ms; max(TReport, ceil(1.5*M*P*N)*max(TDRX,TCSI-RS)). DRX cycle > 320ms; ceil(M*P*N)*TDRX. Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3.. - Where the variable M, P, N are the same as Section 9.5.4.2 for L1-RSRP reporting..

Issue-4: Measurement restriction/scheduling availability

Agreement from RRM Session (Wednesday) For CSI-RS configured as CMR: - In FR2 it is that possible to configure CSI-RS with “repetition = on” - Measurement restriction is applied if this CSI-RS with “repetition = on” is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement.

Issue-5: Side condition

Agreement from RRM Session (Wednesday) Side condition for CMR only scenario - In addition to lower bound, the accuracy requirements for L1-SINR in CMR only scenarios is applicable for CSI-RS Es/Iot <= 25 dB as upper bound (Same note 3 applied as L1-RSRP: “NOTE 3: The requirements apply for SSB Ês/Iot ≤ 25 dB”).

2.2  SCell Beam failure recovery

Issue-1: Link recovery procedure general

Agreement from RRM Session (Wednesday) Link recovery procedure general UE is not required to perform BFD and CBD on a deactivated SCell UE is only required to perform BFD and CBD on SCell within the active BWP of the configured CC

Issue-2: BFD on SCell:

Agreement from RRM Session (Wednesday) CSI-RS and SSB based BFD for SCell BFD can be applied

# 3	Way Forward

3.1  L1-SINR Measurement

##### 3.1.1 Measurement period for L1-SINR measurement:

- Applicable condition for one-shot L1-SINR measurement report without averaging (for both CMR only and CMR+IMR)

  - Option-1: L1-SINR is configured with aperiodic CSI-RS and/or measurement restriction (timeRestrictionForChannelMeasurement or timeRestrictionForInterferenceMeasurements if applicable).

  - Other options are not excluded.

- For L1-SINR measurement report with averaging over multiple shot for CMR+IMR scenario

  - CMR+IMR scenario (SSB-based CMR + IMR, and CSI-RS-based CMR+IMR):

    - In RAN4 requirement, only CMR periodicity = IMR periodicity is assumed;

  - Measurement period: For L1-SINR measurement report with averaging over multiple shot for CMR+IMR scenario

    - only using CMR periodicity to define measurement period

##### 3.1.2 Measurement restriction/scheduling availability

- CSI-RS configured as NZP-IMR:

  - For CSI-RS configured as NZP-IMR in FR2, FFS whether or not CSI-RS with “repetition = on” can be configured, i.e., beam refinement based on NZP-IMR.

    - Option-1: NZP-IMR only with “repetition = off”.

    - Option-2: NZP-IMR can be configured with “repetition = off” or “repetition = on”.

- CSI-RS configured as ZP-IMR:

  - For CSI-RS configured as ZP-IMR in FR2, FFS whether or not CSI-RS with “repetition = on” can be configured, i.e., beam refinement based on ZP-IMR.

    - Option-1: ZP-IMR only with “repetition = off”.

    - Option-2: ZP-IMR can be configured with “repetition = off” or “repetition = on”.

##### 3.1.3 Side condition

- CMR+IMR scenario:

  - Side condition for CMR:

    - FFS the range of CMR Es/Iot by considering CMR condition and resultant L1-SINR range.

  - Side condition for IMR:

    - FFS the range of Iot on IMR

##### 3.1.4 For CMR+IMR (dedicated IMR configured), how to calculate L1-SINR:

- For CMR+IMR (dedicated IMR configured), how to calculate L1-SINR:

  - the numerator is the signal power measured on CMR and the denominator (interference and noise) is the total received power on associated IMR.

- Measurement accuracy for L1-SINR

  - Only resultant L1-SINR shall be considered.

##### 3.1.5 Assumptions for L1-SINR accuracy evaluation

- General assumption:

  - Number of CSI-RS port for CMR (if applicable), and NZP-IMR:

    - N=1

- L1-SINR accuracy evaluation simulation assumption for CMR only:

| Parameters | Values |
| --- | --- |
| SCS | 15kHz (FR1), 30kHz (FR1), 120kHz (FR2) |
| Channel measurement resource (CMR) | CSI-RS |
| Interference measurement resource (IMR) configuration | N/A |
| Side condition (SNR) on CMR | -3dB |
| Density (D) | 3 |
| Number of samples (M) | 1, 3, 5 |
| Number of PRBs | 48 |
| Propagation condition | AWGN |

- L1-SINR accuracy evaluation simulation assumption for SSB-based CMR + NZP-IMR:

| Parameters | Values |
| --- | --- |
| SCS | 15kHz (FR1), 30kHz (FR1), 120kHz (FR2) |
| Channel measurement resource (CMR) | SSB |
| Interference measurement resource (IMR) configuration | CSI-RS |
| Periodicity | CMR Periodicity = IMR Periodicity |
| Ideal SINR | -3dB |
| Side condition (SNR) on CMR | option-1: 0dB, other options not excluded |
| Side condition (SNR) on IMR | option-1: 0dB, other options not excluded |
| Density (D) for IMR | 3 |
| Number of samples (M) for IMR/CMR | 1, 3, 5 |
| Number of PRBs for IMR | 48 |
| Propagation condition | AWGN |

- L1-SINR accuracy evaluation simulation assumption for SSB-based CMR + ZP-IMR:

| Parameters | Values |
| --- | --- |
| SCS | 15kHz (FR1), 30kHz (FR1), 120kHz (FR2) |
| Channel measurement resource (CMR) | SSB |
| Interference measurement resource (IMR) configuration | CSI-RS |
| Periodicity | CMR Periodicity = IMR Periodicity |
| Ideal SINR | -3dB |
| Side condition (SNR) on CMR | -3dB |
| Side condition (SNR) on IMR | N/A (only AWGN noise) |
| Density (D) for IMR | 3 |
| Number of samples (M) for IMR/CMR | 1, 3, 5 |
| Number of PRBs for IMR | 48 |
| Propagation condition | AWGN |

- L1-SINR accuracy evaluation simulation assumption for CSI-RS-based CMR + NZP-IMR:

| Parameters | Values |
| --- | --- |
| SCS | 15kHz (FR1), 30kHz (FR1), 120kHz (FR2) |
| Channel measurement resource (CMR) | CSI-RS |
| Interference measurement resource (IMR) configuration | CSI-RS |
| Periodicity | CMR Periodicity = IMR Periodicity |
| Ideal SINR | -3dB |
| Side condition (SNR) on CMR | option-1: 0dB, other options not excluded |
| Side condition (SNR) on IMR | option-1: 0dB, other options not excluded |
| Density (D) for CMR/IMR | 3 |
| Number of samples (M) for CMR/IMR | 1, 3, 5 |
| Number of PRBs for CMR/IMR | 48 |
| Propagation condition | AWGN |

- L1-SINR accuracy evaluation simulation assumption for CSI-RS-based CMR + ZP-IMR:

| Parameters | Values |
| --- | --- |
| SCS | 15kHz (FR1), 30kHz (FR1), 120kHz (FR2) |
| Channel measurement resource (CMR) | CSI-RS |
| Interference measurement resource (IMR) configuration | CSI-RS |
| Periodicity | CMR Periodicity = IMR Periodicity |
| Ideal SINR | -3dB |
| Side condition (SNR) on CMR | -3dB |
| Side condition (SNR) on IMR | N/A (only AWGN noise) |
| Density (D) for CMR/IMR | 3 |
| Number of samples (M) for CMR/IMR | 1, 3, 5 |
| Number of PRBs for CMR/IMR | 48 |
| Propagation condition | AWGN |

3.2  SCell Beam failure recovery

##### 3.2.1  Link recovery procedure general

Background: BFD-RS for a SCell configured in an implicit manner can be transmitted another CC.

- RS within a deactivated SCC is implicitly configured as the BFD-RS for another activated SCell:

  - Option-1: UE is not required to perform BFD on RS within a deactivated SCC which is implicitly configured as the BFD-RS for another activated SCell

  - Option-2: FFS

- RS within an activated SCC is implicitly configured as the BFD-RS for another deactivated SCell:

  - Option-1: UE is not required to perform BFD on RS within an activated SCC which is implicitly configured as the BFD-RS for another deactivated SCell

  - Option-2: FFS

- When more than 2 BFD-RSs are transmitted on a CC for current SCell and (implicitly configured for) other SCell

  - Option-1: UE is allowed to only perform BFD on BFD-RSs for current SCell.

  - Option-2: FFS

##### 3.2.2  BFD on SCell

- RRM core requirement impact by BFD on SCell

  - The evaluation periods for SSB/CSI-RS based BFD on SCell are same as those of SSB/CSI-RS based BFD on PCell/PSCell. Detailed requirement captured in Section 8.5.3/4 shall be reused for SCell BFD.

    - FFS whether to introduce a sharing factor for BFD and CBD measurement period

##### 3.2.3  CBD on SCell

- RRM core requirement impact by CBD on SCell

  - Option-1: The evaluation periods for SSB/CSI-RS based CBD on SCell are same as those of SSB/CSI-RS based CBD on PCell/PSCell. Detailed requirement captured in Section 8.5.5/6 shall be reused for SCell CBD.

    - FFS whether to introduce a sharing factor for BFD and CBD measurement period

##### 3.2.4  SCell Beam Failure Recovery ReQuest (BFRQ) Mechanism

- Step-1 for BFRQ on SCell with DL only: UE reports beam failure event through a dedicated SR like PUCCH resource

  - Option-1: RAN4 requirement is defined for step 1 of BFR, UE reports beam failure event through a dedicated SR like PUCCH resource, in SCells with DL only.

  - Option-2: No RRM core requirement impact by SCell BFRQ mechanism

- Step-2 for BFRQ on SCell with DL only: UE conveys new beam information and failed CC index(es) via MAC-CE

  - Option-1: FFS Defining requirement for step 2

  - Option-2: No RRM core requirement impact by SCell BFRQ mechanism

##### 3.2.5  UE Capability

- UE Capability of Number of SCells for BFR:

  - The maximum number of SCell for which UE performs BFR is a UE capability

  - For BFD and CBD requirements in FR2, up to N serving cell is assumed to perform BFR procedure for one FR2 band:

    - Option-1: N=1

    - Other options FFS

3.3  DL/UL beam indication with reduced latency and overhead

##### 3.3.1  MAC-CE based spatial relation update for aperiodic SRS:

- MAC-CE based spatial relation update for aperiodic SRS:

  - FFS RAN4 RRM requirement impact is identified from the mechanism of spatial relation updates for AP-SRS via MAC-CE

##### 3.3.2  Simultaneous TCI States Activation/Selection across Multiple CCs/BWPs

- Simultaneous TCI States Activation/Selection across Multiple CCs/BWPs:

  - No RAN4 impact on Rel-15 active TCI state switching delay requirement has been identified due to newly introduced simultaneous TCI states activation /selection across multiple CCs/BWPs.

  - FFS new Rel-16 requirement on simultaneous TCI states activation/selection across multiple CCs/BWPs.

3.4 Others

##### 3.4.1 Reply LS on timing for pathloss RS activated/updated by MAC-CE

- UE processing time after the last measurement sample (i.e., the time to apply the new PL-RS measurement results from the last measurement sample)

  - 2ms