3GPP TSG-RAN WG4 Meeting #93	R4-1915851

Reno, US, 18 November – 22 November 2019

Source:	ZTE

Title:	Simulation assumption on SIB1 decoding for NR CGI reading

Agenda item:	9.15.1.3

Document for:	Approval

1. Introduction

In this contribution we provide simulation assumptions on SIB1 decoding for NR CGI reading.

2. Simulation assumptions

The simulation assumptions are provided in Table 1.

Table 1: Parameters for SIB1 reading (RMSI acquisition)

| Parameters | Unit | Value |
| --- | --- | --- |
| Subcarrier spacing | KHz | 30 kHz for 4GHz; 120 kHz for 30GHz |
| Channel bandwidth | MHz | 10 for SCS=30kHz 100 for SCS=120kHz |
| Number of transmitter antennas |  | 1 |
| Number of Rx antennas |  | 2 |
| CP Length |  | Normal |
| Allocated resource blocks for PDSCH |  | 24 (allocated from the lowest edge of the carrier) |
| Number of PDSCH symbols |  | 12 |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| PDSCH mapping type |  | Type A |
| dmrs-TypeA-Position |  | 2 |
| dmrs-Type |  | 1 |
| dmrs-AdditonalPositions |  | 2 |
| Dmrs-maxLength |  | 1 |
| PRB bundling size |  | 2 |
| SIB1 transmission period | ms | 20 |
| Information Bit Payload |  |  |
| For slots with RMSI | Bits | 1864 |
| Number of Code Blocks per slot |  | 1 |
| Binary Channel Bits Per slot |  |  |
| For slots with RMSI | Bits | 6048 |
| SNR |  | -8, -6, -4, -2, 0 |
| Propagation Condition |  | AWGN, TDL-C with RMS delay spread = 300ns |
| UE speed |  | 3km/h |
| Detection Method |  | Option 1: Soft combining with 2, 4, 8 repetitions Option 2: One shot |
| Metrics |  | The number of repetitions required to decode SIB1 with 90%-ile success rate. |