3GPP TSG-RAN WG4 Meeting #93               	R4-1915856

Reno, Nevada, US, Nov 18 - 22, 2019

Source:	China Telecom

Title:	R15/R16 NR UE Demodulation ad-hoc minutes #2

Agenda Item:	7.12.1, 9.18.1

Document for:	Approval

Ad-hoc attendees: Samsung, Intel, Huawei, Qualcomm, Ericsson, CATT, NTT DOCOMO, CMCC, China Telecom, ANRITSU, R&S, Keysight

Rel-15 UE demodulation and CSI requirements

Return to CRs

- General

R4-1913417	CR to TS 38.101-4: Corrections for applicability rules (R15)
					38.101-4	  CR-0009  rev  Cat: F (Rel-15) v15.3.0
					Source: Intel Corporation

Abstract:

Discussion:

QC: discuss the language offline: The test coverage can be considered fulfilled without executing of Test 1-1 in Clauses 5.2.2.1.4 and 5.2.3.1.4.

The other part is agreeable.

Decision:		Revised to R4-1915805 (from R4-1913417)

R4-1915805	CR to TS 38.101-4: Corrections for applicability rules (R15)
					38.101-4	  CR-0009  rev 1 Cat: F (Rel-15) v15.3.0
					Source: Intel Corporation

Abstract:

Make the following editorial change:

For UEs supporting “Alternative additional DMRS position for co-existence with LTE CRS”, if Test 1-2 is tested, the test coverage can be considered fulfilled without executing Test 1-1. Otherwise, only Test 1-1 is tested.

The other part is agreeable.

Recommendation: 		The document was return to.

R4-1915089	Draft CR for TS38.101-4: Angle of arrival for radiated UE demodulation testing
					38.101-4	  CR-0023  rev  Cat: F (Rel-15) v15.3.0
					Source: Qualcomm Incorporated

Abstract:

Discussion:

Recommendation: 		The document was return to.

- PDSCH

R4-1915294	CR: Updates to NR EN-DC SDR tests
					38.101-4	  CR-0030  rev  Cat: F (Rel-15) v15.3.0
					Source: Huawei, HiSilicon

Abstract:

Resubmission of endorsed draftCR R4-1912751

Discussion:

Remove 9.4B.1.2 and 9.4B.1.3 from the CR, to avoid confusion for the MCC.

Decision:		Revised to R4-1915806 (from R4-1915294) 

R4-1915806	CR: Updates to NR EN-DC SDR tests
					38.101-4	  CR-0030  rev  Cat: F (Rel-15) v15.3.0
					Source: Huawei, HiSilicon

Abstract:

Resubmission of endorsed draftCR R4-1912751

Discussion:

The revision in the draft inbox is agreeable.

Recommendation:		Return to

- Control channel

- CSI reporting

R4-1914372	Editorial CR to correct PMI test cases
					38.101-4	  CR-0021  rev  Cat: F (Rel-15) v15.3.0
					Source: Ericsson

Abstract:

In this CR we provide editorial corrections to Rel-15 PMI tests

Discussion:

In Table 6.3.2.2.1-1 and Table 6.3.2.2.2-1, some text are in bold. Need to be corrected.

Decision:		Revised to R4-1915807 (from R4-1914372) 

R4-1915807	Editorial CR to correct PMI test cases
					38.101-4	  CR-0021  rev  Cat: F (Rel-15) v15.3.0
					Source: Ericsson

Abstract:

In this CR we provide editorial corrections to Rel-15 PMI tests

Reason for change: Correction of nomenclature in 38.101-4

Summary of change:	Changing eNB to gNB

Discussion:

Recommendation:		Return to

- Channel model

- Requirements for NE-DC, NGEN-DC, NR-NR DC

R4-1915130	CR on NE-DC and NGEN-DC performance requirements
					38.101-4	  CR-0027  rev  Cat: B (Rel-15) v15.3.0
					Source: Huawei, HiSilicon

Abstract:

Resubmission of endorsed draftCR R4-1912806

Discussion:

QC: UE may support EN-DC but not support NGEN-DC.

-	For UEs supporting NGEN-DC, the test coverage of CSI reporting requirements can be considered fulfilled, if the CSI reporting requirements of EN-DC in Section 610 are executed for UE under test.

Huawei: so far, no such kind of UEs. There is no band combination supporting NGEN-DC but not supporting EN-DC. You suggested change is Ok.

Intel: For normal demodulation requirements, add the applicability rule for NGEN-DC in 9.1.1.

Other parts are agreeable.

Decision:		Revised to R4-1915808 (from R4-1915130) 

R4-1915808	CR on NE-DC and NGEN-DC performance requirements
					38.101-4	  CR-0027  rev  Cat: B (Rel-15) v15.3.0
					Source: Huawei, HiSilicon

Abstract:

Resubmission of endorsed draftCR R4-1912806

Discussion:

The revision in the draft inbox is agreeable.

Recommendation:		Return to

R4-1915131	CR on NR-DC performance requirements
					38.101-4	  CR-0028  rev  Cat: B (Rel-15) v15.3.0
					Source: Huawei, HiSilicon

Abstract:

Resubmission of endorsed draftCR R4-1912748

Discussion:

QC: Change “thet” to “the”

R&S: FR1 Test 1-1 is 2x2 test, need to use 1x2 test.

Huawei: check offline.

Intel: no strong view

QC: no strong view. If use 1x2, refer to the SDR test.

Huawei: for SDR, high SNR is needed.

QC: follow the antenna configuration in SDR test, FFS on the MCS.

Intel: For LTE setup in EN-DC test, no specific MCS is defined.

Huawei: acceptable to change to 1x2.

Decision:		Revised to R4-1915809 (from R4-1915131) 

R4-1915809	CR on NR-DC performance requirements
					38.101-4	  CR-0028  rev  Cat: B (Rel-15) v15.3.0
					Source: Huawei, HiSilicon

Abstract:

Resubmission of endorsed draftCR R4-1912748

Discussion:

The revision in the draft inbox is agreeable.

Recommendation:		Return to

Rel-16 UE demodulation and CSI requirements

General

- Contributions list and summary of proposals

| Tdoc | Title | Source | Proposal |
| --- | --- | --- | --- |
| R4-1913185 | CR work split for NR performance requirement enhancement WI | China Telecom | CR work split |
| R4-1913186 | On CA CQI requirements in Rel-16 | China Telecom | Proposal 1: Describe in the WID that CA CQI requirements will be defined under AWGN condition. |
| R4-1914349 | Release independent UE demodulation and CSI reporting requirement for Rel-16 | Ericsson | Observation: The following UE demodulation and CSI reporting requirements are the candidate of release independent features from Release 15: - UE demodulation and CSI reporting test for NR CA - PMI reporting test with Tx ports more than 8 - LTE-NR co-existence scenario - High speed train Proposal: RAN4 discuss what NR UE demodulation and CSI reporting requirements are release independent from Release 15 onward. |

- Discussion

Open issues:

- CR work split for UE part (R4-1913185)

|  |  |  |  | CR Responsibility |
| --- | --- | --- | --- | --- |
| UE demodulation and CSI (38.101-4/307) | CA normal demodulation for NR CA, EN-DC, NE-DC, NR-DC | FR1 | Applicability | Intel |
|  |  |  | 2Rx requirements | CMCC |
|  |  |  | 4Rx requirements | Huawei |
|  |  |  | FRC | Intel |
|  |  | FR2 | Applicability | Intel |
|  |  |  | 2Rx requirements | Qualcomm |
|  |  |  | FRC | Intel |
|  | CA CQI for NR CA, EN-DC, NE-DC, NR-DC |  |  | To be added later |
|  | Tx ports larger than 8 and up to 32 | Applicability |  | Huawei |
|  |  | Requirements for type I single-panel codebook |  | Ericsson |
|  |  | Requirements for type II codebook |  | Samsung |
|  |  | FRC |  | Samsung |
|  | TDD LTE-NR co-existence | Applicability and requirements |  | Intel |
|  |  | FRC |  | Intel |
|  | CA power imbalance |  |  | To be added later |
|  | Release independence |  |  | China Telecom |

- On CA CQI requirements

  - Describe in the WID that CA CQI requirements will be defined under AWGN condition. (China Telecom)

- Release independent aspect

  - RAN4 discuss what NR UE demodulation and CSI reporting requirements are release independent from Release 15 onward. (Ericsson)

    - UE demodulation and CSI reporting test for NR CA

    - PMI reporting test with Tx ports more than 8

    - LTE-NR co-existence scenario

Samsung: release independent is usually for RF requirements.

CMCC: this WI mainly focus on the Rel-15 leftovers. LTE HST RRM/demod requirements are also release independent from an earlier release.

Discussion:

E///: two separate CRs for type I FRC and type II FRC.

Samsung provide FRC CR for type II; E/// provide FRC CR for type I

Agreement:

- CR work split in R4-1913185 will be revised:

  - E/// provide FRC CR for type I; Samsung provide FRC CR for type II

- Release independent aspect

  - RAN4 to discuss what NR UE demodulation and CSI reporting requirements are release independent from Release 15 onward.

    - UE demodulation and CSI reporting test for NR CA

    - PMI reporting test with Tx ports more than 8

    - LTE-NR co-existence scenario

PDSCH CA requirements

- Contributions list and summary of proposals

| Tdoc | Title | Source | Proposal |
| --- | --- | --- | --- |
| R4-1913183 | On NR CA PDSCH normal demodulation requirements | China Telecom | Single carrier SCS Proposal 1: For single carrier SCS, cover FR1 TDD 15kHz. Use TDD UL-DL pattern of 3D1S1U with S=10:2:2 for FR1 TDD 15kHz. Rank and MCS for FR2 Proposal 2: For FR2, use rank 2 and MCS 10 or MCS 11. PUCCH capacity Proposal 3: To carry ACK/NACK feedback, use PUCCH format 1 for no more than 2 DL CCs, and use PUCCH format 3 for more than 2 DL CCs. Observation 1: PUCCH capacity issue is not observed based on proposal 3. HARQ process number for TDD-FDD CA and TDD-TDD CA with different SCSs Proposal 4-1: For FDD 15 kHz + TDD 30 kHz CA and TDD 15 kHz + TDD 30 kHz CA, the HARQ process number for Pcell is same with that for single carrier test. Proposal 4-2: In FDD 15 kHz Pcell + TDD 30 kHz Scell CA, the HARQ process number for TDD Scell is 8. Proposal 4-3: In FDD 15 kHz Scell + TDD 30 kHz Pcell CA, the HARQ process number for FDD Scell is 8. Proposal 4-4: In TDD 15 kHz Pcell + TDD 30 kHz Scell CA, the HARQ process number for 30kHz Scell is 11. Proposal 4-5: In TDD 15 kHz Scell + TDD 30 kHz Pcell CA, the HARQ process number for 15kHz Scell is 8. Proposal 4-6: Summary of proposed HARQ process number for NR FDD 15 kHz + TDD 30 kHz CA, and TDD 15 kHz + TDD 30 kHz CA. Table 4: Summary of proposed HARQ process number for NR CA NR CA; HARQ process number. FDD 15 kHz + TDD 30 kHz CA; FDD Pcell; 4 for FDD CC; 8 for TDD CC. TDD Pcell; 8 for TDD CC; 8 for FDD CC. TDD 15 kHz + TDD 30 kHz CA; 15kHz SCS PCell; 8 for 15 kHz CC; 11 for 30 kHz CC. 30kHz SCS PCell; 8 for 30 kHz CC; 8 for 15 kHz CC. Single carrier performance for TDD-FDD CA and TDD-TDD CA with different SCSs Observation 2: The link-level performance difference at 70% throughput is negligible for round-trip time of 10 slots and 20 slots. Proposal 5: Apply the same single carrier requirement for Pcell and Scell in CA with the same duplex mode and SCS, CA with different duplex modes, CA with the same duplex mode and different SCSs. Test applicability Proposal 6: Test all the supported CA duplex mode. Proposal 7: Discuss whether to conduct test for both FDD PCell and TDD Pcell for FDD 15 kHz + TDD 30 kHz CA, and whether to conduct test for both 15 kHz PCell and 30 kHz Pcell for TDD 15 kHz + TDD 30 kHz CA. Observation 3: For inter-band CA with different numbers of bands, the largest aggregated channel bandwidth can be different. Proposal 8: Reuse the LTE approach for CA capability categorization, i.e., define different capabilities for intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA with different numbers of bands. Proposal 9: Test all the supported CA capabilities. Proposal 10: If only up to rank 2 and up to MCS 13 are used in the test, select any one of the supported CA configurations in each CA capability with largest aggregated CA bandwidth combination. |
| R4-1913420 | Discussion on NR CA UE demodulation requirements | Intel Corporation | Proposal 1: Define requirements for FR1 TDD with 15 kHz SCS and test this configuration for FDD-TDD inter-band CA scenario in case UE does not support different SCS on different CCs. Proposal 2: Use Rank 1 MCS13 for FR2 NR Normal CA requirements. Proposal 3: Align categorizing of CA capabilities for NR Normal CA requirements with RF specifications Proposal 4: Consider the following CA capabilities for NR Normal CA testing: Intra-band contiguous CA, Intra-band non-contiguous CA and Inter-band CA with the largest number of bands Proposal 5: Use the following approach for selection of CA configuration for NR FR1 Normal CA testing: - Step 1: Select CA configurations with maximum number of CCs, on which UE capability field supportedSubCarrierSpacingDL is equal to SCSreq, among all supported CA configurations - Step 2: Select CA configurations with maximum number of CCs, on which UE capability field maxNumberMIMO-LayersPDSCH is higher or equal to νLayersreq, among all the selected CA configurations from Step 1 - Step 3: Select any one of CA configurations, which contain CBW combination with the largest data rate not exceeding DataRatereq, among all the selected CA configurations from Step 2. Proposal 6: Use the following approach for selection of CA configuration for NR FR2 Normal CA testing: - Step 1: Select CA configurations, which contain CBW combinations with SNRTEmax higher or equal to SNRreq, among all supported CA configurations - Step 2: Select CA configurations with maximum number of CCs, on which UE capability field supportedSubCarrierSpacingDL is equal to SCSreq, among all the selected CA configurations from Step 1 - Step 3: Select CA configurations with maximum number of CCs, on which UE capability field maxNumberMIMO-LayersPDSCH is higher or equal to νLayersreq, among all the selected CA configurations from Step 2 - Step 4: Select any one of CA configurations, which contain CBW combination with the largest data rate not exceeding DataRatereq and aggregated bandwidth with SNRTEmax higher or equal to SNRreq, among all the selected CA configurations from Step 3. |
| R4-1913421 | Simulation results for NR FR1 Normal CA requirements | Intel Corporation | Simulation results |
| R4-1913422 | Summary of Normal CA simulation results (NR FR1 FDD) | Intel Corporation | Summary of simulation results |
| R4-1913423 | Summary of Normal CA simulation results (NR FR1 TDD) | Intel Corporation | Summary of simulation results |
| R4-1913424 | CR to TS 38.101-4: FRC for FR1 Normal CA requirements (R16) | Intel Corporation | CR |
| R4-1913496 | Views on NR CA PDSCH Demodulation Performance Tests | Qualcomm Incorporated | Proposal 1: For FDD+TDD or TDD+FDD CA scenarios, PCell will follow same HARQ timelines as single carrier test cases. Proposal 2: Single carrier requirements from FDD+FDD and TDD+TDD CA scenarios can be reused for PCell in FDD+TDD and TDD+FDD CA. Observation 1: For FDD+TDD or TDD+FDD CA scenarios, SCell cannot follow same HARQ timelines as single carrier test cases. Proposal 3: Further discuss whether single carrier requirements from FDD+FDD and TDD+TDD CA scenarios can be reused for SCell in FDD+TDD and TDD+FDD CA. Observation 2: The table of SNR limitations for FR2 in WF [1] is pessimistic compared to the SNR calculator spreadsheet in 38.810. Observation 3: Commercial test equipment can achieve 1% EVM for 28GHz and 39GHz carrier frequency. We can have at least 40dB DL SNR with current test equipment capability. Observation 4: EVM and phase noise in the test equipment are not the bottleneck for FR2 DL testing, and hence only the link budget shall be the focus from the testability point of view. Proposal 4: Use MCS 13, Rank 2 for defining FR2 CA normal demodulation requirements. Proposal 5: If testable SNR is lower than the requirement SNR for FR2 CA, that test case will be skipped. |
| R4-1913573 | Views on normal PDSCH demodulation test for CA | NTT DOCOMO, INC. | Proposal 1: The number of HARQ process for TDD-FDD CA with FDD PCell should be same as the values, which was agreed at RAN4#92, i.e., 4 for FDD and 8 for TDD. Proposal 2: Applicability of the CA test is designed using following alternatives. - Alt. 1: Maximum number of CCs - Alt. 2: Maximum number of bands - Alt. 3: Maximum aggregated channel bandwidth |
| R4-1913683 | Discussion on test applicability for NR CA PDSCH normal demodulation | CMCC | Proposal 1: It is proposed to specify FR1 TDD 15KHz for CA normal demodulation requirements in Rel-16 - All configurable channel bandwidths for TDD 15 kHz will be considered - TDD 15 kHz is only tested in inter-band CA scenario with different SCSs on different CCs: FR1 TDD 15 kHz SCS and TDD 30 kHz SCS Proposal 2: It is proposed to align the categorizing of CA capabilities with RF core specification in TS 38.101. Proposal 3: Test all the different CA capabilities. Proposal 4: Test all the supported CA duplex mode. Proposal 5: Test any one of the supported CA configurations in each CA capability for each duplex mode among supported CA configurations. Proposal 6: Test the largest aggregated CA bandwidth combination for selected CA configuration. |
| R4-1914347 | Simulation results of NR PDSCH demodulation requirements with CA | Ericsson | Proposal: RAN4 will choose MCS and Rank for FR2 CA scenario from the following options: Option 1: MCS13 with rank 1 Option 2: MCS10 with rank 2 |
| R4-1915110 | Simulation results for NR FR1 normal CA UE performance requirements for FDD | Huawei, HiSilicon | Simulation results |
| R4-1915111 | Simulation results on NR FR1 normal CA UE performance requirements for TDD | Huawei, HiSilicon | Simulation results Observation: There is almostly the same for different channel bandwidth for NR FR1 normal CA for TDD. |
| R4-1915112 | Discsussion on NR normal CA performance requirements | Huawei, HiSilicon | Observation: No PUCCH capability limitation for HARQ ACK/NACK feedback transmission for NR CA. Proposal 1: One PUCCH group should be assumed for NR CA cases with same and mixed numerology, and PUCCH is transmitted on the PCell with no UL CA assumption. Proposal 2: Consider to use the number of HARQ process for scenario 3 and scenario 4 of CA with mixed numerology as shown in Table 1: - FDD PCell: 5 for FDD CC and 8 for TDD CC - TDD PCell: 5 for FDD CC; 12 for TDD CC Proposal 3: Choose FR2 Rank 1, FR2 MCS 4 for FR2 CA cases Proposal 4: For the test applicability rule, we have the following proposal: - UE should be tested for all the supported CA duplex mode - CA capability category: intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA as per the different frequency range FR1, FR2 and FR1+FR2 - Testing of different CA capabilities, CA configurations and CBW combinations: Tests; CA capability where the tests apply; CA configuration from the selected CA capability where the tests apply; CA Bandwidth combination to be tested in priority order. All CA tests; Any one of the supported CA capabilities with largest aggregated CA bandwidth combination; Any one of the supported FDD/TDD CA configurations with largest aggregated CA bandwidth combination; Largest aggregated CA bandwidth combination with the largest number of CCs. |
| R4-1915113 | draftCR: test structure for NR FR1 CA normal demodulation requirements for 4Rx | Huawei, HiSilicon | CR |

- SCS, Channel bandwidth

Agreements in RAN4 #92bis (R4-1912832):

- Single carrier SCS

    - FR1

    - FR1 FDD:15kHz

    - FR1 TDD: 30kHz, FFS 15kHz

    - FR2: 120kHz

- Single carrier channel bandwidth for the FR1 FDD 15kHz, FR1 TDD 30kHz and FR2 120kHz SCS

    - All configurable channel bandwidths

- Scenarios with different SCSs on different CCs for inter-band CA

    - FR1 FDD 15 kHz SCS and TDD 30 kHz SCS

- If TDD 15 kHz requirements will be defined then

    - All configurable channel bandwidths for TDD 15 kHz will be considered

    - TDD 15 kHz is only tested in inter-band CA scenario with different SCSs on different CCs: FR1 TDD 15 kHz SCS and TDD 30 kHz SCS

Open issues:

- Whether to define requirements for FR1 TDD 15 kHz

    - Yes (China Telecom, Intel, CMCC)

- CA scenario for FR1 TDD 15kHz

    - Option 1: FR1 inter-band CA with TDD 15 kHz + TDD 30 kHz (Agreement in the last meeting, CMCC)

    - Option 2: FR1 inter-band CA with TDD 15 kHz + TDD 30 kHz, and FR1 inter-band CA with TDD 15 kHz + FDD 15 kHz (Intel, CMCC)

    - Scenario to test TDD+FDD CA in case UE does not support different SCS on different CCs. (Intel)

- Single carrier channel bandwidth for FR1 TDD 15 kHz

    - Option 1: All configurable channel bandwidths  (Agreement in the last meeting, CMCC)

    - Option 2: All configurable channel bandwidths  up to [30] MHz

E///: all CBW means up to 50MHz?

Intel/CTC: Yes

E///: not sure if 50MHz CBW is available for the re-farming band.

CMCC: 50MHz CBW for n40.

- TDD UL-DL pattern for FR1 TDD 15kHz

    - Option 1: 3D1S1U with S=10:2:2  (China Telecom)

Discussion:

Agreement:

- Define requirements for FR1 TDD 15 kHz

- CA scenario for FR1 TDD 15kHz

    - FR1 inter-band CA with TDD 15 kHz + TDD 30 kHz

    - FFS: FR1 inter-band CA with TDD 15 kHz + FDD 15 kHz

    - Scenario to test TDD+FDD CA in case UE does not support different SCS on different CCs.

    - QC would like to check

- Single carrier channel bandwidth for FR1 TDD 15 kHz

    - All configurable channel bandwidths

- TDD UL-DL pattern for FR1 TDD 15kHz

    - 3D1S1U with S=10:2:2

- Test parameters

Agreements in RAN4 #92bis (R4-1912832):

- Rank:

    - FR1: Rank 2 for 2Rx and 4Rx

    - FR2:

    - Option 1: Rank 2

    - Option 2: Rank 1

- MCS:

    - FR1: MCS 13 (16QAM, CR 1/2)

    - FR2:

    - Option 1: MCS 13 (16QAM, CR 1/2)

    - Option 2: MCS 4 (QPSK, CR 1/3)

    - Option 3: MCS 10 or 11 (16 QAM)

    - Other options are not precluded

- Receiver type: MMSE-IRC

- Not cover 16 HARQ processes

- PDSCH scheduling

    - Self-carrier scheduling for each carrier

- PUCCH capacity

    - Check if there is PUCCH capacity issue for the maximum numbers of CCs in the Sep version of Rel-16 core spec

- HARQ process number for TDD-FDD CA: FFS

- Single carrier performance for TDD-FDD CA

    - Further discuss whether the requirement of one TDD/FDD carrier in TDD-TDD/ FDD-FDD CA can be applied for TDD-FDD CA with TDD Pcell or FDD Pcell.

Open issues:

- Rank and MCS for FR2

    - Option 1: rank 2 and MCS 10 (China Telecom, Ericsson)

    - Option 2: rank 2 and MCS 11 (China Telecom)

    - Option 3: rank 1 and MCS13 (Intel, Ericsson)

    - Option 4: rank 2 and MCS13 (Qualcomm)

    - Option 5: rank 1 and MCS4 (Huawei)

HW: for OTA, prefer to use rank 1. Questionable how to conduct tests with 2Tx and ULA in practical.

QC: we already have rank 2 test for single carrier. CPA can provide sufficient isolation for rank 2.

R&S: in the test, apply the correlation matrix in wireless cable. For each direction, we can apply the correlation matrix.

HW: Not sure if all the UE have good isolation.

R&S: RAN5 is discussing what kind of isolation can be achieved. RAN5 discussion will not impact RAN4 agreement. No testability issue to introduce FR2 test with rank 2.

Intel: the testable SNR should be taken into account. No a good way to introduce test which can only be tested, for example, with 600MHz aggregated CBW.

QC: rank 2 and MCS 11 is also feasible from testable SNR point of view.

Intel: rank 2 and MCS 11 can only be tested for 400MHz CBW.

E///: use different MCS/rank for different bands.

Intel: additional workload

QC: prefer to define band agnostic requirements. Applicability rule is needed if use different MCS/rank for different bands.

Intel: why we need to use rank 2 if the test purpose is to check CA capability?

QC: the test purpose to check high throughput can be achieved for CA capable UE.

Intel: for FR1, we only consider 16QAM but not 64QAM or 256QAM

QC: due to UE capability

Intel: testability need to be taken into account

QC: we did analysis based on commercial test equipment.

- Rank and MCS for FR2

    - Option 1: rank 2 and MCS 10

    - Option 2: rank 2 and MCS 11

    - Option 3: rank 1 and MCS[13]

    - Down-select to one option

Simulation results from companies:

| SNR at 70% TP (dB) | Rank 2 |  |  |  | Rank 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MCS 4 | MCS 10 | MCS 11 | MCS 13 | MCS 4 | MCS 10 | MCS 11 | MCS 13 |
| China Telecom |  | 8.9 | 9.8 | 12.1 |  |  |  |  |
| Intel |  | 8.1 |  | 11.9 |  |  |  | 5.0 |
| Ericsson (400MHz CBW) | 1.4 | 7.8 | 9.0 | 11.8 | -2.8 | 2.1 | 3.0 | 5.1 |
| Average alignment result | 1.4 | 8.3 | 9.4 | 11.9 | -2.8 | 2.1 | 3.0 | 5.1 |
| Average alignment result + ~2dB (IM and extra margin) | 3.4 | 10.3 | 11.4 | 13.9 | -0.8 | 4.1 | 5.0 | 7.1 |

Analysis on testable SNR from companies (Qualcomm, R4-1913496):

    - Observation 2: The table of SNR limitations for FR2 in WF [R4-1912832] is pessimistic compared to the SNR calculator spreadsheet in 38.810.

    - Observation 3: Commercial test equipment can achieve 1% EVM for 28GHz and 39GHz carrier frequency. We can have at least 40dB DL SNR with current test equipment capability.

    - Observation 4: EVM and phase noise in the test equipment are not the bottleneck for FR2 DL testing, and hence only the link budget shall be the focus from the testability point of view.

    - Proposal 5: If testable SNR is lower than the requirement SNR for FR2 CA, that test case will be skipped.

- PUCCH group

    - Option 1: one PUCCH group, i.e., A/N feedback on Pcell  with no UL CA (China Telecom, Huawei)

- PUCCH format for A/N feedback

    - Option 1: use PUCCH format 1 for no more than 2 DL CCs, and use PUCCH format 3 for more than 2 DL CCs. (China Telecom)

- PUCCH capacity

    - Option 1: no issue (China Telecom, Huawei)

- HARQ process number for FDD 15 kHz + TDD 30 kHz CA

    - Pcell:

    - Option 1: Same with that for single carrier test, i.e., 4 for FDD and 8 for TDD (China Telecom, Qualcomm, DOCOMO, Intel)

    - Option 2: 5 for FDD and 12 for TDD (Huawei)

    - Scell:

    - Option 1 (China Telecom, Intel)

      - FDD Pcell: 4 for FDD CC; 8 for TDD CC

      - TDD Pcell: 8 for TDD CC; 8 for FDD CC

    - Option 2 (Huawei)

      - FDD PCell: 5 for FDD CC; 8 for TDD CC

      - TDD PCell: 5 for FDD CC; 12 for TDD CC

CTC: take into account that the HARQ process number is even numbers but not odd numbers.

Intel: not all the even numbers are possible.

- HARQ process number for TDD 15 kHz + TDD 30 kHz CA

    - Pcell:

    - Option 1: Same with that for single carrier test, i.e., 8 for15kHz  and 30kHz SCS (China Telecom)

    - Scell:

    - Option 1 (China Telecom)

      - 15kHz SCS Pcell: 8 for 15 kHz CC; 11 for 30 kHz CC

      - 30kHz SCS PCell: 8 for 30 kHz CC; 8 for 15 kHz CC

- Single carrier performance for TDD-FDD CA and TDD-TDD CA with different SCSs

    - Pcell:

    - Option 1: Reuse single carrier requirements from FDD+FDD and TDD+TDD CA with the same SCS (China Telecom, Qualcomm)

    - Scell:

    - Option 1: Reuse single carrier requirements from FDD+FDD and TDD+TDD CA with the same SCS (China Telecom)

      - Observation: The link-level performance difference at 70% throughput is negligible for round-trip time of 10 slots and 20 slots. (China Telecom)

    - Option 2: Further discuss whether single carrier requirements from FDD+FDD and TDD+TDD CA can be reused (Qualcomm)

Discussion:

Agreement:

- Rank and MCS for FR2

    - Option 1: rank 2 and MCS 10

    - Option 2: rank 2 and MCS 11

    - Option 3: rank 1 and MCS[13]

    - Down-select to one option

- PUCCH group

    - one PUCCH group, i.e., A/N feedback on Pcell with no UL CA

- PUCCH format for A/N feedback

    - use PUCCH format 1 for no more than 2 DL CCs, and use PUCCH format 3 for more than 2 DL CCs.

- No PUCCH capacity issue

- HARQ process number for FDD 15 kHz + TDD 30 kHz CA

    - Pcell:

    - Option 1: Same with that for single carrier test, i.e., 4 for FDD and 8 for TDD

HW would like to check.

    - Scell: FFS

- HARQ process number for TDD 15 kHz + TDD 30 kHz CA: FFS

- Single carrier performance for TDD-FDD CA and TDD-TDD CA with different SCSs:

    - Further discuss whether single carrier requirements from FDD+FDD and TDD+TDD CA with the same SCS can be reused.

    - Encourage more performance analysis in the next meeting.

- Test applicability rule

Agreements in RAN4 #92bis (R4-1912832):

- Test applicability for different CA duplex mode

    - Option 1: Test all the supported CA duplex mode

    - Other options are not precluded

- FFS categorizing of CA capabilities

- FFS on test of different CA capabilities

- FFS on selection of CA configuration(s) for test among supported CA configurations

- FFS on selection CBW combination for test for selected CA configuration

Open issues:

- Test applicability for different CA duplex mode

    - Option 1: Test all the supported CA duplex mode (China Telecom, CMCC, Huawei)

- Pcell in TDD+FDD CA, TDD+TDD CA with different SCSs

    - Option 1 (China Telecom)

    - Discuss whether to conduct test for both FDD PCell and TDD Pcell for FDD 15 kHz + TDD 30 kHz CA

    - Discuss whether to conduct test for both 15 kHz PCell and 30 kHz Pcell for TDD 15 kHz + TDD 30 kHz CA.

QC: run simulation to see if there is performance difference. If the performance is similar, just test one of them.

CMCC: for LTE, we have UE capability whether to support TDD Pcell. For NR, tend to agree with QC.

- Categorizing of CA capabilities

    - Option 1: Define different capabilities for intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA with different numbers of bands. (China Telecom, Intel, CMCC)

    - Option 2: Define different capabilities for intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA (Huawei)

    - Other options are not precluded.

    - Companies to bring proposals on the demod spec structure for CA, with the motivation to minimize future maintenance.

Intel: do we need to update demod spec following the update on RF spec?

CMCC: yes, the same approach as LTE. Updates for each release but not each meeting

Intel: might be better to add RF spec as reference, but not update the demod spec accordingly.

QC: why test CA with different numbers of bands?

R&S: in RAN5, only test the lowest and highest numbers of bands.

QC: if follow RF spec, it should be endless.

- Test of different CA capabilities

    - Option 1: Test intra-band contiguous CA, Intra-band non-contiguous CA and Inter-band CA with the largest number of bands (Intel)

    - Option 1a: Test intra-band contiguous CA, Intra-band non-contiguous CA and Inter-band CA with the largest aggregated CBW (Intel)

    - Option 2: Test all the supported CA capabilities, including intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA with different numbers of bands (China Telecom, CMCC)

    - For inter-band CA with different numbers of bands, the largest aggregated channel bandwidth can be different, e.g., 30+2x100=230 MHz for CA_n1A-n78C, 30+20+100=150 MHz for CA_n3A-n8A-n78A.  (China Telecom)

    - Option 3: Any one of the supported CA capabilities (Huawei)

- Selection of CA configuration(s) and CBW combination

    - Option 1: Select any one of the supported CA configurations in each CA capability with largest aggregated CA bandwidth combination (China Telecom, DOCOMO, CMCC, Huawei)

    - Take into account that only up to rank 2 and up to MCS 13 are used in the test (China Telecom)

    - Prioritize the largest aggregated CA bandwidth combination with the largest number of CCs (Huawei)

    - Option 2: Maximum number of CCs (DOCOMO)

    - Option 3: Maximum number of bands (DOCOMO)

    - Option 4 (Intel)

    - For FR1:

      - Step 1: Select CA configurations with maximum number of CCs, on which UE capability field supportedSubCarrierSpacingDL is equal to SCSreq, among all supported CA configurations

      - Step 2: Select CA configurations with maximum number of CCs, on which UE capability field maxNumberMIMO-LayersPDSCH is higher or equal to νLayersreq, among all the selected CA configurations from Step 1

      - Step 3: Select any one of CA configurations, which contain CBW combination with the largest data rate not exceeding DataRatereq, among all the selected CA configurations from Step 2.

    - For FR2:

      - Step 1: Select CA configurations, which contain CBW combinations with SNRTEmax higher or equal to SNRreq, among all supported CA configurations

      - Step 2: Select CA configurations with maximum number of CCs, on which UE capability field supportedSubCarrierSpacingDL is equal to SCSreq, among all the selected CA configurations from Step 1

      - Step 3: Select CA configurations with maximum number of CCs, on which UE capability field maxNumberMIMO-LayersPDSCH is higher or equal to νLayersreq, among all the selected CA configurations from Step 2

      - Step 4: Select any one of CA configurations, which contain CBW combination with the largest data rate not exceeding DataRatereq and aggregated bandwidth with SNRTEmax higher or equal to SNRreq, among all the selected CA configurations from Step 3.

Intel: Not sure if rank 2 MCS 13 is supported for all the CCs in each CA configuration.

- Selection of CA configuration(s) and CBW combination

    - Further discuss by taking into account:

    - The supportedSubCarrierSpacingDL and maxNumberMIMO-LayersPDSCH and the scaling factor are reported for each CC for FR1 and FR2.

    - The testable SNR for FR2.

Discussion:

Agreement:

- Test applicability for different CA duplex mode

    - Test all the supported CA duplex mode

- For FDD PCell and TDD Pcell in FDD 15 kHz + TDD 30 kHz CA, and for 15 kHz PCell and 30 kHz Pcell in TDD 15 kHz + TDD 30 kHz CA

    - Conduct test based on UE capability (if any)

    - If Pcell in both carriers are supported and the performance is similar, FFS which one is used for the tests.

- Categorizing of CA capabilities

    - Option 1: Define different capabilities for intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA with different numbers of bands.

    - Option 2: Define different capabilities for intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA

    - Other options are not precluded.

    - Companies to bring proposals on the demod spec structure for CA, with the motivation to minimize future maintenance.

- Test of different CA capabilities

    - Option 1: Test intra-band contiguous CA, Intra-band non-contiguous CA and Inter-band CA with the largest number of bands

    - Option 2: Test intra-band contiguous CA, Intra-band non-contiguous CA and Inter-band CA with the largest aggregated CBW

    - Option 3: Test all the supported CA capabilities, including intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA with different numbers of bands

    - Option 4: Any one of the supported CA capabilities

    - Other options are not precluded.

- Selection of CA configuration(s) and CBW combination

    - Further discuss by taking into account:

    - The supportedSubCarrierSpacingDL and maxNumberMIMO-LayersPDSCH and the scaling factor are reported for each CC for FR1 and FR2.

    - The testable SNR for FR2.

- Related CRs

R4-1913424	CR to TS 38.101-4: FRC for FR1 Normal CA requirements (R16)
					38.101-4	  CR-0012  rev  Cat: B (Rel-16) v15.3.0
					Source: Intel Corporation

Abstract:

Discussion:

CTC: checked the numbers, and they are ok.

Recommendation: 		The document was postponed.

R4-1915113	draftCR: test structure for NR FR1 CA normal demodulation requirements for 4Rx
					38.101-4	  CR-  rev  Cat: B (Rel-16) v15.3.0
					Source: Huawei, HiSilicon

Abstract:

test structure for NR FR1 CA 4Rx normal demodulation requirements

Discussion:

Recommendation: 		The document was postponed.

Intel will draft the WF and simulation assumptions on CA normal demodulation requirements.

Tx ports larger than 8 and up to 32

- Contributions list and summary of proposals

| Tdoc | Title | Source | Proposal |
| --- | --- | --- | --- |
| R4-1913184 | Discussion on PMI reporting requirements for larger number of Tx ports | China Telecom | Proposal 1: Configure wideband PMI for 32 Tx ports and subband PMI for 16 Tx ports. Proposal 2: Cover rank 4 for 4Rx UE, and use rank 2 and rank 4 in tests with 32 and 16 Tx ports respectively. Proposal 3: For NZP CSI-RS, (k0, k1, k2, k3) = (2, 4, 6, 8), l0 = 5 for 16 Tx ports, and (k0, k1, k2, k3) = (2, 4, 6, 8), (l0, l1) = (5,7) for 32 Tx ports. Proposal 4: For Rel-15 type II codebook, use (N1, N2) = (4, 2) for 16 Tx ports, and (N1, N2) = (4, 4) for 32 Tx ports. |
| R4-1913372 | Simulation results of Type I PMI test cases | Samsung | Proposal 1: The subband PMI requirement with type I single panel codebook for 32 Tx ports should be defined in RAN4 Proposal 2: If RAN4 agree to introduce the requirement with subband PMI for type I single panel codebook, other channel model with high delay spread should be considered. |
| R4-1913373 | Initial simulation results for PMI reporting with type II codebook | Samsung | Observation1: As for Type II, it achieves significant gain compared to Type I codebook, the gain is about 9dB and 6.6dB for rank1 and rank 2 at SNR point at 90% of peak throughput, respectively. Observation2: The gap of Type II between following PMI and random PMI is extremely high than Type I codebook case Observation3: SNR point at 90% of peak throughput is lower than before due to the more accurate channel compression for Type II. Observation4: The TDL-A channel model has small delay spread which lacks of channel selectivity in frequency domain, there is no obvious performance difference between ‘SubbandAmplitude=OFF’ and ‘SubbandSmplitude=ON’ in the current cases, consequently, other channel model with high delay spread can be considered if RAN4 agreed to configure ‘SubbandSmplitude=ON’. Proposal1: Under beam steering model with dual-cluster beams, there is larger performance gap between Type II codebook and Type I –single Panel codebook, RAN4 should define new test cases for Type II codebook. Proposal 2: Regarding test metric, relative throughput ratio between following PMI with Type II codebook and following PMI with Type I codebook can considered as a candidate option. |
| R4-1913379 | Test case design for PMI reporting with Type II codebook | Samsung | Proposal 1: For NR Type II codebook construction, using below configurations - Number of ports: 16 Tx ports with (N1, N2) = (4,2) and (O1, O2) = (4,4) - L (numberOfBeams): 2 - Npsk (phaseAlphabetSize) : FFS for beam combining coefficient (phase) set size, Npsk =4 can be taken for initial simulation purpose - subbandAmplitude: FFS for 'true' or 'false' Proposal 2: Reusing beam steering approach with dual-cluster beams as as specified in B.2.3B.4A of TS 36.101 - Relative power ratio among two beams can be fixed as 1 (p =1) Proposal 3: For test metric, several approaches can be further considered: - Alt1 : TP ratio between following PMI under Type II codebook and following PMI under Type-I single panel codebook - Alt2: TP ratio between following PMI and rand PMI Proposal 4: below parameters can be used as starting point for initial summation purpose: - MCS and rank: 16QAM ½, rank2 - MIMO correlation: XP High - Channel model: TDLA30-35 |
| R4-1913485 | Simulation results for PMI test cases with 16,32 ports | Intel Corporation | Observation #1: With high antenna correlation and 4 layers maximum throughput is not achieved with 16 TX ports Observation #2: With 16 TX ports the throughput gains are measurable with subband PMI for 2 and 4RX Observation #3: The throughput gain with 32 TX and subband PMI is high and not measurable in a test |
| R4-1913486 | Discussion on PMI test cases with 16, 32 ports | Intel Corporation | Proposal #1: Introduce subband PMI test cases with 16 TX ports Observation #1: With high antenna correlation and 4 layers maximum throughput is not achieved with 16 TX ports Observation #2: It is desirable to have same test parameters for 2RX and 4RX test cases to ease test set up Observation #3: PMI test with rank 4 doesn’t provide any additional benefit Proposal #2: Do not introduce PMI test cases with Rank 4 for 16 or 32 TX with Type-I single panel codebook Proposal #3: Companies are encouraged to bring inputs on simulation and parameters to be discussed for defining PMI tests with Type II single panel codebook |
| R4-1914367 | Simulation results for CSI - PMI test cases | Ericsson | Proposal 1: Cover subband PMI reporting with 16Tx port test cases with TDLA30-5. Proposal 2: For subband PMI reporting test, set subband size to 8 for FDD and 16 for TDD. Proposal 3: Cover wideband PMI reporting with 32Tx port test cases. |
| R4-1914369 | Way forward on PMI reporting requirements for Tx ports larger than 9 and up to 32 | Ericsson | Way forward |
| R4-1914374 | Simulation assumptions for NR PMI reporting requirements for more than 8 Tx ports | Ericsson | Simulation assumptions |
| R4-1914745 | Simulation results for PMI test of Tx ports larger than 8 and up to 32 | Huawei, Hisilicon | Simulation results |
| R4-1914746 | Discussion on PMI test for Tx ports larger than 8 and up to 32 | Huawei, Hisilicon | Proposal1: Define test cases only for rank = 2 for both 2Rx and 4Rx, remove “FFS for rank = 4”. Proposal2: Define test cases only for wideband, remove “FFS for subband PMI”. |

- Test parameters for type I single-panel codebook

Related agreements in RAN4 #92bis (R4-1912834):

- Number of Tx ports

    - 16 and 32 for defining requirements.

    - Test applicability:

    - Conduct the tests based on UE capability. For UE supporting both 16 and 32 Tx ports, conduct test for 32 Tx ports.

- PMI tests:

    - Wideband PMI

    - FFS on subband PMI

- Antenna correlation:

    - XP high (as baseline)

- Propagation condition:

    - TDLA30-5 for wideband PMI

    - FFS: for subband PMI

- Rank:

    - For 2Rx UE, rank 2

    - For 4Rx UE, rank 2, FFS for rank 4

- MCS:

    - As baseline, for rank 2, 64QAM MCS 20 for 64QAM MCS table.

    - Other options will be considered if technical issues are identified based on simulation results.

Open issues:

- Whether to introduce subband PMI test

  - Yes (China Telecom, Samsung, Intel, Ericsson)

    - Option 1a: subband PMI for 16 ports, wideband PMI for 32 ports (China Telecom, Ericsson, Intel)

    - Option 1b: subband PMI for 16 ports (Intel)

    - Option 1c: subband PMI for 32 ports (Samsung)

  - No (Huawei)

QC: first discuss the propagation condition. Decide which requirement can be defined based on the simulation assumptions.

HW: Only wideband PMI is introduced in Rel-15. Why consider subband PMI test in Rel-16 for larger number of Tx ports.

Intel: In Rel-15, subband PMI is deprioritized with the aim to minimize the number of test cases. Subband PMI is an important feature.

Samsung: share Intel’s view. Considering NR is deployed in higher carrier frequency, large CBW is available.

HW: how can we handle the case with up to 8 Tx ports?

Intel: Larger than 8 Tx ports is considered in the WI scope.

Samsung: subband PMI provides larger gain over wideband PMI when the number of Tx ports is larger. Open on whether to define subband PMI for up to 8Tx ports.

Intel: prefer 16 ports for subband PMI considering that the gain over random PMI is too high for 32 ports.

Samsung: the gain is about ~7dB for subband PMI over random PMI for 32 ports in our simulation. Fine to use 16Tx ports.

- If subband PMI test for 16 Tx ports will be introduced, only 32 Tx ports will be covered in wideband PMI test.

- If subband PMI test for 16 Tx ports will not be introduced, both 16 and 32 Tx ports will be covered in wideband PMI test.

Intel: our understanding is that if subband PMI for 16 Tx ports is introduced, only 32 will be covered in wideband PMI.

Samsung: Already agreed to cover 16 and 32 Tx ports in wideband PMI test. If subband PMI test for 16 Tx ports will be introduced, 16 Tx ports for wideband will still be kept.

E///: agree with Intel. Not duplicate the tests.

- Propagation condition for subband PMI test

  - Option 1: TDLA30-5 (Ericsson)

  - Option 2: other channel model with high delay spread  (Samsung)

  - Option 3: TDLB100-5 (Intel, Samsung)

  - Option 4: TDLC300-5 (Samsung)

  - Option 5: 2-tap channel for Rel-15 subband CQI reporting requirement (QC)

E///: we provide simulation results for TDLA, but fine to use other channel models.

- Correlation matrix for subband PMI test: XP high

For Tx ports larger than 8 and up to 32, firstly decide the simulation assumptions for subband PMI reporting, and then decide whether to define requirements based on the performance gain of subband PMI over wideband/random PMI.

The purpose of the simulation: 1) justify the gain; or 2) check the testability

HW: it makes sense the compare the performance of subband and wideband. Provide simulation results for subband, wideband, random PMI based on the channel.

E///: different channel are used for subband and wideband PMI.

QC: random PMI is different for subband and wideband PMI.

HW: subband, wideband, random wideband PMI

Samsung: compare the follow and random PMI for both wideband and subband. 4 cases.

E///: same or different channel for wideband and subband.

QC: follow subband PMI over random subband PMI as baseline to limit simulation workload.

Intel: want to check the purpose of the simulation: 1) justify the gain; or 2) check the testability and feasible parameter for test. Gain by subband is already observed when RAN1 introduced this feature.

HW:  TDLC is in favor of subband PMI.

Companies are encouraged to run simulation for 4 cases. Consider the following priority if no time to run simulation for all 4 cases.

Priority for simulation: subband PMI, random subband PMI > wideband PMI > random wideband PMI

E///: limit the total number of test cases.

CTC: can be try to have only one option for each parameter for initial simulation?

- Subband size for subband PMI test

  - Option 1: 8 for FDD and 16 for TDD (Ericsson)

- Whether to introduce rank 4 test for 4Rx UE

  - Yes (China Telecom)

    - For 4Rx UE, rank 2 for 32 port, rank 4 for 16 Tx (China Telecom)

  - No (Intel, Huawei, Samsung, Qualcomm, Ericsson)

Samsung: high inter-layer interference with MMSE-IRC and XP channel.

E///: use rank 2 for

- First subcarrier index and first symbol location for NZP CSI-RS

  - Option 1 (China Telecom, HW)

    - (k0, k1, k2, k3) = (2, 4, 6, 8), l0 = 5 for 16 Tx ports, and (k0, k1, k2, k3) = (2, 4, 6, 8), (l0, l1) = (5, 7) for 32 Tx ports.

  - Option 2 (Qualcomm)

    - (k0, k1, k2, k3) = (2, 4, 6, 8), l0 = 5 for 16 Tx ports, and (k0, k1, k2, k3) = (2, 4, 6, 8), (l0, l1) = (5, 12) for 32 Tx ports.

HW: ok with option 1. Need more time to check option 2.

QC: just want to use the typical parameters. OK with both options for test purpose.

E///: option 1 as baseline, check option 2 before the next meeting.

Discussion:

Agreement:

To define requirement for wideband PMI reporting under TDLA30-5, firstly run simulation only for 32 Tx ports.

For Tx ports larger than 8 and up to 32, firstly decide the simulation assumptions for subband PMI reporting, and then decide whether to define requirements based on the simulation results under TDLC300-5.

- If subband PMI test for 16 Tx ports will be introduced, only 32 Tx ports will be covered in wideband PMI test.

- If subband PMI test for 16 Tx ports will not be introduced, both 16 and 32 Tx ports will be covered in wideband PMI test.

Companies are encouraged to run simulation for the following 4 cases. Consider the following priority if no time to run simulation for all 4 cases.

- Priority for simulation: subband PMI, random subband PMI > wideband PMI > random wideband PMI

Parameters for the initial simulation for subband PMI:

- Propagation condition: TDLC300-5

- Correlation matrix: XP high

- Number of Tx ports: 16

- Duplex mode: FDD

- 2Rx

- Subband size: 8 for FDD

Use rank 2 for all the PMI test

- First subcarrier index and first symbol location for NZP CSI-RS

  - Option 1

    - (k0, k1, k2, k3) = (2, 4, 6, 8), l0 = 5 for 16 Tx ports, and (k0, k1, k2, k3) = (2, 4, 6, 8), (l0, l1) = (5, 7) for 32 Tx ports.

  - Option 2

    - (k0, k1, k2, k3) = (2, 4, 6, 8), l0 = 5 for 16 Tx ports, and (k0, k1, k2, k3) = (2, 4, 6, 8), (l0, l1) = (5, 12) for 32 Tx ports.

  - Down-select to one option in the next meeting.

- Test parameters for type II codebook

Open issues:

- Codebook construction for 16 Tx ports

  - Option 1: (N1, N2) = (4, 2) and (O1, O2) = (4,4) (Samsung)

    - L (numberOfBeams): 2

    - Npsk (phaseAlphabetSize) : FFS for beam combining coefficient (phase) set size, Npsk =4 can be taken for initial simulation purpose

    - subbandAmplitude: FFS for  'true' or 'false'

  - Option 2: (N1, N2) = (4, 2) (China Telecom)

Intel, E///: need more time to check.

- Codebook construction for 32 Tx ports

  - Option 1: (N1, N2) = (4, 4) (China Telecom)

- Beam steering model

  - Option 1: beam steering model with dual-cluster beams as specified in B.2.3B.4A of TS 36.101 (Samsung)

    - Relative power ratio among two beams can be fixed as 1 (p =1)

- Test metric:

  - Option 1: Relative throughput ratio between following PMI with Type II codebook and following PMI with Type I codebook (Samsung)

  - Option 2: Relative throughput ratio between following PMI and rand PMI  (Samsung)

- MCS and rank:

  - Option 1: 16QAM 1/2, rank2 (Samsung)

- MIMO correlation:

  - Option 1: XP High (Samsung)

- Channel model:

  - Option 1: TDLA30-35 (Samsung)

Discussion:

Agreement:

Encourage companies to provide inputs on the following test parameters in the next meeting:

- Number of Tx ports

- (N1, N2)

- L (numberOfBeams)

- Npsk (phaseAlphabetSize)

- subbandAmplitude

- Beam steering model

- Test metric

- MCS and rank

- MIMO correlation

- Channel model

In the WF, provide the candidate options for each parameter for information purpose.

E/// will lead the WF and simulation assumptions on PMI tests.

TDD LTE-NR co-existence

- Contributions list and summary of proposals

| Tdoc | Title | Source | Proposal |
| --- | --- | --- | --- |
| R4-1913371 | Simulation results of UE demodulation requirements for LTE NR co-existence scenario in TDD | Samsung | Simulation results |
| R4-1913425 | Simulation results for TDD LTE-NR coexistence requirements | Intel Corporation | Simulation results |
| R4-1913426 | CR to TS 38.101-4: LTE-NR coexistence requirements for TDD mode (R16) | Intel Corporation | CR on requirements and FRC |
| R4-1914348 | Simulation results of NR PDSCH demodulation requirements for TDD LTE-NR coexistence | Ericsson | Simulation results |
| R4-1915109 | Simulation results for LTE-NR coexistence for TDD | Huawei, HiSilicon | Simulation results |

- Simulation results

Open issues:

- Simulation results

    - 5 companies provided simulation results in this meeting. The results are aligned.

    - Huawei summarize the simulation results

Discussion:

Agreement:

- Related CRs

R4-1913426	CR to TS 38.101-4: LTE-NR coexistence requirements for TDD mode (R16)
					38.101-4	  CR-0013  rev  Cat: B (Rel-16) v15.3.0
					Source: Intel Corporation

Abstract:

Discussion:

QC: the second DMRS location is missing.

Intel: covered in applicability CR. FRC is copied from the WF.

Check if the FRC in the CR is aligned with the WF agreed in the last meeting.

Recommendation: 		The document was postponed.