3GPP TSG-RAN WG4 Meeting #93                	R4-1915918

Reno, USA, 18 - 22 Nov, 2019

Source:	CATT

Title:	Ad-hoc for CSI-RS based L3 measurement

Agenda Item:	9.16.1

Document for:	Approval

- CSI-RS measurement bandwidth

Contributions submitted for the topic:

| TDoc | Title | Source | Type |
| --- | --- | --- | --- |
| R4-1914462 | Simulation results for CSI-RS based measurements | Nokia, Nokia Shanghai Bell | discussion |
| R4-1913451 | Simulation results for CSI-RS L3 measurement for FR1 | Intel Corporation | discussion |
| R4-1913569 | CSI-RS parameters for RRM core requirements | NTT DOCOMO, INC. | discussion |
| R4-1913588 | Simulation results for CSI-RS measurement BW | MediaTek inc. | discussion |
| R4-1913744 | Further discussion on CSI-RS measurement bandwidth for RRM measurement requirement | CATT | discussion |
| R4-1914459 | Discussion on CSI-RS measurement bandwidth | Nokia, Nokia Shanghai Bell | discussion |
| R4-1914837 | Simulation results of CSI-RSRP measurements for NR mobility | Huawei, HiSilicon | discussion |
| R4-1914838 | Discussion on CSI-RS based measurement requirements for NR mobility | Huawei, HiSilicon | discussion |

Issue 1: How many set of CSI-RS configuration (bandwidth of CSI-RS and density of CSI-RS resource) are needed to define CSI-RS based measurement requirements?

- Option 1: One set of configuration (MediaTek, Intel, Huawei)

- Option 2: 2 set of configuration (CATT, NTT DOCOMO)

Issue 2: If one set of configuration is agreed, then what’s the configuration for bandwidth of CSI-RS and density of CSI-RS resource?

- Option 1: 48PRBs and D = 3 (MediaTek, Huawei)

- Option 2: 48PRBs and D = 1 (Intel)

Issue 3: If multiple set of configuration is agreed, then what are the configurations for r bandwidth of CSI-RS and density of CSI-RS resource?

- Option 1: (CATT)

| Configuration | Measurement bandwidth [RB] | CSI-RS resource Density | TCSI-RS_measurement_Period |
| --- | --- | --- | --- |
| 0 | 48 | 1 | 5 samples |
| 1 | 48 | 3 | 3 samples |
|  | ≥96 | 1 or 3 |  |

- Option 2: (NTT DOCOMO)

| Configuration | Measurement bandwidth [RB] | CSI-RS resource Density |
| --- | --- | --- |
| 0 | 48 | 1 |
| 1 | 96 | 3 |

- Option 3: Define the CSI-RS measurement requirements respectively for 96 PRBs, 192 PRBs and 264 PRBs. (Nokia)

Agreements

CSI-RS configurations (bandwidth of CSI-RS and density of CSI-RS resource) used to define requirements

- Define requirements at least for 1 set of configurations

  - Option 1: 48PRBs and D = 1

  - Option 2: 48PRBs and D = 3

- FFS whether to define requirements for additional configurations

  - 48PRBs and D = 1

  - 48PRBs and D = 3

  - 96PRBs and D = 1

  - 96PRBs and D = 3

Discussion:

Agreements:

- CSI-RS based intra-frequency and inter-frequency measurement definition

Contributions submitted for the topic:

| TDoc | Title | Source | Type |
| --- | --- | --- | --- |
| R4-1913507 | On defintion of CSI-RS based intra-frequency and inter-frequency layers | Apple | discussion |
| R4-1913383 | [DRAFT] Reply LS on clarification about CSI-RS measurement | Huawei, HiSilicon | LS out |
| R4-1913452 | Discussion about intra-frequency definition of CSI-RS based L3 measurement | Intel Corporation | discussion |
| R4-1913570 | Definition of intra-frequency measurement for CSI-RS based L3 measurement | NTT DOCOMO, INC. | discussion |
| R4-1913589 | Definition of Intra and inter frequency for CSI-RS RRM | MediaTek inc. | discussion |
| R4-1913745 | Further discussion on SI-RS based intra-frequency and inter-frequency measurement definition | CATT | discussion |
| R4-1913746 | LS on CSI-RS based intra-frequency and inter-frequency Measurement definition | CATT | LS out |
| R4-1913861 | Discussion on CSI-RS based RRM requirements | CMCC | discussion |
| R4-1914436 | Definition for the CSI-RS based intra-frequency and inter-frequency measurement | Huawei, HiSilicon | discussion |
| R4-1914460 | CSI-RS based intra-f and inter-f measurement definition | Nokia, Nokia Shanghai Bell | discussion |
| R4-1914461 | CR to 38.133 CSI-RS based intra-frequency and inter-frequency measurements definition | Nokia, Nokia Shanghai Bell | CR |
| R4-1914790 | Discussion on definition of CSI-RS based RRM measurements | ZTE | discussion |

Issue 1: Definition of CSI-RS based intra-frequency and inter-frequency measurement

- Option 1 in WF (R4-1910956): (CMCC/CATT/Huawei):

CSI-RS based intra-frequency measurement: a measurement is defined as a CSI-RS based intra-frequency measurement provided that:

• the center frequency of CSI-RS on the serving cell and neighbor cell is the same, and

• the SCS of CSI-RS on the serving cell and neighbor cell is the same

CSI-RS based inter-frequency measurement: a measurement is defined as a CSI-RS based inter-frequency measurement provided that:

• the center frequency of CSI-RS on the serving cell and neighbor cell is not the same, or

• the SCS of CSI-RS on the serving cell and neighbor cell is not the same

- Option 2 in WF (R4-1910956): (Apple/NTT DOCOMO/Nokia):

CSI-RS based intra-frequency measurement: a measurement is defined as a CSI-RS based intra-frequency measurement provided that:

• the bandwidth of the CSI-RS on the neighbor cell is within the active BWP of the UE, and

• the SCS of CSI-RS on the serving cell and neighbor cell is the same

CSI-RS based inter-frequency measurement: a measurement is defined as a CSI-RS based inter-frequency measurement provided that:

• the bandwidth of the CSI-RS on the neighbor cell is not within the active BWP of the UE, or

• the SCS of the CSI-RS of the serving cell and neighbor cell is not the same

- Option 3: (ZTE)

  - CSI-RS based intra-frequency measurement: a measurement is defined as a CSI-RS based intra-frequency measurement provided

    - he centre frequency of the CSI-RS resource on the neighbour cell configured for measurement is the same as centre frequency of the CSI-RS resource on the serving cell configured for measurement, and

    - the bandwidth of the CSI-RS resource on the neighbour cell configured for measurement is the same as bandwidth of the CSI-RS resource on the serving cell configured for measurement, and

    - the subcarrier spacing of the two CSI-RS resources on the neighbour cell and on the serving cell are the same.

  - CSI-RS based inter-frequency measurement: a measurement is defined as a CSI-RS based inter-frequency measurement provided

    - the centre frequency of the CSI-RS resource on the neighbour cell configured for measurement is not the same as centre frequency of the CSI-RS resource on the serving cell configured for measurement, or

    - the bandwidth of the CSI-RS resource on the neighbour cell configured for measurement is not the same as bandwidth of the CSI-RS resource on the serving cell configured for measurement, or

    - the subcarrier spacing of the two CSI-RS resources on the neighbour cell and on the serving cell are the same.

- Option 4: (Qualcomm)

CSI-RS measurements to be considered intra-frequency if the CSI-RS of neighbor cell has the same location, bandwidth, SCS  and CP duration as that of serving cell and the CSI-RS lies within the active BWP. All CSI-RS measurements that it is not defined as in intra-frequency measurement are considered inter-frequency.

- Option 5: (MediaTek)

CSI-RS based intra-frequency measurement: a measurement is defined as a CSI-RS based intra-frequency measurement provided that:

• the center frequency of CSI-RS on the target cell is the same as center frequency of CSI-RS on the serving cell, and,

• the bandwidth of CSI-RS on the target cell is the same as bandwidth of CSI-RS resources on the serving cell, and,

• the bandwidth of CSI-RS resources of target cell configured for measurement is within the active BWP of serving cell, and,

• the SCS of intra-frequency CSI-RS should have the same SCS as UE’s active DL BWP, and,

• the CSI-RS is configured within the MO indicated as servingCellMO in ServingCellConfig, and,

• the CSI-RS is configured within the MO with intra-frequency SSB-based measurement, if SSB-based measurement is configured.

- Option 6: (Intel)

For the case that CSI-RS resource of serving cell is available:

Two options for the definition for CSI-RS intra frequency measurement

Option 1:

- the centre frequency of CSI-RS resources on the target cell configured for measurement is the same as centre frequency of CSI-RS resources on the serving cell configured for measurement, and,

    - the bandwidth of CSI-RS resources of target cell configured for measurement is within the active BWP of serving cell, and,

    - the SCS of CSI-RS on serving cell and target cell is the same

    - the CP type of CSI-RS on serving cell and target cell is the same

Option 2:

- the centre frequency of CSI-RS resources on the target cell configured for measurement is the same as centre frequency of CSI-RS resources on the serving cell configured for measurement, and,

- the bandwidth of CSI-RS resources on the target cell configured for measurement is the same as bandwidth of CSI-RS resources on the serving cell configured for measurement, and,

    - the SCS of CSI-RS on serving cell and target cell is the same

    - the CP type of CSI-RS on serving cell and target cell is the same

For case that CSI-RS resource of serving cell is not available and only SSB of serving cell is available

Option 1:

    - the bandwidth of CSI-RS resources of target cell configured for measurement is within the active BWP of serving cell,

Option 2:

- the centre frequency of CSI-RS resources on the target cell configured for measurement is the same as centre frequency of SSB on the serving cell configured for measurement, and,

- the bandwidth of CSI-RS resources on the target cell configured for measurement is the same as bandwidth of SSB on the serving cell configured for measurement, and,

    - the SCS of SSB on serving cell and CSI-RS on target cell is the same

    - the CP type of SSB on serving cell and CSI-RS on target cell is the same

Agreement

- Case 1: CSI-RS resource of serving cell is available

  - CSI-RS based intra-frequency measurement: a measurement is defined as a CSI-RS based intra-frequency measurement provided that:

    - the SCS of CSI-RS on the serving cell and neighbor cell is the same

    - the CP type of CSI-RS

      - Option 1: the CP type of CSI-RS on serving cell and target cell is the same

    - the center frequency, bandwidth and active BWP

      - Option 1: the center frequency of CSI-RS on the serving cell and neighbor cell is the same, and

      - Option 2: the bandwidth of the CSI-RS on the neighbor cell is within the active BWP of the UE, and

      - Option 3: the bandwidth of CSI-RS on the target cell is the same as bandwidth of CSI-RS resources on the serving cell

      - Any combination of options above

  - Note: the RAN4 design shall not contradict RAN2 agreements on MO signalling

- Case 2: CSI-RS resource of serving cell is not available

  - Option 1: All MO are inter-frequency

  - …

Discussion:

    - the center frequency, bandwidth and active BWP

      - Option 1: the center frequency of CSI-RS on the serving cell and neighbor cell is the same (CMCC)

      - Option 2: the bandwidth of the CSI-RS on the neighbor cell is within the active BWP of the UE

      - Option 2a: If the bandwidth of the maximum of CSI-RS resources BW in a MO is within active BWP, then the MO is intra-frequency, otherwise, this MO is inter-frequency

      - Option 3: the bandwidth of CSI-RS on the target cell is the same as bandwidth of CSI-RS resources on the serving cell  (QC)

      - Any combination of options above

- Case 2: CSI-RS resource of serving cell is not available

  - Option 1: All MO are inter-frequency

  - Option 2: No requirement for case 2.

CMCC: option 1, have some principle in general

Principle: the definition shall not impact RAN1’s agreement on the definition of the same MO

ZTE: option 1 or option1 combine with option3

Nokia/NTT DOCOMO/Apple: option 2

Agreements:

    - the CP type of CSI-RS

      - The CP type of CSI-RS on serving cell and target cell is the same

        - It is applied for SCS = 60KHz

Principle: the definition of intra-frequency and inter-frequency measurement shall not impact RAN1’s agreement on the definition of the same MO

Additional conditions to be considered for the definition:

Tentative agreement:

Case 1: RAN4 specify the CSI-RS based L3 measurement assuming the same center frequency, BW, SCS, CP type configured for all CSI-RS resources in the same MO.

  - FFS for slot configuration

  - FFS for other scenarios

Case 2: RAN4 specify the CSI-RS based L3 measurement assuming the same center frequency, SCS, CP type configured for all CSI-RS resources in the same MO.

- FFS for slot configuration

- FFS for other scenarios

- UE measurement capability

Contributions submitted for the topic:

| TDoc | Title | Source | Type |
| --- | --- | --- | --- |
| R4-1913590 | Discussion on measurement capability for CSI-RS RRM | MediaTek inc. | discussion |
| R4-1913747 | Discussion on CSI-RS based UE measurement capabilities | CATT | discussion |
| R4-1914789 | Discussion on UE measurement capability of CSI-RS based RRM measurements | ZTE | discussion |
| R4-1915071 | On Measurement capability of CSI-RS L3 measurement | OPPO | discussion |
| R4-1915214 | On CSI-RS measurement capability | Huawei, HiSilicon | discussion |

Issue 1: UE capability of total number of frequency layers to be monitored

- Option 1 (MediaTek, OPPO, Huawei): Shared UE capability by SSB based measurement and CSI-RS based measurement.

  - Option 1a (MediaTek, OPPO): No need to introduce additional frequency layer, reusing the current UE measurement requirements defined in TS38.133

  - Option 1b (Huawei): Increase the total number of SSB and CSI-RS frequency layers.

- Option 2 (ZTE): Separated UE capability for CSI-RS based measurement.

- Option 3 (CATT, Huawei):

  - UE shall be able to measure at least 3 CSI-RS frequency layers.

  - UE shall be able to measure at least 8 NR frequency layers in total, including SSB frequency layers and CSI-RS frequency layers.

  - UE shall be able to measure at least 14 carriers of all RATs in total.

Discussion:

Agreements:

Issue 2: Value of number of cells to be monitored per layer

- Option 1 (MediaTek, OPPO): No need to introduce additional number of cells to be monitored per layer based on L3 CSI-RS on top of the requirements already specified for SSB.

- Option 2 (Huawei): FFS on the total number of cells and CSI-RS resources UE should measure per MO.

- Option 3 (ZTE): New UE capabilities for number of cells

  - FR1: at least 8 identified cells in FR1 for intra frequency measurement and inter frequency measurement

  - FR2: at least 6 identified cells in FR2 for intra frequency measurement and inter frequency measurement.

Discussion:

Agreements:

Issue 3: Number of CSI-RS (resource/beams) to be monitored per layer

- Option 1(MediaTek): The values defined for SSB could be a starting point for further discussion.

- Option 2(ZTE): Large number of CSI-RS resources than number of SSBs in current spec should be specified for CSI-RS based measurement.

- Option 3 (CATT, OPPO): 32 CSI-RS resources per frequency layer.

Discussion:

Agreements:

Issue 4: UE capability to indicate maximum number of CSI-RS resources

- Option 1 (Huawei): Reuse existing UE capability maxNumberCSI-RS-RRM-RS-SINR and update the value.

- Option 2 (Huawei): Define another capability for UE to indicate maximum number of CSI-RS resources per MO in a slot.

Discussion:

Agreements:

- Intra and inter frequency measurement requirements

Contributions submitted for the topic:

| TDoc | Title | Source | Type |
| --- | --- | --- | --- |
| R4-1913571 | QCL assumptions for CSI-RS based L3 measurement | NTT DOCOMO, INC. | discussion |
| R4-1913591 | Cell identification requirements for CSI-RS RRM | MediaTek inc. | discussion |
| R4-1913748 | Discussion on CSI-RS based measurement requirements | CATT | discussion |
| R4-1914437 | Discussion on CSI-RS based L3 measurement requirements | Huawei, HiSilicon | discussion |

Issue 1: Applicability of CSI-RS based measurement requirements

- Option 1 (MediaTek): Do not specify requirements if associatedSSB is not configured.

- Option 2 (CATT, Huawei): Specify requirements for both cases associatedSSB is not configured and associatedSSB is configured.

- Option 3 (NTT DOCOMO): Study how to design RRM specifications when isQuasiColocated is OFF or not configured, e.g., larger scaling factor for Rx beam sweeping, no support, etc.

Discussion:

Agreements:

Issue 2: CSI-RS identification time

- Option 1 (Huawei):

- If associatedSSB is not configured, CSI-RS identification time is the CSI-RS measurement periodicity.

- If associatedSSB is configured, CSI-RS identification time includes Tpss/sss, PBCH decoding, AGC adjustment time and CSI-RS measurement periodicity.

- If CSI-RS is not QCL-ed to the associated SSB, UE needs to sweep the RX beam. If the CSI-RS is QCL-ed to the associated SSB, no Rx sweeping is needed.

- Option 2 (CATT):

- If associated SSB is not configured for CSI-RS resources, the CSI-RS identification time can be expressed as: TCSI-RS_identification_without_associatedSSB = TCSI-RS_measurement_period.

- If associated SSB is configured for CSI-RS resources, the CSI-RS identification time can be expressed as: TCSI-RS_identification_with_associatedSSB = TPSS/SSS_sync + TSSB_time_index + TCSI-RS_measurement_period.

- If the CSI-RS resource is configured with associated SSB and QCLed to the associated SSB, the CSI-RS based measurement requirement does not need to consider the scaling factor due to Rx beam sweeping.

- Option 3 (MediaTek):

- If associated SSB is configured for CSI-RS resources , CSI-RS based cell identification consists: 1) Cell search via SSB with AGC margin, 2) PBCH decoding and 3) CSI-RS measurement with AGC margin.

Discussion:

Agreements:

Issue 3: Scheduling restriction for CSI-RS based L3 measurement

- Option 1 (MediaTek):

- The scheduling restriction on the additional OFDM symbols before and after SSB is not needed for CSI-RS based L3 measurement.

- Option 2 (Huawei):

- If UE is not able to support mixed numerology of data and CSI-RS L3 mobility, the following scheduling restrictions apply due to intra-frequency CSI-RS based L3 measurement:

  - if the associatedSSB is configured, UE is not expected to transmit or receive on 2 data OFDM symbols impacted by CSI-RS resource symbol to be measured.

  - if the associatedSSB is not configured, UE is not expected to transmit or receive on the data OFDM symbol impacted by CSI-RS resource symbol to be measured, provided timing difference between the CSI-RS resource and the serving cell should be less than half CP corresponding to the SCS of the CSI-RS.

- When UE performs CSI-RS intra-frequency measurements in a TDD band, UE is not expected to transmit and receive on 2 data OFDM symbols impacted by CSI-RS resource symbol to be measured.

- Scheduling restriction shall be considered when UE performs RX beam sweeping.

Discussion:

Agreements:

Issue 4: Whether to restrict CSI-RS resources outside of DRX/MG duration

- Proposal from CATT:

Option 1: UE is allowed not to measure the CSI-RS resources that are not within DRX on-duration or measurement gap;

Option 2: No UE performance requirement is defined for the CSI-RS resources that are not within DRX on-duration or measurement gap;

Option 3: the network needs to make sure the configured CSI-RS resources falls within the configured measurement gaps

Discussion:

Agreements:

- Others

Contributions submitted for the topic:

| R4-1914959 | On synchronization assumption for CSI-RS measurement requirements | Huawei, HiSilicon | discussion |
| --- | --- | --- | --- |

Issue 1: Synchronization assumptions for CSI-RS based measurement

The discussion can be based on Huawei’s paper (R4-1914959).

- Proposals from Huawei:

- When CSI-RS resource for mobility is configured without associated SSB, the CSI-RS measurement requirements apply provided that receive timing difference between the CSI-RS resource and the serving cell is within X, where X is [CP/2 corresponding the SCS of CSI-RS].

- RAN4 to further study the measurement performance with timing error larger than half CP.

- When CSI-RS resource for mobility is configured with associated SSB, the CSI-RS measurement requirements is not conditioned on network synchronization.

- When CSI-RS resource for mobility is configured with associated SSB, but is not QCL-ed with the associated SSB, timing error should be considered in the CSI-RS measurement requirements.

Discussion:

Agreements: