3GPP TSG-RAN WG4 Meeting #93	R4-1916147

Reno, USA, 18th – 22th Nov. 2019

Title: 	TP on channel raster for NR V2X UE

Source: 	CATT, vivo

Agenda item:	9.4.3.2

Document for:	Approval

- Introduction

Based on the agreed way forward [1], this document presents a TP for TR 38.886 v0.4.0 [2] for the channel arrangement design for 5G V2X UE in Section 3 Annex.

- Reference

[1]	R4-19xxxxx, Way forward on the channel arrangement design for 5G V2X UE,

[2]	3GPP TR38.886 v0.4.0, V2X Services based on NR: User Equipment (UE) radio transmission and reception

- Annex: TP for channel arrangement in TR38.886

============================ Start of TP ============================

7.3	Channel arrangement

7.3.1	Channel arrangement in FR1

7.3.1.1	Channel raster

7.3.1.1.1	NR-ARFCN and channel raster

The NR-ARFCN and channel raster defined in subclause 5.4.2.1 in TS38.101-1 are applied for NR V2X.

For NR V2X UE, the reference frequency can be shifted by configuration.

FREF_V2X = FREF + Δshift + N * 5 kHz

Where

Δshift = 0 kHz or 7.5 kHz indicated in IE (frequencyShift7p5khz), and

N can be set as one of following values {-1, 0, 1}, are signalled by the network in higher layer parameters or configured by pre-configuration parameters.

7.3.1.1.2	Channel raster to resource element mapping

Channel raster to resource element mapping defined in subclause 5.4.2.2 in TS38.101-1 are applied for NR V2X.

7.3.1.1.3	Channel raster entries for each operating band

The channel raster entries for each operating band defined in subclaue 5.4.2.3 in TS38.101-1 are applied for NR V2X. The RF channel positions on the channel raster in each NR V2X operating band are given through the applicable NR-ARFCN in Table 7.3.1.1-1, using the channel raster to resource element mapping in subclause 7.3.1.1.2.

Table 7.3.1.1-1: Applicable NR-ARFCN for NR V2X operating band

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n47 | 15 | 790334 – <1> – 795000 | 790334 – <1> – 795000 |
|  | [30] | [790334 – <2> – 795000] | [790334 – <2> – 795000] |
|  | [60] | [790334 – <4> – 795000] | [790334 – <4> – 795000] |

7.3.1.2	Synchronization raster

There is no synchronization raster definition for NR V2X in the specification.

7.3.2	Channel arrangement in FR2

============================ End of TP ============================