3GPP TSG-WG RAN4 Meeting #94-e	R4-2000175

# Online, 24th February – 6th March, 2020

Source: 	Nokia, Nokia Shanghai Bell, [AT&T]

Title: 					TP to TR 36.716-02-01: CA_2-26

Agenda Item: 			7.2.3

Document for:	Approval

# Introduction

This TP introduces the following LTE inter-band CA for 2 bands DL with 1 band UL for the CA bands CA_2-26.

- CA_2A-26A-BCS0

# Discussion

Since the revised WID has 20 MHz channel bandwidth which is undefined for band 26, it shall be updated to remove 20MHz. A proposed WID update and TP to the TR 36.716-02-01 is provided in the following.

Required revions to Revised WID: Rel16 LTE inter-band CA for 2 bands DL with 1 band UL (RP-192525).

**************************** Start of changes ********************************************

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | BCS |  |
| CA_2A-26A | - | 2 |  | Yes | Yes | Yes | Yes | Yes | 35 |  | 0 |  |
|  |  | 26 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |

**************************** End of changes ********************************************

TP to TR 36.716-02-01

**************************** Start of changes ********************************************

## 5.X	CA_2-26

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: Supported E-UTRA bandwidths per CA configuration for 2-band DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| CA_2A-26A | 2 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  | 26 |  |  | Yes | Yes | Yes |  |  |  |

### 5.X.2	∆TIB and ∆RIB values

Since ther is no harmonic relation for this band combination, the proposed ∆TIB and ∆RIB values of CA_2-26 are based on a typical high-low band dixpler architecture without a harmonic trap filter, which is 0.3dB for Tx and 0.0dB for Rx.

Table 5.X.2-1: ΔTIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2-26 | 2 | 0.3 |
|  | 26 | 0.3 |

Table 5.X.2-2: ΔRIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_2-26 | 2 | 0 |
|  | 26 | 0 |

### 5.X.3	REFSENS requirements

There is no REFSENS exception for CA_2A-26A.

**************************** End of changes ********************************************