3GPP TSG-RAN WG4 Meeting #94-e						R4-2000271

Electronic Meeting, 24 Feb. – 6 Mar., 2020

Source:	Samsung

Title:	Discussion on beam correspondence in Rel-16

Agenda item:	8.14.1.2

Document for:	Discussion

- Background

In RAN4#94 WG meeting, the WF [1] on side conditions of SSB based and CSI-RS based beam correspondence tests was approved. Open issues need to be further discussed including feasibility of SSB based beam correspondence, configuration and side condition of CSI-RS based beam correspondence.

In RAN#86 plenary meeting, the WF [2] on Rel-16 Beam correspondence enhancement was approved which allows discussion on additional beam correspondence enhancement by utilizing the existing UE measurement:

- For BC performance enhancement for both bit #0 and bit #1 UE, RAN4 may continue discussing the additional performance enhancement and test configuration enhancement by utilizing the existing UE measurement including RSRP and/or L1-SINR

Based on above, in this paper we further discuss on Rel-16 beam correspondence enhancement topics focusing on SSB base BC feasibility, CSI-RS based BC side condition, and bit-0 UE further enhancement aspects.

Discussion

Feasibility of SSB based BC

In last RAN4 WG meeting, it is addressed in WF [1] that feasibility of SSB based BC needs further study:

- By RAN4 #94, RAN4 could specify BC based on SSB only unless RAN4 find critical technical problems, considering:

  - Side condition feasibility

  - Common understanding of UE behavior

  - Performance difference with SSB + CSI-RS based BC

In Rel-16 beam correspondence, SSB is assumed with wide beam and CSI-RS is assumed with narrow beam, however the test equipment has only one beam width that is narrow beam. According to TS 38.133, there is 7dB gain difference between wide beam and narrow beam. To show the gain difference between wide beam and narrow beam, 7dB lower PSD of SSB at reference point is required compared with CSI-RS. Rel-15 BC side condition is SNR>=6dB, accordingly SSB side condition of SNR>=-1dB in Rel-16 matches with that of Rel-15, which is too low to ensure BC performance.

Even we ignore the wide beam assumption and just apply higher SNR condition, there is limitation of testability SNR range. Rel-15 BC side condition SNR>=6dB is already the upper SNR range of testability considering 400MHz channel BW test. However, there is no CSI-RS configured in SSB based BC test which indicates less RS available compared with Rel-15 BC. To compensate the absence of CSI-RS, SNR of SSB is expected to be increased to higher than 6dB, but SNR has no room to increase due to testability. Moreover, the absence of CSI-RS for beam refinement means no RS for P3 beam refinement. Even UE can totally rely on SSB for P3 beam refinement, negative impact can still be expected since less RS available compared with SSB + CSI-RS based BC.

Observation 1:	SNR side condition for SSB based BC is limited by testability SNR range, and BC performance is impacted due to less RS available.

On the other hand, different from RRM, RF test focuses on narrow beam test after beam refinement, CSI-RS based BC is more suitable for this purpose. Beam correspondence is essential feature and is verified with MOP test, in other words, MOP test results are beam correspondence test results. As analyzed in [3], if SSB based BC is also specified, there will be two sets of MOP requirements which makes the specification divergent, and there will be two sets of MOP test results for single UE which seems not so reasonable: two sets of “Maximum” output power, two sets of beam correspondence capability, and so on. So it is better not to specify SSB based BC from the RF requirement perspective. Even though SSB based BC is to be specified finally, it is better to use other metric instead of MOP metric. As discussed on the topic of BC in initial access, SSB based BC is also not suitable to verify with MOP metric.

Observation 2:	From RF test point of view, SSB based BC is not suitable to specify with MOP metric.

Based on above analysis, it is proposed that SSB based BC is not specified, or to be specified with other metric instead of MOP metric.

Proposal 1:	SSB based BC is not specified, or to be specified with other metric instead of MOP metric.

Side condition of CSI-RS based BC

Based on the discussions in previous meetings, it is the consensus that CSI-RS based BC will be specified and the parameter configuration is getting converged.

P1 CSI-RS is not important in Rel-15 BC because SSB and CSI-RS are both present with the same PSD. For Rel-16 CSI-RS based BC, SSB is restricted to a low power level, thus P1 CSI-RS is better to be configured with higher PSD with QCL type D to SSB.

About P2 CSI-RS, we think it is not necessary in test environment. P2 CSI-RS is for the DL TX beam refinement at gNB side, however, the antenna of gNB simulator in test equipment is not antenna array but a single probe without beam forming capability. So it will be a waste of resource to configure P2 CSI-RS.

P3 CSI-RS plays the dominant role in CSI-RS based BC. Since P2 CSI-RS is not necessary to be configured, P3 CSI-RS can be configured with QCL type D to P1 CSI-RS. To achieve best beam correspondence performance, it is suggested to use maxNumberRxBeam based on UE capability rather than a fixed value as P3 CSI-RS repetitions per resource set.

Proposal 2:	P2 CSI-RS is not necessary for CSI-RS based BC, and Alt.1 is proposed for most of parameters as shown in the table.

| Parameter | Value in WF | Our proposal |
| --- | --- | --- |
| P1 CSI-RS periodicity | Alt.1: P1 CSI-RS is configured with [TBD] ms periodicity, the QCL (qcl-TypeD) relation is configured as ‘SSB’ Alt.2: P1 CSI-RS is not configured; instead aperiodic P2 CSI-RS can be considered if necessary. If P2 CSI-RS is supported, its qcl-TypeD is ‘SSB’ [2] Alt.3: P1 CSI-RS is configured with [TBD] ms periodicity, the QCL (qcl-TypeD) relation is configured as ‘none’ | Alt.1 |
| P3 CSI-RS repetitions per resource set | Alt. 1: maxNumberRxBeam in UE capability IE of MIMO-ParametersPerBand Alt. 2: 8 | Alt.1 |
| P3 CSI-RS configuration repetition | on |  |
| P3 CSI-RS trigger | Alt.1: once P1 CSI-RS is finished Alt.2: once every SSB cycle (20 ms) if P1 CSI-RS is not configured * The test time for Alt.1 is assumed less than or equal to Alt.2 | Alt.1 |
| Tracking CSI-RS periodicity | reuse Rel-15 60 kHz SCS: 40 slots for CSI-RS resources 1 and 2 120 kHz SCS: 80 slots for CSI-RS resources 1 and 2 |  |
| P3 CSI-RS QCL info | Alt.1: Type D to P1 CSI-RS Alt.2: If P2 CSI-RS is transmitted; - Type A to TRS - Type D to P2 CSI-RS Otherwise; - Type C to SSB - Type D to SSB | Alt.1 |
| P1 CSI-RS QCL info | Alt.1: P1 CSI-RS is transmitted and the QCL relation is configured as ‘SSB’ [14] Alt.2: P1 CSI-RS is not transmitted [2] Alt.3: P1 CSI-RS is transmitted and the QCL relation is configured as ‘none’ | Alt.1 |

Another issue for CSI-RS based BC is the side condition. As shown in the WF [1], two methods to achieve “CSI-RS only” condition are listed:

- How to achieve “CSI-RS only” condition

  - Method 1: DUT is configured with an active BWP containing no SSB

  - Method-2: SSB in wide beam and CSI-RS in fine beam from TE

  - Method-3: SSB and CSI-RS are present, but SSB’s PSD is back-off by XdB from CSI-RS

    - X will be chosen during the RAN4#94 meeting

  - A single method will be chosen during the RAN4 #94 meeting

As commented in last meeting, neither Method-1 nor Method-3 is perfect. For Method-1, the BWP mechanism will cause UE switch back and forth to receive SSB frequently; moreover, this method does not work for the test of 400MHz channel BW. So Method-1 is not feasible. Method-3 is feasible but it can not guarantee “CSI-RS only” condition at all AoAs. Assume the CSI-RS SNR=6dB and SSB SNR=-1dB at the AoA of 50% spherical coverage, then at the AoA of beam peak direction the estimated CSI-RS SNR=17dB and SSB SNR=10dB (calculated based on the around 11dB delta between peak EIRP spec and spherical coverage spec).

Observation 3:	Neither Method-1 nor Method-3 is perfect to achieve “CSI-RS only” condition. Method-1 is not feasible and Method-3 may has too high SSB SNR at many AoAs (measurement grids)

To overcome the drawback of Method-3, UE measurement report of SS-SINR can be utilized. For each AoA of CSI-RS based BC test, the power level of SSB is initially set the same as that of CSI-RS, and then decrease SSB power until UE SSB based SS-SINR measurement reporting getting lower than certain level, e.g. SS-SINR<=-1dB (7dB lower than CSI-RS SNR considering gain difference between wide beam and narrow beam). With this kind of optimization to Method-3, SSB power is controlled to a low level for all AoAs, consequently “CSI-RS only” condition is achieved.

To compensate the UE measurement accuracy of SS-SINR, 2dB extra margin can be considered, i.e., SS-SINR<=-3dB as the threshold for SSB power level control. Detailed configuration is illustrated in the following table:

Table 2.2-1 side condition for CSI-RS based beam correspondence

| Angle of arrival | NR operating bands | Maximum SSB_RP Note 2 | SSB Ês/Iot | Minimum CSI-RS_RP Note 2 | CSI-RS Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSCSI-RS | dB | dBm / SCSCSI-RS | dB |
|  |  | SCSCSI-RS = 120 kHz |  | SCSCSI-RS = 120 kHz |  |
| All angles Note 1 | n257 | power level (PSD) of SSB is initially set the same as that of CSI-RS, and then decrease SSB power until UE SSB based SS-SINR measurement reporting is within the threshold (≤-3dB) | ≤-3 | -96.4 | ≥6 |
|  | n258 |  |  | -96.4 |  |
|  | n260 |  |  | -92.1 |  |
|  | n261 |  |  | -96.4 |  |
| Note 1: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ΣMBP and Spherical coverage values are increased by ΣMBS, the UE multi-band relaxation factor in dB specified in clause 6.2.1. Note 2: Values specified at the Reference point to give maximum SSB Ês/Iot and minimum CSI-RS Ês/Iot, with no applied noise. |  |  |  |  |  |

Proposal 3:	An optimization to Method-3 is proposed to effectively achieve “CSI-RS only” condition by utilizing UE measurement reporting of SS-SINR. The side condition for CSI-RS is SNR>=6dB with fixed PSD for all AoAs, and the side condition for SSB is SNR<=-3dB with dynamic PSD for each AoA.

BC bit-0 UE further enhancement

About beam correspondence capability, it is still controversial on how to handle BC bit-0 UE in Rel-16. Some companies think that BC bit-0 UE shall not be allowed in Rel-16, and some companies think that Rel-16 BC capability shall apply the same rule as Rel-15. As a compromise, we propose that BC bit-0 UE can be kept in Rel-16 but be also further enhanced.

As discussed in previous paper [3], in Rel-15 it is assumed the same RSRP error for all AoAs which indicates the same baseband SNR for different AoAs, therefore a fixed M_SRS value for all AoAs is used for uplink beam sweeping during EIRP2 testing. In fact, the SNR at UE baseband is different for different AoAs under noise-free test environment. It is the conflict between simulation assumption and practice. After L1-SINR reporting is introduced to Rel-16 beam correspondence, the EIRP2 measurement can be changed from fixed M_SRS value to adaptive M_SRS value depending on L1-SINR UE measurement reporting. Table 2.3-1 is shown as an example for comparison between Rel-15 and Rel-16 to illustrate the enhancement for BC bit-0 UE.

Table 2.3-1 Illustration of BC bit-0 UE enhancement between Rel-15 and Rel-16

| L1-SINR reporting value | M_SRS value for UL beam sweeping in Rel-15 | M_SRS value for UL beam sweeping in Rel-16 |
| --- | --- | --- |
| L1-SINR >= A | 8 | Adaptive, e.g. 0 |
| B =< L1-SINR < A | 8 | Adaptive, e.g. 2 |
| C =< L1-SINR < B | 8 | Adaptive, e.g. 4 |
| L1-SINR < C | 8 | Adaptive, e.g. 8 |

EIRP2 has many disadvantages due to UL beam sweeping.  It takes more network resource, more test time and more power consumption, etc. According to above table, we can see that the UE measurement reporting enhancement of Rel-16 BC bit-0 UE can save network resource and test time and so on due to less M_SRS value per L1-SINR reporting.

Observation 4:	After introducing L1-SINR reporting, the Rel-16 BC bit-0 UE can be enhanced in terms of saving network resource, test time and so on.

Based on above analysis, it is proposed to introduce L1-SINR reporting in Rel-16 beam correspondence as enhancement to BC bit-0 UE.

Proposal 4:	Introduce L1-SINR reporting in Rel-16 beam correspondence as enhancement to BC bit-0 UE.

Conclusion

Observation 1:	SNR side condition for SSB based BC is limited by testability SNR range, and BC performance is impacted due to less RS available.

Observation 2:	From RF test point of view, SSB based BC is not suitable to specify with MOP metric.

Proposal 1:	SSB based BC is not specified, or to be specified with other metric instead of MOP metric.

Proposal 2:	P2 CSI-RS is not necessary for CSI-RS based BC, and Alt.1 is proposed for most of parameters as shown in the table.

Observation 3:	Neither Method-1 nor Method-3 is perfect to achieve “CSI-RS only” condition. Method-1 is not feasible and Method-3 may has too high SSB SNR at many AoAs (measurement grids)

Proposal 3:	An optimization to Method-3 is proposed to effectively achieve “CSI-RS only” condition by utilizing UE measurement reporting of SS-SINR. The side condition for CSI-RS is SNR>=6dB with fixed PSD for all AoAs, and the side condition for SSB is SNR<=-3dB with dynamic PSD for each AoA.

Observation 4:	After introducing L1-SINR reporting, the Rel-16 BC bit-0 UE can be enhanced in terms of saving network resource, test time and so on.

Proposal 4:	Introduce L1-SINR reporting in Rel-16 beam correspondence as enhancement to BC bit-0 UE.

References

- R4-1916172 	“WF on side conditions of SSB based and CSI-RS based beam correspondence tests”, Apple

- RP-193204 	“Proposed Way Forward on Rel-16 Beam correspondence enhancement”, Samsung, Nokia, Intel, Sony, LG, Ericsson, CATT, ZTE

- R4-1913251 	“Discussion on more enhancement to beam correspondence in Rel-16”, Samsung