3GPP TSG-RAN WG4 Meeting #94-e              	R4-2000310

Online, 24th February– 6th March, 2020

Agenda item:	7.13.4.1

Source:	Samsung

Title:	Simulation results for HST PUSCH in LTE Rel-16

Document for:	Discussion

# 1	Introduction

During the HST PUSCH for LTE Rel-16 discussion in RAN4#92b meeting, the simulation assumption for PUSCH requirement is captured in WF [1].

In the last meeting, interested companies have provided the results. Considering the span of results are large, one more meeting cycle is agreed to further align the result during the last RAN plenary meeting.

In this contribution, the updated ideal and impairment simulation results are provided for LTE HST Rel-16 requirements finalization. Meanwhile, the view of how to handle the requirement is provided.

# 2	Simulation results

## 2.1	Simulation assumption for PUSCH

As captured in the WF [1] as following

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
|  | Open space |  | Tunnel for multi-antennas |  |
|  | Bi-directional (1st priority) | Uni-directional | Bi-directional (1st priority) | Uni-directional |
| Ds | 1000 m |  | 300 m |  |
| Dmin | 50m |  | 2m |  |
| V | 500km/h |  | 500km/h |  |
| fd | 1944Hz 1750Hz |  |  |  |

In table 1, we provide the simulation assumptions for BS demodulation based on agreement.

Table 1: simulation assumptions for PUSCH demodulation performance evaluation

| Parameters | Unit | Values |
| --- | --- | --- |
| Bandwidth | MHz | 1.4/3/5/10/15/20 |
| MCS for PUSCH |  | QPSK 1/3 |
| Propagation condition and correlation matrix |  | AWGN |
| Antenna configuration |  | 1x2 |
| Reference receiver |  | MRC |
| Noise estimation |  | Practical |
| Time and frequency track |  | Practical |

## 2.2	Simulation Results

In this subsection, the ideal and impairment results for PUSCH under two scenarios are provided to finalize the requirement of HST.

Table 2: ideal results and impairment results for HST PUSCH requirements in open space environment

| SCS, BW | MCS | Antenna Configuration | Doppler Value | Test Metric | SNR (ideal results) | SNR (impairment results) |
| --- | --- | --- | --- | --- | --- | --- |
| 15KHz, 1.4MHz | QPSK 1/3 | 1x2 | 1944Hz | 30% | -7.2 | -5.2 |
|  |  |  |  | 70% | -2.5 | -0.5 |
| 15KHz, 3MHz | QPSK 1/3 | 1x2 | 1944Hz | 30% | -7.1 | -5.1 |
|  |  |  |  | 70% | -3 | -1 |
| 15KHz, 5MHz | QPSK 1/3 | 1x2 | 1944Hz | 30% | -7.8 | -5.8 |
|  |  |  |  | 70% | -3.5 | -1.5 |
| 15KHz, 10MHz | QPSK 1/3 | 1x2 | 1944Hz | 30% | -7.8 | -5.8 |
|  |  |  |  | 70% | -3.3 | -1.3 |
| 15KHz, 15MHz | QPSK 1/3 | 1x2 | 1944Hz | 30% | -7.9 | -5.9 |
|  |  |  |  | 70% | -3.45 | -1.45 |
| 15KHz, 20MHz | QPSK 1/3 | 1x2 | 1944Hz | 30% | -7.9 | -5.9 |
|  |  |  |  | 70% | -3.75 | -1.75 |
| 15KHz, 1.4MHz | QPSK 1/3 | 1x2 | 1750Hz | 30% | -7.2 | -5.2 |
|  |  |  |  | 70% | -2.75 | -0.75 |
| 15KHz, 3MHz | QPSK 1/3 | 1x2 | 1750Hz | 30% | -7.2 | -5.2 |
|  |  |  |  | 70% | -3.15 | -1.15 |
| 15KHz, 5MHz | QPSK 1/3 | 1x2 | 1750Hz | 30% | -7.8 | -5.8 |
|  |  |  |  | 70% | -3.7 | -1.7 |
| 15KHz, 10MHz | QPSK 1/3 | 1x2 | 1750Hz | 30% | -7.8 | -5.8 |
|  |  |  |  | 70% | -3.4 | -1.4 |
| 15KHz, 15MHz | QPSK 1/3 | 1x2 | 1750Hz | 30% | -7.9 | -5.9 |
|  |  |  |  | 70% | -3.5 | -1.5 |
| 15KHz, 20MHz | QPSK 1/3 | 1x2 | 1750Hz | 30% | -7.9 | -5.9 |
|  |  |  |  | 70% | -4.1 | -2.1 |

Table 3: ideal results and impairment results for HST PUSCH requirements in tunnel environment

| SCS, BW | MCS | Antenna Configuration | Doppler Value | Test Metric | SNR (ideal results) | SNR (impairment results) |
| --- | --- | --- | --- | --- | --- | --- |
| 15KHz, 1.4MHz | QPSK 1/3 | 1x1 | 1944Hz | 30% | -3.7 | -1.7 |
|  |  |  |  | 70% | 0.6 | 2.6 |
| 15KHz, 3MHz | QPSK 1/3 | 1x1 | 1944Hz | 30% | -4.3 | -2.3 |
|  |  |  |  | 70% | 0.6 | 2.6 |
| 15KHz, 5MHz | QPSK 1/3 | 1x1 | 1944Hz | 30% | -4.8 | -2.8 |
|  |  |  |  | 70% | -0.4 | 1.6 |
| 15KHz, 10MHz | QPSK 1/3 | 1x1 | 1944Hz | 30% | -4.8 | -2.8 |
|  |  |  |  | 70% | 0.5 | 2.5 |
| 15KHz, 15MHz | QPSK 1/3 | 1x1 | 1944Hz | 30% | -4.8 | -2.8 |
|  |  |  |  | 70% | -0.35 | 1.65 |
| 15KHz, 20MHz | QPSK 1/3 | 1x1 | 1944Hz | 30% | -4.9 | -2.9 |
|  |  |  |  | 70% | -0.4 | 1.6 |
| 15KHz, 1.4MHz | QPSK 1/3 | 1x1 | 1750Hz | 30% | -3.9 | -1.9 |
|  |  |  |  | 70% | 0.55 | 2.55 |
| 15KHz, 3MHz | QPSK 1/3 | 1x1 | 1750Hz | 30% | -4.5 | -2.5 |
|  |  |  |  | 70% | 0.35 | 2.35 |
| 15KHz, 5MHz | QPSK 1/3 | 1x1 | 1750Hz | 30% | -4.8 | -2.8 |
|  |  |  |  | 70% | -0.5 | 1.5 |
| 15KHz, 10MHz | QPSK 1/3 | 1x1 | 1750Hz | 30% | -4.8 | -2.8 |
|  |  |  |  | 70% | 0 | 2 |
| 15KHz, 15MHz | QPSK 1/3 | 1x1 | 1750Hz | 30% | -4.8 | -2.8 |
|  |  |  |  | 70% | -0.4 | 1.6 |
| 15KHz, 20MHz | QPSK 1/3 | 1x1 | 1750Hz | 30% | -4.9 | -2.9 |
|  |  |  |  | 70% | -0.8 | 1.2 |

## 2.3	Requirements handling

The HST requirement defined in LTE Rel-13 is specified as the following tables

| Channel Bandwidth [MHz] | Cyclic prefix | FRC (Annex A) | Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.4 | Normal | A3-2 | 1 | 1 | HST Scenario 3 | 30% | -1.5 |
|  |  |  |  |  |  | 70% | 1.9 |
|  |  |  |  | 2 | HST Scenario 1 Low | 30% | -3.9 |
|  |  |  |  |  |  | 70% | -0.6 |
| 3 | Normal | A3-3 |  | 1 | HST Scenario 3 | 30% | -2.1 |
|  |  |  |  |  |  | 70% | 1.6 |
|  |  |  |  | 2 | HST Scenario 1 Low | 30% | -4.5 |
|  |  |  |  |  |  | 70% | -1.0 |
| 5 | Normal | A3-4 |  | 1 | HST Scenario 3 | 30% | -2.6 |
|  |  |  |  |  |  | 70% | 1.3 |
|  |  |  |  | 2 | HST Scenario 1 Low | 30% | -5.1 |
|  |  |  |  |  |  | 70% | -1.4 |
| 10 | Normal | A3-5 |  | 1 | HST Scenario 3 | 30% | -2.7 |
|  |  |  |  |  |  | 70% | 1.2 |
|  |  |  |  | 2 | HST Scenario 1 Low | 30% | -5.4 |
|  |  |  |  |  |  | 70% | -1.5 |
| 15 | Normal | A3-6 |  | 1 | HST Scenario 3 | 30% | -2.7 |
|  |  |  |  |  |  | 70% | 1.2 |
|  |  |  |  | 2 | HST Scenario 1 Low | 30% | -5.2 |
|  |  |  |  |  |  | 70% | -1.4 |
| 20 | Normal | A3-7 |  | 1 | HST Scenario 3 | 30% | -2.7 |
|  |  |  |  |  |  | 70% | 1.2 |
|  |  |  |  | 2 | HST Scenario 1 Low | 30% | -5.3 |
|  |  |  |  |  |  | 70% | -1.4 |

With small bandwidth, such as 1.4MHz and 3MHz, the SNR of 70%TP is lower than large bandwidth performance, around 1dB. Due to the limited DMRS resource in small bandwidth, it is reasonable that there is performance loss with considering the inaccuracy frequency offset estimation. While as indicated the simulation summary [2], the performance with different bandwidth in some companies results are very similar.

Observation 1: Due to the limited DMRS resource in small bandwidth, it is reasonable that there is performance loss with considering the inaccuracy frequency offset estimation.

As indicated in table, the SNR requirement with 70% TP is about -0.6 to -1.4dB. The alignment results should be around with -2.6 to -3.4dB with considering 2dB implementation margin. Considering the span of each company alignment result, the best performance should be around with -3.6 to -4.4dB.

Compare with LTE Rel-13 with 1340Hz Doppler value in HST open scenario, only the Doppler value is increasing with 1944Hz in the simulation assumption for LTE HST Rel-16. The maximum Doppler value with 1944Hz is out of range of DMRS frequency offset tracking ability, even the DMRS in PUCCH. To some extent, with large Doppler value, there should be some performance loss.

While as indicated in the simulation summary [2], the SNR with 70% under maximum Doppler value 1944Hz is better than the SNR with 70% under maximum Doppler value 1340Hz.

Observation 2:  The maximum Doppler value with 1944Hz is out of range of DMRS frequency offset tracking ability, even the DMRS in PUCCH. To some extent, with large Doppler value, there should be some performance loss.

Based on our analysis, we suggest that simulation result with ideal frequency offset estimation should also be encouraged to submit for further alignment.

Proposal 1:  Encourage companies to provide the simulation results with ideal frequency offset for further alignment.

Generally, frequency offset estimation is controlled based on the uplink physical single, such as DMRS in PUSCH. However, the need of the configuration of PUCCH can be benefit to achieve a sufficient performance for frequency offset estimation, considering the frequency offset range of DMRS in PUCCH format2 can up to 1750Hz.

As mentioned in the WF [1], the configuration of PUCCH (format2) is optional, which means it would depend on BS implementation and could not be mandated how BS estimate the frequency offset.

Since the PUSCH is considered as the measurement channel to specify the eNB performance for the high-speed train, it should be the baseline of requirement specified. With additional PUCCH configuration, the performance can be improved. Either using DMRS in PUCCH or DMRS in PUSCH for frequency offset estimation depends on BS implementation. From my understanding, the minimum requirement specified in RAN4 is based on the worst scenario, to guarantee. Considering the diverse of frequency tracking ability for two DMRS structure, we propose to add the additional margin for requirement

Proposal 2:  Additional margin should be considered for deriving requirement, due to the diverse of two DMRS structure in PUSCH and PUCCH.

# 3	Conclusion

In this contribution, we presented the updated ideal and impairment results for finalizing the requirements of HST PUSCH.

Meanwhile, the views of further align the simulation results and how to handling the requirements are provided.

Observation 1: Due to the limited DMRS resource in small bandwidth, it is reasonable that there is performance loss with considering the inaccuracy frequency offset estimation.

Observation 2:  The maximum Doppler value with 1944Hz is out of range of DMRS frequency offset tracking ability, even the DMRS in PUCCH. To some extent, with large Doppler value, there should be some performance loss.

Proposal 1:  Encourage companies to provide the simulation results with ideal frequency offset for further alignment.

Proposal 2:  Additional margin should be considered for deriving requirement, due to the diverse of two DMRS structure in PUSCH and PUCCH.

# References

R4-1907747, Way forward on LTE HST BS demodulation performance requirement, NTT DOCOMO

R4-1913369, Summary of ideal and impairment results for LTE Rel-16 HST BS demodulation requirement,  Samsung