3GPP TSG-RAN WG4 Meeting #94-e	                                       R4-2000391

Electronic Meeting, 24 Feb. – 6 Mar., 2020

Agenda Item:	8.8.2.1.6

Source:	Intel Corporation

Title:	Link-level simulation results for RSTD measurement

Document for:		Discussion

# Introduction

In the previous RAN4 meeting the link-level simulation assumption for PRS RSTD measurement in NR positioning was agreed [1]. In this contribution, we provided a set of initial link-level simulation results and observations for PRS RSTD measurements.

# Simulation Results

The general parameters and PRS-specific parameter agreed in the last RAN4 meeting are provided in Table 1 and Table 2, respectively in Appendix.

It was noted that in the last RAN4 meeting, the performance of metric in terms of RSTD error was also defined as:

At least the following performance characteristics are to be provided for PRS RSTD: - RSTD error CDFs for 3 cells - 90%-ile of the RSTD errors for each cell At least the following performance characteristics are to be provided for PRS RSRP: - RSRP error CDFs for 3 cells - 90%-tile of the RSRP errors for each cell

However, since RSTD is the timing difference between the reference cell’s TDoA and TDoA of the neighbor cells, only two cells ( e.g. cell2 and cell3) RSTD error can be evaluated in our simulations.

In the figures below the results for the difference PRS measurement configuration (e.g. PRS comb size and measurement bandwidth) were given.

Figure 1. RSTD simulation results with different PRS BW

Figure 2. RSTD simulation results with different PRS Comb size

Observation 1: There is obvious performance gap when PRS RSTD measurement BW is different.

Observation 2: There is obvious performance gap when PRS resource number and comb size is different.

Thus in RAN4 it is not feasible to cover all measurement PRS configuration (e.g. PRS bandwidth and PRS resource symbols ) with a single requirement. It is better to define multiple performance requirements for the different PRS resource configuration.

Based on the above observations and analysis, the proposal can be drawn as:

Proposal 1: Multiple PRS measurement performance requirements shall be defined at least regarding to:

- Different PRS measurement bandwidth (e.g. <=52RBs and >52RBs)

- Different PRS comb size (e.g. <=comb2 and >comb2)

# Conclusion

In this contribution, link level simulation results for PRS RSTD are provided.

Observation 1: There is obvious performance gap when PRS RSTD measurement BW is different.

Observation 2: There is obvious performance gap when PRS resource number and comb size is different.

Proposal 1: Multiple PRS measurement performance requirements shall be defined at least regarding to:

- Different PRS measurement bandwidth (e.g. <=52RBs and >52RBs)

- Different PRS comb size (e.g. <=comb2 and >comb2)

# References

- R4-1915802, “Link-level simulation assumptions for PRS RSTD and PRS RSRP”

# Appendix

Table 1: General parameters

| Parameter | Value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Cell layout | 3 cells at distinct locations: <cell 1, cell 2, cell 3>, where cell 1 is the reference cell |  |
| Network synchronization | • Synchronous with time shifts <0, 0, 3 us> • Asynchronous with time shifts: <0, 7 symbols, -7 symbols> |  |
| Duplex modes | FDD and TDD |  |
| TDD specific parameters (TTD configuration is in 38.133, section A.3.1.4) | - TDDConf.1.1 (15 kHz) - TDDConf.2.1 (30 kHz) | - TDDConf.3.1 (120 kHz) |
| Data and CCH load in PRS symbols | no other cell transmissions in its positioning symbols, except PRS |  |
| Data and CCH load in non-PRS symbols | - 50% utilization in time - 100% RE utilization |  |
| Cyclic prefix | Normal |  |
| DRX | OFF |  |
| Carrier frequency / BW / SCS / duplex mode | - 2 GHz - 10 MHz, 20 MHz, 50 MHz - 15 kHz - FDD, TDD - 4 GHz - 20 MHz, 50 MHz, 100 MHz - 30 kHz - FDD, TDD | - 40 GHz - 50 MHz, 100 MHz, 200 MHz - 120 kHz - TDD |
| Propagation conditions [TS 38.101-4] | AWGN, TDL-C (300 ns delay spread, 100 Hz), TDL-A (30 ns delay spread, 5Hz), TDL-B (100 ns delay spread, 200Hz) | TDL-C (60 ns delay spread, 300 Hz) |
| Es/Iot for three cells (cell 1, cell 2, cell 3), [dB] | (-6, -13, -13) (-3, -13, -13) | (-6, -13, -13) (-3, -10, -10) |
| Number of UE receive antennas | 2 rx (uncorrelated with equal gain, no rx beamforming) |  |
| UE measurement bandwidth | Full carrier bandwidth |  |

Table 2: PRS transmission configuration parameters

| Parameter | Value |  |
| --- | --- | --- |
| Number of transmit PRS antennas | 1 |  |
| Cell ID, TRP ID, PRS Resource Set ID | Selected to ensure non-overlapping PRS REs in frequency |  |
| Number of DL PRS Resource sets for a positioning fix | 1 (including all PRS resource repetitions) |  |
| PRS transmission bandwidth (in PRBs) - full carrier BW | - 15 kHz: - 52 (10MHz), 104 (20MHz), 268 (50MHz) - 30 kHz: - 48 (20MHz),132 (50MHz), 272 (100MHz) | - 120 kHz: - 32(50MHz),64(100MHz), 128 (200MHz) |
| Comb | Comb-4, comb-6, comb-2 (with muting) |  |
| DL-PRS-ResourceRepetitionFactor | 1, 2, 4, 16 |  |
| PRS muting | No muting (comb-4, comb-6), muting (comb-2) |  |
| Power boosting | No power boosting |  |
| ExpectedRSTD-Uncertainty | 5 us (15 kHz), 2.5 us (30 kHz), 0.625 us (120 kHz) |  |