3GPP TSG-RAN WG4 Meeting #94-e                 	 R4-2000457

Electronic Meeting, 24 Feb. – 6 Mar., 2020

Agenda Item:	8.13.2.1

Source:	MediaTek Inc.

Title:	Interruption for Tx switching between two uplink carriers

Document for:		Discussion

# Introduction

In last meeting, whether to specify interruption due to Tx switching between 2 UL carriers was discussed. Only conclusions reached on chairman’s note were to further study the issue.

Conclusion: - Further study if interruption requirements shall be defined - Companies to bring analysis on the expected interruption length

Same time, a WF [1] was agreed in RF session about DL interruption.

- Option A: Define different capabilities for UEs with and without DL reception interruption - If UE does not report this capability, it means there is no DL reception interruption. - Option B: DL reception interruption is not allowed. - Option C: - No DL reception interruption for the following duplex mode combinations: (carrier 1 + carrier 2) - SUL+TDD - FDD+TDD - TDD+TDD with synchronous UL-DL configurations the same UL-DL pattern - Other duplex mode combinations: allowed for one or two DL carriers based on UE capability reporting - Send LS to RAN1 and ask RAN1’s feedback on RAN1 spec impact if there is DL reception interruption in some scenarios.

In this paper, we provide our view on this issue.

# Discussion

In our view, interruption will occur when UE is performing carrier switch. Optimization is possible to minimize the interruption for certain band combinations. However, concerning implementation cost, it is not easy to do this optimization for all band combinations. Therefore, we should consider interruption as the baseline, while allowing UE to optimize and inform network through capabilities. Anyhow, this is the discussion belonging to RF session. RRM session should wait for the conclusion of RF session.

Observation 1: Whether to introduce interruption requirements in RRM session is pending on the conclusion in RF session.

Another discussion is about where to capture this interruption requirements (if agreed in RF session). Traditionally, interruption requirements were specified in TS38.133 with the interruption duration in the unit of slot. However, we also observe some description about allowing interruption in TS38.101-3, as captured below.

### 6.3B.2 Output power dynamics for intra-band EN-DC without dual PA capability For both non-contiguous intra-band EN-DC and contiguous EN-DC for DC_(n)41 and DC_(n)71 without dual PA capability, maximum UL switching time is defined as 120 us and DL reception interruption is allowed during UL switching. Time masks in Figure 6.3B.2-1 and Figure 6.3B.2-2 shall apply. Unless otherwise stated, for intra-band contiguous EN-DC of other band combinations the switching time in 6.3B.1.1 applies. Figure 6.3B.2-1: E-UTRA to NR switching time mask for intra-band EN-DC without dual PA capability when single UL is allowed

For spec consistency, we prefer to address this interruption requirement in TS38.133. It is better for the maintenance of the spec in future releases, e.g., if we identify any new general rule that should be applied to all interruption requirements.

Observation 2: Capturing interruption requirements in TS38.133 is better for spec consistency and maintenance in the future.

On the other hand, we also understand the worry from operators. Currently, the unit used in TS38.133 for interruption requirements is ‘slot’, even if the actual UE action causing interruptions can be done in 1 or 2 OFDM symbols. Interruption requirements in the unit of slot sound rather pessimistic to this feature.

In our view, RAN4 can try to address operators’ concern via 2 angles. One is to introduce UE capability so that UE which doesn’t need this interruption can tell network about its capability. (This is a discussion in RF session) The other one is to discuss whether to specify this interruption requirement in the unit of OFDM symbol. Note that both the interruption duration and the starting time of the interruption needs to be specified in symbol-level granularity. One may also think to update other interruption requirements (e.g., BWP switch), but we prefer to keep them as they are now to minimize the workload in RAN4 RRM session.

Proposal 1: If interruption is agreed in RF session, the requirements are specified in TS38.133 with the starting time of the interruption and the interruption duration specified in the unit of OFDM symbol.

# Summary

In this paper, we provide our view on the interruption requirements for Tx switching between two uplink carriers. We have the following observations and proposal.

Observation 1: Whether to introduce interruption requirements in RRM session is pending on the conclusion in RF session.

Observation 2: Capturing interruption requirements in TS38.133 is better for spec consistency and maintenance in the future.

Proposal 1: If interruption is agreed in RF session, the requirements are specified in TS38.133 with the starting time of the interruption and the interruption duration specified in the unit of OFDM symbol.

# Reference

[1] R4-1916084, “Way forward on Tx switching between two uplink carriers”, China Telecom