3GPP TSG-RAN WG4 Meeting#94-e                	R4-2000654

E-meetings, February 24th – March 6th, 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	Measurement accuracy for CLI SRS-RSRP measurement

Agenda item:	8.2.3.1

Document for:	Discussion

Introduction

During RAN4#93 meeting, the core part of the CLI measurements requirements was completed and the CRs were agreed in [1-2]. The remaining issue is to conclude on the performance aspects i.e. to determine the measurement accuracy per frequency range and dependent on SCS. In this contribution, we show the simulation results under agreed side conditions and provide our recommendation on the accuracy requirements for CLI SRS-RSRP measurement.

Discussion

From previous RAN4 meetings, it was agreed to define the CLI SRS-RSRP measurement requirements under following side conditions. The simulation results of accuracy are therefore provided in Figure 1 and Figure 2 with these assumptions:

- Residual timing error is 4.67us for FR1 and 3.67us for FR2

- The bandwidth of the SRS resource is 48 PRBs

- 3 samples

- SNR = 1dB

Based on the simulation results, the baseband accuracy values under AWGN channel are listed in Table 1 below. Assuming 2.5dB RF margin for FR1 and 4dB RF margin for FR2, the CLI measurement accuracy requirements can be derived accordingly.

| FR | SCS | Baseband Accuracy | Accuracy Requirement |
| --- | --- | --- | --- |
| FR1 | 15kHz | ±0.38 | ±2.9 |
|  | 30kHz | ±0.95 | ±3.5 |
|  | 60kHz | ±2.47 | ±5 |
| FR2 | 60kHz | ±1.77 | ±5.8 |
|  | 120kHz | ±4.39 | ±8.4 |

Table 1:Accuracy values with reference to AWGN channel simulation results in Figure 1 and 2

Proposal1: It is proposed to adopt the CLI accuracy requirements values as defined in Table 1.

Figure 1. Measurement accuracy for CLI SRS-RSRP measurements, FR1

Figure 2: Measurement accuracy for CLI SRS-RSRP measurements, FR2

Conclusion

This contribution discusses the performance of CLI SRS-RSRP measurements based on the agreed side conditions. The proposals are summarized as below:

Proposal1: It is proposed to adopt the CLI accuracy requirements values as defined in Table 1.

References

- R4-1915517, CR for cross link interference measurements (section 9), LG Electronics

- R4-1915780, CR to introduce CLI measurement accuracy requirements (section 10), Huawei, HiSilicon