3GPP TSG-RAN WG4 Meeting #94-e	R4-2000733

Online, 24 February – 6 March, 2020

Agenda item:	8.8.2.1.4

Source: 	Qualcomm Incorporated

Title: 	On UE Rx-Tx time difference measurements for NR positioning

Document for:	Discussion

# Introduction

In RAN4#93 meeting, the following agreements were reached on UE Rx-Tx time difference measurements:

UE Rx-Tx time difference measurement period:

- When UE Rx-Tx time difference measurement is configured to be measured along with PRS-RSRP using the same assistance data:

  - then whether the measurement periods of UE Rx-Tx time difference measurement and PRS-RSRP are the same or not, is FFS.

- Otherwise:

  - In non-DRX the UE Rx-Tx time difference measurement period is FFS.

  - When DRX is used then whether or not the UE Rx-Tx time difference measurement period depends on DRX cycle may depend on the positioning method; details are FFS.

- RAN4 needs to check RAN1/2 agreements if UE Rx-Tx time difference measurement can be configured to be measured along with RSTD using the same assistance data

Side conditions for UE Rx-Tx time difference:

- Serving cell:

  - Side conditions (PRS Es/Iot) for UE Rx-Tx time difference in FR1 are FFS.

  - Side conditions (PRS Es/Iot) for UE Rx-Tx time difference in FR2 are FFS.

- Neighbour cell:

  - Side conditions (PRS Es/Iot) for UE Rx-Tx time difference and for RSTD measurement in FR1 are the same.

  - Side conditions (PRS Es/Iot) for UE Rx-Tx time difference and for RSTD measurement in FR2 are the same.

UE Rx-Tx time difference measurement under cell change:

- If the cell change occurs on the serving cell where the SRS is configured then after the serving cell change:

  - the UE shall restart the UE Rx-Tx time difference measurement;

- otherwise the UE shall continue the ongoing UE Rx-Tx time difference measurement.

In this paper, we discuss some of the remaining issues related to UE Rx-Tx time difference measurement.

# Measurement requirements

## Measurement period

Per RAN1 recommendation, the following table links different measurement types and positioning methods in R16 NR positioning WI:

Table 1 UE measurements to support positioning techniques

| DL/UL Reference Signals | UE Measurements | To facilitate support of the following positioning techniques |
| --- | --- | --- |
| Rel.16 DL PRS | DL RSTD | DL-TDOA |
| Rel.16 DL PRS | DL PRS RSRP | DL-TDOA, DL-AoD, Multi-RTT |
| Rel.16 DL PRS / Rel.16 SRS for positioning | UE Rx-Tx time difference | Multi-RTT |
| Rel. 15 SSB / CSI-RS for RRM | SS-RSRP(RSRP for RRM), SS-RSRQ(for RRM), CSI-RSRP (for RRM), CSI-RSRQ (for RRM) | E-CID |

UE Rx-Tx time difference measurement is used only in multi-RTT positioning method using DL PRS and R16 UL SRS for obtaining UE Rx-Tx and gNB Rx-Tx time difference measurements. Although the measurement type is different than in DL-TDOA, the same high-level requirements in terms of final positioning error and time-to-fix also applies to multi-RTT. In RAN4#93 meeting, the following agreement was reached regarding RSTD measurements in DL-TDOA [1]:

RSTD measurement period:

- Non-DRX requirements apply for RSTD measurement period, regardless of whether and which DRX configuration is configured for the UE.

In our view, there is no reason for the above agreement to not apply to UE Rx-Tx time difference measurement as well. It is noted that unlike LTE, UE Rx-Tx time difference is not part of E-CID measurement in R16 NR. Moreover, given the sensitivity of timing measurements to latency, UE and gNB clock drift, and UE mobility (as discussed in Section 5), non-DRX mode in UE Rx-Tx time difference measurement is as important.

Observation 1. The same high-level requirements in terms of final positioning error and time-to-fix also applies to multi-RTT. Given the sensitivity of timing measurements to latency, UE and gNB clock drift, and UE mobility, non-DRX mode in UE Rx-Tx time difference measurement is as important.

Proposal 1. Non-DRX requirements apply for UE Rx-Tx time difference measurement period, regardless of whether and which DRX configuration is configured for the UE.

As discussed in [2], in multi-RTT, reporting delay requirements will depend on several factors as in the following:

- Periodicity of PRS and SRS resource set(s)

- Total number of PRS resources to be measured subject to UE capability

- UE processing power per measurement window

- UE buffering (memory) budget per measurement window

- Delay related to a single PRS resource including sampling and processing time

- Number of frequency layers to be measured

- Repetition factor for PRS as configured by DL-PRS-ResourceRepetitionFactor

In the RAN1 agreement (and also UE feature list document [3]), capabilities regarding maximum number of PRS resources that UE can advertise to be capable of measuring are listed as:

Agreement:

UE can be configured for DL PRS processing according to the following table:

| Description | Maximum numbers for DL PRS resources | Values that can be signaled as part of UE Capability |
| --- | --- | --- |
| Max number of frequency layers (X1) | X1=4 | Values = {1,4} FFS: other values |
| Max number of TRPs per frequency layer (X2) | X2=64 |  |
| Max number of PRS resource sets per TRP (X3) per frequency layer | X3=2 | Values = {1,2} |
| Max number of Resources per PRS resource set (X4) | X4=64 | FFS: values |
| Max number of DL PRS Resources per UE (X5) | NA | FFS: values |
| Max number of TRPs for all frequency layers (X6) per UE | 256 | FFS: values |
| Max number of Resources per frequency layer (X7) | NA | FFS: values |

Note: This does not constrain in any way how features and feature sets are defined. The values in the table above may or may not be signalled to be different for different features or feature sets.

In addition to the above, [3] introduces two more UE capabilities as duplets {N1,T1} and {N2,T2} to signal maximum UE processing load and maximum UE memory budget, respectively:

- A limit on the maximum number N1 of DL PRS resources the UE is expected to measure across all TRPs and frequency layers within a measurement window of T1 msec reported as a list of duplets {N1, T1} when measurement gaps are configured

- A limit on the maximum number of symbols N2 containing PRS resources of maximum bandwidth the UE is expected to measure within a measurement window of T2 msec reported as a list of duplets {N2, T2} when measurement gaps are configured

Similar duplets also exist for when measurement gaps are not configured.

The following analysis focuses on the derivation of measurement delay for one frequency layer when measurement gaps are configured and UE signals one duplet {N1,T1} and one duplet {N2,T2} as a starting point for discussion. In this analysis, it is assumed that SRS periodicity has sufficient regularity to not be the bottleneck for UE Rx-Tx time difference measurement period. This assumption is further discussed in Section 5 and is reasonable given that gNB relies on SRS reception to measure gNB Rx-Tx time difference.

The measurement delay requirement is determined by:

- The maximum number of DL PRS resources that UE measures subject it its capability

- The time required to process DL PRS resources that UE is configured to measure

- The time required to buffer DL PRS resources that UE is configured to measure

Maximum number of PRS processes:

For each frequency layer configured, the maximum number of DL PRS resources can be

Ntotal=min(X2.X3.X4 , X7)

Where the first term X2.X3.X4, determines the maximum number of PRS resources that can be configured to a UE per frequency layer, and X7 is the upper bound on the number of PRS resources per frequency layer. X2, X3, X4, X7 are signaled UE capabilities for multi-RTT positioning method.

Observation 2. Max number of DL PRS resources per frequency layer constrained by UE capability is Ntotal=min(X2.X3.X4 , X7)

Time required for PRS processing:

The time required for PRS processing of configured resources is a function of the duplet {N1, T1}. Figure 1 illustrates an example of several PRS resources spanning a time duration (LPRS) in ms, corresponding to the time lapse from the first symbol of the earliest PRS resource to the last symbol of latest PRS resource. The periodicity of these PRS resources are assumed to be all equal to TPRS. The time required to consume Ntotal PRS resources according to duplet {N1, T1} is:

Tproc=Ntotal.T1N1

However, this is subject to presence of PRS resources in a slot which is periodic. Given length LPRS appearing every TPRS , the total number of required periods, in units of TPRS , for PRS processing can be expressed as:

Nproc=LPRSTproc

Figure 1 Time duration spanned by all DL PRS resources (LPRS) and PRS periodicity (TPRS) with SCS = 15 kHz

Observation 3. The total number of required PRS periods, in units of TPRS to process Ntotal PRS resources is  Nproc=LPRSTproc where  Tproc=Ntotal.T1N1 and {N1, T1} is the signalled capability duplet for PRS processing.

Time required for PRS buffering:

Similarly, the time required for PRS buffering of configured resources is a function of duplet {N2, T2}. The time required to consume Ntotal,symbols PRS symbols according to duplet {N2, T2} is:

Tmem=Ntotal,symbols.T2N2

Where Ntotal,symbols corresponds to the total number of PRS symbols from Ntotal PRS resources. This number depends on the PRS configurations (e.g., comb pattern, DL-PRS-ResourceRepetitionFactor, …). The total number of PRS periods, in units of TPRS , required for PRS buffering can be expressed as:

Nmem=LPRSTmem

Observation 4. The total number of PRS periods, in units of TPRS, required to process Ntotal,symbols PRS resources is  Nmem=LPRSTmem where  Tmem=Ntotal,symbols.T2T2 and {N2, T2} is the signalled capability duplet for PRS buffering.

Finally, the measurement period formula can be expressed as:

TPRS, meas=maxTproc, Tmem.TPRS

Which accounts for both processing and buffering capability of UE.

Proposal 2. PRS measurement period for Ntotal PRS resources in one frequency layer can be expressed as 
TPRS, meas=maxNproc, Nmem.TPRS

Where

- Nproc is the UE processing time as expressed in Observation 3 (FFS)

- Nmem is the UE buffering time as expressed in Observation 4 (FFS)

- When UE signals a list of {N1,T1} and {N2,T2}, as opposed to one {N1,T1} and one {N2,T2}, a separate Nproc, Nmem is derived for each pair and max of all Nproc, Nmem is used.

## Measurement accuracy

As proposed in [2], the measurement accuracy requirements for UE Rx-Tx time difference should be based on one-shot measurement. Combining measurements across repetition windows, if available, to improve performance should be left to UE implementation.

Proposal 3. UE Rx-Tx time difference measurement accuracy requirements to be defined using samples from only one DL PRS resource repetition, i.e., the time duration spanned by one DL PRS resource after repetition by DL-PRS-ResourceRepetitionFactor and one SRS resource repetition.  Combining measurements across PRS/SRS repetitions, if possible and available, to improve performance should be left to UE implementation.

An issue that needs to be carefully considered in NR is the Timing advance (TA) consistency across the SRS transmissions during one repetition window. Consider the case that the UE reports one “UE Rx-Tx timing difference” measurement report associated with a frame number during which two or more SRS slots are transmitted, and in between them, a TA MAC CE command is received, as shown in the figure below. If the TRP then attempts to average the channel energy responses across the successive SRS receptions, there will be an error due to the different timing advance between successive SRS transmissions.

Figure 2 TA update during UE Rx-Tx time difference measurements

This issue also exists in LTE. In this case, in LTE UL E-CID (see TS 36.133), the accuracy of UE Rx-Tx is only defined assuming no changes in the uplink transmit timing during the measurement period as shown in the text below:

A similar approach is proposed to be used in NR Rel-16, due to its specification simplicity, but indeed for scenarios of high mobility, where TA commands are received too often, the positioning measurements can deteriorate, and its accuracy cannot be guaranteed.

Proposal 4. With respect to TA commands during a positioning session, accuracy requirements are valid under the condition that no changes to UL transmission timing are applied during the measurement period.

- A measurement period includes an instance of PRS and its associated SRS as in Proposal 3. Exact definition is FFS.

Next, the accuracy requirements of Rx-Tx time difference measurement is discussed. It is noted that:

Observation 5. Two factors impact the accuracy requirements of gNB/UE Rx-Tx time difference measurement:

- Time-of-arrival (TOA) estimation error from DL PRS (for UE) or UL SRS (for gNB) resources

- Rx-Tx calibration error

Figure 3 (left) shows the gNB/UE Rx-Tx calibration error which reflects the pipeline delay from antenna to baseband when gNB/UE transmits/receives PRS and when gNB/UE receives/transmits SRS. Figure 3 (right) shows the measurement of TOA from PRS and a mirror operation in gNB side on SRS which leads to Rx-Tx time difference measurement reporting.

Figure 3 Calibration error and group delay variation in RTT (left) and UE/gNB Rx-Tx time difference (right)

In our view, RAN4 should consider the error budget from both of these factors to arrive at accuracy requirements for UE Rx-Tx time difference measurements. For TOA error estimation, it is proposed to use the same simulation assumptions as in RSTD link-level simulation assumption [4] and study the distribution of TOA error, as opposed to RSTD error, across different channel profiles and PRS configurations.

Proposal 5. RAN4 to use the same simulation assumptions as in RSTD link-level simulation assumptions to provide CDFs of TOA estimation error where TOA error = estimated TOA – ideal TOA.

In regard to the Rx-Tx calibration error, the error budget should be viewed holistically from both gNB and UE perspective. The performance and accuracy of multi-RTT depends on Rx-Tx calibration error on both UE and gNB side as illustrated in Figure 3 (left). Hence, allocating a small error budget on one side (UE or gNB) while permitting a disproportionately larger error budget on the other side is nonsensical.

Observation 6. The performance and accuracy of multi-RTT depends on Rx-Tx calibration error on both UE and gNB. Hence, the error budget should be viewed holistically from both sides. Allocating a small error budget on one side (UE or gNB) while permitting a disproportionately larger error budget on the other side is nonsensical.

## Side conditions

In our view, there is no need to define side conditions for serving cell or reference cell in UE Rx-Tx time difference measurement. It is noted that even in multi-RTT positioning technique, the presence of PRS reference resource (or set) is necessary as UE needs to know what is the relative timing of DL PRS resources to be able to measure its TOA correctly. However, TOA measurements are not relative, so no reference point is needed. Hence, only one side condition is required and applicable to the PRS resources in the assistance data.

Proposal 6. In multi-RTT positioning technique, only one side condition is needed and applicable to PRS resources in the assistance data for UE Rx-Tx time difference measurement. This side condition is proposed to be the same as neighbour cell side condition in RSTD requirements.

# Measurement report mapping

In LTE, UE Rx-Tx time difference report mapping table used a non-uniform step size with 2Ts  resolution in lower values and 8Ts resolution in larger values. In previous RAN4 meetings, there were proposals to use the same approach in NR positioning. However, the following agreement was made in RAN1#99:

Agreement:

- The reporting granularity for the UE/gNB timing measurements (DL RSTD, the UE Rx-Tx time difference, UL RTOA, gNB Rx-Tx time difference) is defined as T=Tc2k, where k is a configuration parameter with a minimum value of at most 0.

  - Note: RAN4 can determine if -1 can be a minimum value

- RAN1 assumes that the details of the reporting granularity and ranges for the UE/gNB timing measurements (DL RTSD, the UE Rx-Tx time difference, UL RTOA, gNB Rx-Tx time difference) will be determined by RAN4, including the potential relation of the parameter k to DL PRS bandwidth.

- Include above information in an LS to RAN2/RAN3/RAN4

Based on the above agreement, the reporting granularity cannot be non-uniform. RAN4 can adjust the reporting granularity based on DL PRS bandwidth but using a different granularity for smaller vs. larger values in the same DL PRS BW is a violation of RAN1 agreement. Moreover, in multi-RTT positioning technique, UE measures UE Rx-Tx time difference from multiple cells (unlike E-CID in LTE where only serving cell is measured). Measurements with larger UE Rx-Tx time difference values correspond to farther cells and they are as important in the positioning fix. So there is no reason to use a coarser granularity for them.

Observation 7. Using a different granularity for smaller vs. larger UE Rx-Tx time difference values of the same DL PRS BW is a violation of RAN1 agreement. In multi-RTT, measurements with larger UE Rx-Tx time difference values correspond to farther cells and they are as important in the positioning fix. There is no reason to use a coarser granularity for them.

Proposal 7. RAN4 to use uniform report mapping table(s) for UE Rx-Tx time difference measurements, i.e., smaller and larger UE Rx-Tx time difference values to have the same reporting granularity.

The UE’s reporting granularity should depend on configured DL PRS BW. Based on AWGN results tabulated in [5], the accuracy of RSTD and/or TOA estimation can approach 1/(2*BW) which matches the theoretical expectation. For instance, in 200 MHz PRS BW, the accuracy level in AWGN can be ~2.5 ns which is ~5Tc. The reporting granularity, however, should be significantly better than the achievable accuracy to not affect the positioning error. Hence, in the following table, the proposed reporting granularity for different PRS BW is summarized which is smaller than achievable accuracy by at least a power of 2.

Proposal 8. Reporting granularity for UE Rx-Tx time difference measurement to be as in Table 2.

Table 2 Proposed reporting granularity for UE Rx-Tx time difference measurement

| DL PRS BW (MHz) | Theoretical accuracy (ns) | Reporting Granularity (ns) | k |
| --- | --- | --- | --- |
| ≤ 25 | 20 | 8.1 | 4 |
| ≤ 50 | 10 | 4.1 | 3 |
| ≤ 100 | 5 | 2.0 | 2 |
| ≤ 200 | 2.5 | 1.0 | 1 |
| ≤ 400 | 1.25 | 0.5 | 0 |

Next, the report mapping table is discussed. In LTE, the minimum value of report mapping table was dictated by NTA,Offset value which is 0 for FDD and 624Ts for TDD. The maximum value was based on maximum value of TA update which is 20512Ts.

In NR FR1, the range of TA in response to random access can be [0, 3846*16*64]Tc. However, the maximum value corresponds to a cell radius of 300 km which is impractical. It is noted that RAN1 made the following agreement regarding maximum possible RSTD value:

Agreement:

The expected RSTD value is a single value defined as the RSTD the UE is expected to measure (at the UE location).

- The value range of the expected RSTD is +/- 500 us.

- The value range for the uncertainty of the expected RSTD is

  - When any of the resources used for the DL positioning measurement are in FR1: +/- 32 us

  - When all of the resources used for the DL positioning measurement are in FR2: +/- 8 us

Assuming that the maximum RSTD can occur if reference cell is very close to UE and the neighbor cell is the farthest, it is implied that the max UE Rx-Tx time difference does not need to be larger than 2*(500 us + 32) us where the factor of 2 accounts for round-trip propagation delay. Considering the 4Tc granularity (k=2) for max possible PRS BW of 100 MHz in FR1, each UE Rx-Tx time difference report will need a maximum of log2 (2*532 us / (4*Tc) ) = 19 bits with report mapping table as in Table 3:

Table 3 UE Rx-Tx time difference measurement report mapping for FR1 with 100 MHz PRS BW

| Reported Value | Measured quantity value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFF_0 | NTA,Offset ≤TUE,Rx-Tx ≤NTA,Offset+ 4 | Tc |
| RX-TX_TIME_DIFF_1 | NTA,Offset+4 ≤TUE,Rx-Tx ≤NTA,Offset+8 | Tc |
| … | … | … |
| RX-TX_TIME_DIFF_524286 | NTA,Offset+2097144 ≤TUE,Rx-Tx ≤NTA,Offset+2097148 | Tc |
| RX-TX_TIME_DIFF_524287 | NTA,Offset+2097148 <TUE,Rx-Tx | Tc |

Where NTA,Offset is either configured through n-TimingAdvanceOffset or default values in Table 7.1.2-3 of TS 38.133 is used.

Proposal 9. Use Table 3 for UE Rx-Tx time difference measurement report mapping for FR1 with 100 MHz PRS BW when UE also measures PRS with 100 MHz BW where NTA,Offset is either configured through n-TimingAdvanceOffset or default values in Table 7.1.2-3 of TS 38.133 is used.

In NR FR2, the range of TA in response to random access can be [0, 3846*16]Tc which maps to cell radius of 4.7 km. To cover this range with Tc granularity, 16 bits are needed with the report mapping as in Table 4:

Table 4 UE Rx-Tx time difference measurement report mapping for FR2 with 400 MHz PRS BW

| Reported Value | Measured quantity value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFF_0 | NTA,Offset ≤TUE,Rx-Tx ≤NTA,Offset+ 1 | Tc |
| RX-TX_TIME_DIFF_1 | NTA,Offset+1 ≤TUE,Rx-Tx ≤NTA,Offset+2 | Tc |
| … | … | … |
| RX-TX_TIME_DIFF_65534 | NTA,Offset+65534 ≤TUE,Rx-Tx ≤NTA,Offset+65535 | Tc |
| RX-TX_TIME_DIFF_65535 | NTA,Offset+65535<TUE,Rx-Tx | Tc |

Proposal 10. Use Table 4 for UE Rx-Tx time difference measurement report mapping for FR2 with 400 MHz PRS BW when UE also measures PRS with 400 MHz BW where NTA,Offset is either configured through n-TimingAdvanceOffset or default values in Table 7.1.2-3 of TS 38.133 is used.

For Proposals 7-8, an important consideration is that PRS BW used for measurement report mapping is the minimum of configured DL PRS BW and the BW that UE can use for measurement subject to its capability. For instance, if UE is only capable of 200 MHz PRS measurement in FR2 but gNB configures PRS BW of 400 MHz, then 200 MHz PRS BW is used for measurement report mapping. In the same example, if UE supports measurement of 400 MHz PRS but only through measurement gaps, yet gNB does not configure the proper measurement gap, then 200 MHz PRS BW is again used for measurement report mapping.

Proposal 11. PRS BW used for measurement report mapping is the minimum of configured DL PRS BW and the BW that UE can use for measurement subject to its capability.

Next, the need for differential PRS-RSTD reporting is discussed. From [6]:

| NR_pos-Core | NR DL PRS Report Configuration | UE can be configured to report up to M = 4 UE-Rx-Tx time difference measurements for different DL PRS resources or DL PRS resource sets per TRP A UE can be configured to report multiple Rx–Tx time difference measurements corresponding to a single SRS resource/resource set for positioning with each measurement corresponding to a single DL PRS resource/resource set. The DL PRS resource/resource sets can be in different positioning frequency layers | {1,2,3,4} |
| --- | --- | --- | --- |

The M UE Rx-Tx time difference report belong to the same TRP and are logically close in value as they are a measure of round-trip delay to that TRP. The difference in their values arise from beamforming of PRS resources from that TRP.

Observation 8. The M UE Rx-Tx time difference report belong to the same TRP and are logically close in value as they are a measure of round-trip delay to that TRP.

Moreover, per the analysis in this section, each UE Rx-Tx time difference report requires a maximum of 19 bits in FR1 and 16 bits in FR2 which are larger than those in LTE (i.e., 14 bits) for each RSTD report. With increased number of DL resources that UE may need to measure, the overall impact is a significant growth in measurement report payload.

To address this issue, it is proposed to use differential UE Rx-Tx time difference reporting within the set of M-1 measurements of the same TRP and absolute UE Rx-Tx time difference reporting for only one of them. Since the values of these M measurements are close, a smaller number of bits are required to report each of the M-1 measurements resulting in reduction of report payload while fulfilling the measurement requirements.

To achieve this, UE can report the minimum UE Rx-Tx time difference value from the set of M UE Rx-Tx time difference values with absolute report mapping as in Tables 3-4, and use positive differential UE Rx-Tx time difference value for the remaining M-1 values. Each UE Rx-Tx time difference will be identified by its ID per RAN2 agreement.

Proposal 12. RAN4 to define differential UE Rx-Tx time difference report mapping table. UE to report the minimum UE Rx-Tx time difference value from the set of M UE Rx-Tx time difference values of the same TRP with absolute report mapping as in Tables 3-4 and use positive differential UE Rx-Tx time difference value for the remaining M-1 UE Rx-Tx time difference values.

- FFS: the maximum range of differential UE Rx-Tx time difference report mapping

A final note is on whether UE should use the same report mapping table as in Tables 3-4 for smaller PRS BW. In our view, such level of optimization does not provide a significant reduction in reporting payload compared to differential UE Rx-Tx time difference reporting in Proposal 9. In our view, the same report mapping table as in Tables 3-4 should be used for smaller PRS BW.

Proposal 13. The same report mapping table as in Tables 3-4 should be used for smaller PRS BW as reduction in reporting payload is not as significant as differential UE Rx-Tx time difference reporting in Proposal 9.

# Measurement reporting criteria

In multi-RTT, UE can be configured with multiple frequency layers for UE Rx-Tx time difference measurement where each frequency layer can consist of multiple TRPs, each TRP can consist of multiple resource sets, and each set can consist of multiple resources. In Section 2.1, these advertised capabilities are described.

For each frequency layer configured, the maximum number of DL PRS resources is:

Ntotal=min( X2.X3.X4 , X7)

Where the definition of X2, X3, X4, and X7 can be found in Section 2.1. Consequently, each measurement reporting criterion corresponding to UE Rx-Tx time difference in multi-RTT positioning method corresponds to Ntotal measurement reports.

Proposal 14. In multi-RTT positioning, each UE Rx-Tx time difference measurement reporting criterion corresponds to one frequency layer with Ecat = 1 indicating Ntotal UE Rx-Tx time difference measurement reports.

- FFS: Ntotal=min( X2.X3.X4 ,  X7)

# Scheduling of SRS transmission

Ideally, all measurements used in generating a positioning fix should be made concurrently. If measurements are performed at different points in time for generating a position fix, UE motion as well as changes to UE clock and gNB clocks result in measurement errors that ultimately can produce position errors. For instance, traveling at highway speed of 30 m/s can result in a 1 s * 30 m/s = 30 m measurement error if two measurements are performed 1 second apart.  Similarly, a 10 part-per-billion (ppb) UE clock drift can produce a 1 s * 10 ns/s = 10 ns ~ 3m measurement error for two measurements taken 1 second apart.

With the stricter 3 - 10m accuracy targets for commercial use cases in NR, it is vital that sources of error within our control, unlike effects such as multipath, are minimized.

Observation 9. UE motion, UE clock drift and gNB clock drift can all result in significant errors for measurements that are conducted apart in time but are all used to generate the same position fix.

In multi-cell RTT, it is imperative to conduct and associate as close in time as possible the Rx-Tx offset measurements produced by the UE with Rx-Tx offset measurements produced by the corresponding gNBs.

Figure X shows an example of an RTT pair where the DL-PRS transmission of TRP1 and SRS transmission of UE 1 are scheduled close in time for instance#5. DL-PRS transmission is scheduled according to DL-PRS-ResourceSetSlotOffset and DL-PRS-Periodicity.  Similarly, SRS is scheduled in instances according to SRS-slot-offset and SRS-Periodicity.  However, keeping with the on-demand nature of NR, there may be a start instance that specifies the first time during a session that SRS will be transmitted by a given UE.  Start of SRS transmission for positioning may for example be given by an SRS activation command for the semi-persistent use case (described in TS 38.214 clause 6.2.1).  Similarly, a session may be limited in time by a QoS or by an SRS deactivation command.  In Figure X, the SRS instances are numbered according to their proximity to DL-PRS instance for simplicity.  It is noted that a corresponding SRS instance can appear before or after DL-PRS instance in an RTT pair.  In Figure X, this is referred to as DL-to-UL offset.  Furthermore, different UEs may utilize different sets of SRS instances.  The large-scale (inter-instance) and small-scale (intra-instance) time-division-multiplexing provides opportunities to support more UEs while minimizing signal congestion/interference on the radio link and reducing peak processing load requirements on the gNBs.

It is thus proposed that core measurement and performance requirements to be applicable when the SRS transmissions are within [-X, X] ms of at least one DL PRS resource from each of the TRPs in the assistance data.

Proposal 15. The core measurement and performance requirements for UE Rx-Tx time difference applies if the configured SRS-Slot-offset and SRS-Periodicity parameters for SRS resource for positioning are such that any SRS transmission is within [-X, X] msec of at least one DL PRS resource from each of the TRPs in the assistance data.

- FFS: X = 25 msec

- Note: Such a RS configuration may enable sufficiently close measurements in time in both the UE and TRPs

# Conclusions

Observation 1. The same high-level requirements in terms of final positioning error and time-to-fix also applies to multi-RTT. Given the sensitivity of timing measurements to latency, UE and gNB clock drift, and UE mobility, non-DRX mode in UE Rx-Tx time difference measurement is as important.

Proposal 1. Non-DRX requirements apply for UE Rx-Tx time difference measurement period, regardless of whether and which DRX configuration is configured for the UE.

Observation 2. Max number of DL PRS resources per frequency layer constrained by UE capability is Ntotal=min(X2.X3.X4 , X7)

Observation 3. The total number of required PRS periods, in units of TPRS to process Ntotal PRS resources is  Nproc=LPRSTproc where  Tproc=Ntotal.T1N1 and {N1, T1} is the signalled capability duplet for PRS processing.

Observation 4. The total number of PRS periods, in units of TPRS, required to process Ntotal,symbols PRS resources is  Nmem=LPRSTmem where  Tmem=Ntotal,symbols.T2T2 and {N2, T2} is the signalled capability duplet for PRS buffering.

Proposal 2. PRS measurement period for Ntotal PRS resources in one frequency layer can be expressed as 
TPRS, meas=maxNproc, Nmem.TPRS

Where

- Nproc is the UE processing time as expressed in Observation 3 (FFS)

- Nmem is the UE buffering time as expressed in Observation 4 (FFS)

- When UE signals a list of {N1,T1} and {N2,T2}, as opposed to one {N1,T1} and one {N2,T2}, a separate Nproc, Nmem is derived for each pair and max of all Nproc, Nmem is used.

Proposal 3. UE Rx-Tx time difference measurement accuracy requirements to be defined using samples from only one DL PRS resource repetition, i.e., the time duration spanned by one DL PRS resource after repetition by DL-PRS-ResourceRepetitionFactor and one SRS resource repetition.  Combining measurements across PRS/SRS repetitions, if possible and available, to improve performance should be left to UE implementation.

Proposal 4. With respect to TA commands during a positioning session, accuracy requirements are valid under the condition that no changes to UL transmission timing are applied during the measurement period.

- A measurement period includes an instance of PRS and its associated SRS as in Proposal 3. Exact definition is FFS.

Observation 5. Two factors impact the accuracy requirements of gNB/UE Rx-Tx time difference measurement:

- Time-of-arrival (TOA) estimation error from DL PRS (for UE) or UL SRS (for gNB) resources

- Rx-Tx calibration error

Proposal 5. RAN4 to use the same simulation assumptions as in RSTD link-level simulation assumptions to provide CDFs of TOA estimation error where TOA error = estimated TOA – ideal TOA.

Observation 6. The performance and accuracy of multi-RTT depends on Rx-Tx calibration error on both UE and gNB. Hence, the error budget should be viewed holistically from both sides. Allocating a small error budget on one side (UE or gNB) while permitting a disproportionately larger error budget on the other side is nonsensical.

Proposal 6. In multi-RTT positioning technique, only one side condition is needed and applicable to PRS resources in the assistance data for UE Rx-Tx time difference measurement. This side condition is proposed to be the same as neighbour cell side condition in RSTD requirements.

Observation 7. Using a different granularity for smaller vs. larger UE Rx-Tx time difference values of the same DL PRS BW is a violation of RAN1 agreement. In multi-RTT, measurements with larger UE Rx-Tx time difference values correspond to farther cells and they are as important in the positioning fix. There is no reason to use a coarser granularity for them.

Proposal 7. RAN4 to use uniform report mapping table(s) for UE Rx-Tx time difference measurements, i.e., smaller and larger UE Rx-Tx time difference values to have the same reporting granularity.

Proposal 8. Reporting granularity for UE Rx-Tx time difference measurement to be as in Table 2.

Table 2 Proposed reporting granularity for UE Rx-Tx time difference measurement

| DL PRS BW (MHz) | Theoretical accuracy (ns) | Reporting Granularity (ns) | k |
| --- | --- | --- | --- |
| ≤ 25 | 20 | 8.1 | 4 |
| ≤ 50 | 10 | 4.1 | 3 |
| ≤ 100 | 5 | 2.0 | 2 |
| ≤ 200 | 2.5 | 1.0 | 1 |
| ≤ 400 | 1.25 | 0.5 | 0 |

Proposal 9. Use Table 3 for UE Rx-Tx time difference measurement report mapping for FR1 with 100 MHz PRS BW when UE also measures PRS with 100 MHz BW where NTA,Offset is either configured through n-TimingAdvanceOffset or default values in Table 7.1.2-3 of TS 38.133 is used.

Table 3 UE Rx-Tx time difference measurement report mapping for FR1 with 100 MHz PRS BW

| Reported Value | Measured quantity value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFF_0 | NTA,Offset ≤TUE,Rx-Tx ≤NTA,Offset+ 4 | Tc |
| RX-TX_TIME_DIFF_1 | NTA,Offset+4 ≤TUE,Rx-Tx ≤NTA,Offset+8 | Tc |
| … | … | … |
| RX-TX_TIME_DIFF_524286 | NTA,Offset+2097144 ≤TUE,Rx-Tx ≤NTA,Offset+2097148 | Tc |
| RX-TX_TIME_DIFF_524287 | NTA,Offset+2097148 <TUE,Rx-Tx | Tc |

Proposal 10. Use Table 4 for UE Rx-Tx time difference measurement report mapping for FR2 with 400 MHz PRS BW when UE also measures PRS with 400 MHz BW where NTA,Offset is either configured through n-TimingAdvanceOffset or default values in Table 7.1.2-3 of TS 38.133 is used.

Table 4 UE Rx-Tx time difference measurement report mapping for FR2 with 400 MHz PRS BW

| Reported Value | Measured quantity value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFF_0 | NTA,Offset ≤TUE,Rx-Tx ≤NTA,Offset+ 1 | Tc |
| RX-TX_TIME_DIFF_1 | NTA,Offset+1 ≤TUE,Rx-Tx ≤NTA,Offset+2 | Tc |
| … | … | … |
| RX-TX_TIME_DIFF_65534 | NTA,Offset+65534 ≤TUE,Rx-Tx ≤NTA,Offset+65535 | Tc |
| RX-TX_TIME_DIFF_65535 | NTA,Offset+65535<TUE,Rx-Tx | Tc |

Proposal 11. PRS BW used for measurement report mapping is the minimum of configured DL PRS BW and the BW that UE can use for measurement subject to its capability.

Observation 8. The M UE Rx-Tx time difference report belong to the same TRP and are logically close in value as they are a measure of round-trip delay to that TRP.

Proposal 12. RAN4 to define differential UE Rx-Tx time difference report mapping table. UE to report the minimum UE Rx-Tx time difference value from the set of M UE Rx-Tx time difference values of the same TRP with absolute report mapping as in Tables 3-4 and use positive differential UE Rx-Tx time difference value for the remaining M-1 UE Rx-Tx time difference values.

- FFS: the maximum range of differential UE Rx-Tx time difference report mapping

Proposal 13. The same report mapping table as in Tables 3-4 should be used for smaller PRS BW as reduction in reporting payload is not as significant as differential UE Rx-Tx time difference reporting in Proposal 9.

Proposal 14. In multi-RTT positioning, each UE Rx-Tx time difference measurement reporting criterion corresponds to one frequency layer with Ecat = 1 indicating Ntotal UE Rx-Tx time difference measurement reports.

- FFS: Ntotal=min( X2.X3.X4 ,  X7)

Observation 9. UE motion, UE clock drift and gNB clock drift can all result in significant errors for measurements that are conducted apart in time but are all used to generate the same position fix.

Proposal 15. The core measurement and performance requirements for UE Rx-Tx time difference applies if the configured SRS-Slot-offset and SRS-Periodicity parameters for SRS resource for positioning are such that any SRS transmission is within [-X, X] msec of at least one DL PRS resource from each of the TRPs in the assistance data.

- FFS: X = 25 msec

- Note: Such a RS configuration may enable sufficiently close measurements in time in both the UE and TRPs

# References

[1] R4-1915854

[2] R4-1915191

[3] R1-2000984

[4] R4-1915802

[5] R4-2000735

[6] R1-1913674