3GPP TSG-RAN4 Meeting #94-e	R4-2000784

# Online, 24th Feb 2020 - 6th Mar 2020

Title:	LS on RSTD measurement report mapping for NR positioning

Reply To:

Release:	Rel-16

Work Item:	NR_pos-Core

Source:	RAN WG 4

To:	RAN WG 1, RAN WG 2

Contact Person:

Name:	Jie Cui

E-mail Address:	jie_cui@apple.com

Attachments:	N.A.

1. Overall Description:

RAN4 has discussion on the RSTD measurement report mapping for NR positioning, and following conclusions are achieved,

- The RSTD report mapping table and relative quantity mapping table for FR1 is designed as in table x-1 and x-2,

For FR1, the UE shall report a reference quantity based on Table x-1 and a relative quantity defined in Table x-2, so that the difference between the measured RSTD quantity and the lower bound of the corresponding range from Table x-1 is between and + resolutionStep. resolutionStep is 16Tc for 15kHz SCS, 8Tc for 30kHz SCS and 8Tc for 60kHz in the range from RSTD_2260 to RSTD_10451, and 32Tc for 15kHz SCS, 16Tc for 30kHz SCS and 16Tc SCS for 60kHz in the range from RSTD_0000 to RSTD_2259 or in the range from RSTD_10452 to RSTD_12711. The applicability of relative quantity value is defined in Table x-2. Table x-1 RSTD report mapping for FR1 Reported Value; Measured Quantity Value; Unit. RSTD_0000; -15391 > RSTD; 64Tc. RSTD_0001; -15391 RSTD < -15386; 64Tc. …; …. RSTD_2258; -4106 RSTD < -4101; 64Tc. RSTD_2259; -4101 RSTD < -4096; 64Tc. RSTD_2260; -4096 RSTD < -4095; 64Tc. RSTD_2261; -4095 RSTD < -4094; 64Tc. …. RSTD_6353; -3 RSTD -2; 64Tc. RSTD_6354; -2 RSTD -1; 64Tc. RSTD_6355; -1 RSTD 0; 64Tc. RSTD_6356; 0 < RSTD 1; 64Tc. RSTD_6357; 1 < RSTD 2; 64Tc. RSTD_6358; 2 < RSTD 3; 64Tc. …; …; …. RSTD_10450; 4094 < RSTD 4095; 64Tc. RSTD_10451; 4095 < RSTD 4096; 64Tc. RSTD_10452; 4096 < RSTD 4101; 64Tc. RSTD_10453; 4101 < RSTD 4106; 64Tc. …; …; …. RSTD_12709; 15381 < RSTD 15386; 64Tc. RSTD_12710; 15386 < RSTD 15391; 64Tc. RSTD_12711; 15391 < RSTD; 64Tc. Table x-2 Relative quantity mapping for different SCS in FR1 Reported Relative Quantity Value; Measured Relative Quantity Value,; Unit; Applicable SCS(kHz); Applicable RSTD report range in table x-1. RSTD_delta_0; 0; Tc; 15; all RSTD reported values in table x-1. 30/60. RSTD_delta_1; 8; Tc; 15; NA. 30/60; from RSTD_2260 to RSTD_10451 in table x-1. RSTD_delta_2; 16; Tc; 15; from RSTD_2260 to RSTD_10451 in table x-1. 30/60; all RSTD reported values in table x-1. RSTD_delta_3; 24; Tc; 15; NA. 30/60; from RSTD_2260 to RSTD_10451 in table x-1. RSTD_delta_4; 32; Tc; 15; all RSTD reported values in table x-1. 30/60. RSTD_delta_5; 40; Tc; 15; NA. 30/60; from RSTD_2260 to RSTD_10451 in table x-1. RSTD_delta_6; 48; Tc; 15; from RSTD_2260 to RSTD_10451 in table x-1. 30/60; all RSTD reported values in table x-1. RSTD_delta_7; 56; Tc; 15; NA. 30/60; from RSTD_2260 to RSTD_10451 in table x-1. RSTD_delta_8; 64; Tc; 15; all RSTD reported values in table x-1. 30/60. RSTD_delta_9; 80; Tc; 15; NA. 30/60; all RSTD reported values in table x-1. RSTD_delta_10; 96; Tc; 15; all RSTD reported values in table x-1. 30/60. RSTD_delta_11; 112; Tc; 15; NA. 30/60; all RSTD reported values in table x-1. RSTD_delta_12; 128; Tc; 15; all RSTD reported values in table x-1. 30/60. RSTD_delta_13; 144; Tc; 15; NA. 30/60; all RSTD reported values in table x-1. RSTD_delta_14; 160; Tc; 15; all RSTD reported values in table x-1. 30/60. RSTD_delta_15; 176; Tc; 15; NA. 30/60; all RSTD reported values in table x-1. RSTD_delta_16; 192; Tc; 15; all RSTD reported values in table x-1. 30/60. RSTD_delta_17; 208; Tc; 15; NA. 30/60; all RSTD reported values in table x-1. RSTD_delta_18; 224; Tc; 15; all RSTD reported values in table x-1. 30/60. RSTD_delta_19; 240; Tc; 15; NA. 30/60; all RSTD reported values in table x-1. RSTD_delta_20; 256; Tc; 15; all RSTD reported values in table x-1. 30/60. RSTD_delta_21; 272; Tc; 15; NA. 30/60; all RSTD reported values in table x-1. RSTD_delta_22; 288; Tc; 15; all RSTD reported values in table x-1. 30/60. RSTD_delta_23; 304; Tc; 15; NA. 30/60; all RSTD reported values in table x-1.

- The RSTD report mapping table and relative quantity mapping table for FR2 is designed as in table y-1 and y-2,

For FR2, the UE shall report a reference quantity based on Table y-1 and a relative quantity defined in Table y-2, so that the difference between the measured RSTD quantity and the lower bound of the corresponding range from Table y-1 is between and + resolutionStep. resolutionStep is 4Tc for 60kHz SCS and 2Tc for 120kHz SCS in the range from RSTD_2260 to RSTD_10451, and 8Tc for 60kHz SCS and 4Tc for 120kHz SCS in the range from RSTD_0000 to RSTD_2259 or in the range from RSTD_10452 to RSTD_12711. The applicability of relative quantity value is defined in Table y-2. Table y-1 RSTD report mapping for FR2 Reported Value; Measured Quantity Value; Unit. RSTD_0000; -15391 > RSTD; 16Tc. RSTD_0001; -15391 RSTD < -15386; 16Tc. …; …. RSTD_2258; -4106 RSTD < -4101; 16Tc. RSTD_2259; -4101 RSTD < -4096; 16Tc. RSTD_2260; -4096 RSTD < -4095; 16Tc. RSTD_2261; -4095 RSTD < -4094; 16Tc. …. RSTD_6353; -3 RSTD -2; 16Tc. RSTD_6354; -2 RSTD -1; 16Tc. RSTD_6355; -1 RSTD 0; 16Tc. RSTD_6356; 0 < RSTD 1; 16Tc. RSTD_6357; 1 < RSTD 2; 16Tc. RSTD_6358; 2 < RSTD 3; 16Tc. …; …; …. RSTD_10450; 4094 < RSTD 4095; 16Tc. RSTD_10451; 4095 < RSTD 4096; 16Tc. RSTD_10452; 4096 < RSTD 4101; 16Tc. RSTD_10453; 4101 < RSTD 4106; 16Tc. …; …; …. RSTD_12709; 15381 < RSTD 15386; 16Tc. RSTD_12710; 15386 < RSTD 15391; 16Tc. RSTD_12711; 15391 < RSTD; 16Tc. Table y-2 Relative quantity mapping for FR2 Reported Relative Quantity Value; Measured Relative Quantity Value,; Unit; Applicable SCS(kHz); Applicability. RSTD_delta_0; 0; Tc; 60; all RSTD reported values in table y-1. 120. RSTD_delta_1; 2; Tc; 60; NA. 120; from RSTD_2260 to RSTD_10451 in table y-1. RSTD_delta_2; 4; Tc; 60; from RSTD_2260 to RSTD_10451 in table y-1. 120; all RSTD reported values in table y-1. RSTD_delta_3; 6; Tc; 60; NA. 120; from RSTD_2260 to RSTD_10451 in table y-1. RSTD_delta_4; 8; Tc; 60; all RSTD reported values in table y-1. 120. RSTD_delta_5; 10; Tc; 60; NA. 120; from RSTD_2260 to RSTD_10451 in table y-1. RSTD_delta_6; 12; Tc; 60; from RSTD_2260 to RSTD_10451 in table y-1. 120; all RSTD reported values in table y-1. RSTD_delta_7; 14; Tc; 60; NA. 120; from RSTD_2260 to RSTD_10451 in table y-1. RSTD_delta_8; 16; Tc; 60; all RSTD reported values in table y-1. 120. RSTD_delta_9; 20; Tc; 60; NA. 120; all RSTD reported values in table y-1. RSTD_delta_10; 24; Tc; 60; all RSTD reported values in table y-1. 120. RSTD_delta_11; 28; Tc; 60; NA. 120; all RSTD reported values in table y-1. RSTD_delta_12; 32; Tc; 60; all RSTD reported values in table y-1. 120. RSTD_delta_13; 36; Tc; 60; NA. 120; all RSTD reported values in table y-1. RSTD_delta_14; 40; Tc; 60; all RSTD reported values in table y-1. 120. RSTD_delta_15; 44; Tc; 60; NA. 120; all RSTD reported values in table y-1. RSTD_delta_16; 48; Tc; 60; all RSTD reported values in table y-1. 120. RSTD_delta_17; 52; Tc; 60; NA. 120; all RSTD reported values in table y-1. RSTD_delta_18; 56; Tc; 60; all RSTD reported values in table y-1. 120. RSTD_delta_19; 60; Tc; 60; NA. 120; all RSTD reported values in table y-1. RSTD_delta_20; 64; Tc; 60; all RSTD reported values in table y-1. 120. RSTD_delta_21; 68; Tc; 60; NA. 120; all RSTD reported values in table y-1. RSTD_delta_22; 72; Tc; 60; all RSTD reported values in table y-1. 120. RSTD_delta_23; 76; Tc; 60; NA. 120; all RSTD reported values in table y-1.

2. Actions:

To RAN WG1 and WG2 group.

ACTION: 	RAN4 kindly asks RAN1 and RAN2 to take the above information into account in their future work.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 #94bis			20th - 24th Apr 2020 		Canada

TSG-RAN4 #95		  25th - 29th May 2020		Athens, Greece