3GPP TSG-RAN WG4 Meeting #94-e                       	 R4- 2001029

# Electronic Meeting, 24 Feb. – 6 Mar., 2020

Agenda Item:	8.4.5.4

Source:	MediaTek Inc.

Title:	Discussion on NR V2X interruption requirement

Document for:		Discussion

- Introduction

In last RAN4 #93 meeting, a WF about NR V2X RRM requirement was approved. The open issue about interruption is listed below.

- Interruption to SL due to synchronization source change - Allow interruption of 1ms on SL Tx and Rx during synchronization source change for all SCS in NR V2X

In this paper, we will discuss the interruption requirement for synchronization source change.

- Requirement

Currently, NR sidelink UE supports multiple synchronization sources:

- GNSS, UE directly synced to GNSS, UE indirectly synced to GNSS

- eNB, UE directly synced to eNB, UE indirectly synced to eNB

- gNB, UE directly synced to gNB, UE indirectly synced to gNB

Different as LTE sidelink, NR sidelink UE supports both eNB and gNB as synchronization source.

RAN4 had already agreed single carrier sidelink with EN-DC/NE-DC Uu link is not an applicable scenario. Thus, the synchronization source can only be changed between GNSS and eNB or between GNSS and gNB. RAN4 won’t need to define the interruption requirement for synchronization source change between eNB and gNB.

RAN4 #93 Agreement: - Not applicable scenarios - Single carrier in NR V2X SL + EN-DC/NE-DC/NR-DC/NR-CA/LTE-CA/LTE-DC in Uu - NR V2X SL is dedicated to NR V2X SL and is not used for Uu link

Proposal 1: RAN4 only define the interruption requirement for synchronization source change between GNSS and eNB and between GNSS and gNB.

- Summary

In this paper, we give our views on interruption requirement for synchronization source change.

Proposal 1: RAN4 only define the interruption requirement for synchronization source change between GNSS and eNB and between GNSS and gNB.