3GPP TSG-RAN WG4 Meeting #94-e	R4-2001359

Electronic Meeting, 24 February – 6 March 2020

Agenda Item:	8.1.4.3

Source:	Ericsson

Title:	SI acquisition in NR-U

Document for:	Discussion

# 1	Introduction

In the way forward on the RRM requirements for NR-U [1], it was agreed to analyze how to extend TSI-NR.

SIB Reading in RRC Re-establishment, RRC Release with Redirection, and Paging Interruption Requirements - SIB reading in RRC re-establishment, RRC release with redirection, and paging interruption requirements - Further study SIB reading time including MIB and SIB1 decoding time - Option 1: SIB reading time 1280 ms - Other options are not precluded - FFS whether soft combining or single shot detection without soft combining shall be applied - The above also addresses extending the time needed to acquire the MIB and SIB1 due to LBT failure in paging interruption requirements, based on the simulation study (simulation assumptions are to be further discussed off-line)

- Option 2: SI reading time is derived based on the necessary number of samples and the LBT impact - Number of PBCH samples for MIB is 5 - Number of PDSCH samples for SIB1 is based on simulation results with 95% of decoding success rate, assuming: Parameters; Unit; Value. Channel bandwidth; MHz; 20. SCS; kHz; 15. Number of transmitter antennas; 1. Number of receive antennas; 2. Allocated resource blocks for PDSCH; 24 (allocated in the lower edge of the carrier). MCS index; 4. Modulation; QPSK. Target Coding Rate; 0.33. Number of control symbols; 2. PDSCH mapping type; Type A. Information Bit Payload. For slots with RMSI; Bits; 1864. Number of Code Blocks per slot; 1. Binary Channel Bits Per slot. For slots with RMSI; Bits; 6048. Side condition (PDSCH SNR); dB; -6. Propagation channel model; AWGN, TDLA30-10, TDLB100-400, TDLC300-100. Receiver assumption; Single shot detection Soft-combining (2, 4 and 8 samples). SIB1 transmission period; ms; 20. Metric; 95% of SIB1 acquisition success rate. Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1..

# 2	SI acquisition time for RRC Release with Redirection and RRC Re-establishment

SI acquisition time (TSI-NR) is the time required for receiving all the relevant system information according to the reception procedure. SI acquisition time can be divided into MIB acquisition time (TMIB-NR), SIB1 acquisition time (TSIB1-NR), and other SI acquisition time (TSIBx-NR). If we focus on RRC release with redirection and RRC re-establishment, we can assume UE needs to acquire RACH configuration to be able to perform the random access procedure. Since RACH configuration is included in SIB1, and the SIB1 scheduling information is provided by MIB, SI acquisition time, TSI-NR, is given by TSI-NR = TMIB + TSIB1, where TMIB is the MIB acquisition time and TSIB1 is the SIB1 acquisition time.

Observation 1: SI acquisition time for RRC Release with Redirection and RRC Re-establishment procedures, TSI-NR, are given by TSI-NR = TMIB + TSIB1, where TMIB is the MIB acquisition time and TSIB1 is the SIB1 acquisition time.

Proposal 1: Define SI acquisition time for NR-U as TSI_CCA = TMIB_CCA + TSIB1_CCA, where TMIB_CCA is the MIB acquisition time for NR-U and TSIB1_CCA is the SIB1 acquisition time for NR-U.

# 3	SI acquisition time for NR-U

## 3.1	MIB acquisition time

Considering the LBT failure, the MIB acquisition time is given by

TMIB_CCA = TSMTC * GMIB

where TSMTC is SMTC period = [5, 10, 20, 40, 80, 160] (ms), and GMIB = NMIB + M MIB_CCA-failure, where NMIB is the required number of PBCH transmissions and M MIB_CCA-failure is the number of LBT failures by the time BS can transmit NMIB PBCH samples.

Since RAN4#93 agreed to set NMIB=5, we propose the MIB acquisition time as follows:

Proposal 2: Set TMIB_CCA = TSMTC * GMIB, where TSMTC is the configured SMTC period and GMIB = NMIB + MMIB_CCA-failure, where NMIB=5 and M MIB_CCA-failure is the number of LBT failures by the time BS transmits NMIB PBCH samples.

## 3.2	SIB1 acquisition time

Like MIB acquisition, if we assume the necessary number of PDSCH samples to achieve 95% success rate for SIB1 acquisition to NSIB1, the SIB1 acquisition time considering LBT failure is given by

TSIB1-NR-U = TSIB1 * GSIB1

where TSIBI is SIB1 transmission period and it is fixed to 20ms in the case of FR1 by default, and GSIB1 = NSIB1 + MSIB1_CCA-failure, where NSIB1 is the number of PDSCH transmissions to achieve 95% of SIB1 decoding success rate and MSIB1_CCA-failure is the number of slots UE does not receive SIB1 on the SIB1 transmission occasions by the time UE receives NSIB1 PDSCH samples. There are two possibilities UE does not receive SIB1 on the SIB1 transmission occasions. One is because gNB does not schedule SIB1 transmission, as TS38.133 mentions ‘the actual transmission repetition periodicity is up to network implementation.’ Another reason is because gNB scheduled SIB1 but cannot transmit due to the LBT failure.

Proposal 3: Set TSIB1_CCA = TSIB1 * GSIB1, where TSIB1 is the SIB1 transmission repetition period (=20ms for FR1) and GSIB1 = NSIB1 + MSIB1_CCA-failure, where MSIB1_CCA-failure is the number of slots UE does not receive SIB1 on the SIB1 transmission occasions by the time UE receives NSIB1 PDSCH samples.

Table 1 shows our simulation result of PSDCH demodulation assuming SIB1 reception based on the agreed simulation assumption in [1]. The result shows the PDSCH decoding success rate with single shot receiver and multi-shot receiver with the side condition of SNR = -6dB.

Table 1	SIB1 decoding success rate with the side condition of SNR=-6dB.

|  | AWGN | TDLA10-30 | TDLB100-400 | TDLC300-100 |
| --- | --- | --- | --- | --- |
| Single shot | 0.00 | 0.42 | 0.12 | 0.12 |
| 2 samples | 1.00 | 0.80 | 0.70 | 0.74 |
| 4 samples | 1.00 | 0.94 | 1.00 | 1.00 |
| 8 samples | 1.00 | 1.00 | 1.00 | 1.00 |

It is observed from the simulation result that, it cannot achieve 95% of PDSCH decoding success rate with single shot receiver. With the soft-combining of 4 or more PDSCH samples can achieve 95% of PDSCH decoding success rate for most cases. If UE cannot perform soft-combining, UE need to continue to decode PDSCH until it is successful (a.k.a. keep-trying). However if we look the simulation results with AWGN, the success rate of single shot receiver is 0% with SNR=-6dB, this means UE has no chance to read SIB1 even with keep-trying. We therefore propose to set the requirements based on the soft-combining receiver, and set the number of PDSCH samples for SIB1, NSIB1, to 4, targeting 95% of decoding success rate.

Proposal 4: Set the number of PDSCH samples for SIB1, NSIB1, to 4.

Considering NMIB=5 and NSIB1=4, the SI reading time option 1, i.e., 1280ms, is pessimistic. We therefore propose to set SI reading delay based on the number of PBCH samples for MIB and the number of PDSCH samples for SIB1, but RAN4 need discuss further how to derive the SI reading dela.

Proposal 5: Set the SI acquisition time based on the number of PBCH samples for MIB and the number of PDSCH samples for SIB1.

# 4	Summary

Observation 1: SI acquisition time for RRC Release with Redirection and RRC Re-establishment procedures, TSI-NR, are given by TSI-NR = TMIB + TSIB1, where TMIB is the MIB acquisition time and TSIB1 is the SIB1 acquisition time.

Proposal 1: Define SI acquisition time for NR-U as TSI_CCA = TMIB_CCA + TSIB1_CCA, where TMIB_CCA is the MIB acquisition time for NR-U and TSIB1_CCA is the SIB1 acquisition time for NR-U.

Proposal 2: Set TMIB_CCA = TSMTC * GMIB, where TSMTC is the configured SMTC period and GMIB = NMIB + MMIB_CCA-failure, where NMIB=5 and M MIB_CCA-failure is the number of LBT failures by the time BS transmits NMIB PBCH samples.

Proposal 3: Set TSIB1_CCA = TSIB1 * GSIB1, where TSIB1 is the SIB1 transmission repetition period (=20ms for FR1) and GSIB1 = NSIB1 + MSIB1_CCA-failure, where MSIB1_CCA-failure is the number of slots UE does not receive SIB1 on the SIB1 transmission occasions by the time UE receives NSIB1 PDSCH samples.

Proposal 4: Set the number of PDSCH samples for SIB1, NSIB1, to 4.

Proposal 5: Set the SI acquisition time based on the number of PBCH samples for MIB and the number of PDSCH samples for SIB1.

# References

- R4-1915777, “WF on NR-U RRM Requirements (all agreements in RAN4#93)”, Ericsson, Qualcomm.

# Appendix – Simulation parameters

| Parameters | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 10 |
| SCS | kHz | 15 |
| Number of transmitter antennas |  | 1 |
| Number of receive antennas |  | 2 |
| Allocated resource blocks for PDSCH |  | 24 (allocated in the lower edge of the carrier) |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 0.33 |
| Number of control symbols |  | 2 |
| PDSCH mapping type |  | Type A |
| Information Bit Payload |  |  |
| For slots with RMSI | Bits | 1608 |
| Number of Code Blocks per slot |  | 1 |
| Binary Channel Bits Per slot |  |  |
| For slots with RMSI | Bits | 5184 |
| Side condition (PDSCH SNR) | dB | -6 |
| Propagation channel model |  | AWGN, TDLA30-10, TDLB100-400, TDLC300-100 |
| Receiver assumption |  | Single shot detection Soft-combining (2, 4 and 8 samples) |
| SIB1 transmission period | ms | 20 |
| Metric |  | 95% of SIB1 acquisition success rate |
| Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. |  |  |