3GPP TSG-RAN WG4 Meeting #94-e	R4-2001364

Electronic Meeting, 24 February – 6 March 2020

Agenda Item:	8.15.1.3

Source:	Ericsson

Title:	PDSCH simulation result for SI reading

Document for:	Discussion

# 1	Introduction

RAN4#93 agreed with the way forward on CGI reading for Rel-16 NR RRM enhancements [1], where it is agreed that the SIB1 decoding delay is derived based on the simulation results. RAN4#93 also agreed with the PDSCH simulation assumption considering the SIB1 transmission [2].

This contribution provides our simulation on PDSCH to derive the number of PDSCH transmissions for SIB1 reading.

# 2	Simulation results

Table 1 and Table 2 show our simulation result of SIB1 reading for FR1 and FR2, respectively, according to the simulation assumption in Appendix. It is observed from the simulation result that it requires the soft-combining of 4 PDSCH transmissions to achieve 90% of SIB1 decoding success rate with SNR=-6dB for both FR1 and FR2.

If we assume the keep-trying with single-shot receiver (i.e., UE continues receiving PDSCH for SIB1 until it decodes successfully), it needs at least 10 SIB1 occasions for the fading channel case. For AWGN case, the simulation result shows UE never decode SIB1 with keep-trying with single-shot because the SIB1 decoding success rate is 0%.

Observation 1: For both FR1 and FR2, it requires the soft-combining of 4 PDSCH samples to achieve 90% if SIB1 decoding success rate with the side condition of SNR=-6dB.

Table 1	SIB1 decoding success rate (FR1, 4GHz, SNR=-6dB)

|  | AWGN | TDLC300 3km/h |
| --- | --- | --- |
| Single shot | 0.00 | 0.20 |
| 2 samples | 1.00 | 0.78 |
| 4 samples | 1.00 | 0.97 |
| 8 samples | 1.00 | 1.00 |

Table 2	SIB1 decoding success rate (FR2, 30GHz, SNR=-6dB)

|  | AWGN | TDLC60 3km/h |
| --- | --- | --- |
| Single shot | 0.00 | 0.13 |
| 2 samples | 1.00 | 0.77 |
| 4 samples | 1.00 | 0.98 |
| 8 samples | 1.00 | 1.00 |

TS 38.331 specified the SIB1 transmission as follows:

the SIB1 is transmitted on the DL-SCH with a periodicity of 160 ms and variable transmission repetition periodicity within 160 ms as specified in TS 38.213 [13], clause 13. The default transmission repetition periodicity of SIB1 is 20 ms but the actual transmission repetition periodicity is up to network implementation. For SSB and CORESET multiplexing pattern 1, SIB1 repetition transmission period is 20 ms. For SSB and CORESET multiplexing pattern 2/3, SIB1 transmission repetition period is the same as the SSB period (TS 38.213 [13], clause 13). SIB1 includes information regarding the availability and scheduling (e.g. mapping of SIBs to SI message, periodicity, SI-window size) of other SIBs with an indication whether one or more SIBs are only provided on-demand and, in that case, the configuration needed by the UE to perform the SI request. SIB1 is cell-specific SIB

In the case of SSB and CORESET multiplexing pattern 1 (configured in FR1), UE assumes the SIB1 transmission repetition periodicity is 20 ms and gNB may transmit 8 PDSCH samples for SIB1 within the SIB1 transmission periodicity of 160ms.

In the case of SSB and CORESET multiplexing pattern 2/3 (configured in FR1 and FR2), on the other hand, UE assumes the SIB1 transmission repetition periodicity is same as SSB period, which corresponds to 20ms in RAN4 assumption for the initial cell search. In the real network, however, gNB can configure SSB periodicity with {5, 10, 20, 40, 80, 160ms}.

It is also important to emphasise that TS38.331 mentions ‘the actual transmission repetition periodicity is up to network implementation’. This means gNB may transmit only one SIB1 within the SIB1 transmission periodicity even with the SSB and CORESET multiplication pattern 1. In this case it is not guaranteed that UE can read SIB1 within 160ms even if UE is capable of soft-combining of PDSCH for SIB1 acquisition.

Observation 2: Number of SIB1 transmissions within the SIB1 transmission periodicity is up to network implementation.

It is also up to network implementation but if gNB does not change the SIB1 information across the SIB1 transmission periodicities, then UE can also do the soft-combining of PDSCH across the SIB1 transmission periodicities. It is noted that the LTE eMTC/NB-IoT SI reading requirements were specified based on whether soft-combining across SIB1 transmission periodicities.

Observation 3: If gNB does not change the SIB1 information across the SIB1 transmission periodicities, UE may also do the soft-combining of PDSCH samples across the SIB1 transmission periodicities.

Based on the observations, we propose to derive the SIB1 reading delay based on the soft-combining of 4 PDSCH samples. However it is also important to discuss in RAN4 how to ensure UE can soft-combine the PDSCH samples within the SIB1 transmission periodicity and across the SIB1 transmission periods.

Proposal 1: Derive the SIB1 reading delay based on the soft-combining of 4 PDSCH samples. RAN4 discuss further how to ensure UE can soft-combine the PDSCH symbols within the SIB1 transmission periodicity and across the SIB1 transmission periods.

# 3 Summary

Observation 1: For both FR1 and FR2, it requires the soft-combining of 4 PDSCH samples to achieve 90% if SIB1 decoding success rate with the side condition of SNR=-6dB.

Observation 2: Number of SIB1 transmissions within the SIB1 transmission periodicity is up to network implementation.

Observation 3: If gNB does not change the SIB1 information across the SIB1 transmission periodicities, UE may also do the soft-combining of PDSCH samples across the SIB1 transmission periodicities.

Proposal 1: Derive the SIB1 reading delay based on the soft-combining of 4 PDSCH samples. RAN4 discuss further how to ensure UE can soft-combine the PDSCH symbols within the SIB1 transmission periodicity and across the SIB1 transmission periods.

# 4	References

- R4-1915932, “Way forward on R16 NR RRM enhancements – CGI reading”, ZTE.

- R4-1915851, “Simulation assumption on SIB1 decoding for NR CGI reading”, ZTE.

# Appendix – Simulation parameters

| Parameters | Unit | Value |
| --- | --- | --- |
| Subcarrier spacing | KHz | 30 kHz for 4GHz; 120 kHz for 30GHz |
| Channel bandwidth | MHz | 10 for SCS=30kHz 100 for SCS=120kHz |
| Number of transmitter antennas |  | 1 |
| Number of Rx antennas |  | 2 |
| CP Length |  | Normal |
| Allocated resource blocks for PDSCH |  | 24 (allocated from the lowest edge of the carrier) |
| Number of PDSCH symbols |  | 12 |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| PDSCH mapping type |  | Type A |
| dmrs-TypeA-Position |  | 2 |
| dmrs-Type |  | 1 |
| dmrs-AdditonalPositions |  | 2 |
| Dmrs-maxLength |  | 1 |
| Number of PDSCH DMRS CDM group(s) without data |  | 2 |
| PRB bundling size |  | 2 |
| SIB1 transmission period | ms | 20 |
| Information Bit Payload |  |  |
| For slots with RMSI | Bits | 1608 |
| Number of Code Blocks per slot |  | 1 |
| Binary Channel Bits Per slot |  |  |
| For slots with RMSI | Bits | 5184 |
| SNR |  | -8, -6, -4, -2, 0 |
| Propagation Condition |  | AWGN, TDL-C with RMS delay spread = 300ns for FR1 TDL-C with RMS delay spread = 60ns for FR2 |
| UE speed |  | 3km/h |
| Detection Method |  | Option 1: Soft combining with 2, 4, 8 repetitions Option 2: One shot |
| Metrics |  | The number of repetitions required to decode SIB1 with 90%-ile success rate. |