3GPP TSG-RAN WG4 Meeting #94-e		                    R4-2001805

Online, 24th February – 6 March 2020

Source: 	Keysight Technologies

Title: 	Study on NR Test Model signal characteristic by data content choice 
Agenda Item:	6.8.3.3

Document for:	Discussion

# Introduction

In last RAN4#93 meeting, WF[1] created for further study on data field content impact on Test Model signal characteristics. This document provides result of further study on choice of data field content and proposals.

# Description

In last RAN4#93 meeting, question raised to see if “All Zero” data content is appropriate as NR Test model design and proposed to do further study and investigate “random” data to be used to improve Test Model signal characteristic.

As this issue described in contribution document [2], CCDF plot result is shown and used for describing characteristic of signal. For evaluating signal characteristic, CCDF curve is very useful tool to see power characteristic of signal including evaluating Peak-to-Average ratio as well as statistical representation on power distribution of signal especially for noise like signal such as OFDM modulated signal (ex. LTE and NR, etc.). Here is description of CCDF curve from application note [3];

“A CCDF curve shows how much time the signal spends at or above a given power level. The power level is expressed in dB relative to the	average power. For example, each of the lines across	the	waveform	shown	in Figure 1A represents a specific power level above the average. The percentage of time the signal spends at or above each line defines the probability	for	that particular power level. A CCDF curve	is a	 plot of relative power levels versus probability.”

Fig 1A sample signal in power versus time display					Fig 1B CCDF plot of sample signal (left)

In Fig 1B plot, gaussian distribution is shown as reference, which is close to real-life use so that plot close to this reference is more realistic signal to be used as test signal.

One thing to note about CCDF curve is, very precise detail of curve also depends on where (starting point and stopping point) on signal taken for curve plot and sampling timing and rate of measurement equipment not just signal characteristics itself. So that, checking too much of detail and comparing in very detail on curve doesn’t have much meaning. It’s good to see overall characteristics.

For further evaluation on choice of data, “All zero” versus “random” and then which PN sequence to use for “random”, based on WF[2], 5MHz and 20 MHz bandwidth test model, TM1.1, TM1.2, TM3.1 with 15k, 30k, 60k SCS are looked. In addition, 10MHz BW signal was looked at. For PN sequence variation, PN23 and PN31 compared.

- Regarding with CCDF curve, random data are close to gaussian (real-life) in most of the cases in comparison with “All zero”.

- “All Zero” in some cases shows very similar curve as “random” data (ex. TM1.2 20MHz BW 15kSCS) but in some other cases, off from gaussian. Amount of off is about 2dB as largest (ex. TM1.1 5MHz BW 30kSCS)

- For the Peak point, as we pointed in last meeting, in some cases, “All zero” shows lower peak even from gaussian as largest about 1.5dB. but this also means off from real-life cases. (ex. TM3.1 20MHz BW 60kSCS)

- Largest difference in peak between three data types are about less than 2dB (in plus and minus).

- Overall, characteristic from CCDF curve with these samples are in favor of use of “random” data.

Regarding with choice of PN sequence, between PN23 and PN31, (note, for PN15 vs PN23, please see our contribution [4] in previous meeting);

- Both PN23 and PN31 shows very similar result with these samples. In some cases, PN23 shows better and some cases PN31 shows better. With these sample, difficult to judge which is better.

- Regarding with largest payload size vs length of entire PN sequence, for example, largest payload size as 400MHz BW with 120kSCS, CCDF curve are very close to gaussian in both PN sequence cases so that it doesn’t matter with which PN sequence by this result.

In other aspect that more randomization on data itself, because these CCDF curve already shows good enough result in many cases, it doesn’t necessary to do further consideration.

Regarding with different cell ID, Cell ID=2 is experimented with the same set of combination. Each CCDF curve plot becomes different from Cell ID=1 cases, however, overall, summary is the same.

For the case of multiple CC configuration, when having multiple CC, it adds more randomness and resulted CCDF curve closer to gaussian in comparison with one carrier. Here is CCDF curve example of two TM combination with “All zero” and “PN23” are in data.

Fig 2a TM1.1 20MHz BW 30k SCS CCDF of 1CC, 2CC			Fig 2b TM1.2 5MHz BW 15k SCS CCDF of 1CC, 2CC

In both cases, 2CC curve plot are closer to gaussian or about the same. There is no need to worry about multiple CC configuration.

Regarding with EVM evaluation with use of TM2 signal, its CCDF plot has larger off from gaussian in comparison with other TMs. With closer look, this is caused by PDCCH channel not PDSCH channel. See Fig 3 below. This example is from 20MHz BW 30k SCS. On Fig3a which is power envelop of TM2 signal, pointed by small red circle where higher peak is shown in period where PDCCH exist. On Fig3b, comparison of CCDF plot, black plot is TM2 as current definition and blue plot is CCDF without PDCCH (only with PDSCH) which shows much smaller peak. Other samples from 5MHz BW, 20MHz BW, 200MHz BW shows the same characteristic, larger peak and larger off plot are by PDCCH. While PDCCH is necessary for better EVM measurement result on TM2 (note, without PDCCH, number of DMRS is too small. This has been studied and discussed when TM2 design was made in past meeting), and changing data payload on PDCCH from “All Zero” to “random” (either PN23 or PN31) doesn’t show much of improvement on peak. If this was concern of equipment venders, further study would be needed.

Fig 3a. Power envelope (Time Domain) of TM2 (20M, 30kscs)	  Fig 3b. CCDF plot of TM2 (20M, 30kscs) w or w/o PDCCH

# Conclusion/Proposal

Observation 1. CCDF plot is very useful tool to evaluate power characteristic of signal

Observation 2. “random” sequence shows close to gaussian (real-life) in most of the cases in comparison with “All zero” data.

Observation 3. There is not much of difference between PN23 and PN31.

Observation 4. For data sequence choice with case of multiple CC, multiple CC itself increases randomness and result CCDF plot closer to real-life. No further study is needed for multiple CC with different cell ID.

Observation 5. TM2 CCDF curve is not improved by changing data type. If CCDF plot and higher peak of TM2 is concern, RAN4 needs to conduct further study.

Proposal1. Use CCDF curve as tool to evaluate data content of Test model.

Proposal2. Use random data rather All zero for test model data content.

Proposal3. Between PN23 and PN31, either PN sequence is good enough. And no further randomization study is not necessary.

Proposal4. No further study necessary for multiple CC case and cell ID.

Proposal5. Further study may be needed on TM2 if RAN4 sees concern on CCDF plot of TM2

# References

[1] R4-1916052 “WF for random data content of physical channels for NR test modes”

[2] R4-1914558 “Further elaboration on NR Test Model” by Ericsson

[3] “Characterizing Digitally Modulated Signals with CCDF Curves” Keysight Technologies Application note 5968-6875E

[4] R4-1916050 “CCDF analysis result with use of random data in Test Model payload” by Keysight Technologies

# Appendix CCDF curve from sample combination

Here is sample CCDF curves from 20MHz 30kscs, TM1.1, TM1.2, TM3.1, all with CellID=1 and TM1.1 20MHz 30kscs with CellID=2

| TM1.1 20MHz BW, 30kscs, CellID=1 | TM1.2 20MHz BW, 30kscs, CellID=1 |
| --- | --- |
| TM3.1 20MHz BW, 30kscs, CellID=1 | TM1.1 20MHz BW, 30kscs, CellID=2 |