3GPP TSG RAN WG4 Meeting #94-e	R4-2001857

Electronic Meeting, February 24-March 06, 2020

Agenda Item:	8.5.5.2

Source: 	Ericsson

Title:	TP to TS 38.174 v0.0.1: IAB MT TA accuracy requirements

Document for:	Approval

- Introduction

In RAN4#92bis according to the approved WF it was agreed to introduce TA adjustment accuracy requirements for IAB MT. Section 2 provides text proposal on IAB MT TA adjustment accuracy requirements for the IAB specification TS 38.171 v0.0.01 (Integrated access and backhaul radio transmission and reception).

# Text Proposal

--------------------------------------------------Start of TP------------------------------------------------------

### 12.2.3 IAB-MT timing advance

### 12.2.3.1	Introduction

The timing advance is initiated from IMT-DU with MAC message that implies and adjustment of the timing advance, as defined in clause 5.2 of TS 38.321 [TBD].

### 12.2.3.2	Requirements

#### 12.2.3.2.1	Timing Advance adjustment delay

IAB-MT shall adjust the timing of its uplink transmission timing at time slot n+ k+1 for a timing advance command received in time slot n, and the value of k is defined in clause 4.2 in TS 38.213 [TBD]. The same requirement applies also when the IAB-MT is not able to transmit a configured uplink transmission due to the channel assessment procedure.

#### 12.2.3.2.2	Timing Advance adjustment accuracy

The IAB-MT shall adjust the timing of its transmissions with a relative accuracy better than or equal to the IAB-MT Timing Advance adjustment accuracy requirement in Table 12.2.3.2.2-1, to the signalled timing advance value compared to the timing of preceding uplink transmission. The timing advance command step is defined in TS 38.213 [TBD].

Table 12.2.3.2.2-1: IAB-MT Timing Advance adjustment accuracy

| UL Sub Carrier Spacing (kHz) | 15 | 30 | 60 | 120 |
| --- | --- | --- | --- | --- |
| IAB-MT Timing Advance adjustment accuracy | ±256 Tc | ±256 Tc | ±128 Tc | ±32 Tc |

--------------------------------------------------End of TP------------------------------------------------------

- References

- R4-1912829, Way forward on scope on NR IAB RRM requirements, Qualcomm