3GPP TSG-RAN WG4 Meeting #94-e	R4-2002026

Online, 24th February – 6th March, 2020

Source: 	Skyworks Solutions, Inc.

Title: 	UL Configuration for ULSUP TDM Combinations

Agenda Item:	9.8.2 UE RF [NR_SUL_combos_R16-Core]

Document for:	Discussion

# Introduction

In RAN4#93 meeting, we discussed in [1] the fact that MSD and MPR requirements are missing for FDM mode of ULSUP which was already observed in [2]. In this contribution, we have looked at some potential clarification needed in TDM for SUL UL configurations to avoid LTE de-sense for ULSUP TDM combinations using the corresponding LTE DL band.

# Discussion

## SUL Band Definition

Within current Release 15/16 38.010-3 specification, there are a few SUL bands defined and some of them support higher channel bandwidth than their LTE band counterpart as they usually inherit from the corresponding re-farmed NR channel configurations.

It is understood that TDM mode does not have simultaneous 2 UL in the same channel, however, it is unclear how the NR SUL UL configurations compare with the LTE case. Especially, as for example in current Release 15 specification, there are SUL band definitions that have channel bandwidths that are higher than for the corresponding LTE band at shown in Table 1.

Table 1: NR SUL band and corresponding LTE channel bandwidths

| SUL band | CH BW [MHz] | Corresponding LTE band | CH BW [MHz] | NR BW > LTE BW |
| --- | --- | --- | --- | --- |
| n80 | 30 | 3 | 20 | Yes |
| n81 | 20 | 8 | 10 | Yes |
| n82 | 20 | 20 | 20 | No |
| n83 | 20 | 28 | 20 | No |
| n84 | 20 | 1 | 20 | No |
| n86 | 40 | 66 | 20 | Yes |

It is unclear if the NR SUL UL allocations are contained within the LTE UL BW of the associated ULSUP TDM combination when the NR channel bandwidth is larger.

For example, for DC_3A_n80A_ULSUP-TDM_n78/79A:

- Is the UL allocation for the n80 SUL band contained within the corresponding LTE 20 MHz UL channel of band 3 and thus paired with the band 3 DL channel?

- Or anywhere within 30 MHz of band n80 and thus potentially at the top of the UL band while the LTE DL channel is at the bottom of band and suffer de-sense from the spectral regrowth and noise of SUL UL?

If the first assertion is valid and since all channel configurations are supported for DC, there is no point in defining larger channel bandwidth for the SUL band compared to the LTE band.

If the second assertion is right, then there should be some further restrictions on the UL configuration of the SUL band or potential MSD issues assessed. Note that the issue is present whether the SUL channel bandwidth is the same or higher than for the corresponding LTE band.

Observation:

- SUL UL configuration is unclear for ULSUP TDM combination using the corresponding LTE band

- Depending on the understanding of the SUL transmission behavior, some further clarification on channel and UL configuration may be needed or potential MSD studied.

# Conclusions

In this contribution, we discuss our understanding of the potential SUL UL behavior in ULSUP TDM combination that uses the corresponding LTE band and made the following observation.

Observation:

- SUL UL configuration is unclear for ULSUP TDM combination using the corresponding LTE band

- Depending on the understanding of the SUL transmission behavior, some further clarification on channel and UL configuration may be needed or potential MSD studied.

# References

[1] R4-1915319, MSD and MPR for ULSUP FDM Combinations, Skyworks Solutions Inc., RAN4#93